### Maintenance of NR Mobility Enhancements

R1-2003747	Issue Summary for NR Mobility Enhancements	Intel Corporation

#### Physical Layer Aspects for Mobility Enhancements during HO and SCG Change

R1-2003330	Remaining issues on NR mobility enhancements in physical layer	ZTE

R1-2003506	Remaining issues on DAPS-HO	Huawei, HiSilicon

R1-2003676	Remaining issues on Physical Layer Aspects for  DAPS-HO	MediaTek Inc.

R1-2003748	Corrections to Physical layer aspects of NR mobility enhancement	Intel Corporation

R1-2003890	Remaining issues for NR Mobility Enhancement	Samsung

R1-2004202	Remaining issues on mobility enhancements	Ericsson

R1-2004235	On remaining issues on NR mobility enhancements	Apple

R1-2004580	Remaining physical layer aspects of dual active protocol stack based HO	Nokia, Nokia Shanghai Bell

[101-e-NR-Mob-Enh-01]	Email discussion/approval of Issue #1 (UL cancellation for DAPS) and #3 (UL overlapping transmission) in R1-2003747 by 5/29; if necessary, endorse the associated TPs by 6/4 – Daewon (Intel)

[101-e-NR-Mob-Enh-02]	Email discussion/approval of Issue #5 (Power sharing mode for UL DAPS-HO) in R1-2003747 by 5/28; if necessary, endorse the associated TPs by 6/3– Daewon (Intel)

[101-e-NR-Mob-Enh-03]	Email discussion/approval of Issue #6 (PDCCH monitoring in DL DAPS-HO) in R1-2003747 by 5/28; if necessary, endorse the associated TPs by 6/2– Daewon (Intel)

#### Others

R1-2003331	Discussion on FR2 mobility interruption enhancements	ZTE

R1-2004148	Remaining PHY aspects for CHO	Huawei, HiSilicon