3GPP TSG-RAN WG1 Meeting #101-e	R1- 20xxxxx

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	6.2.1.3

Source:	Qualcomm Incorporated

Title:	TP for TDD HARQ-ACK bundling

Document for:	Discussion/Decision

| Reason for change: | HARQ-ACK bundling mechanism is not defined for multi-TB in TDD. |
| --- | --- |
| Summary of change: | For TDD, when multiple TBs are scheduled from the same DCI, follow the same procedure as FDD HARQ-ACK bundling, with the additional constraint that HARQ-ACKs for different set of PDSCH (or SPS release) do not overlap in the same subframe(s). |
| Consequences if not approved: | HARQ-ACK bundling is not supported for multi-TB in TDD. |
| Clauses affected: | 5.3.3.1.12 (TS 36.212), 7.3.1, 10.1.3 (TS 36.213) |

--------------------------------------------Start of Text Proposal for 36.212----------------------------------------- ##### 5.3.3.1.12 Format 6-1A <Unchanged parts are omitted> - Downlink Assignment Index – number of bits as specified in Table 5.3.3.1.2-2. This field is reserved when multi-TB-DL-config is enabled and multiple TBs are scheduled, or when the configured maximum repetition number is larger than 1 for MPDCCH, and not present when the format 6-1A CRC is scrambled with G-RNTI, or when the higher layer parameter csi-NumRepetitionCE-r13 indicates more than one subframe. <Unchanged parts are omitted> --------------------------------------------End of Text Proposal for 36.212----------------------------------------- --------------------------------------------Start of Text Proposal for 36.213----------------------------------------- #### 7.3.2.1 TDD HARQ-ACK reporting procedure for same UL/DL configuration <Unchanged parts are omitted> For TDD and a BL/CE UE, - if the UE is configured with multi-TB-DL-config, and multiple TBs are scheduled by a single DCI - the UE is not expected to receive any other PDSCH transmission(s) or MPDCCH indicating downlink SPS release, corresponding to which the UE shall report HARQ-ACK in any subframe(s) in which HARQ-ACKs are reported for the multiple TBs scheduled by the single DCI, according to subclause 10.2 - The UE behaviour for HARQ-ACK reporting is the same as that of a BL/CE UE with FDD, except: - PUCCH resource(s) is (are) determined according to Subclause 10.1.3.1; and - PUCCH(s) is (are) transmitted in a set of BL/CE UL subframe(s) according to Subclause 10.2 for TDD and BL/CE UEs. - else if, the UE is configured with csi-NumRepetitionCE equal to 1 and mPDCCH-NumRepetition equal to 1, - the UE behaviour for HARQ-ACK reporting is the same as that of a non-BL/CE UE with TDD, except: - PDCCH/EPDCCH is replaced by MPDCCH; and - DCI format 1/1A/1B/1D/2/2A/2B/2C/2D is replaced by DCI format 6-1A; and - DCI format 0/4 is replaced by DCI format 6-0A; and - PUCCH is transmitted in a set of BL/CE UL subframe(s) according to Subclause 10.2 for TDD and BL/CE UEs; - else - the UE is not expected to receive more than one PDSCH transmission, or more than one of PDSCH and MPDCCH indicating downlink SPS releases, with transmission ending within subframe(s) , where and is defined in Table 10.1.3.1-1 intended for the UE; - The UE behavior for HARQ-ACK reporting is the same as that of a BL/CE UE with FDD, except: - PUCCH resource is determined according to Subclause 10.1.3.1; and - PUCCH is transmitted in a set of BL/CE UL subframe(s) according to Subclause 10.2 for TDD and BL/CE UEs. <Unchanged parts are omitted> ### 10.1.3 TDD HARQ-ACK feedback procedures <Unchanged parts are omitted> For TDD and a BL/CE UE, - if multiple TBs are not scheduled by a single DCI - if the UE is configured with csi-NumRepetitionCE equal to 1 and mPDCCH-NumRepetition equal to 1, - the UE may be configured with HARQ-ACK bundling or HARQ-ACK multiplexing; - HARQ-ACK multiplexing can be configured only if pucch-NumRepetitionCE-format1 equal 1 and HARQ-ACK multiplexing is performed according to the set of Tables 10.1.3-5/6/7 - else - the UE is not expected to receive more than one PDSCH transmission, or more than one of PDSCH and MPDCCH indicating downlink SPS releases, with transmission ending within subframe(s) , where and is defined in Table 10.1.3.1-1 intended for the UE; <Unchanged parts are omitted> --------------------------------------------End of Text Proposal for 36.213-----------------------------------------