3GPP TSG RAN WG1 Meeting #101-e	R1-200xxxx

E-Meeting, May 25 – June 5, 2020

Agenda Item:	6.2.3.1.1

Source:	Moderator (Huawei)

Title:	Text proposal on additional SRS for carrier based switching

Document for:	Discussion and Decision

Introduction

This document provides the text proposal as outcomes of the following email discussion [1]:

[101-e-LTE-LTE_DL_MIMO_EE-01] Support of additional SRS for carrier based switching, dual connectivity and LAA/eMTC by 5/29 – Yubo (Huawei)

- Issues 2, 3, 4 in R1-2004706

Discussion

TP on resource reservation to special subframes

Reason for changes:

The additional SRS symbols for carrier based switching has not been captured.

Summary of changes:

The collision handling of addition SRS symbol for carrier based switching is captured.

Specs/sections impacted:

36.213 sections 8.2

Consequences if not approved:

The collision handling of additional SRS symbols configured for carrier based switching is not complete.

-----------------------------------------------------Start of Text Proposal-----------------------------------

<unchanged parts are omitted>

## 8.2	UE sounding procedure

<unchanged parts are omitted>

For a TDD serving cell d not configured for PUSCH/PUCCH transmission, denote as s0(d) the corresponding serving cell whose UL transmissions may be interrupted as signalled by srs-SwitchFromServCellIndex. Define the set S(d)= {s0(d)… sN-1(d)} as the set of serving cells that meet the all the following conditions:

-	{s0(d)… sN-1(d)} are in the same band as s0(d).

-	{s0(d)… sN-1(d)} have the same CP as s0(d).

-	{s0(d)… sN-1(d)} are in the same TAG as s0(d).

The following prioritization rules shall be applied in case of collision between a transmission of SRS over serving cell d and transmission of a physical signal/channel over a serving cell in set S(d):

-	If PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/PTI/CRI/wideband PMI only (PUCCH reporting type 2a in Subclause 7.2.2) and/or PRACH on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) on serving cell d, then the UE shall not transmit trigger type 0/1 SRS or drop the overlapped symbol(s) of type 2 SRS (including any interruption due to uplink or downlink RF retuning time [10]). Otherwise,

-	if PUSCH transmission carrying aperiodic CSI on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) in serving cell d, and if the SRS transmission is a type 0 SRS transmission, then the UE shall not transmit the type 0 SRS. Otherwise,

-	if PUSCH transmission on a serving cell in set S(d) overlaps in more than one symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) in serving cell d, then the UE shall drop the PUSCH transmission. If PUCCH/trigger type 0/1 SRS transmission on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) on serving cell d, the UE shall drop the PUCCH/trigger type 0/1 SRS transmission. If a subset of symbol(s) for type 2 SRS transmission on a serving cell in set S(d) overlaps with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) on serving cell d, the UE shall drop the subset of the symbol(s) for type 2 SRS on a serving cell in set S(d).

In case an SRS transmission in subframe N on serving cell d is dropped due to a collision with a higher priority transmission (as defined above) in subframe N+1, and there is a lower priority transmission (as defined above) in subframe N that would have been dropped had the transmission in subframe N+1 not occurred, the UE is not required to transmit the lower priority transmission in subframe N.

The UE is not expected to be triggered to transmit type 2 SRS on serving cell d in subframe N that overlaps (including any interruption due to uplink or downlink RF retuning time [10]) with uplink subframe N-1 on a serving cell in set S(d).

<unchanged parts are omitted>

For a serving cell that is not a LAA SCell, a non-BL/CE UE configured for type 1/2 triggered SRS transmission on serving cell c upon detection of a positive SRS request in subframe n, slot 2n or slot 2n+1 of serving cell c shall commence SRS transmission in the first subframe satisfying , and

-	 if the positive SRS request in PDCCH/SPDCCH with DCI format 7-0A/7-1A is detected in slot 2n or slot 2n+1, for TDD

-	 if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI with DCI format other than DCI format 7-0A/7-0B/7-1E/7-1F/7-1G is in the UE-specific search space,

-	otherwise, and

for TDD serving cell c with  and for FDD serving cell c,

for TDD serving cell c with

where for FDD serving cell c  is the subframe index within the frame , for TDD serving cell c, if the UE is configured with the parameter srs-UpPtsAdd for trigger type 1,  is defined in Table 8.2-6; otherwise  is defined in Table 8.2-3. For a TDD serving cell not configured for PUSCH/PUCCH transmission and the positive SRS request detected in PDCCH/EPDCCH scheduling PDSCH and the UE configured with soundingRS-FlexibleTiming-r14 by higher layer signalling, if the trigger type 1 SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) in the first subframe  happens to overlap with a HARQ-ACK transmission for any serving cell, the UE shall commence trigger type 1 SRS transmission in subframe n + k + l, where l = max( 5, ). The soundingRS-FlexibleTiming-r14 if configured by higher layer signaling is not applied to trigger type 2 SRS.

<unchanged parts are omitted>

------------------------------------------------------End of Text Proposal------------------------------------

- References

- R1-200xxxx	Feature summary on 101-e-LTE-LTE_DL_MIMO_EE-01	Moderator (Huawei)