3GPP TSG RAN WG1 Meeting #101-e	R1-200xxxx

E-Meeting, May 25 – June 5, 2020

Agenda Item:	6.2.3.1.1

Source:	Moderator (Huawei)

Title:	Feature summary #1 on LTE DL MIMO efficiency enhancement

Document for:	Discussion and Decision

Introduction

Agreements and conclusions in previous meeting for the LTE DL MIMO efficiency enhancements (WI code LTE_DL_MIMO_EE-Core; WID in RP-182901) are summarized in [1].

In this paper, the inputs from companies submitted to RAN1#100-e meeting ([2]-[9]) on remaining issues are summarized.

Discussion

  - Transmission pattern

### Issue 1: Indexing of additional SRS symbols.

- As explained in [4], the parameter l in nSRS=lR may be misleading as the range of the parameter l is not defined. The interpretation that l is OFDM symbol index within the subframe should be avoided.

- Proposal: Endorse the following text proposal to TS 36.211

Mapping to physical resources shall be done according to clause 5.5.3.2.1 with the following exceptions: - frequency hopping between OFDM symbols is supported and if a UE is configured by higher layer parameter additionalSRS-GuardSymbolFH, a guard symbol is added between every frequency hop; - nSRS=l'R where l' is the index of the OFDM symbol number carrying additional SRS transmission number within the subframe, where l'=0 corresponds to the first OFDM symbol of SRS transmission not counting guard symbol(s) , and R∈1,2,3,4,6,7,8,9,12,13 is the repetition factor given by the higher-layer parameter additionalSRS-RepNum;

  - Collision handling

### Issue 2: Support of additional SRS for DC

There are two options considering the support of DC:

- Option 1: As explained in [3][7][8], the additional SRS can be applied in DC cases.

  - Proposal:

    - Legacy UE behavior related to dual connectivity power control mode 1 and 2, including simultaneous transmission of SRS in a cell group and PUCCH/PUSCH in the other cell group, SRS power scaling within a cell group, and dropping rules, should be inherited for additional SRS.

- In [3], the following is proposed:

=========================Start of text proposal 2 to 36.213=========================== #### 5.1.4.1 Dual connectivity power control Mode 1 If the UE PUSCH/PUCCH transmission(s) in subframe of CG1 overlap in time with PUSCH/PUCCH transmission(s) in more than one symbol of subframe of CG2 or if any symbol the UE PUSCH/PUCCH transmission(s) in subframe of CG1 overlap in time with SRS transmission(s) of subframe , and <unchanged parts are omitted> - is determined as follows - if the PUSCH/PUCCH is not transmitted in the last symbol of subframe of CG1 and UE does not have a trigger type 2 SRS transmission in subframe of serving cell , or if the UE does not have an SRS transmission in subframe of serving cell or if the UE drops SRS transmission in subframe of serving cell due to collision with PUCCH in subframe of serving cell - ; - if the UE has an SRS transmission and does not have a PUCCH/PUSCH transmission in subframe of serving cell - ; - if the UE has a trigger type 0/1 SRS transmission and has a PUCCH transmission, and does not have a PUSCH transmission in subframe of serving cell - - if the UE has a trigger type 0/1 SRS transmission and has a PUSCH transmission, and does not have a PUCCH transmission in subframe of serving cell - - if the UE has a trigger type 0/1 SRS transmission and has a PUSCH transmission and a PUCCH transmission in in subframe of serving cell - <unchanged parts are omitted> #### 5.1.4.2 Dual connectivity power control Mode 2 <unchanged parts are omitted> - is determined as follows - if the UE does not have an SRS transmission in subframe of serving cell or if the UE drops the SRS transmission in subframe of serving cell due to collision with a PUCCH/PUSCH transmission in subframe of serving cell - ; - if the UE has an SRS transmission and does not have a PUCCH/PUSCH transmission in subframe of serving cell - ; - if the UE has a trigger type 0/1 SRS transmission and has a PUCCH transmission, and does not have a PUSCH transmission in subframe of serving cell - - if the UE has a trigger type 0/1 SRS transmission and has a PUSCH transmission, and does not have a PUCCH transmission in subframe of serving cell - - if the UE has a trigger type 0/1 SRS transmission and has a PUSCH transmission and a PUCCH transmission in subframe of serving cell - where is the linear value of described in Subclause 5.1.3.1. <unchanged parts are omitted> =========================End of text proposal to 36.213===========================

- Option 2: As explained in [5], DC scheduling between MCG and SCG requires coordination over X2 interface. It would be difficult to deal with the collision between additional SRS and PUCCH/PUSCH in DC.

  - Proposal:

    - Additional SRS is not applied to DC

    - Endorse the following TP

< Unchanged parts are omitted> ### 5.1.4 Power allocation for EUTRA dual connectivity If a UE is configured with multiple cell groups, the UE is not expected to be configured with SRS trigger type 2, and - if the UE supports synchronous dual connectivity but does not support asynchronous dual connectivity, or if the UE supports both synchronous dual connectivity and asynchronous dual connectivity and if the higher layer parameter powerControlMode indicates dual connectivity power control mode 1 < Unchanged parts are omitted>

### Issue 3: Support of additional SRS for carrier based switching.

- As explained in [5][8], including the retuning time, the configured additional SRS may have impact the uplink transmission in the previous and/or next subframe.

  - For collision of additional SRS in subframe N with uplink transmission in subframe N-1:

    - Option 1: The UE is not expected to be triggered to transmit type 2 SRS on a PUSCH-less CC in subframe N that overlaps (including the retuning time) with uplink subframe N-1 on a different CC.

    - Option 2: In addition to current dropping/puncturing rules defined for SRS carrier switching, UE may drop additional SRS transmission in the symbols of the first slot of the subframe

  - For collision of additional SRS in subframe N with uplink transmission in subframe N+1:

    - In case a subset of the symbols for type 2 SRS transmission in subframe N on a PUSCH-less CC collide (including the retuning time) with a higher priority uplink transmission in subframe N+1 on a different CC, the UE shall drop the subset of type 2 SRS symbols.

    - Endorse the following TP

<TP1 for TS 36.213> ## 8.2 UE sounding procedure < Unchanged parts are omitted> For a TDD serving cell d not configured for PUSCH/PUCCH transmission, denote as s0(d) the corresponding serving cell whose UL transmissions may be interrupted as signalled by srs-SwitchFromServCellIndex. Define the set S(d)= {s0(d)… sN-1(d)} as the set of serving cells that meet the all the following conditions: - {s0(d)… sN-1(d)} are in the same band as s0(d). - {s0(d)… sN-1(d)} have the same CP as s0(d). - {s0(d)… sN-1(d)} are in the same TAG as s0(d). The following prioritization rules shall be applied in case of collision between a transmission of SRS over serving cell d and transmission of a physical signal/channel over a serving cell in set S(d): - If PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/PTI/CRI/wideband PMI only (PUCCH reporting type 2a in Subclause 7.2.2) and/or PRACH on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) on serving cell d, then the UE shall not transmit type 0/1 SRS or drop the overlapped symbol(s) of type 2 SRS (including any interruption due to uplink or downlink RF retuning time [10]). Otherwise, - if PUSCH transmission carrying aperiodic CSI on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) in serving cell d, and if the SRS transmission is a type 0 SRS transmission, then the UE shall not transmit the type 0 SRS. Otherwise, - if PUSCH transmission on a serving cell in set S(d) overlaps in more than one symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) in serving cell d, then the UE shall drop the PUSCH transmission. If PUCCH/type 0/1 SRS transmission on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) on serving cell d, the UE shall drop the PUCCH/type 0/1 SRS transmission. If a subset of symbol(s) for type 2 SRS transmission on a serving cell in set S(d) overlaps in with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) on serving cell d, the UE shall drop the subset of the symbol(s) for type 2 SRS on a serving cell in set S(d). In case an SRS transmission in subframe N on serving cell d is dropped due to a collision with a higher priority transmission (as defined above) in subframe N+1, and there is a lower priority transmission (as defined above) in subframe N that would have been dropped had the transmission in subframe N+1 not occurred, the UE is not required to transmit the lower priority transmission in subframe N. The UE is not expected to be triggered to transmit type 2 SRS on serving cell d in subframe N that overlaps (including any interruption due to uplink or downlink RF retuning time [10]) with uplink subframe N-1 on a serving cell in set S(d). < Unchanged parts are omitted>

- As explained in [5], with soundingRS-FlexibleTiming configured, the legacy SRS can be postponed once to the next SRS occasion, which can be used to type 2 SRS.

  - Proposal:

    - In case a subset of the symbols for type 2 SRS transmission on a PUSCH-less CC collide (including the retuning time) with HARQ-ACK transmission, the UE shall postpone the subset of the type 2 SRS symbol(s) once to next SRS occasion if the UE is configured with soundingRS-FlexibleTiming.

    - Endorse the following TP

<TP2 for TS 36.213> ## 8.2 UE sounding procedure < Unchanged parts are omitted> For a serving cell that is not a LAA SCell, a non-BL/CE UE configured for type 1/2 triggered SRS transmission on serving cell c upon detection of a positive SRS request in subframe n, slot 2n or slot 2n+1 of serving cell c shall commence SRS transmission in the first subframe satisfying , and - if the positive SRS request in PDCCH/SPDCCH with DCI format 7-0A/7-1A is detected in slot 2n or slot 2n+1, for TDD - if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI with DCI format other than DCI format 7-0A/7-0B/7-1E/7-1F/7-1G is in the UE-specific search space, - otherwise, and for TDD serving cell c with and for FDD serving cell c, for TDD serving cell c with where for FDD serving cell c is the subframe index within the frame , for TDD serving cell c, if the UE is configured with the parameter srs-UpPtsAdd for trigger type 1, is defined in Table 8.2-6; otherwise is defined in Table 8.2-3. For a TDD serving cell not configured for PUSCH/PUCCH transmission and the positive SRS request detected in PDCCH/EPDCCH scheduling PDSCH and the UE configured with soundingRS-FlexibleTiming-r14 by higher layer signalling, if the type 1 SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) in the first subframe happens to overlap with a HARQ-ACK transmission for any serving cell, the UE shall commence type 1 SRS transmission in subframe n + k + l, where l = max( 5, ); if a subset of the symbols corresponding to the type 2 SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) in the first subframe n+k, k≥4 happen to overlap with a HARQ-ACK transmission for any serving cell, the UE shall commence the transmission of the subset of the type 2 SRS symbols in subframe n+k+l, where l=max⁡(5,TSRS,1). < Unchanged parts are omitted>

  - Other

### Issue 4: Additional SRS for LAA, eMTC.

- As explained in [5][6][8],

  - Due to limited bandwidth, the BL/CE UEs will use frequency hopping to achieve the frequency diversity gain, which requires RF retuning, which has not been discussed in RAN1.

  - For LAA UEs, if additional SRS is triggered to be transmitted in the unlicensed band, there may be impact on the channel access before the additional SRS transmission, which has not been discussed in RAN1.

  - The discussion is out of scope of R16 WID.

- Proposal: Additional SRS is not applied to LAA, eMTC, and endorse the following TP

< Unchanged parts are omitted> ## 8.2 UE sounding procedure < Unchanged parts are omitted> A UE is not expected to be configured with SRS trigger type 0 and trigger type 2 on a LAA SCell. < Unchanged parts are omitted> - A UE may be configured with SRS parameters for trigger type 0 and trigger type 1/2 on each serving cell. A BL/CE UE configured with CEModeB is not expected to be configured with SRS parameters for trigger type 0 and trigger type 1. A BL/CE UE is not expected to be configured with SRS parameters for trigger type 2. The following SRS parameters are serving cell specific and semi-statically configurable by higher layers for trigger type 0 and for trigger type 1/2. < Unchanged parts are omitted>

### Issue 5: Term alignment between specs.

- As explained in [2][3][7][9], the term used in 36.211 is additional SRS symbol while in 36.213 it’s trigger type 2. But it is not specified that trigger type 2 SRS corresponds to additional SRS, and it should be clarified that trigger type 0 SRS and trigger type 1 SRS correspond to periodic SRS and aperiodic SRS of basic SRS respectively.

- Proposal: Endorse one of the following text proposal

Text proposal to 36.213 - 8.2 UE sounding procedure If the UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group unless stated otherwise - When the procedures are applied for the primary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell', and 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell or serving cells belonging to the primary PUCCH group respectively unless stated otherwise. - When the procedures are applied for secondary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell' and 'serving cells' in this clause refer to secondary cell, secondary cells (not including the PUCCH-SCell), serving cell, serving cells belonging to the secondary PUCCH group respectively unless stated otherwise. The term 'primary cell' in this clause refers to the PUCCH-SCell of the secondary PUCCH group. A UE shall transmit Sounding Reference Symbol (SRS) on per serving cell SRS resources based on three trigger types: - trigger type 0: higher layer signalling - trigger type 1: DCI formats 0/0A/0B/4/4A/4B/1A/6-0A/6-1A for FDD, TDD, and frame structure type 3 and DCI formats 2B/2C/2D/3B for TDD, and frame structure type 3, and DCI format 7-0A/7-0B/7-1E/7-1F/7-1G for TDD if the UE is configured by higher layers for SRS triggering via DCI format 7-0A and has indicated the capability srs-DCI7-Triggering-FS2-r15/ srs-DCI7-Triggering-FS2-r16 and the UE is configured for SRS triggering with srs-DCI7-TriggeringConfig-r15/ srs-DCI7-Triggering-FS2-r16. - trigger type 2: DCI formats 0/4/1A/6-0A/6-1A for FDD and TDD, and DCI formats 2B/2C/2D/3B for TDD, and DCI format 7-0A/7-0B/7-1E/7-1F/7-1G for TDD if the UE is configured by higher layers for SRS triggering via DCI format 7-0A and has indicated the capability srs-DCI7-Triggering-FS2-r16 and the UE is configured for SRS triggering with srs-DCI7-TriggeringConfig-r16. Trigger type 0 and type 1 SRS correspond to periodic SRS and aperiodic SRS of basic SRS respectively in Subclause 5.5.3 of [1]. Trigger type 2 SRS corresponds to additional SRS in Subclause 5.5.3 of [1].

Or

Text proposal to 36.213 <Unchanged parts are omitted> ## 8.2 UE sounding procedure If the UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group unless stated otherwise - When the procedures are applied for the primary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell', and 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell or serving cells belonging to the primary PUCCH group respectively unless stated otherwise. - When the procedures are applied for secondary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell' and 'serving cells' in this clause refer to secondary cell, secondary cells (not including the PUCCH-SCell), serving cell, serving cells belonging to the secondary PUCCH group respectively unless stated otherwise. The term 'primary cell' in this clause refers to the PUCCH-SCell of the secondary PUCCH group. A UE shall transmit Sounding Reference Symbol (SRS) on per serving cell SRS resources based on three trigger types: - trigger type 0: higher layer signalling - trigger type 1: triggering of basic sounding reference signals as defined in subclause 5.5.3 of [3], by DCI formats 0/0A/0B/4/4A/4B/1A/6-0A/6-1A for FDD, TDD, and frame structure type 3 and DCI formats 2B/2C/2D/3B for TDD, and frame structure type 3, and DCI format 7-0A/7-0B/7-1E/7-1F/7-1G for TDD if the UE is configured by higher layers for SRS triggering via DCI format 7-0A and has indicated the capability srs-DCI7-Triggering-FS2 and the UE is configured for SRS triggering with srs-DCI7-TriggeringConfig . - trigger type 2: triggering of additional sounding reference signals as defined in subclause 5.5.3 of [3], by DCI formats 0/4/1A/6-0A/6-1A for FDD and TDD, and DCI formats 2B/2C/2D/3B for TDD, and DCI format 7-0A/7-0B/7-1E/7-1F/7-1G for TDD if the UE is configured by higher layers for SRS triggering via DCI format 7-0A and has indicated the capability srs-DCI7-Triggering-FS2 and the UE is configured for SRS triggering with srs-DCI7-TriggeringConfig. <Unchanged parts are omitted>

Or

Text proposal to 36.211 ---- Unchanged parts are omitted ---- ### 5.5.3 Sounding reference signal Two types of sounding reference signals can be configured: - basic sounding reference signal, supporting periodic or aperiodic transmission - additional sounding reference signal, supporting aperiodic transmission only Basic SRS corresponds to either SRS trigger type 0 or type 1 in 8.2 of [4]. Additional SRS corresponds to SRS trigger type 2 in 8.2 of [4]. ---- Unchanged parts are omitted ----

Or

-------------------------- Start of proposed TP1 for TS 36.211 ------------------------------------ #### 5.5.1.5 Determining virtual cell identity for sequence generation The definition of depends on the type of transmission. Transmissions associated with PUSCH: - if no value for is configured by higher layers or if the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure, - otherwise. Transmissions associated with SPUCCH/PUCCH: - if no value for is configured by higher layers, - otherwise. Basic sSounding reference signals transmissions of types 0 and 1 as defined in 8.2 of [4]: - nIDRS=nIDSRS if the higher-layer parameters nSRS-Identity-Legacy and nSRS-Identity both are configured, where nIDSRS equals the higher-layer parameter nSRS-Identity - nIDRS=NIDcell otherwise. Additional sSounding reference signals transmissions of type 2 as defined in 8.2 of [4]: - nIDRS=NIDcell if no value for nIDSRS is configured by the higher-layer parameter nSRS-Identity - nIDRS=nIDSRS otherwise. ----------------------------------------------- Unchanged text omitted ------------------------------------------- ### 5.5.3 Sounding reference signal Three types of sounding reference signals can be configured as defined in 8.2 of [4]: SRS trigger type 0, SRS trigger type 1, and SRS trigger type 2. Two types of sounding reference signals can be configured: - basic sounding reference signal, supporting periodic or aperiodic transmission - additional sounding reference signal, supporting aperiodic transmission only #### 5.5.3.1 Sequence generation ##### 5.5.3.1.1 Sequence generation for SRS trigger types 0 and 1 basic SRS The sounding reference signal sequence is defined by clause 5.5.1, where is the sequence-group number defined in clause 5.5.1.3, is the base sequence number defined in clause 5.5.1.4, and . The cyclic shift of the sounding reference signal is given as , where is configured separately for periodic and each configuration of aperiodic sounding by the higher-layer parameters cyclicShift and cyclicShift-ap, respectively, for each UE and is the number of antenna ports used for sounding reference signal transmission. The parameter if , otherwise . The parameter is given by the higher layer parameter transmissionCombNum if configured, otherwise . ##### 5.5.3.1.2 Sequence generation for SRS trigger type 2 additional SRS The sounding reference signal rSRS(p)n is defined by clause 5.5.3.1.1 with the following exceptions - nSRScs is given by the higher-layer parameter additionalSRS-cyclicShift - Nap is given by the higher-layer parameter additionalSRS-AntennaPort - KTC is given by the higher-layer parameter additionalSRS-transmissionComb - the function fgh in clause 5.5.1.3 is given by fghns,l=0if group hopping is disabledi=072ic8nsNsymbUL+l+i mod 30if group hopping is enabled where l is the OFDM symbol index within the slot ns and NsymbUL is the number of OFDM symbols per slot - the function v in clause 5.5.1.4 is given by v=cnsNsymbUL+lif group hopping is disabled and sequence hopping is enabled0otherwise #### 5.5.3.2 Mapping to physical resources ##### 5.5.3.2.1 Mapping to physical resources for SRS trigger types 0 and 1 basic SRS The sequence shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in clause 5.1.3.1 in 3GPP TS 36.213 [4], and mapped in sequence starting with to resource elements on antenna port according to ----------------------------------------------- Unchanged text omitted ------------------------------------------- ##### 5.5.3.2.2 Mapping to physical resources for SRS trigger type 2 additional SRS An additional SRS of trigger type 2 spans one or more OFDM symbols in the time domain, where - the starting OFDM symbol l0 within the subframe is given by the higher-layer parameter additionalSRS-startPos; - the duration N in number of OFDM symbols, including potential guard symbols, is given by the higher-layer parameter additionalSRS-duration; Mapping to physical resources shall be done according to clause 5.5.3.2.1 with the following exceptions: - frequency hopping between OFDM symbols is supported and if a UE is configured by higher layer parameter additionalSRS-GuardSymbolFH, a guard symbol is added between every frequency hop; - nSRS=lR where l is the index of the OFDM symbol number carrying additional SRS of trigger type 2 within the subframe not counting guard symbol(s), and R∈1,2,3,4,6,7,8,9,12,13 is the repetition factor given by the higher-layer parameter additionalSRS-RepNum; - BSRS is given by the higher-layer parameter additionalSRS-Bandwidth; - bhop is given by the higher-layer parameter additionalSRS-HoppingBandwidth; - NFH is the number of frequency hops for additional SRS of trigger type 2, derived from N=RNFH+NFH-1GFH if antenna switching is not configured for additional SRS of trigger type 2, and from N=RNASNFH+NAS-1GAS+NFH-1GFH if antenna switching is configured for additional SRS of trigger type 2, where R is the repetition factor given by the higher-layer parameter additionalSRS-RepNum, NAS is the number of antenna switches for additional SRS of trigger type 2 defined in 8.2 of [4], GAS∈0, 1 is the guard-symbol configuration for antenna switching given by the higher-layer parameter additionalSRS-GuardSymbolAS, GFH∈0, 1 is the guard symbol configuration for frequency hopping given by the higher-layer parameter additionalSRS-GuardSymbolFH, and N is given by the higher-layer parameter additionalSRS-duration; - nRRC is given by the higher-layer parameter freqDomainPosition-additionalSRS; - Nap is given by the higher-layer parameter additionalSRS-AntennaPort; - nSRScs is given by the higher-layer parameter additionalSRS-cyclicShift; - KTC is given by the higher-layer parameter additionalSRS-transmissionCombNum; - kTC is given by the higher-layer parameter additionalSRS-transmissionComb. ------------------------------------------- End of proposed TP 1 ----------------------------------------------------

### Issue 6: Aligning the higher layer parameters for virtual cell ID.

- As explained in [2], high layer parameters srs-VirtualCellID and srs-VirtualCellID-AllSRS are introduced, which are not aligned with the name of nSRS-Identity and nSRS-Identity-Legacy in TS 36.211

- Proposal: nSRS-Identity and nSRS-Identity-Legacy in TS 36.211 should be replaced with srs-VirtualCellID and srs-VirtualCellID-AllSRS.

- 5.5.1.5 Determining virtual cell identity for sequence generation The definition of depends on the type of transmission. Transmissions associated with PUSCH: - if no value for is configured by higher layers or if the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure, - otherwise. Transmissions associated with SPUCCH/PUCCH: - if no value for is configured by higher layers, - otherwise. Basic sounding reference signals: - nIDRS=nIDSRS if the value of higher-layer parameter srs-VirtualCellID-AllSRS is TRUE and srs-VirtualCellID is configured, where nIDSRS equals the higher-layer parameter srs-VirtualCellID - nIDRS=NIDcell otherwise. Additional sounding reference signals: - nIDRS=NIDcell if no value for nIDSRS is configured by the higher-layer parameter srs-VirtualCellID - nIDRS=nIDSRS otherwise.

### Issue 7: Typos for power control in 36.213.

- As explained in [7], there are several typos in 36.213 for power control.

- Proposal: Endorse the following text proposal to TS 36.213

Text proposal to 36.213 ---- Unchanged parts are omitted ---- ### 5.1.3 Sounding Reference Symbol (SRS) #### 5.1.3.1 UE behaviour The setting of the UE Transmit power for the SRS transmitted on subframe i for serving cell is defined by: for SRS transmission given trigger type 2 or for serving cell with frame structure type 2, and not configured for PUSCH/PUCCH transmission [dBm] otherwise [dBm] where - is the configured UE transmit power defined in [6] in subframe i for serving cell . - is semi-statically configured by higher layers for m=0 and m=1 for serving cell . For SRS transmission given trigger type 0 then m=0 and for SRS transmission given trigger type 1 then m=1. - is the bandwidth of the SRS transmission in subframe i for serving cell expressed in number of resource blocks. - is the current PUSCH power control adjustment state for serving cell , see Subclause 5.1.1.1. - and are parameters as defined in Subclause 5.1.1.1 for subframe , where . - is the higher layer parameter alpha-SRS for SRS transmission given trigger type 0, 1, or is the higher layer parameter alpha-additionalSRS for SRS transmission given trigger type 2, configured by higher layers for serving cell . - is a parameter composed of the sum of a component which is p0-Nominal-PeriodicSRS, p0-Nominal-AperiodicSRS, or p0_Nominal_AdditionalSRS provided from higher layers for m=0, 1 or 2 respectively, and a component which is p0-UE-PeriodicSRS, p0-UE-AperiodicSRS, or p0-Nominal-AdditionalSRS provided by higher layers for m=0, 1 or 2 respectively, for serving cell . For SRS transmission given trigger type 0 then m=0 and for SRS transmission given trigger type 1 then m=1 and for SRS transmission given trigger type 2 then m=2. ---- Unchanged parts are omitted ----

### Issue 8: UL SC-FDMA symbol for additional SRS.

- As explained in [7], a SC-FDMA symbol instead of an OFDM symbol is used for a UL symbol.

- Proposal: Endorse the following text proposal to TS 36.211 and 36.213

Text proposal to 36.211 < Unchanged parts are omitted> ##### 5.5.3.1.2 Sequence generation for additional SRS The sounding reference signal rSRS(p)n is defined by clause 5.5.3.1.1 with the following exceptions - nSRScs is given by the higher-layer parameter additionalSRS-cyclicShift - Nap is given by the higher-layer parameter additionalSRS-AntennaPort - KTC is given by the higher-layer parameter additionalSRS-transmissionComb - the function fgh in clause 5.5.1.3 is given by fghns,l=0if group hopping is disabledi=072ic8nsNsymbUL+l+i mod 30if group hopping is enabled where l is the SC-FDMA symbol index within the slot ns and NsymbUL is the number of SC-FDMA symbols per slot - the function v in clause 5.5.1.4 is given by v=cnsNsymbUL+lif group hopping is disabled and sequence hopping is enabled0otherwise < Unchanged parts are omitted> ##### 5.5.3.2.2 Mapping to physical resources for additional SRS An additional SRS spans one or more SC-FDMA symbols in the time domain, where - the starting SC-FDMA symbol l0 within the subframe is given by the higher-layer parameter additionalSRS-startPos; - the duration N in number of SC-FDMA symbols, including potential guard symbols, is given by the higher-layer parameter additionalSRS-duration; Mapping to physical resources shall be done according to clause 5.5.3.2.1 with the following exceptions: - frequency hopping between SC-FDMA symbols is supported and if a UE is configured by higher layer parameter additionalSRS-GuardSymbolFH, a guard symbol is added between every frequency hop; - antenna switching within a subframe is supported and if a UE is configured by higher layer parameter additionalSRS-GuardSymbolAS, a guard symbol is added between every antenna switching; - nSRS=lR where l is the index of the SC-FDMA symbol number carrying additional SRS within the subframe not counting guard symbol(s), and R∈1,2,3,4,6,7,8,9,12,13 is the repetition factor given by the higher-layer parameter additionalSRS-RepNum; < Unchanged parts are omitted>

Text proposal to 36.213 ## 8.2 UE sounding procedure < Unchanged parts are omitted> - SRS subframe for each configuration of trigger type 1 for a serving cell that is a LAA SCell and DCI format 4B - Starting SC-FDMAsymbol l0 and duration N and repetition number R as defined in Subclause 5.5.3.2.2 of [3] for each configuration of SRS trigger type 2. < Unchanged parts are omitted>

Summary

FL’s view on the issues that are prioritized for discussion in this e-meeting are as following:

- Support of additional SRS for carrier based switching and dual connectivity

    - Issues 2, 3

- Indexing of additional SRS symbols

    - Issues 1

- Support of additional SRS to LAA/eMTC

    - Issue 4

- Alignment/correction of terms, higher layer parameters, typos

    - Issues 5, 6, 7, 8

- References

- R1-1913596, “RAN1 agreements for DL MIMO efficiency enhancements for LTE”, Huawei, Reno, USA, November 2019.

- R1-2003466	Maintenance of additional SRS symbols	ZTE

- R1-2003541	Corrections on additional SRS symbols	Huawei, HiSilicon

- R1-2003723	Corrections to additional SRS	Intel Corporation

- R1-2003784	Additional SRS symbols	Qualcomm Incorporated

- R1-2003818	Additional SRS symbols	Lenovo, Motorola Mobility

- R1-2003925	Text proposals on additional SRS symbols	LG Electronics

- R1-2003937	Remaining details of additional SRS symbols	Nokia, Nokia Shanghai Bell

- R1-2004431	Maintenance on additional SRS symbols	Ericsson