3GPP TSG RAN WG1 #101-e			R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Title:	[Draft] LS to RAN2 on SL_RESOURCE_RESELECTION_COUNTER and period value provided to L1

Response to:

Release:	Rel-16

Work Item:	5G_V2X_NRSL

Source:	Intel Corporation [RAN1]

To:	RAN2

Cc:

Contact Person:

#### Name:	Sergey Panteleev

Tel. Number:

E-mail Address:	sergey.panteleev@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN1 made the following agreement which handles the value range for SL_RESOURCE_RESELECTION_COUNTER in cases of TX periodicity < 100 ms and expects RAN2 to capture it in MAC specification:

Agreements: - Reuse LTE rule to calculate C_resel from SL_RESOURCE_RESELECTION_COUNTER: - C_resel=10*SL_RESOURCE_RESELECTION_COUNTER - Introduce the following scaling to SL_RESOURCE_RESELECTION_COUNTER range and inform RAN2 about this decision: - SL_RESOURCE_RESELECTION_COUNTER is the value randomly selected from the range (the range as a working assumption) - Note: this intends to capture details of the RAN1#99 agreement which are still missing in specifications

In addition, RAN1 made the following two agreements related to periodicity signalling in SCI:

Agreements: - A UE is expected to be (pre-)configured with a set sl-ResourceReservePeriod containing value of 0 ms Agreements: - A UE sets “Resource reservation period” in SCI 1-A to correspond to value of the period provided by higher layers from (pre-)configured set sl-ResourceReservePeriod - RAN1 assumes that at least in cases if higher layer decides not to keep the resource for the transmission in the next period or there is no associated period, then higher layer provides 0 ms periodicity - Send LS to RAN2 to inform this decision

2. Actions:

To RAN2

RAN1 respectfully requests RAN2 to specify the aspects related to the provided agreements.

3. Date of Next TSG-RAN1 Meetings:

TSG-RAN WG1#102 		24 August – 4 September 2020       			E-Meeting