3GPP TSG RAN WG1 Meeting #101-E						     R1- 200xxxx

e-Meeting, May 25th – June 5th, 2020

Source:	Moderator (Intel Corporation)

Title:	TPs based on outcome of [101-e-NR-5G_V2X_NRSL-Mode-2-04]

Agenda item:	7.2.4.2.2

Document for:	Discussion and Decision

Introduction

The following agreements to be captured in RAN1 specifications were made in [101-e-NR-5G_V2X_NRSL-Mode-2-04].

Agreements: - A UE is expected to be (pre-)configured with a set sl-ResourceReservePeriod containing value of 0 ms Agreements: - For conversion of Prsvp_TX and Prsvp_RX measured in ms to P’rsvp_TX and P’rsvp_RX in logical slots, LTE principle is reused by the following formula: - P’rsvp = ceiling(N/20ms Prsvp) where N is the number of slots that can be used for SL transmission within 20 ms of the configured UL-DL configuration Agreements: - In 38.214, section 8.1.4, Tscal is set - the selection window length in ms Agreements: - Reuse LTE rule to calculate C_resel from SL_RESOURCE_RESELECTION_COUNTER: - C_resel=10*SL_RESOURCE_RESELECTION_COUNTER - Introduce the following scaling to SL_RESOURCE_RESELECTION_COUNTER range and inform RAN2 about this decision: - SL_RESOURCE_RESELECTION_COUNTER is the value randomly selected from the range (the range as a working assumption) - Note: this intends to capture details of the RAN1#99 agreement which are still missing in specifications Agreements: - A UE sets “Resource reservation period” in SCI 1-A to correspond to value of the period provided by higher layers from (pre-)configured set sl-ResourceReservePeriod - RAN1 assumes that at least in cases if higher layer decides not to keep the resource for the transmission in the next period or there is no associated period, then higher layer provides 0 ms periodicity - Send LS to RAN2 to inform this decision

TP to TS 38.213

## 16.4 UE procedure for transmitting PSCCH A UE can be provided a number of symbols in a resource pool, by timeResourcePSCCH, starting from a second symbol that is available for SL transmissions in a slot, and a number of PRBs in the resource pool, by frequencyResourcePSCCH, for a PSCCH transmission with a SCI format 1-A. A UE that transmits a PSCCH with SCI format 1-A using sidelink resource allocation mode 1 [6, TS38.214] - sets a value of a HARQ process ID field as indicated by higher layers - for an initial transmission of a TB that is scheduled by a DCI format 3_0 with CRC scrambled by SL-RNTI, the UE - toggles the NDI field value in SCI format 1-A, if the NDI field value in DCI format 3_0 is toggled - does not toggle the NDI field value in SCI format 1-A, if the NDI field value in DCI format 3_0 is not toggled - for subsequent transmissions of the TB that are scheduled by the DCI format 3_0 with CRC scrambled by SL-RNTI, the UE does not toggle the NDI field value in SCI format 1-A. A UE that transmits a PSCCH with SCI format 1-A using sidelink resource allocation mode 2 [6, TS38.214] - if sl-MultiReserveResource is configured - sets “Resource reservation period” as an index in the set sl-ResourceReservePeriod corresponding to the reservation period in units of ms provided by higher layer

| Source | Comments |
| --- | --- |
| Nokia, NSB | Should “correspond to the value of the period…” be elaborated? E.g. “index of the period … in set sl-ResourceReservePeriod” FL: tried to address |
| OPPO | Following the new naming convention for SCI formats in R1-2003169, SCI format 0_1 should be 1-A. FL: accepted, but the part related to Mode-1 is actually outdated and there is no need to correct it |
| Huawei/HiSilicon | There are two “provided by higher layer” in the last bullet. I assume the first “provided by higher layer” means “sl-ResourceReservePeriod is provided by higher layer”, and the second “provided by higher layer” means “the reservation period is provided by higher layer”. To improve the reading, I think maybe it’s ok to remove the first one. Otherwise, the sentence is a little bit hard to understand. We suggest the following changes (marked in red). Either Alt1 or Alt2 is ok for us. Alt1: “- sets “Resource reservation period” as an index in the set sl-ResourceReservePeriod provided by higher layer corresponding to the reservation period in units of ms provided by higher layer” Alt2: “- sets “Resource reservation period” as an index in the set sl-ResourceReservePeriod provided by higher layer corresponding to the reservation period in units of ms provided by higher layer, where sl-ResourceReservePeriod is provided by higher layer” FL comment: Alt.1 looks good, thanks. Adopted. |

TP to TS 38.214

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission: - the resource pool from which the resources are to be reported; - L1 priority, prioTX; - the remaining packet delay budget; - the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH; - optionally, the resource reservation interval, Prsvp_TX, in units of ms. The following higher layer parameters affect this procedure: - t2min_SelectionWindow: internal parameter T2min is set to the corresponding value from higher layer parameter t2min_SelectionWindow for the given value of prioTX. - SL-ThresRSRP_pi_pj: this higher layer parameter provides an RSRP threshold for each combination pi, pj, where pi is the value of the priority field in a received SCI format 1-A and pj is the priority of the transmission of the UE selecting resources; for a given invocation of this procedure, pj = prioTX. - RSforSensing selects if the UE uses the PSSCH-RSRP or PSCCH-RSRP measurement, as defined in clause 8.4.2.1. - reservationPeriodAllowed - t0_SensingWindow: internal parameter T0 is defined as the number of slots corresponding to t0_SensingWindow ms. The resource reservation interval, Prsvp_TX, if provided, is converted from units of ms to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7. Notation: t0SL,t1SL,t2SL,... denotes the set of slots which can belong to a sidelink resource pool and is defined in [TBD]. The following steps are used: 1) A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot tySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource, where - selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1 , where Tproc,1 is TBD; - if T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to T2min ≤ T2 ≤ remaining packet budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots). The total number of candidate single-slot resources is denoted by Mtotal. 2) The sensing window is defined by the range of slots [n –T0,n–Tproc,0) where T0 is defined above and Tproc,1 is TBD. The UE shall monitor slots which can belong to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. 3) The internal parameter Th(pi) is set to the corresponding value from higher layer parameter SL-ThresRSRP_pi_pj for pj equal to the given value of prioTX and each priority value pi. 4) The set SA is initialized to the set of all the candidate single-slot resources. 5) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: - the UE has not monitored slot tmSL in Step 2. - for any periodicity value allowed by the higher layer parameter reservationPeriodAllowed and a hypothetical SCI format 1-A received in slot tmSL with "Resource reservation period" field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met. 6) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: a) the UE receives an SCI format 1-A in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause [TBD] in [6, TS 38.213]; b) the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX; c) the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A, is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of ms. 7) If the number of candidate single-slot resources remaining in the set SA is smaller than 0.2⋅Mtotal, then Th(pi) is increased by 3 dB for each priority value Th(pi) and the procedure continues with step 4. The UE shall report set SA to higher layers. ### 8.1.5 UE procedure for determining slots and resource blocks for PSSCH transmission associated with an SCI format 1-A The set of slots and resource blocks for PSSCH transmission is determined by the resource used for the PSCCH transmission containing the associated SCI format 1-A, and fields "Frequency resource assignment", "Time resource assignment" of the associated SCI format 1 as described below. "Time resource assignment" carries logical slot offset indication of N = 1 or 2 actual resources when sl-MaxNumPerReserve is 2, and N = 1 or 2 or 3 actual resources when sl_MaxNumPerReserve is 3, in a form of time RIV (TRIV) field which is determined as follows: if N=1 TRIV=0 elseif N=2 TRIV=t1 else if t2-t1-1≤15 TRIV=30t2-t1-1+t1+31 else TRIV=3031-t2+t1+62-t1 end if end if where the first resource is in the slot where SCI format 1-A was received, and ti denotes i-th resource time offset in logical slots of a resource pool with respect to the first resource where for N = 2, 1≤t1≤31; and for N = 3, 1≤t1≤30, t1<t2≤31. The starting sub-channel nsubCH,0start of the first resource is determined according to clause 8.1.2.2. The number of contiguously allocated sub-channels for each of the N resources LsubCH≥1 and the starting sub-channel indexes of resources indicated by the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received, are determined from "Frequency resource assignment" which is equal to a frequency RIV (FRIV) where. If sl-MaxNumPerReserve is 2 then FRIV=nsubCH,1start+i=1LsubCH-1N subchannel SL+1-i If sl-MaxNumPerReserve is 3 then FRIV=nsubCH,1start+nsubCH,2start⋅N subchannel SL+1-LsubCH+i=1LsubCH-1N subchannel SL+1-i2 where - nsubCH,1start denotes the starting sub-channel index for the second resource - nsubCH,2start denotes the starting sub-channel index for the third resource - N subchannel SL is the number of sub-channels in a resource pool provided according to the higher layer parameter numSubchannel If TRIV indicates N < sl-MaxNumPerReserve, the starting sub-channel indexes corresponding to sl-MaxNumPerReserve minus N last resources are not used. The number of slots in one set of the time and frequency resources for transmission opportunities of PSSCH is given by Cresel where Cresel= 10*SL_RESOURCE_RESELECTION_COUNTER [10, TS 38.321] if configured else Cresel is set to 1. If a set of sub-channels in slot tmSL is determined as the time and frequency resource for PSSCH transmission corresponding to the configured sidelink grant (described in [10, TS 38.321]), the same set of sub-channels in slots tm+j×Prsvp_TX'SL are also determined for PSSCH transmissions corresponding to the same sidelink grant where j=1, 2,…, Cresel-1, Prsvp_TX, if provided, is converted from units of ms to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7, and t0SL,t1SL,t2SL,... is determined by [TBD]. Here, Prsvp_TX is the resource reservation interval indicated by higher layers. << UNCHANGED PART OMITTED>> ### 8.1.7 UE procedure for determining number logical slots for a given reservation period A given resource reservation period Prsvp in milliseconds is converted to a period Prsvp' in logical slots as: Prsvp'=N20 ms×Prsvp where N is the number of slots that can be used for SL transmission within 20 ms of the configured UL-DL configuration.

| Source | Comments |
| --- | --- |
| Nokia, NSB | Typo “If a set of sub-channels in slots” should read “slot”. FL: fixed |
| QC | Tscal is set to selection window size T2 converted to units of ms. There can be case T2> Prsvp_RX>T2-T1. If that happen, clause 6c) only count Q = 1 with FL’s wording. Then only one resource reserved at m, where n+T1> m>n is accounted for, but not the resource reserved at m+Prsvp_RX. Among m and m+Prsvp_RX, m+Prsvp_RX is in selection window while m is not. The proposed change will fix this case. FL: I’m not sure if the agreement “resource selection window size” refers to T2 or T2-T1. But I tend to think T2 is more suitable here. |
| OPPO | - Following the new naming convention for SCI formats in R1-2003169, SCI format 0-1 should be 1-A. - Regarding the issue brought up by QC above, in our understanding, Prsvp_RX needs to fall in between T2-T1 and T2, where the time gap T1 is around 1ms. If we want to optimize this case, some further discussion may be needed between T2-T1 and T2. FL: I’m not sure if the agreement “resource selection window size” refers to T2 or T2-T1. But I tend to think T2 is more suitable here. |
| Huawei/HiSilicon | We think T2 seems more accurate and reasonable. It seems in the submission to ITU, we cannot have [TBD]. So how to handle [TBD] then? Replace [TBD] with something else? FL comment: TBD is meant to refer to a section where resource pool slot determination is written. I guess in PHY structure this TP is going to be developed this meeting. During editor CR phase we can fix the reference to the actual clause. |

- References

- R1-2003310	Remaining details of Resource Allocation Mode 2	Nokia, Nokia Shanghai Bell

- R1-2003379	Remaining issues on mode 2 resource allocation mechanism	vivo

- R1-2003495	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

- R1-2003549	Remaining issues in Mode-2	ZTE, Sanechips

- R1-2003559	Remaining Issues on Sidelink Mode 2 Resource Allocation	Panasonic Corporation

- R1-2003563	Discussion on resource allocation for Mode 2	LG Electronics

- R1-2003613	Remaining issues on Mode 2 resource allocation in NR V2X	CATT

- R1-2003653	Remaining Issues on Resource Allocation in NR Sidelink Mode 2	ITRI

- R1-2003671	Sidelink mode-2 resource allocation	MediaTek Inc.

- R1-2003703	Remaining issues for Mode 2 resource allocation in NR V2X	ASUSTeK

- R1-2003735	Remaining details of Mode-2 NR V2X sidelink design	Intel Corporation

- R1-2003807	Remaining details on mode-2 resource allocation	Futurewei

- R1-2003874	On Mode 2 for NR Sidelink	Samsung

- R1-2003991	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

- R1-2004043	Remaining details on mode 2 resource allocation for NR V2X	Fujitsu

- R1-2004074	Discussion on remaining open issue for mode 2	OPPO

- R1-2004171	Resource allocation for NR sidelink Mode 2	TCL Communication Ltd.

- R1-2004217	Remaining Issues of Mode 2 Resource Allocation	Apple

- R1-2004295	Remaining Issues on NR Sidelink Mode 2 Resource Allocation	InterDigital, Inc.

- R1-2004310	Remaining issues on resource allocation Mode 2	NEC

- R1-2004328	Remaining issues on resource allocation mode 2 for NR sidelink	Sharp

- R1-2004385	Remaining issues on resource allocation mechanism mode 2	NTT DOCOMO, INC.

- R1-2004452	Sidelink Resource Allocation Mode 2	Qualcomm Incorporated

- R1-2004531	Remain details on mode-2 resource allocation for NR V2X	ITL

- R1-2004544	Resource allocation Mode 2 for NR SL	Ericsson