3GPP TSG RAN WG1 #101							R1-2004931

e-Meeting, May 25th – June 5th, 2020

______________________________________________________________________Agenda item: 7.2.4.5

Source: Moderator (LG Electronics)

Title: Text proposal from Email discussion thread #1 for AI 7.2.4.5 Physical layer procedures for sidelink

Document for: Discussion and decision

- Introduction and proposal

RAN1 made the agreements copied in Appendix in [101-e-NR-5G_V2X_NRSL-SL_PHY_Procedure-01] Email discussion/approval regarding: power control. This contribution includes the text proposal endorsed. It is proposed to adopt the text proposal in Section 2 for the following reasons

- Reason for change: RAN1 made the agreements to complete the UE procedure for sidelink power control. The text proposal is to implement these agreements.

- Summary of change: DL pathloss used in the sidelink open loop power control is specified. Rules to determine the number of simultaneously transmitted PSFCH and the transmit power of each PSFCH are specified in the cases missing in the current specifications.

- Consequences if not approved: The specification is incomplete in supporting sidelink power control.

- Text proposal

  - Text proposal for TS 38.213

===========================<Start of change #1>=======================

### 16.2.1	PSSCH

< Unchanged parts are omitted>

where

-	PO,D is a value of p0-DL-PSCCHPSSCH if provided

-	αD is a value of alpha-DL-PSCCHPSSCH, if provided; else, αD=1

-	PLD=PLb,f,c(qd) as described in Clause 7.1.1 with following changes:

-  Using the RS resource used for power control for PUSCH transmission scheduled by DCI format 0_0 when the UE monitors DCI format 0_0.

-  Using a RS resource from the SS/PBCH block that the UE uses to obtain MIB when the UE does not monitor DCI format 0_0.

-	MRBPSSCH(i) is a number of resource blocks for the PSSCH transmission occasion i and μ is a SCS configuration

< Unchanged parts are omitted>

============================<End of change #1>=======================

===========================<Start of change #2>=======================

### 16.2.3	PSFCH

A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i as

-	if p0-DL-PSFCH is provided,

PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm]

where

-	PO,PSFCH is a value of p0-DL-PSFCH

-	αPFSCH is a value of alpha-DL-PSFCH, if provided; else, αPFSCH=1

-	PL=PLb,f,c(qd) as described in Clause 7.1.1

-	if Nsch,Tx,PSFCH≤Nmax,PSFCH

-	if PPSFCH,one+10log10Nsch,Tx,PSFCH≤PCMAX, where PCMAX is determined for Nsch,Tx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]

-	NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm]

-	else

-	UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending priority order as described in Clause 16.2.4.2 such that NTx,PSFCH≥X where max1,i=1KMi where Mi is the number of PSFCHs with priority value i and K is the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for the transmissions of all the PSFCHs assigned with the priority values 1, 2, …, K TBD and

PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm]

where PCMAX	is defined in [8-1, TS 38.101-1] and is determined for the NTx,PSFCH PSFCH transmissions

-	else

-	the UE autonomously selects Nmax,PSFCH PSFCH transmissions with ascending priority order as described in Clause 16.2.4.2

-	if PPSFCH,one+10log10Nmax,PSFCH≤PCMAX, where PCMAX is determined for the Nmax,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]

-	NTx,PSFCH=Nmax,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm]

-	else

-  	the UE autonomously selects NTx,PSFCH PSFCH transmissions in ascending order of corresponding priority field values as described in Clause 16.2.4.2 such that NTx,PSFCH≥X where  max1,i=1KMi where Mi is the number of PSFCHs with priority value i and K is the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for the transmissions of all the PSFCHs assigned with the priority values 1, 2, …, K TBD and

-	PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm]

where PCMAX is determined for the  NTx,PSFCH simultaneous PSFCH transmissions according to [8-1, TS 38.101-1]

-	else

PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH) [dBm]

where UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending priority order as described in Clause 16.2.4.2 such that NTx,PSFCH≥1

============================<End of change #2>=======================

Appendix: Agreements made in the email discussion [101-e-NR-5G_V2X_NRSL-SL_PHY_Procedure-01]

Agreements:

For open-loop power control based on DL pathloss, the UE calculates DL pathloss using

- The RS resource used for power control for PUSCH transmission scheduled by DCI format 0_0 when the UE monitors DCI format 0_0.

- A RS resource from the SS/PBCH block that the UE uses to obtain MIB when the UE does not monitor DCI format 0_0.

Agreements:

For Case 1-2 and Case 2-2 of simultaneous transmissions of PSFCH, the lower bound X of the number of actually transmitted PSFCH is given by X = max {1, Y = M_1+M_2+…+M_K} where

- M_i is the number of PSFCHs assigned with the priority value i

- K is the largest value which does not lead to the power limited case when the UE transmits all the PSFCHs with the priority value 1, 2, …, K.

Agreements:

When P_(O,PSFCH) is not provided,

- The number of actually transmitted PSFCH N is up to UE implementation with X=1.

- The transmit power of each PSFCH is PCMAX-10log10N where PCMAX determined for the N PSFCH transmission.