3GPP TSG RAN WG1 #101	R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Source:	moderator (vivo)

Title:	Outcome of [101-e-NR-eMIMO-ULFPTx-02] Email discussion

Agenda Item:	7.2.6.4

Document for:	Discussion and Decision

Summary

TP for TS 38.213 section 7.1

Reason for change: SRI can be either indicated in DCI or by RRC configuration in the case of Type1 configured grant. In the current spec the description for Type1 configured grant is missing

Summary of change: description on SRI indication for Type1 configured grant is added

Consequences if not approved: SRI indication by Type1 configured grant is not supported

Clauses affected: 38.213, section 7.1

--------------------------------------------start------------------------------------------------------------

<<unchanged part omitted>>

if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode2

-	s=1 for full power TPMIs reported by the UE [16, TS 38.306], and s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over a number of SRS ports for remaining TPMIs, where the number of SRS ports is associated with a SRS resource indicated by a SRI field in a DCI format scheduling the PUSCH transmission if more than one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or the number of SRS ports is associated with the SRS resource if only one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook',

-	s=1, if a SRS resource with a single port is indicated by a SRI field in a DCI format scheduling the PUSCH transmission when more than one SRS resource is provided in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or if only one SRS resource with a single port is provided in the SRS-ResourceSet with usage set to 'codebook', and

-	if ul-FullPowerTransmission in PUSCH-Config is set to fullpower, s=1

<<unchanged part omitted>>

---------------------------------------------end-----------------------------------------------------------------