3GPP TSG RAN WG1 #102-e                                                                                   R1-2005466

# e-Meeting, August 17th – 28th, 2020

# Title:	Draft reply LS on NR SRS carrier switching

# Response to:	R2-2006362

# Release:	Release 15

# Work Item:	NR_newRAT-Core

# Source:	ZTE [RAN1]

# To:	RAN2

# Contact Person:

Name:

E-mail Address:

# 1. Overall Description:

RAN1 would like to thank RAN2 for the LS R2-2006362 on NR SRS carrier switching.

In R2-2006362, for SRS carrier switching type A, RAN2 asks RAN1 whether the following two cases on NR SRS carrier switching are allowed:

- Case 1: SRS carrier switching to a SUL without PUCCH/PUSCH while PUCCH/PUSCH is configured in NUL;

- Case 2: SRS carrier switching to both NUL and SUL, neither of which are configured with PUCCH/PUSCH.

From RAN1 perspective, SRS transmission for NR SRS carrier switching is used for a serving cell without PUSCH and PUCCH in order to achieve DL CSI via UL/DL channel reciprocity in a TDD band. However, it does not have the corresponding DL symbols configured in the same frequency band as the SUL carrier. Thus, it is not valid that a SUL without any PUSCH and PUCCH transmission is configured with SRS carrier switching for DL CSI acquisition. Therefore, the above two cases are not allowed from RAN1 perspective.

# 2. Actions:

# To RAN2 group

RAN1 respectfully requests RAN2 to take the above information into consideration.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #103-e	                          October 26th – November 13th, 2020