3GPP TSG RAN WG1 #102-e	                                                                 R1-200XXXX

# e-Meeting, August 17th – 28th, 2020

Agenda item:	7.1

Source:	Moderator (Nokia)

Title:	Summary for [102-e-NR-7.1CRs-18] “Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations NCPU” (Issue #16)

Document for:	Discussion and Decision

# Introduction

This document is created to facilitate the email discussion [102-e-NR-7.1CRs-18] “Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations N_CPU”. This thread is triggered by Issue #16 of [1] and originates from a draft CR to TS38.214 in [2].

| Issue# | Tdoc# | Source | Issue description |
| --- | --- | --- | --- |
| 16 | R1-2006851 | Nokia, NSB | Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations N_CPU. Carry over from previous meeting. |

# Company views

Please provide company’s view in the table below:

| Company | View |
| --- | --- |
| Nokia/NSB | In subclause 5.2.1.6 of TS38.214, the parameter NCPU is defined as the number of simultaneous CSI calculations supported by a UE. The text specifies that this parameter is indicated by the UE, but it is not clear which UE capability component indicates it and that the indication is given per-CC as well as across all active CCs. Without this clarification, a UE may report the maximum supported number of simultaneous CSI reports in simultaneousCSI-ReportsPerCC and simultaneousCSI-ReportsAllCC, instead of the maximum number of CPUs, which creates an underreporting problem because one CSI report can occupy multiple CPUs. |
| Qualcomm | Additionally clarify that per-CC capability is “hard restriction”, while across-all-CC capability is a “soft restriction” – It means that gNB can configure/trigger >N_CPU across all CCs, UE just do not update the CSI who exceeds the limit. Modified text proposal: “UE indicates the number of supported simultaneous CSI calculations NCPU' with parameter simultaneousCSI-ReportsPerCC in a component carrier, and NCPU simultaneousCSI-ReportsAllCC across all component carriers. A UE does not expect to have more than NCPU' simultaneous CSI calculations on each component carrier. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports across all configured cells.” |
| Huawei/HiSilicon | With regarding to newly suggested change, “A UE does not expect to have more than N_CPU^' simultaneous CSI calculations on each component carrier”, it actually means new gNB implementation restriction on top of Rel-15 spec. At least in our understanding, all priority of CSI reporting (if gNB triggering CSI reports exceed N_CPU) shall apply to per CC and across CCs simultaneously so that the UE will provide non-updated report(s) to the gNB per CC and also across CCs. Spec has defined a series of clarifications how to rank CSI reports per CC and across CCs. Therefore, would original text proposal from Nokia may be sufficient enough? |
| ZTE | Similar understanding as Huawei. Based on the previous text from Nokia, it means the specified rules for Rel-15 apply on both N_CPU per CC and N_CPU across all CCs. However, the new text from Qualcomm means those rules only apply on N_CPU across all CCs, and there should be a new rule for per CC restriction of N'_CPU, which can be NBC. Hence, we think it is better to keep the original text from Nokia, as we don't want to introduce new behavior at such late stage. |

# Conclusion

To be added after the discussion.

# Reference

- R1-2006958 RAN1#102-e preparation phase on NR Rel-15 CRs, Ad-hoc chair (Samsung)

- R1-2006851 38.214CRdraft (Rel-15, F) Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations NCPU, Nokia, Nokia Shanghai Bell