3GPP TSG RAN WG1 #102-e	                                                                 R1-200XXXX

# e-Meeting, August 17th – 28th, 2020

Agenda item:	7.1

Source:	Moderator (Nokia)

Title:	Summary for [102-e-NR-7.1CRs-18] “Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations NCPU” (Issue #16)

Document for:	Discussion and Decision

# Introduction

This document is created to facilitate the email discussion [102-e-NR-7.1CRs-18] “Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations N_CPU”. This thread is triggered by Issue #16 of [1] and originates from a draft CR to TS38.214 in [2].

| Issue# | Tdoc# | Source | Issue description |
| --- | --- | --- | --- |
| 16 | R1-2006851 | Nokia, NSB | Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations N_CPU. Carry over from previous meeting. |

# Company views

Please provide company’s view in the table below:

| Company | View |
| --- | --- |
| Nokia/NSB | In subclause 5.2.1.6 of TS38.214, the parameter NCPU is defined as the number of simultaneous CSI calculations supported by a UE. The text specifies that this parameter is indicated by the UE, but it is not clear which UE capability component indicates it and that the indication is given per-CC as well as across all active CCs. Without this clarification, a UE may report the maximum supported number of simultaneous CSI reports in simultaneousCSI-ReportsPerCC and simultaneousCSI-ReportsAllCC, instead of the maximum number of CPUs, which creates an underreporting problem because one CSI report can occupy multiple CPUs. |
| Qualcomm | Additionally clarify that per-CC capability is “hard restriction”, while across-all-CC capability is a “soft restriction” – It means that gNB can configure/trigger >N_CPU across all CCs, UE just do not update the CSI who exceeds the limit. Modified text proposal: “UE indicates the number of supported simultaneous CSI calculations NCPU' with parameter simultaneousCSI-ReportsPerCC in a component carrier, and NCPU simultaneousCSI-ReportsAllCC across all component carriers. A UE does not expect to have more than NCPU' simultaneous CSI calculations on each component carrier. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports across all configured cells.” |
| Huawei/HiSilicon | With regarding to newly suggested change, “A UE does not expect to have more than N_CPU^' simultaneous CSI calculations on each component carrier”, it actually means new gNB implementation restriction on top of Rel-15 spec. At least in our understanding, all priority of CSI reporting (if gNB triggering CSI reports exceed N_CPU) shall apply to per CC and across CCs simultaneously so that the UE will provide non-updated report(s) to the gNB per CC and also across CCs. Spec has defined a series of clarifications how to rank CSI reports per CC and across CCs. Therefore, would original text proposal from Nokia may be sufficient enough? |
| ZTE | Similar understanding as Huawei. Based on the previous text from Nokia, it means the specified rules for Rel-15 apply on both N_CPU per CC and N_CPU across all CCs. However, the new text from Qualcomm means those rules only apply on N_CPU across all CCs, and there should be a new rule for per CC restriction of N'_CPU, which can be NBC. Hence, we think it is better to keep the original text from Nokia, as we don't want to introduce new behavior at such late stage. |
| Moderator | Based on the comments so far, it seems we can agree on Proposal 1 summarised below, where I omitted the rest of the paragraph from 5.2.1.6 for brevity. Proposal 1. Clarification on the two UE capabilities indicating NCPU restriction per-CC and across all CCs. The original text proposal implies that a UE does not update the CSI reports exceeding the CPU count in each CC and across all CCs, following the priority rules of 5.2.5 (“soft” NCPU restriction per CC and across all CCs). “The UE indicates the number of supported simultaneous CSI calculationsNCPU with parameter simultaneousCSI-ReportsPerCC in a component carrier, and simultaneousCSI-ReportsAllCC across all component carriers.” Proposal 2. Under this proposal, simultaneous CSI reports in a CC should not occupy more CPU calculations than the per-CC capability. A UE does not update the CSI reports exceeding the CPU count across all CCs, following the priority rules of 5.2.5 (“Hard” NCPU restriction per CC and “soft” restriction across all CCs). “UE indicates the number of supported simultaneous CSI calculations NCPU' with parameter simultaneousCSI-ReportsPerCC in a component carrier, and NCPU simultaneousCSI-ReportsAllCC across all component carriers. A UE does not expect to have more than NCPU' simultaneous CSI calculations on each component carrier.” |
| Apple | To make is clearer, we can have the following TP (to amend the sentence after the first sentence in the current 38.214) The UE indicates the number of supported simultaneous CSI calculations 𝑁𝐶𝑃𝑈 with parameter simultaneousCSI-ReportsPerCC in a component carrier, and simultaneousCSI-ReportsAllCC across all component carriers. If a UE supports 𝑁𝐶𝑃𝑈 simultaneous CSI calculations it is said to have 𝑁𝐶𝑃𝑈 CSI processing units for processing CSI reports across all configured cells for simultaneousCSI-ReportsAllCC and within the configured cell for simultaneousCSI-ReportsPerCC. Or The UE indicates the number of supported simultaneous CSI calculations 𝑁𝐶𝑃𝑈 with parameter simultaneousCSI-ReportsPerCC in a component carrier, and simultaneousCSI-ReportsAllCC across all component carriers. If a UE supports 𝑁𝐶𝑃𝑈 simultaneous CSI calculations it is said to have 𝑁𝐶𝑃𝑈 CSI processing units for processing CSI reports across all configured cells. |
| Ericsson | We have a similar understanding as Nokia, Huawei, ZTE and Apple. We can support Proposal 1. Also, the following clarification suggested by Apple is preferable from our perspective: The UE indicates the number of supported simultaneous CSI calculations 𝑁𝐶𝑃𝑈 with parameter simultaneousCSI-ReportsPerCC in a component carrier, and simultaneousCSI-ReportsAllCC across all component carriers. If a UE supports 𝑁𝐶𝑃𝑈 simultaneous CSI calculations it is said to have 𝑁𝐶𝑃𝑈 CSI processing units for processing CSI reports across all configured cells. |
| vivo | Our understanding is that the following rule is applied per CC with restriction of simultaneousCSI-ReportsPerCC or across CC with the restriction of simultaneousCSI-ReportsAllCC. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L holds.. Our understanding is that without any update the spec is already clear with the following definition of component 8 and component 9 of 2-35. 8) UE can process Y CSI report(s) simultaneously in a CC. CSI reports can be P/SP/A CSI and any latency class and codebook type. 9) UE can process X CSI report(s) simultaneously across all CCs. CSI reports can be P/SP/A CSI and any latency class and codebook type.. |
| Qualcomm | We think the spec is unclear how to honor per-CC capability, because the rule and the wording in 5.2.1.6 is describing w.r.t. N_CPU across all cells. Proposal 1 is to treat per-CC capability soft. That is, allowing configuring/triggering >N_CPU CSI process per cell, but UE only update N_CPU. If UE reports per-CC NCPU = 2, network can configure/trigger 3 CSIs, CSI 1-3, but UE only update CSI 1-2. Proposal 2 is to treat per-CC capability hard. That is, it is an error case if configuring/triggering >N_CPU CSI process per cell. In other words, if UE reports per-CC NCPU = 2 and network can configure/trigger 3 CSIs, CSI 1-3, then it’s upto UE implementation how to handle it. We are fine with either option, but wish to hear the common understanding so that companies can align. |
| Oppo | We support Proposal 1. We also share the same view as Ericsson that Apple’s following proposal is preferable. The UE indicates the number of supported simultaneous CSI calculations 𝑁𝐶𝑃𝑈 with parameter simultaneousCSI-ReportsPerCC in a component carrier, and simultaneousCSI-ReportsAllCC across all component carriers. If a UE supports 𝑁𝐶𝑃𝑈 simultaneous CSI calculations it is said to have 𝑁𝐶𝑃𝑈 CSI processing units for processing CSI reports across all configured cells. |
| Qualcomm | The text proposed by Apple works for us. |
| Spreadtrum | We support proposal 2 in principle. In our understanding, the priority rules of 5.2.5 is only used for across all CCs capability. Otherwise, UE behavior is not clear. For example, a UE can process 2 CSI report(s) simultaneously in a CC, and 4 CSI report(s) simultaneously across all CCs. When the UE is configured to report CSI1/CSI2/CSI3 in CC1 and CSI4/CSI5/CSI6 in CC2. If the CSI priority is CSI1à CSI2à CSI3à CSI4à CSI5à CSI6, UE is not required to calculate CSI5 and CSI6. However, according to per CC capability, CSI3 should not be calculated, which is not align with the priority rules of 5.2.5. |
| Samsung | We support Proposal 1 |
| Moderator | Proposal 1 now reads: “The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC in a component carrier, and simultaneousCSI-ReportsAllCC across all component carriers. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports across all configured cells.” @Spreadtrum: your example would be an error case under Proposal 2 because the reports per CC exceed the per-CC capability. Hence, different UEs may handle it differently, as pointed out by Chenxi and Min: in your example the UE would only update CSI1, CSI2 and CSI4 (hard per-CC restriction applied after the soft across-CCs restriction), but another UE may update CSI1, CSI2 and CSI4,CSI5 (hard per-CC restriction applied before the soft across-CCs restriction), etc. Conversely, under Proposal 1, this is not an error case and a UE would update the reports in order of priority until both the per-CC and across-CC restrictions allow, i.e. CSI1, CSI2 and CSI4,CSI5. @vivo: let’s consider, for example, a UE that can process 1 CSI report requiring OCPU=2 in CC1, 1 CSI report requiring OCPU=4 in CC2, and it can process both reports simultaneously. Without the clarification, from the cap description you quoted in 822 or 306, a UE may indicate 1,1,2 as simultaneousCSI-ReportsPerCC in CC1 and CC2, and simultaneousCSI-ReportsAllCC, respectively. However, with the text proposal in 214, it’s clear that a UE should indicate 2,4,6 instead. Besides, without the clarification, it is not clear from 214 that the CPU restriction formula is applied per-CC as well as across CCs, so if the CPU cap is exceeded per-CC, different UEs may behave differently: trigger an error, or apply “hard” or “soft” restriction per CC, etc. |
| ZTE | We are fine with the updated proposal 1 |
| Huawei/HiSilicon | We are also fine with the update in Proposal 1 |

# Conclusion

Proposal

“The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC in a component carrier, and simultaneousCSI-ReportsAllCC across all component carriers. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports across all configured cells.”

# Reference

- R1-2006958 RAN1#102-e preparation phase on NR Rel-15 CRs, Ad-hoc chair (Samsung)

- R1-2006851 38.214CRdraft (Rel-15, F) Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations NCPU, Nokia, Nokia Shanghai Bell