3GPP TSG RAN WG1 #102-e   	 	  	                 R1-2007006

e-Meeting, August 17th – 28th, 2020

Agenda item: 7.2.6

Source: Moderator (LG Electronics)

Title: Summary#1 of email thread [102-e-NR-eMIMO-01]

Document for: Discussion and Decision

- Introduction

This contribution summaries discussion in email thread [102-e-NR-eMIMO-01]

- Background

Codebook based or non-codebook based PUSCH can be scheduled by either DCI format 0_1 or DCI format 0_2. However, in TS 38.213 the UE behavior on determining default pathloss RS for PUSCH transmission is only specified for the PUSCH scheduled by DCI format 0_1. For this reason, OPPO proposed following TP.

TP from OPPO for clause 7.1.1 of TS 38.213

### 7.1.1 UE behavior ## *** Unchanged text is omitted *** - If the PUSCH transmission is scheduled by a DCI format 0_0, and if the UE is provided a spatial setting by PUCCH-SpatialRelationInfo for a PUCCH resource with a lowest index for active UL BWP b of each carrier f and serving cell c, as described in Clause 9.2.2, the UE uses the same RS resource index qd as for a PUCCH transmission in the PUCCH resource with the lowest index - If the PUSCH transmission is scheduled by a DCI format 0_1 or a DCI format 0_2, and if the UE is provided enableDefaultBeamPlForSRS and is not provided PUSCH-PathlossReferenceRS and PUSCH-PathlossReferenceRS-r16, the UE uses the same RS resource index qd as for a SRS resource set with an SRS resource associated with the PUSCH transmission - If - the PUSCH transmission is scheduled by a DCI format 0_0 and the UE is not provided a spatial setting for a PUCCH transmission, or - the PUSCH transmission is scheduled by a DCI format 0_1 or a DCI format 0_2 that does not include an SRI field, or - SRI-PUSCH-PowerControl is not provided to the UE, the UE determines a RS resource index qd with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking ## *** Unchanged text is omitted ***

- Discussion

Based on the discussion during preparation phase and due to the fact that DCI0_1 and DCI0_2 are quite similar in terms of power control, it seems reasonable to adopt OPPO’s TP.

FL’s proposal: Adopt the TP in section 2.

Companies’ view (to be updated)

| Company name | View |
| --- | --- |

- Conclusion (to be updated)

Reference

| TDoc | Title | Source |
| --- | --- | --- |
| R1-2005976 | Text proposals for Multi-beam Operation Enhancement | OPPO |