3GPP TSG RAN WG1 #102-e-	R1-200xxxx

e-Meeting, August 17th – 28th, 2020

Source:	moderator (vivo)

Title:	Feature lead summary on ULFPTx

Agenda Item:	7.2.6

Document for:	Discussion and Decision

Introduction

In this contribution, following issue is discussed according to agreement of preparatory email discussion

- [102-e-NR-eMIMO-07] TPMI grouping for mode 2 – Rakesh (vivo)

Remaining issues

Issue 1: TPMI grouping for Mode 2

Alt1. Add new TPMI group(s)

Alt2. No change in TPMI group, and UE can report >1 TPMI groups

Alt3. Revise at least one TPMI group

| Company | Comments |
| --- | --- |
| ZTE | There are two questions ahead of us: - Q1: The number of PA architectures corresponds to non-coherent and partial-coherent case is different, so how to make it in equal? - Q2: In order to enable the diversity of UE implementation with this functionality, how to support more newly typical PA architectures? Here, we would like to provide a solution to address this issue, which consist of the following two aspects. For Q1, compared to the existing G0-G6 for partial-coherent 4-port UEs, two PA architectures have been missed for non-coherent 4-port UEs, which are {20 17 20 17} dBm and {20 20 20 20} dBm. Correspondingly, two new groups (G4’, G5’) correspond to the two missed PA architectures should be added to Table 1 in RAN1 #99 agreement. Table 1’ 4Tx, nonCoherent; 4Tx, partialCoherent. G0; G0. G1; G1. G2; G2. G3; G3. G4’; G4. G5’; G5. G6. <Precoders of G4’ and G5’ correspond to non-coherent 4-port UEs> G4’: G5’:{, , , , , },. <PA architectures of Table 1> 4Tx, nonCoherent; PA architecture; 4Tx, partialCoherent; PA architecture. G0; [23 17 17 17] dBm; G0; [23 17 17 17] dBm. G1; [23 17 23 17] dBm; G1; [23 17 23 17] dBm. G2; [23 23 23 17] dBm; G2; [23 23 23 17] dBm. G3; [20 20 20 17] dBm; G3; [20 20 20 17] dBm. G4’; [20 17 20 17] dBm; G4; [20 17 20 17] dBm. G5’; [20 20 20 20] dBm; G5; [20 20 20 17] dBm. G6; [20 20 20 20] dBm. For Q2, based on the newly added G4’ and G5’ for non-coherent 4-port UEs, Alt2 can be regarded as a good way to address this issue, meanwhile it retains the flexible scalability of the PA architectures. Specifically, for the partial-coherent 4-port UE, it can report up to two TPMI groups, one from {G0~G2} and one from {G3~G6}. For the non-coherent 4-port UE, it also can report up to two TPMI groups, one from {G0~G2} and one from {G3, G4’, G5’}. We echo some examples as follow to elaborate this solution. - Report G1+G4’ can enable the non-coherent 4-port UE with PA={23 17 23 17} dBm to implement full power with this functionality. - Report G1+G4 can enable the partial-coherent 4-port UE with PA={23 17 23 17} dBm to implement full power with this functionality. - Report G2+G5’ can enable the non-coherent 4-port UE with PA={23 23 23 17} dBm to implement full power with this functionality. - Report G2+G5 can enable the partial-coherent 4-port UE with PA={23 23 23 17} dBm to implement full power with this functionality. - Report G1+G5' can enable the non-coherent 4-port UE with PA={23 20 23 20} dBm to implement full power with this functionality. - Report G1+G6 can enable the partial-coherent 4-Tx UE with PA={23 20 23 20} dBm to implement full power with this functionality. - etc. Due to the joint-reporting of TPMI groups, this solution can flexibly to enable many PA architectures of non-coherent & partial-coherent 4-port UEs to implement full power UL transmission. In addition, plenty of entries also have been remained to support some other PA architectures of non/ partial-coherent 4-port UEs in the future. In conclusion, the intention of our solution is to ensure the number of supported PA architectures of non-coherent and partial-coherent 4-port UEs is equal, and enable more other PA architectures for the diversity of UE implement. |
| Samsung | Thanks ZTE for the proposal. Although a solution based on Alt1+2 is not our preference, but for progress, we can be fine with it. It does provide more flexibility to non-cerement UEs by including G4’ and G5’. |

- Answer to RAN2 LS: TPMIs for 4-port non-coherence can be deduced from the reported set of TPMIs for 4-port partial-coherence. 4-port full coherent UE follow the same way as 4-port partial coherent UE to report full power TPMIs.

| Company | Comments |
| --- | --- |
| ZTE | According to our above proposed solution of TPMIs reporting issue, we draft reply LS to RAN2 as below: For a 4 port partial-coherent or full-coherent UE, the UE shall report 2-port {2-bit bitmap}, one 4-port non-coherent TPMI group from {G0~G2} in Table 1’ in Annex and/or one 4-port partial-coherent TPMI group from {G3~G6} in Table 1’ in Annex. For a 4 port non-coherent UE, the UE shall report 2-port {2-bit bitmap}, one 4-port non-coherent TPMI group from {G0~G2} in Table 1' in Annex and/or one 4-port non-coherent TPMI group from {G3, G4', G5'} in Table 1’ in Annex. For a 2 port UE, the UE shall report 2-port {2-bit bitmap}. Annex Table 1’ 4Tx, nonCoherent; 4Tx, partialCoherent. G0; G0. G1; G1. G2; G2. G3; G3. G4’; G4. G5’; G5. G6. Definition of G0~G6 and G4’, G5’ can be found in the table 2’ as below. Table 2’ TPMI group; Precoders. G0. G1; ,. G2; ,,. G3; ,. G4; ,. G5; ,,. G6; ,,,. G4’. G5’; {,,,,, },. |
| Samsung | Support with the clarification that for 4 ports, the UE reports x or y or (x, y) where x is a TPMI group from {G0-G2} and y is a TPMI group from {G3-G5} for NC or {G3-G6} for PC. |

Reference

[1] R1-2006979. “Summary#2 for Rel.16 NR eMIMO maintenance”, moderator (Samsung), RAN1#102-e, 17th -28th August, 2020