3GPP TSG RAN WG1 Meeting #102-e R1-200xxxx e-Meeting, 17th – 28th August, 2020 CR-Form-v12.0. CHANGE REQUEST. 38.213; CR; DRAFT; rev; -; Current version:; 16.2.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR (Rel-16, F) on Tyep-3 CSS monitoring with PS-RNTI on primary cell. Source to WG:; CATT, Samsung, Huawei, HiSilicon. Source to TSG:; RAN WG1. Work item code:; NR_UE_pow_sav_CORE; Date:; 2020-08-26. Category:; F; Release:; Rel-16. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; DCI format 2_6 scrambled by PS-RNTI can be monitored at primary cell only (PCell for CA and SpCell for DC). However, in the paragraph related to Type3-CSS in 38.213 Section 10.1, it is ambiguous whether the PS-RNTI can be monitored in primary cell only or not since PS-RNTI is located wrongly. -. Summary of change:; Locate PS-RNTI to the primary cell only for Type3-CSS.. Consequences if not approved:; The UE behavior is not clear in Type3-CSS when CRC is scrambled by PS-RNTI. Clauses affected:; 10.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: The CR clarifies the UE behavior of Type3-CSS PDCCH monitoring with CRC scrambled by PS-RNTI only at primary cell (PCell for CA and SpCell for DC). The current specification incorrect specifies the PS-RNTI could have Type3-CSS PDCCH monitoring at any cell.. This CR's revision history:; This is the first version for this CR..

## 10.1	UE procedure for determining physical downlink control channel assignment

A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets

-	a Type0-PDCCH CSS set configured by pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell

-	a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a P-RNTI on the primary cell of the MCG

-	a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, or CI-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, CS-RNTI(s), or PS-RNTI and

-	a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI(s), SL-RNTI, SL-CS-RNTI, or SL-L-CS-RNTI.