3GPP TSG RAN WG1 Meeting #102-e R1-200xxxx e-Meeting, 17th – 28th August, 2020 CR-Form-v12.0. CHANGE REQUEST. 38.214; CR; DRAFT; rev; -; Current version:; 16.2.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR (Rel-16, F) on RRM measurements when drx-onDurationTimer does not start. Source to WG:; CATT, Huawei, HiSilicon. Source to TSG:; RAN WG1. Work item code:; NR_UE_pow_sav_CORE; Date:; 2020-08-26. Category:; F; Release:; Rel-16. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; RRM measurement for mobility is performed during active time indicated by drx-onDurationTimer. The start or not of the drx-onDurationTimer is based on the wakeup indication of DCI format 2_6. The drx-onDurationTimer does not start if UE is indicated not to wake up and stay outside active time. The UE should perform RRM measurement for mobility at the scheduled interval regardless drx-onDurationTimer starts or not. Therefore, the current description of “during the active time and during the timer duration indicated by drx-onDurationTimer” may cause misunderstanding that “and during the time duration indicated by drx-onDurationTimer” is during active time. Thus, the description is clarified that the time duration indicated by drx-onDurationTime is outside active time.. -. Summary of change:; Adding “also outside Active Time” after during the timer duration indicated by drx-onDurationTimer. Consequences if not approved:; This CR is to ensure that UE performs RRM measurement “during the timer duration indicated by drx-onDurationTimer” outside active time when drx-onDuratinTimer does not start. Clauses affected:; 5.1.6.1.3. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: Current specification on UE RRM measurements for mobility is not clear with description “the UE may not expect that the CSI-RS resources are available other than during the active time and during the time duration indicated by drx-onDurationTimer. The CR is to clarify the description “during the time duration indicated by drx-onDurationTimer” is for outside active time when drx-onDurationTimer does not start.. This CR's revision history:; This is the first version for this CR..

5.1.6.1.3	CSI-RS for mobility

< Unchanged text is omitted >

If the UE is configured with DRX, the UE is not required to perform measurement of CSI-RS resources other than during the active time for measurements based on CSI-RS-Resource-Mobility. When the UE is configured to monitor DCI format 2_6, the UE is not required to perform measurements other than during the active time and during the timer duration indicated by drx-onDurationTimer also outside active time based on CSI-RS-Resource-Mobility.

If the UE is configured with DRX and DRX cycle in use is larger than 80 ms, the UE may not expect CSI-RS resources are available other than during the active time for measurements based on CSI-RS-Resource-Mobility. If the UE is configured with DRX and configured to monitor DCI format 2_6 and DRX cycle in use is larger than 80ms, the UE may not expect that the CSI-RS resources are available other than during the active time and during the time duration indicated by drx-onDurationTimer also outside active time for measurements based on CSI-RS-Resource-Mobility. Otherwise, the UE may assume CSI-RS are available for measurements based on CSI-RS-Resource-Mobility.

< Unchanged text is omitted >