3GPP TSG RAN WG1 Meeting #103-e	                                                                               R1-2007582

E-meeting, October 26th – November 13th, 2020

Agenda Item:	8.8.1.2

Source:	Huawei, HiSilicon

Title:	Evaluation on the baseline performance for FR2

Document for:	Discussion and Decision

- Introduction

The study item of NR coverage enhancement was approved at RAN#86 meeting [1], and most of the evaluation methods and assumptions were agreed in RAN1#102e [2] and email discussions. MIL [and MCL] and MPL are used for coverage bottleneck(s) identification.

Agreements: - Adopt single link budget template for both FR1 and FR2 based on IMT-2020 self-evaluation with rows for MIL, MCL, MPL, and necessary revisions, including adding/removing/revising/simplifying some parameters - [For LLS based methodology,] coverage bottleneck(s) identification is performed using at least [MCL and] MIL. - [MCL values can also be considered to compare channels with similar antenna (and antenna array) gain] Agreements: - MPL can be used as supplemental information for coverage bottleneck(s) identification - The results based on MPL are to be captured in TR Agreements: - Suburban scenario is deprioritized for NR coverage enhancement SI. Agreements: - Adopt single link budget template for both FR1 and FR2 based on IMT-2020 self-evaluation with rows for MIL, MCL, MPL, and necessary revisions, including adding/removing/revising/simplifying some parameters - For LLS based methodology, coverage bottleneck(s) identification is performed using at least MIL or MCL (assuming the set of simulation assumptions) - Even when SLS is used to obtain some components of MIL or MCL, it is categorized as LLS based methodology. - MCL values can also be used to identify the coverage bottleneck(s) when applicable - “applicable” above means the following situation: - [comparing channels with similar antenna (and antenna array) gain, and/or - the simulation results with MIL from companies are diverse, and the comparison with MIL is not easy]

In this contribution, we provide the FR2 evaluations results of MPL, MIL, and MCL for indoor and urban scenarios.

- FR2 evaluation assumptions

In the link budget template based on IMT-2020 self-evaluation, only some parameters are agreed in the last meeting. The remaining undetermined parameters are important for link budget calculation, including BS antenna element gain, cable loss, interference, shadow fading, penetration, and pathloss model, which are listed in Table 1 and mostly are from [4], [5]. For FR2, the signal from adjacent cells is with large attenuation, thus the interference density is assumed to be the same as thermal noise for both uplink and downlink.

Table 1 Assumptions on the remaining parameters in the link budget

| Parameters | Indoor hotspot | Urban Macro |
| --- | --- | --- |
| BS antenna element gain (dBi) | 5 | 8 |
| Cable loss | BS: 3 dB, UE: 1 dB [5] |  |
| Receiver interference density (dBm/Hz) | All channel: -174 | All channel: -174 |
| Shadow fading (dB) [5] | 5.2 | O2O: 4.85 O2I: 6.41 |
| Penetration margin (dB) | - | 37.95 for 28 GHz Derived according to high loss model defined in TS 38.901 with indoor distance = 0 m [6] |
| Pathloss model | InH [4] [5] | UMa [4] [5] |

For FR2 coverage evaluation, it is considered to focus on a subset of evaluation configurations and technical features. This could facilitate the preparation of the baseline coverage performance. The focused evaluation configuration is shown in Table 2. Link-level simulation is carried to get the required SNR based on the assumptions.

Table 2 evaluated configurations on link budget evaluation

| Parameters | Indoor hotspot | Urban Macro |
| --- | --- | --- |
| System configuration | NR 28 GHz TDD DL/UL: DDDSU | NR 28 GHz TDD DL/UL: DDDSU |
| Channel state | NLOS | NLOS O-to-O NLOS O-to-I |
| Antenna settings [2] | BS: (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) UE: (M, N, P) = (2,2,2) 2T2R for both DL and UL | BS: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2) UE: (M, N, P) = (2,2,2) 2T2R for both DL and UL |
| Analog beamforming | The BS and UE beams, and antenna elements are directed to the strongest cluster in the CDL-A channel model. |  |
| Channel model [2] | CDL-A 30 ns | CDL-A 100 ns |
| SCS | 120 kHz | 120 kHz |
| Waveform [2] | OFDM for DL DFT-s-OFDM for UL | OFDM for DL DFT-s-OFDM for UL |
| Bandwidth [2] | PDSCH/SIB1: 66 RB PUSCH: 30 RB Msg3: 2 RB | PDSCH/SIB1: 66 RB PUSCH: 30 RB Msg3: 2 RB |
| Data and DMRS configuration [2] | Data channel PDSCH/SIB1: 12 OFDM symbols PUSCH/Msg3: 14 OFDM symbols DMRS 1 DMRS symbol, no multiplexing with data PDSCH: Type I w/o additional DMRS PUSCH/Msg3: Type I with 1 additional DMRS |  |
| MCS | PDSCH: 3, SIB1: 0, PUSCH: 5, Msg3: 0 | PDSCH: 3, SIB1: 0, PUSCH: 5, Msg3: 0 |
| UE EIRP | 23 dBm 34 dBm | 23 dBm 34 dBm |
| UE speed | 3 km/h | Indoor: 3 km/h Outdoor: 30 km/h |
| Target ISD and cell coverage | Coverage 50 m ISD 70 m | Coverage1: 267.67 m ISD1: 400 m Coverage2: 333.33 m ISD2: 500 m |

- Baseline coverage performance for FR2

Required SNR based on LLS

Based on the assumptions listed in Table 2, the required SNR is simulated and provided in Table 3. The simulation is based on CDL channel model and the antenna array gain is included in the required SNR.

Table 3 Required SNR (dB) based on LLS

| Scenario and channels | Indoor | Urban |
| --- | --- | --- |
| SS/PBCH (O2O) | -29.71 | -26.33 |
| SS/PBCH (O2I) |  | -27.90 |
| SIB1 (O2O) | -27.03 | -23.65 |
| SIB1 (O2I) |  | -25.22 |
| PDSCH (O2O) | -24.66 | -23.08 |
| PDSCH (O2I) |  | -23.85 |
| PRACH C2 (O2O) | -28.72 | -26.54 |
| PRACH C2 (O2I) |  | -29.75 |
| Msg3 (O2O) | -25.95 | -23.21 |
| Msg3 (O2I) |  | -24.52 |
| PUSCH (O2O) | -22.07 | -22.28 |
| PUSCH (O2I) |  | -23.77 |

Indoor coverage performance

The indoor coverage performance is shown in Figure 1 for UE with maximum EIRP 23 dBm and 34 dBm, and the gap between the achieved and target are given in Table 4 and Table 5, respectively. Based on the results, we have the following observations:

Observation 1: An indoor target coverage of 50 m (ISD 70 m) can be achieved for both 23 dBm and 34 dBm EIRP at UE.

Table 4 MPL performance and gap

| MPL performance | SS/PBCH | SIB1 | PDSCH | PRACH C2 | Msg3 | PUSCH |
| --- | --- | --- | --- | --- | --- | --- |
| Achieved MPL at UE EIRP 23 dBm (dB) | 129.31 | 126.63 | 125.77 | 130.35 | 135.87 | 123.23 |
| Gap to 70 m ISD at UE EIRP 23 dBm (dB) | 21.60 | 18.92 | 18.06 | 22.64 | 28.16 | 15.52 |
| Achieved O2I MPL at UE EIRP 34 dBm (dB) | 134.31 | 131.63 | 130.77 | 141.35 | 146.87 | 134.23 |
| Gap to 70 m ISD at UE EIRP 34 dBm (dB) | 26.60 | 23.92 | 23.06 | 33.64 | 39.16 | 26.52 |

Table 5 Achieved coverage performance and gap

| Coverage performance (C band) | SS/PBCH | SIB1 | PDSCH | PRACH C2 | Msg3 | PUSCH |
| --- | --- | --- | --- | --- | --- | --- |
| Achieved radius at UE EIRP 23 dBm (m) | 96.33 | 82.00 | 77.85 | 102.53 | 142.84 | 66.82 |
| Gap to 70 m ISD at UE EIRP 23 dBm (m) | 49.66 | 35.33 | 31.18 | 55.86 | 96.18 | 20.15 |
| Achieved radius at UE EIRP 34 dBm (m) | 130.11 | 110.75 | 105.15 | 198.63 | 276.74 | 129.44 |
| Gap to 70 m ISD at UE EIRP 34 dBm (m) | 83.44 | 64.08 | 58.48 | 151.97 | 230.07 | 82.78 |

- UE maximum EIRP 23 dBm

- UE maximum EIRP 34 dBm

Figure 1 MPL, MIL, and MCL performance for indoor

Coverage performance of urban

The urban coverage performance is shown in Figure 2 for UE with maximum EIRP 23 dBm. The gap between the achieved and target are given in Table 6 and Table 7, respectively. Based on the results, we have the following observations:

Observation 2: For a target O2O urban coverage of ISD=400 and UE EIRP 23 dBm, SS/PBCH, SIB1, and all uplink channels are unachievable.

Observation 3: For a target O2O urban coverage of ISD=500 and UE EIRP 23 dBm, all downlink and uplink channels are unachievable.

Observation 4: For a target O2I urban coverage of ISD=400 and 500, and UE EIRP 23 dBm, all downlink and uplink channels are unachievable.

- O2O

- O2I

Figure 2 MPL, MIL, and MCL performance for urban with UE EIRP 23 dBm

Table 6 MPL performance and gap for urban with UE EIRP 23 dBm

| MPL performance | SS/PBCH | SIB1 | PDSCH | PRACH C2 | Msg3 | PUSCH |
| --- | --- | --- | --- | --- | --- | --- |
| Achieved O2O MPL (dB) | 137.28 | 134.60 | 135.54 | 122.52 | 127.48 | 117.78 |
| O2O gap to 400 m ISD (dB) | -0.01 | -2.69 | -1.75 | -14.77 | -9.81 | -19.51 |
| O2O gap to 500 m ISD (dB) | -3.80 | -6.48 | -5.54 | -18.55 | -13.60 | -23.29 |
| Achieved O2I MPL (dB) | 108.35 | 105.67 | 105.80 | 95.22 | 98.28 | 88.77 |
| O2I gap to 400 m ISD (dB) | -28.94 | -31.62 | -31.49 | -42.07 | -39.01 | -48.52 |
| O2I gap to 500 m ISD (dB) | -32.73 | -35.41 | -35.28 | -45.86 | -42.80 | -52.31 |

Table 7 Achieved coverage performance and gap for urban with UE EIRP 23 dBm

| Coverage performance | SS/PBCH | SIB1 | PDSCH | PRACH C2 | Msg3 | PUSCH |
| --- | --- | --- | --- | --- | --- | --- |
| Achieved O2O radius (m) | 266.54 | 227.61 | 240.50 | 111.71 | 149.59 | 84.49 |
| O2O gap to 400 m ISD (m) | -0.13 | -39.06 | -26.17 | -154.95 | -117.08 | -182.18 |
| O2O gap to 500 m ISD (m) | -66.79 | -105.73 | -92.84 | -221.62 | -183.74 | -248.84 |
| Achieved O2I radius (m) | 65.05 | 55.55 | 55.98 | 42.75 | 51.20 | 29.23 |
| O2I gap to 400 m ISD (m) | -201.61 | -211.12 | -210.68 | -223.91 | -215.47 | -237.43 |
| O2I gap to 500 m ISD (m) | -268.28 | -277.78 | -277.35 | -290.58 | -282.13 | -304.10 |

The urban coverage performance is shown in Figure 3 for UE with maximum EIRP 34 dBm. The gap between the achieved and target are given in Table 8 and Table 9, respectively. Based on the results, we have the following observations:

Observation 5: For a target O2O urban coverage of ISD=400 and UE EIRP 34 dBm, PRACH and PUSCH are unachievable.

Observation 6: For a target O2O urban coverage of ISD=500 and UE EIRP 34 dBm, SIB1, PDSCH and uplink channels are unachievable.

Observation 7: For a target O2I urban coverage of ISD=400 and 500, and UE EIRP 34 dBm, all downlink and uplink channels are unachievable.

Observation 8: The coverage of the evaluated channels are: PUSCH < PRACH < Msg3 < SIB1 < PDSCH < SS/PBCH.

- O2O

- O2I

Figure 3 MPL, MIL, and MCL performance for urban with UE EIRP 34 dBm

Table 8 MPL performance and gap for urban with UE EIRP 34 dBm

| MPL performance | SS/PBCH | SIB1 | PDSCH | PRACH C2 | Msg3 | PUSCH |
| --- | --- | --- | --- | --- | --- | --- |
| Achieved O2O MPL (dB) | 142.28 | 139.60 | 140.54 | 133.52 | 138.48 | 128.78 |
| O2O gap to 400 m ISD (dB) | 4.99 | 2.31 | 3.25 | -3.77 | 1.19 | -8.51 |
| O2O gap to 500 m ISD (dB) | 1.20 | -1.48 | -0.54 | -7.55 | -2.60 | -12.29 |
| Achieved O2I MPL (dB) | 113.35 | 110.67 | 110.80 | 106.22 | 109.28 | 99.77 |
| O2I gap to 400 m ISD (dB) | -23.94 | -26.62 | -26.49 | -31.07 | -28.01 | -37.52 |
| O2I gap to 500 m ISD (dB) | -27.73 | -30.41 | -30.28 | -34.86 | -31.80 | -41.31 |

Table 9 Achieved coverage performance and gap for urban with UE EIRP 34 dBm

| Coverage performance | SS/PBCH | SIB1 | PDSCH | PRACH C2 | Msg3 | PUSCH |
| --- | --- | --- | --- | --- | --- | --- |
| Achieved O2O radius (m) | 357.85 | 305.58 | 322.89 | 213.59 | 286.00 | 161.54 |
| O2O gap to 400 m ISD (m) | 91.19 | 38.92 | 56.22 | -53.08 | 19.34 | -105.13 |
| O2O gap to 500 m ISD (m) | 24.52 | -27.75 | -10.44 | -119.74 | -47.33 | -171.79 |
| Achieved O2I radius (m) | 65.05 | 55.55 | 55.98 | 42.75 | 51.20 | 29.23 |
| O2I gap to 400 m ISD (m) | -201.61 | -211.12 | -210.68 | -223.91 | -215.47 | -237.43 |
| O2I gap to 500 m ISD (m) | -268.28 | -277.78 | -277.35 | -290.58 | -282.13 | -304.10 |

In light of above observations, we have the following proposals:

Proposal 1: To achieve urban 400m/500m ISD in FR2, PUSCH, Msg3, and PRACH should be enhanced.

Proposal 2: To achieve urban O2I 400m/500m ISD in FR2, PUSCH, Msg3, PRACH, SS/PBCH and SIB1 should be enhanced.

- Conclusions

The contribution provides our considerations on FR2 coverage evaluation, and the observations are listed as following:

Observation 1: An indoor target coverage of 50 m (ISD 70 m) can be achieved for both 23 dBm and 34 dBm EIRP at UE.

Observation 2: For a target O2O urban coverage of ISD=400 and UE EIRP 23 dBm, SS/PBCH, SIB1, and all uplink channels are unachievable.

Observation 3: For a target O2O urban coverage of ISD=500 and UE EIRP 23 dBm, all downlink and uplink channels are unachievable.

Observation 4: For a target O2I urban coverage of ISD=400 and 500, and UE EIRP 23 dBm, all downlink and uplink channels are unachievable.

Observation 5: For a target O2O urban coverage of ISD=400 and UE EIRP 34 dBm, PRACH and PUSCH are unachievable.

Observation 6: For a target O2O urban coverage of ISD=500 and UE EIRP 34 dBm, SIB1, PDSCH and uplink channels are unachievable.

Observation 7: For a target O2I urban coverage of ISD=400 and 500, and UE EIRP 34 dBm, all downlink and uplink channels are unachievable.

Observation 8: The coverage of the evaluated channels are: PUSCH < PRACH < Msg3 < SIB1 < PDSCH < SS/PBCH.

The proposals are:

Proposal 1: To achieve urban 400m/500m ISD in FR2, PUSCH, Msg3, and PRACH should be enhanced.

Proposal 2: To achieve urban O2I 400m/500m ISD in FR2, PUSCH, Msg3, PRACH, SS/PBCH and SIB1 should be enhanced.

- References

RP-193240, “New SID on NR coverage enhancement”, RAN#86, Sitges, Spain, December 9 – 12, 2019.

RAN1#102-e, “Chairman’s notes of RAN1#102-e”, Aug, 2020.

R1-2005256, “Evaluation on the baseline performance for FR1”, Huawei, HiSilicon, RAN1#102e, Aug. 2020.

Report ITU-R M.2412-0, Guidelines for evaluation of radio interface technologies for IMT-2020, 10/2017.

RP-191527, “Link Budget Template - SRIT”, Part 3 of RP-191524 3GPP ITU-R Contribution - Final IMT-2020 Submission Overview, AT&T, Telecom Italia, RAN#84, Jun. 2019.

3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz (Release 16)”, V16.0.0 (2019-10).

Report ITU-R M.2435-1, Guidelines for evaluation of radio interface technologies for IMT-Advanced, 12/2009.