3GPP TSG RAN WG1 #103                                                           R1-2008104

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	7.2.7

Source:	Spreadtrum Communications

Title:	Remaining issues on UE power saving

Document for:	Discussion and decision

# Introduction

In this contribution, we discuss the following remaining issues on power saving signal. This contribution is revision of our contribution [R1-2006289] in RAN1#101-e.

# Discussion

## P-CSI/L1-RSRP measurement/report

It was agreed in RAN1#100e [1] to support UE performing L1-RSRP and CSI measurements outside Active Time when indicated by DCI format 2_6 not to wake up at the next DRX On duration. Due to time limit, the final wording seems a bit ambiguous with too many conditions. In our view, “If(){when(){}},  otherwise{}” is not equivalent to “If() and if(){}, otherwise{}”. We provide some editorial revision in Appendix 5.1 based on TS 38.214 [2].

Necessary justification is provided as follows.

Reason for change:

It was agreed in RAN1#100e [1] to support UE performing L1-RSRP and CSI measurements outside Active Time when indicated by DCI format 2_6 not to wake up at the next DRX On duration. Due to time limit, the final wording seems a bit ambiguous with too many conditions. In our view, “If(){when(){}},  otherwise{}” is not equivalent to “If() and if(){}, otherwise{}”.

Summary of change:

Make the “If(){when(){}},  otherwise{}” structure clearer for the reader without background of meeting discussion.

Consequences if not approved:

The reader without background of meeting discussion may be confused about the “If(){when(){}},  otherwise{}” structure.

Clauses affected:

TS38.214 g30, section 5.2.2.5 and 5.1.6.1.

We kindly propose to accept editorial revision in Appendix 5.1 based on TS 38.214 [2].

Proposal 1: For P-CSI/L1-RSRP measurement/report, consider to adopt TP in Appendix 5.1.

# Conclusion

The following conclusions are proposed:

Proposal 1: For P-CSI/L1-RSRP measurement/report, consider to adopt TP in Appendix 5.1.

# Reference

- R1-2001248, “Summary of Phase 1 Discussion on PDCCH-based Power Saving Signal/Channel”,CATT, RAN1#100e, 17th  – 21st, February 2020.

- TS 38.214, “NR; Physical layer procedures for data”, g20, June 2020.

# Appendix A-Text Proposal

## TP for TS 38.214 (ver. 16.3f.0)

------------------------------------------------------ Start of Text Proposal ----------------------------------------------------------

- 5.2.2.5	CSI reference resource definition

--------------------------------------------------- Unchanged parts omitted ---------------------------------------------------------

When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When DRX is configured, if the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP' and 'ssb-Index-RSRP' and if drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer also outside DRX active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When DRX is configured, if the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP' or 'ssb-Index-RSRP' and if drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer also outside DRX active time according to the procedure described in clause 5.2.1.4 if reportQuantity set to 'cri-RSRP' and receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise.

-------------------------------------------------------- End of Text Proposal --------------------------------------------------------

------------------------------------------------------ Start of Text Proposal ----------------------------------------------------------

- 5.1.6.1	CSI-RS reception procedure

--------------------------------------------------- Unchanged parts omitted ---------------------------------------------------------

If the UE is configured with DRX,

-	if  the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP' and 'ssb-Index-RSRP' and if drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported;

-	if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to cri-RSRP and if drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported;

-	otherwise, the most recent CSI measurement occasion occurs in DRX active time for CSI to be reported.

-------------------------------------------------------- End of Text Proposal --------------------------------------------------------