3GPP TSG RAN WG1 #103-e	                                                                 R1-2008136

e-Meeting, October 26th – November 13th, 2020

Agenda Item:		7.2.5

Source:		Samsung

Title:		Discussion on Type-2 HARQ-ACK codebook

Document for:		Discussion and decision

# Introduction

This contribution discusses the DAI size misalignment issue.

# Discussion

In Rel-16 URLLC, 1 bit C-DAI was introduced, however, the number of bits for UL DAI is fixed as 2. When a HARQ-ACK codebook constructed with 1 bit C-DAI is multiplexed on a PUSCH scheduled by a DCI format, there might be DAI size misalignment issue. In RAN1#101E meeting, we have following agreements

Agreement If UE is configured to monitor DCI format 1_2/0_2, the HARQ-ACK codebook size for type-2 HARQ-ACK codebook is determined by

However, the issue is not corrected solved. In the current spec, UE and gNB may have different understanding of the size of HARQ-ACK codebook if DCI miss detection happens.

For example, the number of bits for the counter DAI is 1, a PDSCH corresponds to 1 bit in the HARQ-ACK codebook. gNB sent 3 DL DCI formats scheduling 3 PDSCHs respectively with HARQ-ACK in slot n, UE received the first 2 DL DCI formats and missed the last DL DCI format. UE received a UL DCI format scheduling a PUSCH in slot n with UL DAI =3.

In the following, the HARQ-ACK codebook construction procedure is illustrated for the given example according to the spec. When current C-DAI is no larger than the last C-DAI, j = j +1, where Vtemp represents the C-DAI, and if T-DAI exists in the DCI format , Vtemp2 represents T-DAI, otherwise, Vtemp2 represents C-DAI.

if end if if else end if

Assuming UE only receives the first two DL DCI formats, values of j, Vtemp and Vtemp2 are given in Table 1.

Table 1

| DCI detected by UE | C-DAI in the DCI format | j: after performed the pseudo code | Vtemp: after performed the pseudo code | Vtemp2: after performed the pseudo code |
| --- | --- | --- | --- | --- |
| DL DCI #1 | 1 | 0 | 1 | 1 |
| DL DCI #2 | 2 | 0 | 2 | 2 |

When UE receives the UL DCI, according the following spec, Vtemp2=3. UE does not receive the last DL DCI, j=0, Vtemp=2.

If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by DCI format that includes a DAI field, the UE generates the HARQ-ACK codebook as described in Clause 9.1.3.1, with the following modifications: - For the pseudo-code for the HARQ-ACK codebook generation in Clause 9.1.3.1, after the completion of the c and m loops, the UE sets Vtemp2=VDAIUL where VDAIUL is the value of the DAI field according to Table 9.1.3-2

UE will perform the following pseudo-code then,

if end if if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell, OACK=2⋅TD⋅j+Vtemp2-1mod TD+1 else OACK=TD⋅j+Vtemp2-1mod TD+1 end if for any

In this case  TD=2, Vtemp2=3, j=0. According to the pseudo-code, UE will determine the size of HARQ-ACK codebook as OACK=1. UE would sent 1 bit HARQ-ACK codebook but gNB expects 3 bits HARQ-ACK codebook, UE receives 2 DL DCIs but can only send HARQ-ACK for the first DCI.

Solution 1 of this issue is converting Vtemp2 (UL DAI) into 1 bit before comparing with Vtemp (C-DAI), the drawback is that it will cause information loss because the MSB of UL DAI is not used and does not help detecting missed DCIs. There can be up to 1 missed DCI detected by UE with this solution. For example, in the above case, the last two DL DCIs are missed by UE, UE and gNB will still have different understanding of the size of HARQ-ACK codebook. UE would send 1 bit HARQ-ACK codebook but gNB would expect 3 bits HARQ-ACK codebook.

UL DAI is not only used to ensure the size of HARQ-ACK codebook but also to ensure the UL data is not impacted due to DL DCI miss detection. Not using the MSB of UL DAI may cause performance loss for UL data as well and should be thus avoided.

Solution 2 is converting Vtemp into 2 bits before comparing with Vtemp2, there will be no information loss and the MSB of UL DAI can help detecting up to 3 missed DCIs.

Firstly, Vtemp can be converted using the following equation.

Vtemp=j mod4TD×4TD+Vtemp

Since the pseudo-code applies to both HARQ-ACK transmitted on PUSCH and PUCCH. The case where C-DAI is configured as 1 bit and HARQ-ACK is transmitted on PUCCH also needs to be taken into consideration. Following pseudo-code can be used.

if UE does not set Vtemp2=VDAIUL and TD=2 Vtemp2=Vtemp end if

If C-DAI is configured as 1 bit, j also needs to be converted accordingly when C-DAI is converted into 2 bits. j can be converted using the following equation.

j=j×TD4

After converting all the above parameters, 2 bits T-DAI/UL DAI can be used to determine the size of HARQ-ACK codebook using the pseudo-code.

if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell, OACK=2⋅4⋅j+Vtemp2 else OACK=4⋅j+Vtemp2 end if

Back to the former example, UE only receives the first two DL DCI format. In the following, the HARQ-ACK codebook construction procedure is clarified using Solution 2.

When UE receive the first DL DCI, j=0, Vtemp=1. When UE receive the second DL DCI, j=0, Vtemp=2. After converting into 2 bits, j=0, Vtemp=2. When UE receives the UL DCI, Vtemp2=3. UE determines the size of HARQ-ACK codebook as OACK=3. The last bit is NACK for the missed DCI.

When UE only receive the first DL DCI, j=0, Vtemp=1. After converting into 2 bits, j=0, Vtemp=1. When UE receives the UL DCI, Vtemp2=3. UE determines the size of HARQ-ACK codebook as OACK=3. The last two bits are NACKs for the missed DCIs.

Similarly, if UE does not receive any DL DCI format, j=0, Vtemp=0. UE still can generate a 3 bit HARQ-ACK codebook on the PUSCH with 3 NACKs.

Based on the above analysis, we have following proposal,

Proposal: 1 bit C-DAI should be converted into 2 bits when determining HARQ-ACK codebook on a PUSCH scheduled by a DCI format. Adopt the following TP,

## 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel ## *** Unchanged text is omitted *** Set – PDCCH with DCI format scheduling PDSCH reception or SPS PDSCH release monitoring occasion index: lower index corresponds to earlier PDCCH monitoring occasion Set Set Set Set Set to the number of serving cells configured by higher layers for the UE - if, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, and is provided ACKNACKFeedbackMode = JointFeedback, the serving cell is counted two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs - if the UE indicates PDSCH-Number-perMOperCell, a serving cell is counted NPDSCHMO times where NPDSCHMO is the number of PDSCH receptions that can be scheduled for the serving cell by DCI formats in PDCCH receptions at a same PDCCH monitoring occasion based on the reported value of PDSCH-Number-perMOperCell Set to the number of PDCCH monitoring occasion(s) while Set – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell while if PDCCH monitoring occasion is before an active DL BWP change on serving cell or an active UL BWP change on the PCell and an active DL BWP change is not triggered in PDCCH monitoring occasion ; else if there is a PDSCH on serving cell associated with PDCCH in PDCCH monitoring occasion , or there is a PDCCH indicating SPS PDSCH release on serving cell if end if if else end if if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of at least one serving cell, o2⋅TD⋅j+2VC-DAI,c,mDL-1ACK = HARQ-ACK information bit corresponding to the first transport block of this cell o2⋅TD⋅j+2VC-DAI,c,mDL-1+1ACK = HARQ-ACK information bit corresponding to the second transport block of this cell Vs=Vs∪2⋅TD⋅j+2VC-DAI,c,mDL-1, 2⋅TD⋅j+2VC-DAI,c,mDL-1+1 elseif harq-ACK-SpatialBundlingPUCCH is provided to the UE and is a monitoring occasion for PDCCH with a DCI format that supports PDSCH reception with two transport blocks and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks in at least one configured DL BWP of a serving cell, oTD⋅j+VC-DAI,c,mDL-1ACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1 else oTD⋅j+VC-DAI,c,mDL-1ACK = HARQ-ACK information bit of this cell Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1 end if end if end if end while end while Vtemp=j mod4TD×4TD+Vtemp if UE does not set Vtemp2=VDAIUL and TD=2 Vtemp2=Vtemp end if j=j×TD4 if end if if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell, OACK=2⋅TD⋅j+Vtemp2-1mod TD+1 OACK=2⋅4⋅j+Vtemp2 else OACK=TD⋅j+Vtemp2-1mod TD+1 OACK=4⋅j+Vtemp2 end if for any ## *** Unchanged text is omitted ***

# Conclusions

This contribution discussed about remaining issues for SPS PDSCH. Following is the summary of this contribution.

Proposal: 1 bit C-DAI should be converted into 2 bits when determining HARQ-ACK codebook on a PUSCH scheduled by a DCI format. Adopt the following TP,

## 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel ## *** Unchanged text is omitted *** Set – PDCCH with DCI format scheduling PDSCH reception or SPS PDSCH release monitoring occasion index: lower index corresponds to earlier PDCCH monitoring occasion Set Set Set Set Set to the number of serving cells configured by higher layers for the UE - if, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, and is provided ACKNACKFeedbackMode = JointFeedback, the serving cell is counted two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs - if the UE indicates PDSCH-Number-perMOperCell, a serving cell is counted NPDSCHMO times where NPDSCHMO is the number of PDSCH receptions that can be scheduled for the serving cell by DCI formats in PDCCH receptions at a same PDCCH monitoring occasion based on the reported value of PDSCH-Number-perMOperCell Set to the number of PDCCH monitoring occasion(s) while Set – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell while if PDCCH monitoring occasion is before an active DL BWP change on serving cell or an active UL BWP change on the PCell and an active DL BWP change is not triggered in PDCCH monitoring occasion ; else if there is a PDSCH on serving cell associated with PDCCH in PDCCH monitoring occasion , or there is a PDCCH indicating SPS PDSCH release on serving cell if end if if else end if if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of at least one serving cell, o2⋅TD⋅j+2VC-DAI,c,mDL-1ACK = HARQ-ACK information bit corresponding to the first transport block of this cell o2⋅TD⋅j+2VC-DAI,c,mDL-1+1ACK = HARQ-ACK information bit corresponding to the second transport block of this cell Vs=Vs∪2⋅TD⋅j+2VC-DAI,c,mDL-1, 2⋅TD⋅j+2VC-DAI,c,mDL-1+1 elseif harq-ACK-SpatialBundlingPUCCH is provided to the UE and is a monitoring occasion for PDCCH with a DCI format that supports PDSCH reception with two transport blocks and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks in at least one configured DL BWP of a serving cell, oTD⋅j+VC-DAI,c,mDL-1ACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1 else oTD⋅j+VC-DAI,c,mDL-1ACK = HARQ-ACK information bit of this cell Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1 end if end if end if end while end while Vtemp=j mod4TD×4TD+Vtemp if UE does not set Vtemp2=VDAIUL and TD=2 Vtemp2=Vtemp end if j=j×TD4 if end if if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell, OACK=2⋅TD⋅j+Vtemp2-1mod TD+1 OACK=2⋅4⋅j+Vtemp2 else OACK=TD⋅j+Vtemp2-1mod TD+1 OACK=4⋅j+Vtemp2 end if for any ## *** Unchanged text is omitted ***