3GPP TSG RAN WG1 #103-e	        							           R1-2008136

e-Meeting, October 26th – November 11th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Type-2 HARQ-ACK codebook |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The number of bits for the counter DAI can be 1 or 2. The number of bits for UL DAI is fixed as 2. When HARQ-ACK codebook is multiplexed on a PUSCH scheduled by a DCI format, there might be DAI size misalignment issue and as a consequence, UE cannot correctly construct the HARQ-ACK codebook. For example, the number of bits for the counter DAI is 1, a PDSCH corresponds to 1 bit in the HARQ-ACK codebook. gNB sent 3 DL DCI formats scheduling 3 PDSCHs respectively with HARQ-ACK in slot n, UE received the first 2 DL DCI formats and missed the last DL DCI format. UE received a UL DCI format scheduling a PUSCH in slot n with UL DAI =3. According to current spec, Vtemp2=3, j=0, OACK=1. gNB expects 3 bits, UE receives 2 DCIs but can only send HARQ-ACK for the first DCI. To void information loss of UL DAI, one solution is converting Vtemp into 2 bits before comparing with Vtemp2. There will be no information loss and the MSB of UL DAI can help finding up to 3 missed DCIs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Converting 1 or 2 bits C-DAI into 2 bits C-DAI after the completion of the c and m loops, HARQ-ACK codebook size is determined based on 2 bits UL-DAI/T-DAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Type-2 HARQ-ACK codebook may not be correctly constructed according to current spec when the HARQ-ACK codebook is transmited on a PUSCH schduled by a DCI format. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel

## *** Unchanged text is omitted ***

Set  – PDCCH with DCI format scheduling PDSCH reception or SPS PDSCH release monitoring occasion index: lower index corresponds to earlier PDCCH monitoring occasion

Set

Set

Set

Set

Set  to the number of serving cells configured by higher layers for the UE

-	if, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, and is provided ACKNACKFeedbackMode = JointFeedback, the serving cell is counted two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs

-	if the UE indicates PDSCH-Number-perMOperCell, a serving cell is counted NPDSCHMO times where NPDSCHMO is the number of PDSCH receptions that can be scheduled for the serving cell by DCI formats in PDCCH receptions at a same PDCCH monitoring occasion based on the reported value of PDSCH-Number-perMOperCell

Set  to the number of PDCCH monitoring occasion(s)

while

Set  – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell

while

if PDCCH monitoring occasion  is before an active DL BWP change on serving cell  or an active UL BWP change on the PCell and an active DL BWP change is not triggered in PDCCH monitoring occasion

;

else

if there is a PDSCH on serving cell  associated with PDCCH in PDCCH monitoring occasion , or there is a PDCCH indicating SPS PDSCH release on serving cell

if

end if

if

else

end if

if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of at least one serving cell,

o2⋅TD⋅j+2VC-DAI,c,mDL-1ACK = HARQ-ACK information bit corresponding to the first transport block of this cell

o2⋅TD⋅j+2VC-DAI,c,mDL-1+1ACK = HARQ-ACK information bit corresponding to the second transport block of this cell

Vs=Vs∪2⋅TD⋅j+2VC-DAI,c,mDL-1, 2⋅TD⋅j+2VC-DAI,c,mDL-1+1

elseif harq-ACK-SpatialBundlingPUCCH is provided to the UE and  is a monitoring occasion for PDCCH with a DCI format that supports PDSCH reception with two transport blocks and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks in at least one configured DL BWP of a serving cell,

oTD⋅j+VC-DAI,c,mDL-1ACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell

Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1

else

oTD⋅j+VC-DAI,c,mDL-1ACK = HARQ-ACK information bit of this cell

Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1

end if

end if

end if

end while

end while

Vtemp=j mod4TD×4TD+Vtemp

if UE does not set Vtemp2=VDAIUL and TD=2

Vtemp2=Vtemp

end if

j=j×TD4

if

end if

if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell,

OACK=2⋅TD⋅j+Vtemp2-1mod TD+1

OACK=2⋅4⋅j+Vtemp2

else

OACK=TD⋅j+Vtemp2-1mod TD+1

OACK=4⋅j+Vtemp2

end if

for any

## *** Unchanged text is omitted ***