3GPP TSG RAN WG1 Meeting #103-e	                                                                          R1-2008601

Oct 26th – Nov 13th, 2020

Agenda item:    7.2.2

Source:              Qualcomm Incorporated

Title:                  TP for Channel access procedures for NR unlicensed

Document for:  Discussion and Decision

# Introduction

This paper captures the TPs for issues identified for agenda item “Channel access procedures for NR unlicensed”.

# ChannelAccess-CPext-CAPC indication for FBE

In RAN1-101e, we have the following agreement

Agreement:

ChannelAccess-CPext-CAPC and ChannelAccess-CPext fields are applicable for DCI 0_1 and 1_1 respectively for FBE as well, though some combinations may not be valid for FBE and the UE does not expect to be configured with those combinations.

- Discuss the need for related TPs in RAN1#102-e

To simply the change to the current version of the spec, we propose to make these fields available for semi-static channel access as well, while introduce the following special handling:

- UE ignores CAPC value

- UE does not expected to be configured entries with cat 4 LBT

- UE does not expect to be configured with entries with CP extension values other than C2*symbol length – 16 us – TA or 0

These can be captured in 37.213 4.3 and 38.212 7.3.1.1.2.

Additionally, in RAN1 #99, we have:

Agreement:

At least for LBE operation:

- For signaling of LBT type & CP extension for both Fallback DL assignment and Fallback UL Grant, the following table is used:

| LBT Type | CP extension |
| --- | --- |
| Cat1 16 µs | C2*symbol length – 16 us – TA |
| Cat2 25 µs | C3*symbol length – 25 us – TA |
| Cat2 25 µs | C1*symbol length – 25 us |
| Cat4 | 0 |

  - CAPC is not indicated explicitly:

    - For the UL grants

      - The UE assumes CAPC=4 was used by the gNB to acquire the CO,

      - For UE initiated COTs (Cat4 case) the UE may select the CAPC by itself.

        - Note: The mapping between priority classes and traffic classes follows the same mechanism as defined for UL CG transmissions.

    - Note: For PUCCH associated with DL assignments may use the highest priority CAPC when CAT4 LBT is used, as agreed earlier.

- If the network indicates FBE operation, for an indication of LBT type of Cat 2 25 us or Cat4 the UE follows the mechanism whereby one 9 microsecond slot is measured within a 25 microsecond interval as in 37.213.

The reinterpretation of Cat 2 LBT or Cat 4 LBT is not clearly captured in 37.213 4.3.

============TP for 38.212==================================

7.3.1.1.2	Format 0_1

----------Unchanged text omitted-----------------------

-	ChannelAccess-CPext-CAPC – 0, 1, 2, 3, 4, 5 or 6 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ULDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16 for operation in a cell with shared spectrum channel; otherwise 0 bit. One or more entries from Table 7.3.1.1.2-35 are configured by the higher layer parameter ULDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16. When ChannelAccessMode-r16=”semi-static”, UE will ignore the CAPC value indicated by ChannelAccess-CPext-CAPC.

----------Unchanged text omitted-----------------------

=======================================================

============TP for 37.213 4.3==================================

4.3	Channel access procedures for semi-static channel occupancy

Channel assess procedures based on semi-static channel occupancy as described in this Clause, are intended for environments where the absence of other technologies is guaranteed e.g., by level of regulations, private premises policies, etc. If a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic' by SIB1 or dedicated configuration, a periodic channel occupancy can be initiated by the gNB every Tx within every two consecutive radio frames, starting from the even indexed radio frame at i∙Txx∙Tx with a maximum channel occupancy timeTy=0.95TxTy=min0.95Tx, Tx-0.1, where Tx= period Tx=Periodin ms, is a higher layer parameter provided in SemiStaticChannelAccessConfig and i∈0,1,…,20Tx-1.

In the following procedures in this subclause, when a gNB or UE performs sensing for evaluating a channel availability, the sensing is performed at least during a sensing slot duration Tsl=9us. The corresponding  XThresh adjustment for performing sensing by a gNB or a UE is described in subclauses 4.1.5 and 4.2.3, respectively.

A channel occupancy initiated by a gNB and shared with UE(s) shall satisfy the following:

-	The gNB shall transmit a DL transmission burst starting at the beginning of the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration Tsl=9us. If the channel is sensed to be busy, the gNB shall not perform any transmission during the current period.

-	The gNB may transmit a DL transmission burst(s) within the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration Tsl=9us if the gap between the DL transmission burst(s) and any previous transmission burst is more than 16us.

-	The gNB may transmit DL transmission burst(s) after UL transmission burst(s) within the channel occupancy time without sensing the channel if the gap between the DL and UL transmission bursts is at most 16us.

-	A UE may transmit UL transmission burst(s) after detection of a DL transmission burst(s) within the channel occupancy time as follows:

-	16usIf the UL transmission is indicated by DCI format 0_1 or DCI format 1_1 to use Type 2C channel access, the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time without sensing the channel.

-	16us If the UL transmission is indicated by DCI format 0_0 or DCI format 1_0 or RAR UL grant to use Type 1 channel access or Type 2A channel access, or if the UL transmission is indicated by DCI format 1_1 or DCI format 0_1 to use Type 2A channel access, the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time after sensing the channel to be idle for at least a sensing slot duration Tsl=9us  before transmission.

-	The gNB and UEs shall not transmit any transmissions in a set of consecutive symbols for a duration of at least Tz=max0.05Tx , 100us before the start of the next period.

=======================================================