3GPP TSG RAN WG1 Meeting #103-e 	R1-2008799

# E-meeting, October 26 - November 13, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | <CR#> | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on power sharing for LTE DAPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  |  | Date: |  | 2020-10-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As agreed, uplink power sharing is introduced for inter-band DAPS and the power sharing modes as defined for LTE DC are reused for DAPS with capability reporting. However, the description of power sharing for LTE DAPS is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the UE indicates support of power control mode 1 and is provided uplinkPowerSharingDAPS-Mode-r16 = 1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 5.1.4.1 by replacing the MCG with the target MCG and the SCG with the source MCG and the UE assumes only Pcell exists in each MCG. If the UE indicates support for power control mode 2 and is provided uplinkPowerSharingDAPS-Mode-r16 = 2, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 5.1.4.2 by replacing the MCG with the target MCG and the SCG with the source MCG and the UE assumes only Pcell exists in each MCG. If UE does not indicate a capability for power sharing between source and target MCGs in DAPS handover, the UE does not expect the transmissions on the target and source cell overlapped in time domain. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Power sharing is not supported for LTE DAPS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### < Unchanged parts are omitted >

# 17	Dual active protocol stack based handover

If a UE indicates a capability for dual active protocol stack based handover (DAPS HO), the UE can be provided with a source MCG and a target MCG.

If a UE is configured with a target MCG and a source MCG, the UE is indidated the guaranteed power γMCG for transmissions on the target MCG by p-DAPS-Target-r16 and the guaranteed power γSCG for transmissions on the source MCG by p-DAPS-Source-r16 and with an inter-CG power sharing mode by uplinkPowerSharingDAPS-Mode-r16.

If the UE indicates support of power control mode 1 and is provided uplinkPowerSharingDAPS-Mode-r16 = 1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 5.1.4.1 by replacing the MCG with the target MCG and the SCG with the source MCG and the UE assumes only Pcell exists in each MCG.

If the UE indicates support for power control mode 2 and is provided uplinkPowerSharingDAPS-Mode-r16 = 2, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 5.1.4.2 by replacing the MCG with the target MCG and the SCG with the source MCG and the UE assumes only Pcell exists in each MCG.

If UE does not indicate a capability for power sharing between source and target MCGs in DAPS handover, the UE does not expect the transmissions on the target and source cell overlapped in time domain.

### < Unchanged parts are omitted >