3GPP TSG RAN WG1 #104-e	R1-2100072

e-Meeting, January 25th - February 5th, 2021

Title:	Remaining issue on the channel access for FBE

Source: 	ZTE, Sanechips

Agenda Item:	7.2.2

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide our views on LBT type and CP extension for FBE.

# LBT type and CP extension for FBE

In RAN1 #99 meeting, the following agreement on FBE mode had been reached:

Agreement:

At least for LBE operation:

- For signaling of LBT type & CP extension for both Fallback DL assignment and Fallback UL Grant, the following table is used:

| LBT Type | CP extension |
| --- | --- |
| Cat1 16 µs | C2*symbol length – 16 us – TA |
| Cat2 25 µs | C3*symbol length – 25 us – TA |
| Cat2 25 µs | C1*symbol length – 25 us |
| Cat4 | 0 |

  - CAPC is not indicated explicitly:

    - For the UL grants

      - The UE assumes CAPC=4 was used by the gNB to acquire the CO,

      - For UE initiated COTs (Cat4 case) the UE may select the CAPC by itself.

        - Note: The mapping between priority classes and traffic classes follows the same mechanism as defined for UL CG transmissions.

    - Note: For PUCCH associated with DL assignments may use the highest priority CAPC when CAT4 LBT is used, as agreed earlier.

- If the network indicates FBE operation, for an indication of LBT type of Cat 2 25 us or Cat4 the UE follows the mechanism whereby one 9 microsecond slot is measured within a 25 microsecond interval as in 37.213.

In RAN1 #101 e-meeting, the following agreement on FBE was reached:

Agreement:

ChannelAccess-CPext-CAPC and ChannelAccess-CPext fields are applicable for DCI 0_1 and 1_1 respectively for FBE as well, though some combinations may not be valid for FBE and the UE does not expect to be configured with those combinations.

- Discuss the need for related TPs in RAN1#102-e

For the first agreement, the following issues need to further be clarified:

- The above Table with yellow highlight is only applied for LBE or both LBE and FBE.

  - If it is the former, then it is not necessary to explicitly indicate what type of DCI cannot be used for the case of No LBT. If it is the latter,  which DCI can indicate that no LBT and at least a sensing slot cat2 LBT need to be clear reflected in the current spec to avoid ambiguity.

  - For the above two understanding, we tend to support the latter.

- The above mentioned Cat 2 LBT is different with Type 2A specified in 37.213.

- It is allowed for gNB to indicate Cat4 LBT to UE.

Besides, the second agreement has not been captured in the current spec.

In order to reflect the existing conclusions more accurately and minimize the changes to current spec, the following TPs can be considered to clarify/resolve the above mentioned issues:

TP#1 on “interpretation of Type 2A channel access procedure, Type 2C channel access procedure and CAPC for FBE ”

--------------------------------------- < Start of TP#1 for 38.212 v16.4.0 [1]> ----------------------------------------- ##### 7.3.1.1.1 Format 0_0 DCI format 0_0 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: Unchanged Texts Omitted - TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS 38.213] - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for operation in a cell with shared spectrum channel access; 0 bit otherwise. Unchanged Texts Omitted The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: Unchanged Texts Omitted - TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS 38.213] - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for operation in a cell with shared spectrum channel access; 0 bit otherwise Unchanged Texts Omitted Table 7.3.1.1.1-4: Channel access type & CP extension for DCI format 0_0 and DCI format 1_0 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2. 1; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 3. 2; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 1. 3; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0. Notes: If the higher layer parameter ChannelAccessMode-r16 is configured as 'semistatic', then - Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] represents “the UE may transmit the transmission after sensing the channel to be idle for at least a sensing slot duration within a interval ending immediately before transmission” - Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] represents “ without sensing”. ##### 7.3.1.1.2 Format 0_1 DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE. Unchanged Texts Omitted Otherwise, all the remaining fields are set as follows: Unchanged Texts Omitted - ChannelAccess-CPext-CAPC – 0, 1, 2, 3, 4, 5 or 6 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-0-1 for operation in a cell with shared spectrum channel access; otherwise 0 bit. One or more entries from Table 7.3.1.1.2-35 are configured by the higher layer parameter ul-AccessConfigListDCI-0-1. Unchanged Texts Omitted Table 7.3.1.1.2-35: Allowed entries for DCI format 0_1, configured by higher layer parameter ul-AccessConfigListDCI-0-1 Entry index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, 38.211]; CAPC. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0; 1. 1; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0; 2. 2; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0; 3. 3; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0; 4. 4; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2; 1. 5; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2; 2. 6; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2; 3. 7; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2; 4. 8; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0; 1. 9; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0; 2. 10; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0; 3. 11; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0; 4. 12; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2; 1. 13; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2; 2. 14; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2; 3. 15; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2; 4. 16; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 0; 1. 17; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 0; 2. 18; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 0; 3. 19; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 0; 4. 20; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 1; 1. 21; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 1; 2. 22; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 1; 3. 23; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 1; 4. 24; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 3; 1. 25; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 3; 2. 26; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 3; 3. 27; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 3; 4. 28; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0; 1. 29; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0; 2. 30; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0; 3. 31; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0; 4. 32; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 1; 1. 33; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 1; 2. 34; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 1; 3. 35; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 1; 4. 36; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 2; 1. 37; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 2; 2. 38; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 2; 3. 39; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 2; 4. 40; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 3; 1. 41; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 3; 2. 42; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 3; 3. 43; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 3; 4. Notes: If the higher layer parameter ChannelAccessMode-r16 is configured as 'semistatic', then - Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] represents “the UE may transmit the transmission after sensing the channel to be idle for at least a sensing slot duration within a interval ending immediately before transmission” - Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] represents “without sensing” - CAPC corresponding to the configured the Entry index should be ignored.. ##### 7.3.1.2.2 Format 1_1 DCI format 1_1 is used for the scheduling of PDSCH in one cell. The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: Unchanged Texts Omitted - ChannelAccess-CPext – 0, 1, 2, 3 or 4 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-1-1 for operation in a cell with shared spectrum channel access; otherwise 0 bit. One or more entries from Table 7.3.1.2.2-6 are configured by the higher layer parameter ul-AccessConfigListDCI-1-1. Table 7.3.1.2.2-6: Allowed entries for DCI format 1_1, configured by higher layer parameter ul-AccessConfigListDCI-1-1 Entry index; Channel Access Type; The CP extension Text index defined in Clause 5.3.1 of [4, TS 38.211]. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0. 1; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2. 2; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0. 3; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2. 4; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 0. 5; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 1. 6; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 3. 7; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0. 8; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 1. 9; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 2. 10; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 3. Notes: If the higher layer parameter ChannelAccessMode-r16 is configured as 'semistatic', then - Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] represents “the UE may transmit the transmission after sensing the channel to be idle for at least a sensing slot duration within a interval ending immediately before transmission” - Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] represents “without sensing”. -------------------------------------------------- < End of text proposal> -----------------------------------------------------

TP#2 for capturing the agreement of RAN1 #101 e-meeting and clarification on how to understand Table of agreement reached in RAN1 #99 meeting. However, a question to be further clarified for this TP is whether configured grant UE can identify the length of gap between the ending of DL and the starting of UL.

--------------------------------------- < Start of TP#2 for 37.213 v16.4.0 [2]> ----------------------------------------- ## 4.3 Channel access procedures for semi-static channel occupancy Channel assess procedures based on semi-static channel occupancy as described in this Clause, are intended for environments where the absence of other technologies is guaranteed e.g., by level of regulations, private premises policies, etc. If a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic' by SIB1 or dedicated configuration, a periodic channel occupancy can be initiated by the gNB every within every two consecutive radio frames, starting from the even indexed radio frame at with a maximum channel occupancy time , where period in , is a higher layer parameter provided in SemiStaticChannelAccessConfig and . In the following procedures in this clause, when a gNB or UE performs sensing for evaluating a channel availability, the sensing is performed at least during a sensing slot duration . The corresponding adjustment for performing sensing by a gNB or a UE is described in clauses 4.1.5 and 4.2.3, respectively. A channel occupancy initiated by a gNB and shared with UE(s) shall satisfy the following: - The gNB shall transmit a DL transmission burst starting at the beginning of the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration . If the channel is sensed to be busy, the gNB shall not perform any transmission during the current period. - The gNB may transmit a DL transmission burst(s) within the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration if the gap between the DL transmission burst(s) and any previous transmission burst is more than . - The gNB may transmit DL transmission burst(s) after UL transmission burst(s) within the channel occupancy time without sensing the channel if the gap between the DL and UL transmission bursts is at most - A UE may transmit UL transmission burst(s) after detection of a DL transmission burst(s) within the channel occupancy time as follows: - If the gap between the UL and DL transmission bursts is at most or if the UE is indicated by DCI format 0_1 or DCI format 1_1 to use Type 2C channel access procedures, the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time without sensing the channel. - If the gap between the UL and DL transmission bursts is more than or if the UE is indicated by DCI format 0_0 or DCI format 1_0 or RAR UL grant to use Type 1 channel access or Type 2A channel access, or if the UE is indicated by DCI format 1_1 or DCI format 0_1 to use Type 2A channel access procedures, the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time after sensing the channel to be idle for at least a sensing slot duration within a interval ending immediately before transmission. - The gNB and UEs shall not transmit any transmissions in a set of consecutive symbols for a duration of at least before the start of the next period. If a UE fails to access the channel(s) prior to an intended UL transmission to a gNB, Layer 1 notifies higher layers about the channel access failure. -------------------------------------------------- < End of text proposal> -----------------------------------------------------

# Conclusion

In this contribution, we present our proposal below and TPs on LBT type and CAPC for FBE.

Proposal 1: Adopt TP#1 for the corresponding change in 38.212 v16.4.0.

Proposal 2: Adopt TP#2 for the corresponding change in 37.213 v16.4.0.

Proposal 3: For TP#2, it is necessary to further clarify whether configured grant UE can identify the length of gap between the ending of DL and the starting of UL.

# Reference

- 38.212 g40, NR Multiplexing and channel coding (Release 16)

- 37.213 g40, NR; Physical layer procedures for shared spectrum channel access(Release 16)