3GPP TSG RAN WG1 #104-e			R1-2100948

e-Meeting, January 25th – February 5th, 2021

Source:	NEC

Title:	UE feedback enhancements for HARQ-ACK

Agenda Item:	8.3.1.1

Document for: 	Discussion and Decision

- Introduction

In RAN1 #103-e meeting, following agreements related to HARQ-ACK enhancement for Rel-17 URLLC/IIoT were achieved [1]:

Agreements: To address the issue of SPS HARQ-ACK dropping for TDD systems, focus on the following two options: - Option 1: Deferring HARQ-ACK until a next (e.g., first) available PUCCH - FFS: Details including the definition of a next (e.g, first) available PUCCH, CB construction / multiplexing - Option 2: Dynamic triggering of a one-shot / Type-3 CB type of re-transmission - FFS: Details on triggering and/or CB construction (incl. potential Type-3 CB optimizations) / multiplexing Decision: As per email decision posted on Nov.13th, Agreement: In the studies on PUCCH carrier switching for HARQ-ACK, PUCCH carrier switching for different cells operated is considered only for cells that are part of the active UL CA configuration. Agreements: For the studies on SPS HARQ skipping for skipped SPS PDSCH, the further discussions should focus on the following reduced sets methods: - ‘NACK skipping’ for (skipped) SPS PDSCH (Alt. 1) - FFS: details including at least when to skip the HARQ-ACK as well as NACK skipping configuration details (per SPS or group of SPS configurations etc.) - Note: this alternative assumes inherently no identification of a skipped SPS PDSCH by the UE - Dynamic indication of skipped SPS PDSCH occasions (Alt. 3) - FFS: details including dynamic indication methods such as e.g. DCI, MAC CE, specific DM-RS instead of SPS DM-RS, … Agreements: For the studies on SPS HARQ payload size reduction (of non-skipped SPS PDSCH), the further discussions should focus on the following reduced sets of methods: - ACK skipping (NACK-only) (Alt. 1) - FFS: Details - NACK skipping (ACK-only) (Alt. 2) - FFS: Details - HARQ bundling / compression (Alt. 3) - FFS: Details including HARQ bundling / compression window, bundling / compression technique - HARQ-ACK disabling /skipping for certain SPS configurations (Alt. 4) - The skipping / disabling is higher-layer configured per SPS configuration - FFS: HARQ-ACK skipping behaviour for Type 1 CB

In this contribution, we firstly discuss the enhancement for Type-1 HARQ codebook based on sub-slot PUCCH configuration. Then we discuss the enhancement for HARQ-ACK feedback for SPS PDSCH. At last, we discuss the PUCCH carrier switching for HARQ-ACK.

- Enhancement on Type-1 HARQ-ACK codebook based on sub-slot PUCCH configuration

For a Rel.16 URLLC UE, sub-slot based HARQ-ACK feedback is supported to reduce the latency. Regarding the HARQ-ACK codebook construction, Type-2 HARQ-ACK codebook for sub-slot-based HARQ-ACK feedback procedure has been supported in Rel-16. For Type-1 HARQ-ACK codebook, it was agreed in 102-e meeting to discuss whether and how to support Type-1 HARQ-ACK codebook based on sub-slot PUCCH configuration in Rel-17. In our view, compared with Type-2 HARQ-ACK codebook, Type-1 HARQ-ACK codebook can provide robustness against missed DCI transmissions, which is beneficial for high reliable URLLC services. Some companies may have the concerns on the redundancy of the semi-static HARQ-ACK codebook, which will lead to high UCI overhead. However, the redundancy of URLLC Type-1 HARQ-ACK codebook can be controlled by network with suitable configuration for TDRA table and HARQ-ACK timing K1 set. Therefore, Tyep-1 HARQ-ACK codebook based on sub-slot PUCCH configuration should be supported in Rel-17.

In order to further reduce the HARQ-ACK codebook size, small enhancements on Type-1 HARQ-ACK codebook construction based on sub-slot PUCCH configuration can be considered. When the numerology configured for DL and UL is same, one simple option for sub-slot based Type-1 HARQ-ACK codebook determination is as following:

- Step 1: Determine the HARQ-ACK multiplexing window based on the HARQ-ACK timing set and sub-slot length.

Fig.1 Example for step 1 with K1={1,2,3} and 7-symbol sub-slot length

- Step 2: Split the TDRA table into N sub-tables based on the sub-slot length and PDSCH-to-UL sub-slot association. N is the number of UL sub-slot within a slot. Which means that the SLIV in the TDRA table is associated with which sub-table depends on the ending symbol of the SLIV is located in which sub-slot. E.g., the first sub-TDRA table is corresponding to the first sub-slot within a slot, the second sub-TDRA table is corresponding to the second sub-slot within a slot.

Fig.2 Example for TDRA table splitting for 7-symbol sub-slot

- Step 3: Do pruning based on TDD configuration and sub-table per sub-slot similar as Rel-15.  E.g., if the sub-slot for HARQ-ACK timing set K1= {1, 2, 3} in the above given example are all DL symbols, the set of PDSCH occasions M is {0, 1, 2, 3, 4}.

Regarding the Type-1 HARQ-ACK codebook construction based on sub-slot PUCCH configuration when DL and UL are configured with different numerologies, following two options can be considered:

- Option 1: Determine the virtual DL sub-slot based on UL sub-slot configuration. The number of symbols for the virtual DL sub-slot is the same as the number of symbols for the UL sub-slot. Then the number of the virtual DL sub-slots within a DL slot is the same as number of the UL sub-slots within a UL slot. Therefore, the Type-1 HARQ-ACK codebook construction based sub-slot PUCCH configuration for different numerologies configuration between DL and UL can follow current slot based Type-1 HARQ-ACK codebook construction mechanism for different numerologies configuration between DL and UL. Which is achieved by replacing the DL slot, the UL slot and TDRA table with the virtual DL sub-slot, the UL sub-slot and the corresponding sub-TDRA table, the corresponding pseudo code is shown in the appendix. For example, when UL sub-slot length is configured as 7-symbol, the number of UL-sub slots within a UL slot and the number of virtual DL sub-slots within a DL slot is 2. The subcarrier spacing configurations for DL and UL are 60 KHz and 15 KHz, respectively. As shown in the Fig.3 below, for a given UL sub-slot, 4 virtual DL sub-slots are determined for k1=1, then for nD=1:2μDL-μUL=1:4, the UE repeats the sub-slot based candidate PDSCH reception occasions determination process for k1=1 as above step 3.

Fig.3 Example for Type-1 HARQ-ACK codebook construction based on sub-slot PUCCH configuration for different numerologies configuration between DL and UL

- Option 2:  In the last meeting, another option was proposed in [2] for  Type-1 HARQ-ACK codebook construction based on sub-slot PUCCH configuration for same or different numerologies configuration for DL and UL:

  - Step 1: Determine the HARQ-ACK multiplexing window based on the HARQ-ACK timing set and sub-slot length.

  - Step 2: If a UL sub-slot in the HARQ-ACK window spans multiple DL slots, create a new TDRA table which is the union of the configured TDRA table and the configured TDRA table offset by 14 symbols.

  - Step 3: Split the TDRA table into N sub-tables based on the sub-slot length and PDSCH-to UL sub-slot association. A PDSCH TDRA is associated with a UL sub-slot if the end of the PDSCH overlaps with the UL sub-slot. N is the number of sub-slots within a slot.

  - Step 4: Do pruning based on TDD configuration and sub-table per sub-slot similar as Rel-15.

From our point of understanding, when DL and UL are configured with different numerologies, option 1 can reuse the current slot based Type-1 HARQ-ACK codebook construction mechanism by replacing the slot with sub-slot, so option 1 seems to have less specification impact. For option 2, it may be simpler since it doesn’t need to introduce the virtual DL sub-slot. In our view, both two options are acceptable, it can be further discussed whether there are other methods and which method is better.

Proposal 1:

- Semi-static HARQ-ACK codebook for sub-slot based HARQ-ACK feedback procedure should be supported in Rel-17.

Proposal 2:

- When DL and UL are configured with same numerology, the sub-slot based semi-static HARQ-ACK codebook can be determined based on following three-steps:

  - Step 1: Determine the HARQ-ACK multiplexing window based on the HARQ-ACK timing set and sub-slot length.

  - Step 2: Split the TDRA table into N sub-tables based on the sub-slot length and PDSCH-to UL sub-slot association. N is the number of sub-slots within a slot.

  - Step 3: Do pruning based on TDD configuration and sub-table per sub-slot similar as Rel-15.

- When DL and UL are configured with different numerologies, further study the sub-slot based semi-static HARQ-ACK codebook determination.

- Enhancement on HARQ-ACK feedback for SPS PDSCH

3.1 Support avoiding SPS HARQ-ACK dropping for TDD

In the last meeting, to address the issue of SPS HARQ-ACK dropping for TDD systems, it was agreed to focus on the following two options:

- Option 1: Deferring HARQ-ACK until a next (e.g., first) available PUCCH

  - FFS: Details including the definition of a next (e.g, first) available PUCCH, CB construction / multiplexing

- Option 2: Dynamic triggering of a one-shot / Type-3 CB type of re-transmission

  - FFS: Details on triggering and/or CB construction (incl. potential Type-3 CB optimizations) / multiplexing

Option 1 is a semi-static method for addressing SPS HARQ-ACK dropping for TDD systems, option 2 is a dynamic method, which is controlled by gNB. In our views, different options can be applied for different use cases. For example, when the earliest available PUCCH resource for delayed SPS HARQ-ACK defined in option 1 doesn’t satisfy the latency requirement, gNB can dynamically trigger option 2 to provide a PUCCH resource for SPS HARQ-ACK. Therefore, both two options can be supported in Rel-17, which option is applied can be configured by gNB.

Deferring HARQ-ACK until a next available PUCCH

For option 1, the definition of the first/next available PUCCH is the earliest PUCCH resource not overlapped with invalid symbol among next PUCCH resources configured for SPS PDSCH and indicated for dynamic scheduled PDSCH. An example is given in Fig.4. For simplicity and avoid the impact of the PDCCH miss detection, the invalid symbol means DL symbol and flexible symbol. If the determined first available PUCCH for delayed SPS PDSCH HARQ-ACK is a dynamically scheduled PUCCH resource, support multiplexing the delayed SPS PDSCH HARQ-ACK and HARQ-ACK for dynamically scheduled PDSCH(s) on a HARQ-ACK codebook. Regarding the HARQ-ACK codebook construction, a simple method is placing the delayed SPS PDSCH HARQ-ACK bit(s) after the HARQ-ACK codebook of dynamically scheduled PDSCH(s). When multiple SPS PDSCH HARQ-ACK bits are delayed to same PUCCH resource, the SPS PDSCH HARQ-ACK bits are ordered as Rel-16.

Fig.4 Example for the first/next available PUCCH resource for the delayed SPS PDSCH HARQ-ACK

In addition, some companies may have the concerns on that option 1 may incur imbalanced HARQ-ACK feedback. There are some simple solutions. For example, configuring a value M for UE controls the maximum number of HARQ-ACK bits for a configured PUCCH resource.  If the number of HARQ-ACK bits carried in a PUCCH resource exceeds the value M, then the PUCCH resource becomes invalid for the delayed HARQ-ACK, UE will find next available PUCCH resource. An example is given in the Fig.5 below, the k1 value for the SPS PDSCH is 2, the M value for the PUCCH is 4, then the first 4-bits HARQ-ACK and the second 4-bits HARQ-ACK are separately transmitted in the PUCCH resource in slot n+8 and the PUCCH resource in slot n+9.

Fig.5 Example for option 1 configured with M

Dynamic triggering of a one-shot / Type-3 HARQ-ACK codebook type of re-transmission

In Rel-16, when one-shot HARQ-ACK request is received by UE, a Type-3 HARQ-ACK codebook containing the all DL HARQ processes for all CCs configured for a UE in the PUCCH group will be generated. For option 2, if directly reuse the Rel-16 one-shot HARQ-ACK feedback mechanism for SPS PDSCH, it will cause large redundancy. To reduce the Type-3 HARQ-ACK codebook size, following enhanced solutions can be considered:

- Alt.1: gNB can trigger the one-shot HARQ-ACK feedback for the SPS PDSCH only by a DL DCI with or without scheduling PDSCH. The requested HARQ-ACK codebook contains the number of all DL HARQ processes for all the configured/activated SPS configuration(s) in the configured CC(s).

- Alt.2: gNB can trigger the one-shot HARQ-ACK feedback for the SPS PDSCH only by a DL DCI without scheduling PDSCH. The requested HARQ-ACK codebook contains only the number of DL HARQ processes for the indicated SPS configuration(s) in the configured CC(s). HARQ-ACK for which SPS configuration will be reported can be indicated by reusing HPN field in the request DCI.

- Alt.3: gNB can trigger the one-shot HARQ-ACK feedback for the SPS PDSCH only by a DL DCI with or without scheduling PDSCH. The requested HARQ-ACK codebook contains a set of DL HARQ processes for the configured/activated SPS configuration(s) in the configured CC(s). The association between the set of the DL HARQ processes and the configured/activated SPS configuration is configured for UE by RRC.

In Rel-16, PUCCH resource set SPS-PUCCH-AN-List-r16 is provided for UE for SPS PDSCH only, while if support the requested Type-3 HARQ-ACK codebook only containing HARQ-ACK for SPS PDSCH only, whether UE determines the PUCCH resource for the Type-3 HARQ-ACK codebook based on UCI payload from SPS-PUCCH-AN-List-r16  or based on PRI indication in the requested DCI from PUCCH-ResourceSet for DG PDSCH may need to be further studied.

Proposal 3:

- In case PUCCH resource for SPS HARQ-ACK is not available due to collision with DL symbol or flexible symbol, support deferring HARQ-ACK for SPS PDSCH until a next available PUCCH

  - The next available PUCCH is the earliest PUCCH resource not overlapped with invalid symbol among next PUCCH resources configured for SPS PDSCH and indicated for dynamically scheduled PDSCH.

    - If the number of HARQ-ACK bits carried in a configured PUCCH resource exceeds the value M, then the PUCCH resource is not valid for the delayed HARQ-ACK.

  - Support multiplexing the delayed HARQ-ACK for SPS PDSCH and HARQ-ACK for dynamically scheduled PDSCH(s) on a HARQ-ACK codebook.

    - Append the delayed HARQ-ACK bits for SPS PDSCH after the HARQ-ACK codebook for dynamically scheduled PDSCH(s).

Proposal 4:

- Support dynamic triggering of a one-shot / Type-3 CB type of HARQ-ACK re-transmission for SPS PDSCH due to collision between PUCCH resource and invalid symbol.

  - Following options can be considered to reduce the Type-3 HARQ-ACK codebook size:

    - Alt.1: The requested HARQ-ACK codebook contains the number of all DL HARQ processes for all the configured/activated SPS configuration(s) in the configured CC(s).

    - Alt.2: The requested HARQ-ACK codebook contains only the number of DL HARQ processes for the indicated SPS configuration(s) in the configured CC(s).

    - Alt.3: The requested HARQ-ACK codebook contains a set of DL HARQ processes for the configured/activated SPS configuration(s) in the configured CC(s).

  - FFS the PUCCH resource determination for the triggered Type-3 HARQ-ACK codebook for SPS PDSCH only.

3.2 SPS HARQ skipping for skipped SPS PDSCH

In Rel-16, multiple SPS configurations with short SPS periodicity in a cell is supported to reduce the latency. For TSC/TSN traffic with non-integer periodicities, multiple SPS configurations maybe configured for one service, some SPS PDSCH occasions may be skipped and not used for DL data transmission. For such case, in order to avoid the unnecessary PUCCH transmission for skipped SPS PDSCH, skipping NACK for skipped SPS PDSCH can be supported in Rel-17. In the last meeting, following two methods were agreed to be further studied.

NACK skipping for (skipped) SPS PDSCH

UE does not need to identify whether the SPS PDSCH is skipped SPS PDSCH (without DL data) or failed to be decoded. UE only skips PUCCH transmission when all HARQ-ACK bits multiplexed in the PUCCH for SPS PDSCH only are NACK. It can reduce the PUCCH transmission for UE and the specification impact is small. Therefore, support NACK skipping for skipped SPS PDSCH in Rel-17.

Dynamic indication of skipped SPS PDSCH occasions

Whether UE skips NACK for SPS PDSCH or not depends on the skipped SPS PDSCH occasion indication by gNB, e.g., by DCI or MAC CE. From our point of understanding, compared with skipping PUCCH transmission if all HARQ-ACK bits in the PUCCH resource for SPS PDSCH only are NACK, it can reduce the UCI payload when HARQ-ACK for the indicated skipped SPS PDSCH and HARQ-ACK for other PDSCH (e.g., dynamically scheduled PDSCH) are indicated to be reported in a same slot. However, it will introduce additional signalling overhead, and the benefits are not obvious when Type-1 HARQ-ACK codebook is configured.  Considering that the benefits are not clear, we suggest that whether support dynamic indication of skipped SPS PDSCH occasions in Rel-17 can be further studied.

Proposal 5:

- Support skipping PUCCH transmission if all HARQ-ACK bits in the PUCCH resource for SPS PDSCH only are NACK.

Proposal 6：

- Further study whether support dynamic indication of skipped SPS PDSCH occasions in Rel-17.

- PUCCH carrier switching for HARQ-ACK feedback

In NR Rel-16, UE can only transmit PUCCH on a cell (Pcell or PUCCH-Scell) within a PUCCH group. Under such restriction, for a URLLC UE, due to there is no early available PUCCH resource in the Pcell or PUCCH-Scell for HARQ-ACK transmission in a given TDD configuration, the latency requirement may not be met in some cases, as shown in Fig.6. Supporting multiple cells for PUCCH transmission for HARQ-ACK when CA is configured can help to reduce the HARQ-ACK feedback latency.

Fig. 6 Example for PUCCH transmission only allowed on a cell

Fig. 7 Example for PUCCH carrier switching for HARQ-ACK

If multiple cells for PUCCH transmission for HARQ-ACK is supported, one key issue that needs to be addressed is how to indicate the cell for PUCCH transmission, following two directions were proposed by companies in the last meeting [3][4]:

- Option 1: Dynamic indication, e.g., by DCI with a new indicator field.

- Option 2: Semi-static indication, e.g., UE determines the cell for PUCCH transmission based on a predefined rule and TDD configuration. Considering that the multiple cells for PUCCH transmission may be configured with different numerologies, the predefined rule below can be considered.

  - Firstly, UE determines a slot for HARQ-ACK feedback transmission based on the HARQ-ACK timing k1 indication with a reference numerology. The reference numerology determination can be down-selected from the two alternatives:

    - Alt.1: The reference numerology is the numerology with the largest 𝜇 value among the 𝜇 values of the configured multiple PUCCH cells.

    - Alt.2: The reference numerology is the numerology of the PUCCH cell with highest priority.

  - Then UE determines a cell in order of priority from highest to lowest with enough available symbols (not collide with DL symbol or DL reception) within the corresponding determined slot for PUCCH transmission. If Alt.2 is applied, when the 𝜇 value of determined cell is larger than the 𝜇 value of the reference numerology, then more than one slot on the cell may be determined based on the k1 value, UE determines the first available slot for PUCCH transmission.

Fig. 8 Example for the first available slot determination of alt.2

Option 1 is simple and flexible, but it will increase the DCI overhead.  Such flexibility of option 1 seems to be unnecessary for PUCCH carrier indication, the semi-static indication method, i.e., option 2 is sufficient and doesn’t need to introduce additional signalling overhead. Therefore, we slightly prefer semi-static indication for PUCCH carrier switching.

As pointed out by other company [5], another open issue related to PUCCH carrier switching is power control of PUCCH. For example, it should be further studied that whether separate or joint closed loop power control for PUCCH transmissions on multiple cells is operated and how to apply the TPC command value in group common DCI format 2_2 for accumulative power control for PUCCH.

Proposal 7：

- Support semi-static indication for PUCCH carrier switching for HARQ-ACK in Rel-17.

Proposal 8：

- Further study the PUCCH power control if PUCCH carrier switching for HARQ-ACK is supported.

- Enhancement on sub-slot based Type-1 HARQ-ACK codebook by the addition of DAI counters

An enhancement to reduce the size of Type-1 HARQ-ACK codebook is proposed as following:

The sub-slot based Type-1 HARQ-ACK codebook ACK/NACK bits will only be present if the corresponding slot or sub-slot has at least one PDCCH transmission, and the reliability of Type-1 HARQ-ACK codebook for URLLC service can be further enhanced by the reuse of DAI counters.

In the following example, a Type-1 HARQ-ACK codebook has a multiplexing window size of 4 slots in total. It is also configured to have 7 sub-slots per slot, with 1 PDCCH monitoring occasion for each sub-slot.

Let’s say a correctly received Rel-16 Type-1 HARQ-ACK codebook is composed of these bits:

ACK/NACK bits = 1000100 0000000 0000000 1000000

With the proposed enhancement, the type 1 slot-based HARQ-ACK codebook will feedback as following:

ACK/NACK bits = 1000100 1000000

The number of bits for type 1 sub-slot based HARQ-ACK codebook can be further reduced if there are 7 sub-slots (2 symbols per sub-slot) per slot:

ACK/NACK bits = 11 1

For the addition of C-DAI counters, the existing C-DAI field can be simply reused:

slot 0	(C-DAI=0, C-DAI=1)

slot 3	(C-DAI=2)

To further improve reliability against missed detection, the existing T-DAI field, which is not currently present for URLLC cases, may be present but redefined:

New Definition of T-DAI for URLLC

The value of the T-DAI, when configured as ‘present’, in DCI format 1_1 (or DCI format 1_2) denotes the total number of slots in which PDCCH/PDSCH transmission(s) associated with DCI format 1_1 (or DCI format 1_2) occur(s), up to the current slot, in increasing order of slot index.

Let x be the number of PDCCH monitoring occasions per slot or per sub-slot. x bits feedback is always present for each value pair of C-DAI/T-DAI (for URLLC) in the enhanced type 1 codebook.

With the proposed enhancement, the following C-DAI/T-DAI settings will be applied in DCI with corresponding HARQ-ACK feedback (x=7 in this case):

ACK/NACK bits = 1000100 1000000

slot 0	(C-DAI=0, T-DAI=0; C-DAI=1, T-DAI=0)

slot 3	(C-DAI=2, T-DAI=1)

| URLLC DCI for DL assignment | Rel-15/Rel-16 DCI | Rel-17 DCI | Comment |
| --- | --- | --- | --- |
| Total Downlink Assignment Index | 0 or 2 | 0 or 2 | 0 or 2 bits can be configured for this field. In Rel-17, 0 bit if it is codebook-less; in Rel-15/Rel-16, 0 bit if there is no CA. |

Table 1: T-DAI field is enhanced for URLLC

Proposal 9:

- The Type-1 HARQ-ACK codebook ACK/NACK bits will only be present if the corresponding slot or sub-slot has at least one PDCCH transmission, and the reliability of Type-1 HARQ-ACK codebook for URLLC service can be further enhanced by the reuse of DAI counters.

- Conclusion

In this contribution, we discussed the possible enhancements for HARQ-ACK feedback for URLLC. Proposals are summarized as following:

Proposal 1:

- Semi-static HARQ-ACK codebook for sub-slot based HARQ-ACK feedback procedure should be supported in Rel-17.

Proposal 2:

- When DL and UL are configured with same numerology, the sub-slot based semi-static HARQ-ACK codebook can be determined based on following three-steps:

  - Step 1: Determine the HARQ-ACK multiplexing window based on the HARQ-ACK timing set and sub-slot length.

  - Step 2: Split the TDRA table into N sub-tables based on the sub-slot length and PDSCH-to UL sub-slot association. N is the number of sub-slots within a slot.

  - Step 3: Do pruning based on TDD configuration and sub-table per sub-slot similar as Rel-15.

- When DL and UL are configured with different numerologies, further study the sub-slot based semi-static HARQ-ACK codebook determination.

Proposal 3:

- In case PUCCH resource for SPS HARQ-ACK is not available due to collision with DL symbol or flexible symbol, support deferring HARQ-ACK for SPS PDSCH until a next available PUCCH

  - The next available PUCCH is the earliest PUCCH resource not overlapped with invalid symbol among next PUCCH resources configured for SPS PDSCH and indicated for dynamic scheduled PDSCH.

    - If the number of HARQ-ACK bits carried in a configured PUCCH resource exceeds the value M, then the PUCCH resource is not valid for the delayed HARQ-ACK.

  - Support multiplexing the delayed HARQ-ACK for SPS PDSCH and HARQ-ACK for dynamically scheduled PDSCH(s) on a HARQ-ACK codebook.

    - Append the delayed HARQ-ACK bits for SPS PDSCH after the HARQ-ACK codebook for dynamically scheduled PDSCH(s).

Proposal 4:

- Support dynamic triggering of a one-shot / Type-3 CB type of HARQ-ACK re-transmission for SPS PDSCH due to collision between PUCCH resource and invalid symbol.

  - Following options can be considered to reduce the Type-3 HARQ-ACK codebook size:

    - Alt.1: The requested HARQ-ACK codebook contains the number of all DL HARQ processes for all the configured/activated SPS configuration(s) in the activated CC(s).

    - Alt.2: The requested HARQ-ACK codebook contains only the number of DL HARQ processes for the indicated SPS configuration(s) in the activated CC(s).

    - Alt.3: The requested HARQ-ACK codebook contains a set of DL HARQ processes for the configured/activated SPS configuration(s) in the activated CC(s).

  - FFS the PUCCH resource determination for the triggered Type-3 HARQ-ACK codebook for SPS PDSCH only.

Proposal 5:

- Support skipping PUCCH transmission if all HARQ-ACK bits in the PUCCH resource for SPS PDSCH only are NACK.

Proposal 6：

- Further study whether support dynamic indication of skipped SPS PDSCH occasions in Rel-17.

Proposal 7：

- Support semi-static indication for PUCCH carrier switching for HARQ-ACK in Rel-17.

Proposal 8：

- Further study the PUCCH power control if PUCCH carrier switching for HARQ-ACK is supported.

Proposal 9:

- The Type-1 HARQ-ACK codebook ACK/NACK bits will only be present if the corresponding slot or sub-slot has at least one PDCCH transmission, and the reliability of Type-1 HARQ-ACK codebook for URLLC service can be further enhanced by the reuse of DAI counters.

References

- 3GPP RAN1 NR #103-e meeting, chairman’s notes.

- R1-2009789, ‘Moderator summary on Rel-17 HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT (AI 8.3.1.1),’ Nokia, Nokia Shanghai Bell.

- R1-2009063, ‘On UE feedback enhancements for HARQ-ACK,’ MediaTek Inc.

- R1- 2009257, ‘HARQ-ACK enhancement for IOT and URLLC,’ Qualcomm Incorporated.

- R1-2008842, ‘HARQ-ACK Feedback Enhancements for URLLC/IIoT,’ Nokia, Nokia Shanghai Bell.

Appendix

TP for Type-1 HARQ-ACK codebook determination based on sub-slot PUCCH configuration

9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel ## *** Unchanged text is omitted *** Set N to the number of UL sub-slots in a slot. In the following pseudo code, slot of a PUCCH means sub-slot of a PUCCH and slot of a PDSCH means sub-slot of a PDSCH if sub-slot based type 1 CB is configured. Set - index of occasion for candidate PDSCH reception or SPS PDSCH release Set Set Set to the cardinality of set Set k =0 – index of slot timing values , in descending order of the slot timing values, in set for serving cell Set Ri=∅, where i=0,…N-1 Set R to the set of rows while R≠∅ Set m to a last OFDM symbol index, as determined by the SLIV, of any row of R with row index r Rm/Nslotsymb∙N=r R=R\r end while If a UE is not provided ca-SlotOffset for any serving cell of PDSCH receptions and for the serving cell of corresponding PUCCH transmission with HARQ-ACK information. while if Set – index of a DL slot within an UL slot while Set to Rmod(nU-K1,k⋅2μDL-μUL+nD,N) Set to the cardinality of Set – index of row in set if slot starts at a same time as or after a slot for an active DL BWP change on serving cell or an active UL BWP change on the PCell and slot nU-K1,k⋅2μDL-μUL+nD is before the slot for the active DL BWP change on serving cell or the active UL BWP change on the PCell nD=nD+1; else while if the UE is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot N∙(nU-K1,k⋅2μDL-μUL+i) where i=nD-NPDSCHrepeat,max+1,…,nD,nU-K1,k⋅2μDL-μUL+nD-NPDSCHrepeat,max+1nU-K1,k⋅2μDL-μUL+nD at least one symbol of the PDSCH time resource derived by row is configured as UL where is the k-th slot timing value in set , ; else ; end if end while if the UE does not indicate a capability to receive more than one unicast PDSCH per slot and , ; ; else Set to the cardinality of Set to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of while Set while if for start OFDM symbol index for row ; - index of occasion for candidate PDSCH reception or SPS PDSCH release associated with row ; ; else ; end if end while ; Set to the smallest last OFDM symbol index among all rows of; end while end if nD=nD+1; end if end while end if ; end while else … end if