## Solutions for NR to support non-terrestrial networks (NTN)

Please refer to RP-202908 for detailed scope of the WI

R1-2100832	Reply LS on SA WG2 assumptions AN-PDB and PER targets for satellite access	THALES

### Timing relationship enhancements

R1-2100156	Discusson on timing relationship enhancement	OPPO

R1-2100222	Discussion on timing relationship enhancements for NTN	Huawei, HiSilicon

R1-2100244	Discussion on timing relationship for NR-NTN	ZTE

R1-2100304	Timing relationship enhancements to support NTN	CAICT

R1-2100381	Timing relationship enhancement for NTN	CATT

R1-2100441	Discussion on timing relationship enhancements for NR-NTN	vivo

R1-2100496	Timing relationship enhancements for NTN	Zhejiang Lab

R1-2100594	Timing relationship enhancements for NR-NTN	MediaTek Inc.

R1-2100654	On timing relationship enhancements for NR NTN	Intel Corporation

R1-2100703	Discussions on timing relationship enhancements in NTN	LG Electronics

R1-2100757	Discussion on NTN timing relationship	Lenovo, Motorola Mobility

R1-2100807	Consideration on timing relationship enhancements	Spreadtrum Communications

R1-2100859	Calculation of timing relationship offsets	Sony

R1-2100912	Discussion on Timing Relationship Enhancements in NR-NTN	China Telecom

R1-2100922	On timing relationship enhancements for NTN	Ericsson

R1-2100971	Timing relationship enhancements in NTN	Asia Pacific Telecom, FGI, III, ITRI

R1-2100984	On timing relationship for NTN	InterDigital, Inc.

R1-2101024	NTN Timing relationship enhancement	Panasonic Corporation

R1-2101042	Discussion on timing relationship enhancements for NTN	CMCC

R1-2101117	Discussion on the timing relationship enhancement for NTN	Xiaomi

R1-2101206	Timing relationship enhancements for NTN	Samsung

R1-2101296	Further views on DL-UL timing relationship for NTN operation	Nokia, Nokia Shanghai Bell

R1-2101383	Enhancement on Timing Relationship in NTN	Apple

R1-2101464	Enhancements on Timing Relationship for NTN	Qualcomm Incorporated

R1-2101616	Discussion on timing relationship enhancements for NTN	NTT DOCOMO, INC.

R1-2101694	Discussion on Timing Relationship Enhancements for NTN	Fraunhofer IIS, Fraunhofer HHI

[104-e-NR-NTN-01] Email discussion/approval on timing relationship enhancements with checkpoints for agreements on Jan-28, Feb-02, Feb-05 – Xingqin (Ericsson)

R1-2101793	Feature lead summary#1 on timing relationship enhancements	Moderator (Ericsson)

R1-2101904	Feature lead summary#2 on timing relationship enhancements	Moderator (Ericsson)

R1-2102019	Feature lead summary#3 on timing relationship enhancements	Moderator (Ericsson)

Agreement:

Confirm the following working assumption:

K_offset can be applied to indicate the first transmission opportunity of PUSCH in Configured Grant Type 2 in the same way as K_offset is applied to the transmission timing of DCI scheduled PUSCH.

Agreement:

Update of K_offset after initial access is supported

Agreement:

For unpaired spectrum, extend the value range of K1 from (0..15) to (0..31)

FFS: Whether there is an impact on the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

Working assumption:

Introduce K_offset to enhance the adjustment of uplink transmission timing upon the reception of a corresponding timing advance command.

### Enhancements on UL time and frequency synchronization

R1-2100157	Discussion on UL time and frequency synchronization	OPPO

R1-2100223	Discussion on UL time and frequency synchronization enhancement for NTN	Huawei, HiSilicon

R1-2100245	Discussion on UL synchronization for NR-NTN	ZTE

R1-2100305	Considerations on Enhancements on UL Time Synchronization in NTN	CAICT

R1-2100382	UL time and frequency compensation for NTN	CATT

R1-2100442	Discussion on UL time and frequency synchronization  enhancements for NR-NTN	vivo

R1-2100520	Considerations on UL timing and frequency synchronization in NR NTN	THALES

R1-2100540	UL time synchronization acquisition for NTN	Mitsubishi Electric RCE

R1-2100595	UE Time and frequency Synchronisation for NR-NTN	MediaTek Inc.

R1-2100655	On UL synchronization for NR NTN	Intel Corporation

R1-2100704	Discussions on UL time and frequency synchronization enhancements in NTN	LG Electronics

R1-2100758	Discussion on NTN uplink time synchronization	Lenovo, Motorola Mobility

R1-2100808	Consideration on enhancements on UL time and frequency synchronization	Spreadtrum Communications

R1-2100860	Enhancement for UL time synchronization	Sony

R1-2100927	On UL time and frequency synchronization enhancements for NTN	Ericsson

R1-2100972	UL time and frequency synchronization in NTN	Asia Pacific Telecom, FGI

R1-2100985	On UL time/frequency synchronization for NTN	InterDigital, Inc.

R1-2101043	Enhancements on UL time and frequency synchronization for NTN	CMCC

R1-2101079	Discussion on UL timing synchronization for NTN	ETRI

R1-2101118	Discussion on UL time and frequency synchronization for NTN	Xiaomi

R1-2101207	Enhancements on UL time and frequency synchronization for NTN	Samsung

R1-2101297	Time and frequency synchronization for NTN systems	Nokia, Nokia Shanghai Bell

R1-2101384	Discussion on Uplink Time and Frequency Synchronization for NTN	Apple

R1-2101465	UL time and frequency synchronization for NTN	Qualcomm Incorporated

R1-2101648	Discussion on UL time and frequency synchronization for NTN	PANASONIC R&D Center Germany

R1-2101716	UL time synchronization methods for NTN systems	CEWiT,IITM,IITH,Tejas Networks, Withdrawn

R1-2101717	UL time synchronization methods for NTN systems	CEWiT,IITM,IITH,Tejas Networks,Reliance Jio

[104-e-NR-NTN-02] Email discussion/approval on UL time and frequency synchronization with checkpoints for agreements on Jan-28, Feb-02, Feb-05 – Mohamed (Thales)

R1-2101871	FL Summary on enhancements on UL time and frequency synchronization for NR NTN	Moderator (Thales)

R1-2102028	Feature lead summary #4 on enhancements on UL timing and frequency synchronization				Moderator (Thales)

R1-2102140	FL Summary#5 on enhancements on UL time and frequency synchronization for NR NTN			Moderator (Thales)

R1-2102182	FL Summary on enhancements on UL time and frequency synchronization for NR NTN	Moderator (Thales)

Agreement:

An NTN UE in RRC_CONNECTED state is required to support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris.

FFS: Operation of closed loop and open loop TA control

Agreement:

For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for NTN.

FFS: Details of the combination of open and closed loop TA control

Conclusion:

It is up to RAN4 to decide whether interruptions or measurement gaps are required for GNSS measurements during NTN operation

Agreement:

RAN1 should send an LS to RAN4 with the following questions:

Question 1: RAN1 would like to ask RAN4, to indicate what are the NTN UL time synchronization requirements?

- For initial access (i.e. PRACH transmission)

- For UL transmissions in RRC Connected State

Question 2: RAN1 would like to ask RAN4, to indicate what are the NTN UL frequency synchronization requirements?

- For initial access (i.e. PRACH transmission)

- For UL transmissions in RRC Connected State

Conclusion:

If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary.

- FFS: support of DL frequency compensation for the service link Doppler.

Agreement:

- RAN1 to support satellite ephemeris broadcast based at least on one of the following format options:

  - Option 1: Ephemeris format based on satellite position and velocity state vectors

    - FFS: Details on state vectors formats

    - FFS: Details on time reference provisioning/format

  - Option 2: Ephemeris format based on orbital elements

    - FFS: Details on orbital elements formats

    - FFS: Details on time reference provisioning/format

- FFS: Whether down-selection is needed or both options are supported

### Enhancements on HARQ

R1-2100158	Discussion on HARQ enhancement	OPPO

R1-2100224	Discussion on HARQ enhancement for NTN	Huawei, HiSilicon

R1-2100246	Discussion on HARQ for NR-NTN	ZTE

R1-2100306	Enhancements on HARQ to support NTN	CAICT

R1-2100383	HARQ operation enhancement for NTN	CATT

R1-2100443	Discussion on HARQ enhancements for NR-NTN	vivo

R1-2100485	INTELLIGENT PACKET REPETITION IN MOBILE SATELLITE SERVICE (MSS) LINKS TO OVERCOME CHANNEL BLOCKAGES	Ligado Networks

Late submission

R1-2100491	Discussion on HARQ enhancement for NTN	BUPT

R1-2100596	HARQ in NR-NTN	MediaTek Inc.

R1-2100656	On HARQ enhancements for NR NTN	Intel Corporation

R1-2100705	Discussions on HARQ enhancements in NTN	LG Electronics

R1-2100759	Enhancements on HARQ for NTN	Lenovo, Motorola Mobility

R1-2100809	Consideration on enhancements on HARQ	Spreadtrum Communications

R1-2100861	Enhancements on HARQ for NTN	Sony

R1-2100913	Discussion on HARQ Enhancements in NR-NTN	China Telecom

R1-2100928	On HARQ enhancements for NTN	Ericsson

R1-2100973	Enhancements on HARQ in NTN	Asia Pacific Telecom, FGI

R1-2100986	On HARQ enhancement for NTN	InterDigital, Inc.

R1-2101025	HARQ enhancement for NTN 	Panasonic Corporation

R1-2101044	Enhancements on HARQ for NTN	CMCC

R1-2101080	Discussion on HARQ Enhancements for NTN	ETRI

R1-2101119	Discussion on the HARQ enhancement for NTN	Xiaomi

R1-2101208	Enhancements on HARQ for NTN	Samsung

R1-2101289	Discussion on HARQ for NTN	THALES

R1-2101298	On HARQ operation for NTN systems	Nokia, Nokia Shanghai Bell

R1-2101385	Discussion on HARQ Enhancements for NTN	Apple

R1-2101466	Enhancements on HARQ for NTN	Qualcomm Incorporated

[104-e-NR-NTN-03] Email discussion/approval on HARQ enhancements with checkpoints for agreements on Jan-28, Feb-02, Feb-05 – Nan (ZTE)

R1-2101781	Summary of AI 8.4.3 for HARQ for NTN	Moderator (ZTE)

R1-2101903	Summary of AI 8.4.3 for HARQ in NTN	Moderator (ZTE)

R1-2102055	Summary#3 of AI 8.4.3 for HARQ in NTN		Moderator (ZTE)

R1-2102143	Summary#4 of AI 8.4.3 for HARQ in NTN		Moderator (ZTE)

Agreement:

For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until X after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process.

- Working assumption: X = T_proc,1

- FFS: Whether X should be changed to X = max(T_proc,1, K1) where K1 is the minimum k1 if it is configured, otherwise k1 = 0

- Note: The TB of the two PDSCHs can be either same or different

Agreement:

For Type-2 HARQ codebook in NTN: Reduce codebook size with HARQ-ACK codebook only including HARQ-ACK of PDSCH with feedback-enabled HARQ processes

- FFS: The details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes

### Others

Including the list of topics to specify if beneficial and needed

R1-2100598	Summary #1 of 8.4.4 Other Aspects of NR-NTN	MediaTek Inc.

R1-2100159	Discussion on other aspects	OPPO

R1-2100247	Discussion on additional enhancement for NR-NTN	ZTE

R1-2100264	IoT NTN Observations and Proposals	Lockheed Martin

R1-2100307	Discusson on beam management of NTN	CAICT

R1-2100384	Beam management and other aspects for NTN	CATT

R1-2100444	Discussion on other aspects for NR-NTN	vivo

R1-2100597	Other Aspects of  NR-NTN	MediaTek Inc.

R1-2100706	Discussions on other aspects of NTN	LG Electronics

R1-2100723	Discussion on channel models for marine and inland river scenes	GDCNI

Late submission

R1-2100760	Discussion on other aspects for NTN	Lenovo, Motorola Mobility

R1-2100862	Discussion on beam management and polarization for NTN	Sony

R1-2100929	On other enhancements for NTN	Ericsson

R1-2100974	Scrambling sequence enhancement in NTN	Asia Pacific Telecom, FGI, ITRI

R1-2100987	On feeder link switch and beam management for NTN	InterDigital, Inc.

R1-2101045	Other Aspects for NTN	CMCC

R1-2101120	Discussion on other design aspects for NTN	Xiaomi

R1-2101209	Remaining issues for NTN	Samsung

R1-2101260	Discussion on other design aspects for NTN	Huawei, HiSilicon

R1-2101288	Beam management and BWP operation in NTN	THALES

R1-2101295	SSB, beam management and polarization signaling for NTN 	PANASONIC

R1-2101299	Additional aspects for NR over NTN	Nokia, Nokia Shanghai Bell

R1-2101386	Other Aspects of NR NTN	Apple

R1-2101467	BWP operation and other issues for NTN	Qualcomm Incorporated

R1-2101693	Discussion on Beam Management for NTN	Fraunhofer IIS, Fraunhofer HHI

[104-e-NR-NTN-04] Email discussion/approval on other enhancements including list of topics to specify if beneficial and needed with checkpoints for agreements on Jan-28, Feb-02, Feb-05 – Hao (Oppo)

R1-2101901	Summary #1 of 8.4.4 Other Aspects of NR-NTN	Moderator (Oppo)

R1-2102033	Summary #2 of 8.4.4 Other Aspects of NR-NTN	Moderator (OPPO)

R1-2102141	Summary #3 of 8.4.4 Other Aspects of NR-NTN	Moderator (OPPO)

Agreement:

Support at least explicit indication of polarization information for DL by the network

- FFS: whether the indication is done by SIB, other RRC signaling, DCI.

- FFS: Whether separate signaling is needed for the UL and if so, whether or not a same polarization is indicated for DL and UL

Conclusion:

Discuss whether or not at least following issues are valid and decide whether or not enhancements are needed in addition to current NR specification for supporting NTN beam management:

- Issue 1: NR BWP is not directly associated with a beam. Thus, when using TCI to change beam from beam 1 to beam 2, it does not trigger NR BWP switching. However, in NTN FRF>1 case, beam switching may result in a BWP switching.

- Issue 2: NR BWP switching in UL and DL are not jointly triggered for FDD. However, in NTN FRF>1 FDD scenario, beam switching may result in a BWP switching in both DL and UL.

- Issue 3: NR dynamic BWP switching requires data scheduling. While in NTN FRF>1 scenario, we may need a fast BWP switching triggering without data scheduling.

- Issue 4: NR BWP switching does not require re-synchronization. However, in NTN FRF>1 scenario, when a satellite beam switching is triggered, UE may need to perform re-synchronization in the switched BWP.

- Issue 5: Since satellite beam switching can be frequent and often highly predictable, mechanisms of configured BWP switching (can be a sequence of BWPs) may be preferred but current NR does not allow it.

- Issue 6: How to deal with BWP switching triggered by bwpInactivityTimer, RA procedure, or simply a need to increase throughput instead of for beam-level mobility.

- Issue 7: NR BWP switching/beam switching is done with UE specific signalling due to UE movement’s. However, in NTN scenario, a satellite BWP/beam switching is common for set of UEs, we may need to a common BWP/beam switching mechanism to save the signalling overhead.

Conclusion:

Discuss the necessity of reporting UE polarization capability considering at least following aspects,

- Deployment scenarios.

- UE implementation aspects with respect to polarization.

- Satellite implementation aspects for switching between polarization states.

- Satellite implementation aspects for realizing multiplexing of UEs having different polarization capabilities.

[104-e-NR-NTN-05] Email discussion/approval of the reply LS to R1-2100014 until Feb-01 – Xiaofeng (Qualcomm)

R1-2101952	[Draft] Reply LS on AN-PDB and PER targets for satellite access	Qualcomm

R1-2102074	Reply LS on AN-PDB and PER targets for satellite access	RAN1, Qualcomm