3GPP TSG RAN WG1 Meeting #104-e

e-Meeting, January 25th – February 5th, 2021

Title:	Session notes for 8.16 (Introduction of DL 1024QAM for NR FR1)

Source:	Ad-hoc chair (Samsung)

## Introduction of DL 1024QAM for NR FR1

Please refer to RP-202886 for detailed scope of the RAN4-led WI, particularly the details to specify high order modulation for PDSCH.

R1-2100215	On support of DL 1024QAM for NR FR1	Huawei, HiSilicon

R1-2100369	DL 1024QAM for NR FR1	CATT

R1-2100484	On supporting DL 1024QAM for NR FR1	vivo

R1-2100532	Discussion on DL 1024QAM for NR FR1	ZTE, Sanechips

R1-2100686	Support of 1024QAM	Intel Corporation

R1-2101071	Discussion on DL 1024QAM for NR FR1	CMCC

R1-2101246	On remaining issues of DL 1024QAM for NR FR1	Samsung

R1-2101421	Support for NR DL 1024 QAM in FR1	Nokia, Nokia Shanghai Bell

R1-2101496	1024-QAM for NR PDSCH	Qualcomm Incorporated

R1-2101564	1024QAM for NR DL	Ericsson

[104-e-NR-1024QAM-01] Email discussion/approval for DL 1024QAM – Ajit (Ericsson)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

1823

Working Assumption (to be confirmed next week)

Reuse LTE CQI table with 1024-QAM entries

- Note: for CQI index 14, the SE value (from LTE) is slightly updated from 8.3321 to 8.3301

- Has 2 CQI entries for 1024-QAM

| CQI index | modulation | code rate x 1024 | Efficiency |
| --- | --- | --- | --- |
| 0 | Out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.377 |
| 3 | QPSK | 449 | 0.877 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3301 |
| 15 | 1024QAM | 948 | 9.2578 |