3GPP TSG RAN WG1 Meeting #104-e

e-Meeting, January 25th – February 5th, 2021

Title:	Session notes for 8.16 (Introduction of DL 1024QAM for NR FR1)

Source:	Ad-hoc chair (Samsung)

## Introduction of DL 1024QAM for NR FR1

Please refer to RP-202886 for detailed scope of the RAN4-led WI, particularly the details to specify high order modulation for PDSCH.

R1-2100215	On support of DL 1024QAM for NR FR1	Huawei, HiSilicon

R1-2100369	DL 1024QAM for NR FR1	CATT

R1-2100484	On supporting DL 1024QAM for NR FR1	vivo

R1-2100532	Discussion on DL 1024QAM for NR FR1	ZTE, Sanechips

R1-2100686	Support of 1024QAM	Intel Corporation

R1-2101071	Discussion on DL 1024QAM for NR FR1	CMCC

R1-2101246	On remaining issues of DL 1024QAM for NR FR1	Samsung

R1-2101421	Support for NR DL 1024 QAM in FR1	Nokia, Nokia Shanghai Bell

R1-2101496	1024-QAM for NR PDSCH	Qualcomm Incorporated

R1-2101564	1024QAM for NR DL	Ericsson

[104-e-NR-1024QAM-01] Email discussion/approval for DL 1024QAM – Ajit (Ericsson)

- 1st check point: Jan 27

- 2nd check point: Feb 1

- 3rd check point: Feb 5

R1-2101823	Initial summary for introduction of 1024QAM for NR DL	Moderator (Ericsson)

Agreement

Reuse LTE CQI table with 1024-QAM entries

- Note: for CQI index 14, the SE value (from LTE) is slightly updated from 8.3321 to 8.3301

- Has 2 CQI entries for 1024-QAM

| CQI index | modulation | code rate x 1024 | Efficiency |
| --- | --- | --- | --- |
| 0 | Out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.377 |
| 3 | QPSK | 449 | 0.877 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3301 |
| 15 | 1024QAM | 948 | 9.2578 |

R1-2101892	Summary for introduction of 1024QAM for NR DL	Moderator (Ericsson)

Agreement

For supporting 1024-QAM in NR downlink, adopt a five-bit MCS table with 1024-QAM entries:

- Remove M explicit MCS entries (from MCS indices 0-27) from the NR 256QAM MCS table and add M new entries for 1024QAM

- M=5

- Add one implicit MCS entry corresponding to 1024-QAM

- Add M-1 explicit MCS entries corresponding to 1024-QAM as follows:

| modulation | code rate x 1024 | Efficiency |
| --- | --- | --- |
| 10 | 805.5 | 7.8662 |
| 10 | 853 | 8.3301 |
| 10 | 900.5 | 8.7939 |
| 10 | 948 | 9.2578 |

R1-2101959	Summary#2 for introduction of 1024QAM for NR	Moderator (Ericsson)

Agreement

For supporting 1024-QAM in NR downlink, remove following M (=5) MCS entries from the NR 256-QAM MCS table

- 2, 4, 6, 8, 10

Agreement

Introduce separate RRC signaling to indicate use of 1024-QAM MCS table for DCI format 1_2.

Agreement

The text proposals in R1-2102023 are endorsed for TS38.214.

R1-2102023	First Set of Agreed TPs for support of 1024QAM in NR DL	Moderator (Ericsson)

2059

Agreement

LS to RAN2 and RAN4 to inform them of RAN1 decisions on 1024QAM and related RRC parameters, CQI/MCS tables is endorsed in R1-210XXXX.

Agreement

The following text proposals in R1-210XXXX are endorsed for TS38.214.

- Proposal 7 v3 with TP

- Proposal 9 with TP

- Proposal 10 with TP