3GPP TSG RAN WG1 Meeting #104-e	R1-21xxxxx

e-Meeting, January 25 – February 05, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | xxxx | rev | - | Current version: | V16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PUR correction on PUSCH Repetition Adjustment and Zero Padding Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Sierra Wireless) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5 -Core |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Specification is not clear that when a UE is configured to decode MPDCCH with CRC scrambled by PUR-RNTI, the target DCI size is based on DCI format 6-0A/6-0B and that zero padding is based on DCI format 6-1A/6-1B. Specification is not clear that the meaning of the DCI field ‘PUSCH repetiion adjustment’ refers to indices n0…n8 in Table 8-2b and indices n0…n4 in Table 8-2c of TS 36.213. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the DCI size and zero padding are based DCI format 6-1A/6-1B when a UE is configured to decode MPDCCH with CRC scrambled by PUR-RNTI. Added text to indicate DCI field ‘PUSCH repetiion adjustment’ refers to indices n0…n4 in Table 8-2b and n0…n8 in Table 8-2c of TS 36.213. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear zero padding procedures when a UE is configured to decode MPDCCH with CRC scrambled by PUR-RNTI Unclear definition of DCI field ‘PUSCH Repetition Adjustment’. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1.10 , 5.3.3.1.11, 5.3.3.1.12 , 5.3.3.1.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.3.3.1.10	Format 6-0A

<Unchanged parts are omitted>

If format 6-0A CRC is scrambled by PUR-RNTI and Resource block assignment is set to all ones, the remaining fields are set as follows:

-	ACK or Fallback indicator – 1 bit, where value 0 indicates ACK and value 1 indicates fallback as defined in clause 9.1.5.3 of [3]

-	PUSCH repetition adjustment – 2 bits refers to indices n1, n2, …, n4 in Table 8-2b of [3]

-	Timing advance adjustment – 6 bits as defined in clause 4.2.3 of [3]. The field is only present if ACK or Fallback indicator is set to 0.

-	All the remaining bits in format 6-0A are set to zero

<Unchanged parts are omitted>

##### 5.3.3.1.11	Format 6-0B

<Unchanged parts are omitted>

If format 6-0B CRC is scrambled by PUR-RNTI and Resource block assignment is set to all ones for sub-PRB resource allocation or Modulation and coding scheme is set to all ones for not sub-PRB resource allocation, the remaining fields are set as follows:

-	ACK or Fallback indicator – 1 bit, where value 0 indicates ACK and value 1 indicates fallback as defined in clause 9.1.5.3 of [3]

-	PUSCH repetition adjustment – 3 bits refers to indices n1, n2, …, n8 in Table 8-2c of [3]

-	Timing advance adjustment – 6 bits as defined in clause 4.2.3 of [3]. The field is only present if ACK or Fallback indicator is set to 0.

-	All the remaining bits in format 6-0B are set to zero

<Unchanged parts are omitted>

##### 5.3.3.1.12	Format 6-1A

<Unchanged parts are omitted>

If the UE is not configured to decode MPDCCH with CRC scrambled by the C-RNTI or PUR-RNTI, and the format 6-1A CRC is not scrambled with a G-RNTI, and the number of information bits in format 6-1A is less than that of format 6-0A, zeros shall be appended to format 6-1A until the payload size equals that of format 6-0A.

If the UE is configured to decode MPDCCH with CRC scrambled by the C-RNTI or PUR-RNTI, and the format 6-1A CRC is not scrambled with a G-RNTI, and the number of information bits in format 6-1A mapped onto a given search space is less than that of format 6-0A for scheduling the same serving cell and mapped onto the same search space, zeros shall be appended to format 6-1A until the payload size equals that of format 6-0A.

<Unchanged parts are omitted>

##### 5.3.3.1.13	Format 6-1B

<Unchanged parts are omitted>

If the UE is not configured to decode MPDCCH with CRC scrambled by the C-RNTI or PUR-RNTI, and the format 6-1B CRC is not scrambled with a G-RNTI, and the number of information bits in format 6-1B is less than that of format 6-0B, zeros shall be appended to format 6-1B until the payload size equals that of format 6-0B.

If the UE is configured to decode MPDCCH with CRC scrambled by the C-RNTI or PUR-RNTI, and the format 6-1B CRC is not scrambled with a G-RNTI, and the number of information bits in format 6-1B mapped onto a given search space is less than that of format 6-0B for scheduling the same serving cell and mapped onto the same search space, zeros shall be appended to format 6-1B until the payload size equals that of format 6-0B.

<Unchanged parts are omitted>