3GPP TSG-RAN WG1 Meeting #104-e	R1-21xxxxx

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	6.2.1

Source:	Moderator (Ericsson)

Title:	FL summary for Multi-TB issues for Rel-16 LTE-MTC

Document for:	Discussion, Decision

# 1	Introduction

This document provides a summary of the following RAN1 email discussion.

[104-e-LTE-eMTC5-02] Multi-TB issues – Johan (Ericsson) - Issue #1: Clarification of DCI definition for SPS validation (R1-2100561) - Issue #2: Clarification of multicast scheduling gap definition (R1-2100761, R1-2101279) - Discussion and decision by 1/29, TPs by 2/5

# Issue #1: Clarification of DCI definition for SPS validation

Contribution [1] discusses the need for clarification of the DCI definition for SPS validation for the case when the Rel-16 LTE-MTC multi-TB scheduling feature is configured and presents a TP for 36.213.

Question: Companies are invited to comment below on the 36.213 TP in [1] for clarification of the DCI definition for SPS validation when multi-TB scheduling is configured.

| Company | Comments |
| --- | --- |

# Issue #2: Clarification of multicast scheduling gap definition

Contributions [2] and [3] discuss clarification of the definition of scheduling gaps for Rel-16 LTE-MTC multi-TB scheduling for multicast SC-PTM transmission and present three alternative TPs for 36.213. Two of the TPs assume that the scheduling gap should be in terms of BL/CE BL subframes, and the third TP assumes that the scheduling gap should be in terms of absolute subframes. The TPs also address the indentation issue discussed in the previous RAN1 meeting [4].

Question: Should the scheduling gap for multi-TB multicast transmission be in terms of BL/CE DL subframes or absolute subframes?

| Company | Comments |
| --- | --- |

# References

- R1-2100561, “Corrections on scheduling enhancement for MTC”, ZTE

- R1-2100761, “Corrections on multicast gap in Multiple TB”, Lenovo, Motorola Mobility

- R1-2101279, “Corrections on multi-TB scheduling for eMTC”, Huawei, HiSilicon

- R1-2009295, “FL summary for Multi-TB issues for Rel-16 LTE-MTC”, Moderator (Ericsson)