3GPP TSG RAN WG1 Meeting #104-e	  R1-20xxxxx

E-meeting, January 25 – February 5, 2021

Agenda Item:	7.2.5

Source:	Moderator (Huawei)

Title:	Summary #1 of email discussion [104-e-NR-L1enh-URLLC-01] on remaining issues on PDCCH enhancements

Document for:	Discussion and Decision

Introduction

The email discussion is to discuss the remaining issues on PDCCH enhancements.

[104-e-NR-L1enh-URLLC-01] Email discussion/approval on remaining issues on PDCCH enhancements – Chengyan (Huawei) by Feb 3

- Issue A-1: Correction on dci-FormatsExt in section 10.1 in TS 38.213

- Issue A-5: PDSCH resource mapping with RE symbol level granularity

This document summarizes the above issue and provide some initial proposals for discussion. Companies are encouraged to provide the first round views by 01/26, 8:00pm PST, then we can adjust the proposals and prepare the TPs for the next step discussions.

Issue A-1: Correction on dci-FormatsExt in section 10.1 in TS 38.213

## Issue A-1: Correction on dci-FormatsExt in section 10.1 in TS 38.213

Sharp R1-2101535 In Rel-16, DCI format 0_2 and DCI format 1_2 have been introduced for Rel-16 URLLC operation. A dci-FormatsExt is used to indicate whether a UE to monitor the DCI format 0_2 and DCI format 1_2 in the USS. To be more specific, in TS 38.331 [1] as below, the dci-FormatsExt is used to indicate whether a UE to monitor PDCCH candidates for the DCI format 0_2 and DCI format 1_2, or for the DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2 in a USS. dci-FormatsExt If this field is present, the field dci-Formats is ignored and dci-FormatsExt is used instead to indicate whether the UE monitors in this USS for DCI format 0_2 and 1_2 or formats 0_1 and 1_1 and 0_2 and 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 10.1).. On the other hand, in TS 38.213 [2] as below, it seems that the dci-FormatsExt can be also used to indicate a UE to monitor PDCCH candidates for the DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, which are not allowed in the TS 38.331. TS 38.213 V16.4.0 (2020-12) ## 10.1 UE procedure for determining physical downlink control channel assignment - if search space set s is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or an indication by dci-FormatsExt to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or for DCI format 0_2 and DCI format 1_2, or, if a UE indicates a corresponding capability, for DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1. Proposal: Adopt the following TP relating to dci-FormatsExt in TS 38.213 to keep insistent with the description of the dci-FormatsExt in TS 38.331. TP ### TS 38.213 V16.4.0 (2020-12) ## 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > - if search space set s is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or an indication by dci-FormatsExt to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or for DCI format 0_2 and DCI format 1_2, or, if a UE indicates a corresponding capability, for DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1 ## < Unchanged parts are omitted >.

Feature lead view: The issue is valid and needs to be addressed.

Proposal A-1: Endorse the text proposal in R1-2xxxxxx for TS 38.213 Section 10.1.

## 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > - if search space set s is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or an indication by dci-FormatsExt to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or for DCI format 0_2 and DCI format 1_2, or, if a UE indicates a corresponding capability, for DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1 < Unchanged parts are omitted >

Please provide your views on the above proposal A-1.

| Company | View |
| --- | --- |

Issue A-5: PDSCH resource mapping with RE symbol level granularity

- Issue A-5: PDSCH resource mapping with RE symbol level granularity

## Sharp (R1-2101536) In Rel-16 URLLC, a new higher layer parameter aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 had been introduced to configure aperiodic zero-power CSI-RS resource sets for DCI format 1_2. A general statement on the top of the subclause 5.1.4.2 of TS 38.214 [1], ‘The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList.’, is used to specify a PDSCH rate matching procedure with the aperiodic ZP CSI-RS resource sets for the PDSCH scheduled by DCI format 1_2. In the meantime, TS 38.214 only states that the REs corresponding to configured resources in aperiodic-ZP-CSI-RS-ResourceSetsToAddModList are available for PDSCH scheduled by DCI format 1_0 as below highlighted in yellow. However, it cannot be inferred from the current TS 38.214 whether the REs corresponding to configured resources in aperiodic-ZP-CSI-RS-ResourceSetsToAddModListForDCI-Format1-2-r16 are available for the PDSCH scheduled by DCI format 1_0. In addition, even taking the general statement into consideration, it is also not clear that, for a given PDSCH scheduled by DCI format 1_0, whether REs corresponding to configured resources in aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 are available for the PDSCH given the general statement only described the rate matching procedures for PDSCH scheduled by DCI format 1_1 apply to PDSCH scheduled by DCI format 1_2. Observation: For a PDSCH scheduled by DCI format 1_0, TS 38.214 describes that the REs corresponding to configured resources in aperiodic-ZP-CSI-RS-ResourceSetsToAddModList are available for the PDSCH, while TS 38.214 does not describe whether REs corresponding to configured resources in aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 are available for the PDSCH. Simply following the principle of Rel-15 rate matching behavior with aperiodic ZP CSI-RS that the UE does not rate match PDSCH around the overlapped aperiodic ZP CSI-RS resource by a DL DCI other than the one which scheduled this PDSCH, TS 38.214 should also describe that, for a PDSCH scheduled by the DCI format 1_0, the REs corresponding to configured resources in aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 are available for the PDSCH. Proposal: Adopt the following TP in TS 38.214 to describe that, for PDSCH scheduled by DCI format 1_0 or PDSCHs with SPS activated by DCI format 1_0, the REs corresponding to configured resources in aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 are available for the PDSCH, just as the REs corresponding to configured resources in aperiodicZP-CSI-RS-ResourceSetsToAddModList are available for the PDSCH. TP ### TS 38.214 V16.4.0 (2020-12) #### 5.1.4.2 PDSCH resource mapping with RE level granularity < Unchanged parts are omitted > The UE may be configured with a DCI field for triggering the aperiodic ZP CSI-RS. A list of ZP-CSI-RS-ResourceSet(s), provided by higher layer parameter aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config, is configured for aperiodic triggering. The maximum number of aperiodic ZP-CSI-RS-ResourceSet(s) configured per BWP is 3. The bit-length of DCI field ZP CSI-RS trigger depends on the number of aperiodic ZP-CSI-RS-ResourceSet(s)configured (up to 2 bits). Each non-zero codepoint of 'ZP CSI-RS' trigger in DCI format 1_1 triggers one aperiodic 'ZP-CSI-RS-ResourceSet' in the list aperiodic-ZP-CSI-RS-ResourceSetsToAddModList by indicating the aperiodic ZP CSI-RS resource set ID. The DCI codepoint '01' triggers the resource set with 'ZP-CSI-RS-ResourceSetId' set to '1', the DCI codepoint '10' triggers the resource set with 'ZP-CSI-RS-ResourceSetId' set to '2', and the DCI codepoint '11' triggers the resource set with 'ZP-CSI-RS-ResourceSetId' set to '3'. Codepoint '00' is reserved for not triggering aperiodic ZP CSI-RS. When receiving PDSCH scheduled by DCI format 1_0 or PDSCHs with SPS activated by DCI format 1_0, the REs corresponding to configured resources in aperiodic-ZP-CSI-RS-ResourceSetsToAddModList or in aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 are available for PDSCH. < Unchanged parts are omitted >.

Feature lead view: The issue is valid and needs to be discussed.

Proposal A-5: Endorse the text proposal in R1-2xxxxxx for TS 38.214 Section 5.1.4.2.

#### 5.1.4.2 PDSCH resource mapping with RE level granularity < Unchanged parts are omitted > The UE may be configured with a DCI field for triggering the aperiodic ZP CSI-RS. A list of ZP-CSI-RS-ResourceSet(s), provided by higher layer parameter aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config, is configured for aperiodic triggering. The maximum number of aperiodic ZP-CSI-RS-ResourceSet(s) configured per BWP is 3. The bit-length of DCI field ZP CSI-RS trigger depends on the number of aperiodic ZP-CSI-RS-ResourceSet(s)configured (up to 2 bits). Each non-zero codepoint of 'ZP CSI-RS' trigger in DCI format 1_1 triggers one aperiodic 'ZP-CSI-RS-ResourceSet' in the list aperiodic-ZP-CSI-RS-ResourceSetsToAddModList by indicating the aperiodic ZP CSI-RS resource set ID. The DCI codepoint '01' triggers the resource set with 'ZP-CSI-RS-ResourceSetId' set to '1', the DCI codepoint '10' triggers the resource set with 'ZP-CSI-RS-ResourceSetId' set to '2', and the DCI codepoint '11' triggers the resource set with 'ZP-CSI-RS-ResourceSetId' set to '3'. Codepoint '00' is reserved for not triggering aperiodic ZP CSI-RS. When receiving PDSCH scheduled by DCI format 1_0 or PDSCHs with SPS activated by DCI format 1_0, the REs corresponding to configured resources in aperiodic-ZP-CSI-RS-ResourceSetsToAddModList or in aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 are available for PDSCH. < Unchanged parts are omitted >

Please provide your views on the above proposal A-5.

| Company | View |
| --- | --- |

- References

- R1-2101535	Correction on inconsistence between TS 38.213 and TS 38.331 in terms of the dci-FormatsExt for NR URLLC  Sharp

- R1-2101177	Remaining issues on PDCCH as PDSCH SLIV reference Samsung

- R1-2101262	Corrections on PDCCH enhancements	Huawei, HiSilicon

- R1-2100792	Remaining issues of PDCCH enhancements for URLLC	Spreadtrum Communications

- R1-2101536	Remaining issue on PDSCH rate matching for DCI format 1_0 for NR URLLC	Sharp