3GPP TSG-RAN WG1 Meeting #103-e	R1- 21NNNN

e-Meeting, January 25th– February 5th, 2021

Agenda Item:	8.5.3

Source:	Moderator (Ericsson)

Title:	FL summary for AI 8.5.3 Accuracy improvements for DL-AoD positioning solutions

Document for:	Discussion, Decision

- Introduction

This FL summary documents the proposals and discussions for agenda item 8.5.3, based on the following chairman decision:

[104-e-NR-ePos-03] Email discussion/approval on accuracy improvements for DL-AoD positioning solutions with checkpoints for agreements on Jan-28, Feb-02, Feb-05 – Florent (Ericsson)

The FL proposals are based on submission to AI 8.5.3 [1-21], but also  including [16] and [21] which where submitted to AI 8.5.2 but contained proposals relevant to 8.5.3.

The proposals in the contributions centered on the following aspects:

Enhancements to UE reporting (LPP)

- Aspect #1 reporting of first path RSRP

- Aspect #2 support of NLOS identification

- Aspect #3 Adjacent beam reporting

- Aspect #4 Rx Beam reporting enhancements

- Aspect #5 CIR reporting for AoD

- Aspect #6 extension of number of reported RSRP measurements

- Aspect #7 Signalling to assist reference UE calibration

Enhancements to gnodeB signalling (NRPPa)

- Aspect #8 beam orientation error handling

- Aspect #9 gNodeB beam Shape information signalling

- Aspect #10 gnodeB based  calculation of AOD

- Aspects #11 TRP antenna Array configuration signalling

- Aspects for discussion

## UE Reporting aspects:

### Aspect #1 reporting of first path RSRP

#### Summary and FL proposal

Most companies support introducing an RSRP measurement of  the earliest received path RSRP for a PRS resource configured for DL AoD, with some companies going further and support reporting more than one path, or the time difference between the first path received at each TRP [1][2][3] [7] [10][11][12][13][14][15][16][18][19][20]. The following proposals were submitted:

| Source | Proposal |
| --- | --- |
| [1] | Proposal 3: The UE uses the LOS path of one DL PRS resource to measure the RSRP measurement report for DL-AoD. Proposal 4: In DL-AoD measurement report, besides the RSRP measurement of DL PRS resources for each TRP, the UE also reports the relative time-of-arrival of those reported DL PRS resources of each TRP. |
| [2] | Proposal 2: Support path specific power reporting for DL-AoD positioning. |
| [3] | Proposal 1: In addition to RSRP measurement, Rel-17 UE should have capability to support timing measurement for DL-AOD positioning method and report the timing information for network based positioning. |
| [6] | Proposal 4: Additional measurement with angle information relative to the first detected path should be further studied including its feasibility to improve AoD based positioning methods. |
| [7] | Proposal 2: Support timing based measurement and report for improving positioning accuracy of DL-AoD. |
| [10] | Proposal 1: Introduce first path coefficient (Hfp) (i.e the amplitude of the first path in the received signal's power delay profile) in DL-AoD measurement report from UE to LS. |
| [11] | Proposal 1: Support the enhancement of reporting PRS-RSRP associated with ToA of the first arrival path. |
| [12] | Proposal 1: Report DL TDoA together with DL PRS-RSRP for DL AoD. Proposal 2: To indicate the first arrival path by reporting the arrival time of each beam in beam measurement report. |
| [13] | Proposal 2: To improve the DL-AoD accuracy in UE-assisted mode, support enhanced UE measurements and reporting by considering the following: - Reporting additional correlation information (ex. CIR reporting) - Reporting of the estimated first arriving path, multipath and noise power - Reporting of timing measurements on the DL-PRS resources along with the RSRP report. |
| [14] | Proposal 2-3: The RSRP measurement may support the single path RSRP measurement. Therefore, new RSRP measurement definition could be needed |
| [15] | Proposal 1: For DL-AoD positioning method, UE can report the RSRP measurement for first arrival path together with a LOS/NLOS indicator. |
| [16] | Proposal 1: Support in Rel-17 that DL-PRS-RSRP calculation for each beam is associated to a time window in which the RSRP measurement is performed. |
| [18]. | Proposal 2: Support UE reporting of RSRP side information and additional measurements to improve the accuracy of the DL-AoD estimate, especially in NLOS scenarios. FFS details such as type of side information and timing measurements, aperiodic/periodic measurement windows, etc. |
| [19] | Observation 1: - Rel-17 can consider using other information (e.g. arrival path timings) in addition to RSRP for UE assisted DL-AoD positioning. |
| [20] | Proposal 1 Define a DL PRS peak-RSRP measurement for the power of a specific peak in the channel impulse response of a received DL-PRS resource. Proposal 2 Include the DL PRS peak-RSRP in the NR DL-AoD Location Information alongside the existing DL PRS RSRP measurement. Proposal 4 Include the DL PRS peak-RSRP in the NR DL-TDOA and multi RTT Location Information alongside the existing DL PRS RSTD measurement. Proposal 5 The UE shall report the DL PRS Peak-RSRP and the corresponding DL PRS resource ID for each additional path in the RSTD and UE Rx-Tx time difference measurements. Proposal 6 The UE shall report the strongest detected paths as additional paths (i.e. in addition to the first path). |

Based on the set of proposals, it is proposed to have an initial agreement to support first-path RSRP measurement and reporting, while keeping open the additional details.

Support the DL-AOD enhancement of measuring and reporting PRS-RSRP based on  first arriving path

  - FFS: Indication of the path arrival time

  - FFS: reporting of additional path to the first path.

  - FFS: Measurement definition

  - FFS: other method to support PRS-RSRP per path among OTDOA, UL TDOA

#### First round of comments

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

### Aspect #2 support of NLOS identification

#### Summary and FL proposal

The support of a LOS/NLOS detection/indication was brought up in [6][4][8][15][21].

| Source | Proposal |
| --- | --- |
| [4] | Proposal 1: RSRP measurements can be associated with a LOS/NLOS identifier for supporting DL AOD positioning. |
| [6] | Proposal 1: For the purpose of improving accuracy, methods to identify NLOS direction and the corresponding mitigation methods for DL-AoD measurements should be supported including the usage of transmissions and measurements of PRS signals with relative changes in their polarization orientation. Proposal 2: Consider reporting additional CIR measurements back to the network for improving NLOS detection and positioning accuracy. Proposal 3: To improve positioning accuracy by regularization techniques the UE should report LOS indicators as soft values for each link for UE-assisted positioning. |
| [8] | Proposal 3: - For the purpose of accuracy improvements, support enhancements so that the LMF knows a LoS direction of a TRP’s transmission beam to the UE. |
| [15] | Proposal 1: For DL-AoD positioning method, UE can report the RSRP measurement for first arrival path together with a LOS/NLOS indicator. |
| [21] | Proposal #3: ● Support signaling indicating the LOS/NLOS link propagation type for DL-AOD positioning ● Support signaling of reliability metric (with probability meaning) for NLOS detection (variable u in the range from 0 to 1, with absolute value showing reliability of decision) |

Since there is a variety of way to indicate LOS/NLOS on the proposals, we propose to agree on the principle of reporting LOS/NLOS information, and leave the details such as granularity (e.g. if the report is a binary indicator or a soft metric) to a future agreement.

For DL-AoD positioning method, UE can associate a measurement on a PRS resource with a report of LOS/NLOS state for the measurement

  - FFS: granularity of the state (binary indicator or soft metric)

#### First round of comments

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

### Aspect #3 adjacent beam reporting

#### Summary and FL proposal

To enhance the granularity of AOD estimation, several contributions proposed to report adjacent beams in addition to the “best beam”[1][5][8].

| Source | Proposal |
| --- | --- |
| [1] | Proposal 2: In DL-AoD UE measurement reporting, support the UE to report the RSRP of a few DL PRS resources carrying adjacent Tx beam directions. |
| [5] | - Report up to 8 DL PRS-RSRP including the strongest PRS resource and adjacent PRS resources. Proposal 4: • To balance the accuracy and robustness, AoD enhancement by adjacent DL PRS-RSRP and limited beam information (the intersection point of multiple beams) need to be considered. |
| [8]. | Proposal 2: - Need discussions on how to utilize the reception beam index for the accuracy improvements of DL-AoD based positioning, such as finding UE’s location when the UE is located between the transmission beams. Proposal 5: - Consider enhancements for the accurate UE’s location estimation when the UE is located in between two different transmission beams. |

For DL-AoD positioning method, UE can associate a measurement on a PRS resource with an additional, adjacent PRS resources measurement

  - FFS: how to define / identify adjacent beams

  - FFS: required signaling to support use of adjacent beams

#### First round of comments

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

### Aspect #4 Rx Beam reporting enhancements

#### Summary and FL proposal

In [9], it was proposed to associate a Tx and Rx beam pair of indices with the RSRP report, indicating the beam pair resulting in the strongest RSRP. In [11], it is proposed to report the Rx beam information to the LMF. [3] proposed to enhance the report by providing detailed Rx beam information including antenna virtualisation and beam direction.  In [7] it was proposed to enhance the report by informing the LMF of the Rx beams relative angular difference.

| Source | Proposal |
| --- | --- |
| [9] | Proposal 4: Introduce an indicator associated with a PRS resource/resource set or RX beam index to indicate TX beam or RX beam corresponding to the highest RSRP measurement. |
| [11] | Proposal 2: Support the enhancement of reporting the UE Rx beam information to the LMF. FFS the details of what/how to report the Rx beam information. |
| [3] | Proposal 2: Rel-17 NR positioning should support detailed Rx beam information, in which UE can provide its antenna virtualization and reception beam direction information. |
| [7] | Proposal 3: Study angle difference measurements for AoA of DL PRS resources in Rel-17. |
| [8] | Proposal 1 - A further restriction would be required so that the UE uses a reception beam to avoid worst case of the reception beam selection, even if the UE can ignore QCL type-D configuration of the PRS resources to use a fixed reception beam for DL-AoD technique. Proposal 2: - Need discussions on how to utilize the reception beam index for the accuracy improvements of DL-AoD based positioning, such as finding UE’s location when the UE is located between the transmission beams. |

For  AOD,  the UE may  optionally indicate the   following Rx beam information

- A) An indicator that the reported resource gives the highest RSRP measurement

- B) Rx beam direction information

- C) Antenna virtualization for the Rx beam

- D) angular difference between Rx beams when using different Rx beams.

#### First round of comments

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

### Aspect #5 CIR reporting for AoD

#### Summary and FL proposal

In [21]  it was proposed to support reporting the  complex channel coeffient per path for DL-AOD, for a number of channel taps. In [13], it is proposed to report the channel impulse response to be able to exploit correlation information.

| Source | Proposal |
| --- | --- |
| [21] | Proposal #1: - Support measured channel reporting per DL-PRS resource from UE to gNB to facilitate accurate DL-AOD estimation, including: - Complex channel coefficient h(m) = A(m) × exp(jφ(m)) for the mth propagation path (channel tap) per DL-PRS resource: - A(m) - channel amplitude for the mth propagation path; and/or - φ(m) – channel phase for the mth propagation path |
| [13] | Proposal 2: To improve the DL-AoD accuracy in UE-assisted mode, support enhanced UE measurements and reporting by considering the following: - Reporting additional correlation information (ex. CIR reporting) - Reporting of the estimated first arriving path, multipath and noise power - Reporting of timing measurements on the DL-PRS resources along with the RSRP report. |

For DL AOD, support reporting of the measured complex channel, h(m) = A(m) × exp(jφ(m)) per path.

- FFS: details on the reporting (including whether to send phase and amplitude separately, quantization, number of paths, etc)

#### First round of comments

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

### Aspect #6 extension of number of reported RSRP measurements

#### Summary and FL proposal

In [4], it was proposed to increase the maximum number of measurements per TRP, in order to account for different RX beam. in [5] it is propose to either fix the RX beam for the 8 reported PRS RSRP, or introduce a new list of PRS Rx Beam indices to map the measurements to Rx beams.

| Source | Proposal |
| --- | --- |
| [4] | Proposal 2: The maximum number of RSRP measurements per TRP for DL-AoD positioning should be increased for reporting the RSRP measured from different RX beams. |
| [5] | Proposal 5 - To improve the accuracy of AoD and to avoid the impact of Rx beam, choose one of the following options. - Option 1: Report up to 8 DL PRS-RSRP corresponding to the same RX beam index - Option 2: Adding an additional DL PRS-RSRP list with different nr-DL-PRS-RxBeamIndex for a PRS resource. |

For DL AOD, the   RSRP measurements per TRP is reported for

- Option 1 : up to 8 measurements per TRP per Rx beam index.  Multiple measurements corresponding to different Rx Beam index may be  reported for a given PRS.

- Option 2 : up to 8 measurements per TRP, for the same Rx beam index

- Option 3: up to N>8 measurements per TRP  for all Rx beam indices, Multiple measurements corresponding to different Rx Beam index may be  reported for a given PRS.

- FFS: value for N.

#### First round of comments

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

### Aspect #7 Signalling to assist reference UE calibration

#### Summary and FL proposal

In [4], it was proposed to support calibration by a reference UE, by allowing to send measurement and location coordinates of a reference UE, as well as TRP beam characteristics, to the LMF. In [12] it is proposed to use a reference node with a very accurate knowledge of the node location to

| Source | Proposal |
| --- | --- |
| [4] | Proposal 3: In order to improve DL-AOD estimation accuracy, the measurements provided by a reference UE with known locations should be supported to assist the calibration of the TRP transmitting beam direction. - The measurements and location coordinates of the reference UE are reported to LMF - The transmitting beam characteristics of TRP should be reported to LMF |
| [12] | Proposal 3: Estimate the angle error by a reference node whose accurate location is known. |

For DL AOD, support introducing high accuracy  reporting of the UE location, for the purpose of supporting reference UEs.

#### First round of comments

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

## gNodeB signalling aspects

### Aspect #8 beam orientation error handling

#### Summary and FL proposal

Several contribution propose to account for the beam orientation error in the gnodeB[5][7].

| Source | Proposal |
| --- | --- |
| [5] | Proposal 3 - Beam orientation errors and potential correction mechanisms need to be considered in R17. |
| [7] | Proposal 1: RAN1 to study beam orientation errors and potential correction mechanisms in order to improve the positioning accuracy achievable with DL-AoD. Including: - UE-based positioning: the beam offset (BO) could be signaled to the UE, as either an indicator, e.g. low/medium/high, each specifying an error range or as a specific value computed by the network - UE-assisted positioning: LMF should be aware of the BO and compensate for the errors when computing the position estimate. - Signaling aspects: - LMF signals to TRPs that a BO recomputation and beam re-tuning is needed. - UE measurement reports to facilitate BO identification and potential correction. |
| [18] | Proposal 3: Support mechanisms to consider beam orientation impairments and related assistance information for improving DL-AoD positioning accuracy. |

Support mechanisms to consider beam orientation impairments and related assistance information for improving DL-AoD positioning accuracy.

  - FFS: gnodeB and UE signalling to support beam orientation impairements mitigation.

#### First round of comments

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

### Aspect #9 gNodeB beam Shape information signalling

#### Summary and FL proposal

in [5][6] [11] [13] [14] [17], it is proposed to have the gnodeB transmit additional beam information to the LMF or the UE for AOD enhancements. In [4] it was proposed to support calibration by a reference UE, by allowing to send measurement and location coordinates of a reference UE, as well as TRP beam characteristics, to the LMF.

| Source | Proposal |
| --- | --- |
| [4] | Proposal 3: In order to improve DL-AOD estimation accuracy, the measurements provided by a reference UE with known locations should be supported to assist the calibration of the TRP transmitting beam direction. - The measurements and location coordinates of the reference UE are reported to LMF - The transmitting beam characteristics of TRP should be reported to LMF |
| [5] | Proposal 2 - Report additional beam information to the LMF or the UE for the enhancement of AoD. - E.g. intersection point of multiple beams: angle ,RSRPintersection point |
| [6] | Proposal 6: Consider reporting beam-shape information to the UE for DL-AoD techniques. |
| [11] | Proposal 3: Support the enhancement of reporting the gNB beam information to the LMF. FFS the details of what/how to report the beam information. |
| [13] | Proposal 1: Introduce the possibility to report the radiation characteristics of the transmitted PRS resources (i.e. main lobe power-level, sidelobe level, etc.). |
| [14] | Proposal 2-1: It is preferred to have a table provided by each TRP which may characterize all the beam responses (EIRP) for covering a range of angles. The table is like a look-up table when UE reports a number of differential RSRP values for finding the angle between a TRP and UE Proposal 2-2: A look-up table with differential EIRP across beams for a range of angles could also be considered |
| [17] | Proposal 1: For the purpose of enhancements to the RSRP-only DL-AoD method, support the signaling of beam responses (or equivalent information) to increase the accuracy precision while ensuring a tractable signaling overhead for both UE-based and UE-assisted solutions. - Note: For UE-Based AoD, the above signaling enhancements correspond to assistance data signaling (unicast or broadcast). For UE-Assisted DL-AoD, the above signaling enhancements correspond to NRPPa signaling enhancements from gNB to LMF. |

Support the enhancement of reporting the gNB beam information to the LMF.

  - FFS the details of what/how to report the beam information.

#### First round of comments

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

### Aspect #9 gnodeB based  calculation of AOD

#### Summary and FL proposal

In [21], it was proposed to have the gnodeB report the calculated AOD to the LMF, using received measurement from the UE. The proposal relies on a companion proposal to support complex channel reporting from the UE to the gnodeB.  In [2] it was proposed to support mechanisms to have the LMF request the gnodeB to calculate the AOD.

| Source | Proposal |
| --- | --- |
| [21] | - Support DL-AOD reporting from gNB to LMF (or from gNB to UE), including: - φ - azimuth angle of departure - θ - zenith angle of departure |
| [2] | Proposal 1: Support LMF to request angle calculation to the gNB for DL-AoD in Rel-17, i.e. the following Option from R3-197794. - The RAN nodes calculate the AoD with the RSRP information based on gNB configuration and send the AoD information back to LMF, who then calculates the UE position. |

DL-AOD reporting from gNB to LMF (or from gNB to UE) is supported

- FFS:  details on the report content

#### First round of comments

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

### Aspects #11 TRP antenna Array configuration signalling

#### Summary and FL proposal

In [21] and [17], it is proposed to provide additional information on the TRP antenna array configuration.

| Source | Proposal |
| --- | --- |
| [21] | Proposal 4 - Support reporting of two DL-AOD azimuth angles φ and (π – φ) in addition to the zenith angle θ from gNB to LMF (or from gNB to UE) Proposal 5 (Alternative to Proposal #4) - Specify the TRP antenna array orientation in the local coordinate system Proposal 6 - Support DL-AOD φ’ = arcsin(sin(φ)×sin(θ)) angle reporting for the linear horizontal array, where φ is the azimuth angle of departure and θ is the zenith angle of departure - Further discuss if it is necessary to indicate information on antenna array structure/type |
| [17] | Proposal 2: For the purpose of enabling a phase-difference-based DL-AoD method, support the following signaling enhancements: - gNBs’ UPA antenna Configuration, PMI Codebook configuration & their association to the transmitted PRS resources - Note: Enhancement should be applicable to both UE-based and UE-assisted DL-AoD |

For both UE-based and UE-assisted DL-AoD, and for the purpose of supporting linear horizontal array types of gnodeB antenna configuration, consider the following options:

  - Support reporting of two DL-AOD azimuth angles φ and (π – φ) in addition to the zenith angle θ from gNB to LMF (or from gNB to UE)

  - Specify the TRP antenna array orientation in the local coordinate system

  - Support DL-AOD φ’ = arcsin(sin(φ)×sin(θ)) angle reporting for the linear horizontal array, where φ is the azimuth angle of departure and θ is the zenith angle of departure

  - Support of signalling gNBs’ UPA antenna Configuration, PMI Codebook configuration & their association to the transmitted PRS resources

#### First round of comments

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

## Other  Aspects for discussion

The following aspects have been proposed by one company. It is proposed to delay discussion on these issues to give a chance to other companies to consider these issues in future contributions.

### Aspect #8 Differential angle reporting

In [15] it was proposed to study angle difference measurement for the AOA of DL PRS.

| Source | Proposal |
| --- | --- |
| [15] | Proposal 2: Support differential beamforming technique for DL-AOD positioning methods. Proposal 3: aspects of PRS resource configuration, DL transmission beam indication and UE measurement and report needs to be considered in order to support differential beamforming technique for DL-AOD positioning methods. |

### Signalling of preferred SSB

In  [8], it is proposed that the you may signal the preferred SSB to obtain timing mesasurements.

| Source | Proposal |
| --- | --- |
| [8] | Proposal #4: - Support enhancements to enable the UE can use the reception beam in a direction corresponding to the minimum propagation time. - E.g., the reporting of the preferred SSB to obtain timing measurements. |

### UE specific beam refinement

In [1] and [6] it was proposed to support configuring UE specific beam refinement on DL PRS resouces:

| Source | Proposal |
| --- | --- |
| [1] | Proposal 1: To enhance the performance of DL AoD, support UE-specific beam refinement on DL PRS resource for DL-AoD measurement. |
| [6] | Proposal 5: Consider two stage beam-sweeping for DL-AoD to improve angular resolution in a time and power efficient manner. |

### UE orientation reporting

In [9], it was proposed to include the UE orientation in the information reported to the LMF from the UE.

| Source | Proposal |
| --- | --- |
| [9] | Proposal 2: The UE should report its orientation information to the LMF. Proposal 3: Define conditions for the UE to report the orientation. |

### UE panel ID reporting

In [9], it was proposed to include the UE panel ID in the information reported to the LMF from the UE.

| Source | Proposal |
| --- | --- |
| [9] | Proposal 1: Include Rx panel ID in the measurement report. |

# Conclusion

TBA

# References

R1-2100130 Enhancements for DL-AoD positioning OPPO

R1-2100238 Enhancement for DL AoD positioning Huawei, HiSilicon

R1-2100295 Accuracy improvements for DL-AoD positioning solutions ZTE

R1-2100387 Discussion on accuracy improvements for DL-AoD positioning solutions CATT

R1-2100447 Discussion on potential enhancements for DL-AoD method vivo

R1-2100489 Discussion on improving the accuracy of DL AoD positioning solutions FUTUREWEI

R1-2100550 Initial views on enhancing DL AoD	Nokia, Nokia Shanghai Bell

R1-2100710 Discussion on accuracy improvement for DL-AoD positioning	LG Electronics

R1-2100750 Accuracy improvements for DL-AoD positioning solutions InterDigital, Inc.

R1-2100864 Discussion on accuracy improvements for DL-AoD positioning method Sony

R1-2101048 Discussion on DL-AoD enhancement CMCC

R1-2101121 Accuracy improvements for DL-AoD positioning solutions Xiaomi

R1-2101133 DL-AoD positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

R1-2101141 Accuracy enhancement for DL-AOD technique MediaTek Inc.

R1-2101212 Accuracy improvements for DL-AoD positioning solutions Samsung

R1-2101388 Accuracy enhancements for UL-AoD positioning technique Apple

R1-2101470 Potential Enhancements on DL-AoD positioning Qualcomm Incorporated

R1-2101501 Potential DL-AoD Positioning Enhancements Lenovo, Motorola Mobility

R1-2101618 Discussion on DL-AoD positioning enhancements NTT DOCOMO, INC.

R1-2101756 Enhancements of DL-AoD positioning solutions Ericsson

R1-2100659 NR positioning enhancements for DL-AoD method	Intel Corporation