3GPP TSG RAN WG1 #104b-e		R1-2102516

e-Meeting, April 12th – 20th, 2021

Source:	vivo

Title:	Discussions on PUCCH enhancements for NR operation from 52.6GHz to 71GHz

Agenda Item:	8.2.3

Document for:	Discussion and Decision

- Introduction

The following objective is included in this work item [1]:

- Physical layer aspects including [RAN1]:

  - Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation.

According to the agreements in RAN1#104-e [2], the link level simulations and the methodology of link budget calculations are listed in appendix. In this contribution, we provide some evaluations for further discussion or down-selecting for the alternatives of enhanced PUCCH format 0/1/4, including the RE mapping for SCS 120kHz, the sequence of PUCCH format 0/1 and the sequence of DMRS of PUCCH format 4, and the block-wise spreading for PUCCH format 4, additionally, the number of RBs of each SCS of each PUCCH format, the UE multiplexing with misalignment or not. Moreover, we also discuss the cyclic shift for a single long sequence, the indication of number of RBs, and the shortage of common PUCCH resource.

- Discussion

Simulation evaluation for PUCCH

In this section, we provide details of our PUCCH evaluation, including the evaluation assumptions and reporting metrics. Then we provide some evaluation results for down-selecting or further discussion.

Evaluation assumption and methodology

Our evaluation assumptions and reporting metrics are based on the baseline as agreed in RAN1#104-e. There are some additional details we provided in the following Table 1. Furthermore, we list all the evaluation cases in Table 3 to express more clearly.

Table 1 The additional evaluation assumptions and reporting metrics

| Parameter | Notes | Values |
| --- | --- | --- |
| PUCCH Frequency Domain Resource Mapping | N_RB contiguous RBs per hop (with all REs allocated per PRB) Note: If alternative RE allocation per PRB is used, companies to report details | For case 0/1/2, all REs are allocated per PRB. For case 3/4/5/6, there are 6 REs allocated per PRB. |
| Frequency hopping details | Frequency offset between hops, | If the PRB number of BWP is N_RB, then the frequency offset between the minimum PRB index in the first hop and the maximum PRB index in the second hop are N_RB – 1. |
| Number of RBs used per hop (N_RB) | N_RB contiguous RBs per hop | The contiguous RBs per hop is {1,2,4,8,16} |
| Sequence construction details | Sequence type for PF0/1 Sequence type for DMRS of PF4 | For case 0/1/3/5, the sequence type is alt.1. For case 2/4/6, the sequence type is alt.2. |
| OCC configuration details | Applicable for PF1, PF4 | 1 user is configured by OCC index 0. OCC length is 2. |
| PUCCH payload encoder type | Reed Muller or Polar for PF4 | Reed Muller when payload is 4 bits or 11 bits. Polar when payload is 22 bits. |
| PUCCH encoding rate(s) | Applicable for PF4 If multiple payload sizes evaluated, encoding rates to be reported for each payload size | The encoding rates are listed in Table 2 for different payloads and different cases. |
| Number of multiplexed users, e.g., by code division, if applicable | 1 user Note: Companies to report if other cases if evaluated | Most cases are evaluated with 1 user while we also provide the evaluation with 2 users for PUCCH format 0. |
| Pmax (dBm) | Maximum allowed conducted power considering combined limit per region (from Table 3). Note: Companies should report if Pmax is considered per region or a combined limit is considered across multiple regions | Pmax is considered per region. |

Table 2 The coding rates for PUCCH format 4

|  | Case 0 | Case 1 (2 PRB) | Case 1 (4 PRB) | Case 1 (8 PRB) | Case 1 (16 PRB) |
| --- | --- | --- | --- | --- | --- |
| 4 bits | 0.0278 | 0.0139 | 0.0069 | 0.0035 | 0.0017 |
| 11 bits | 0.0764 | 0.0382 | 0.0191 | 0.0095 | 0.0048 |
| 22 bits | 0.2292 | 0.1146 | 0.0573 | 0.0286 | 0.0143 |

Table 3 The cases of PUCCH evaluation

| Case | RE mapping: (X, Y) is used to state X contiguous PRBs per hop and Y REs are allocated per PRB for one user. | Sequence type and sequence length |
| --- | --- | --- |
| Case 0 | (1, 12) | Alt 1, 12 |
| Case 1 | (X≥2, 12) | Alt 1, X*12 |
| Case 2 | (X≥2, 12) | Alt 2, 12 |
| Case 3 | (X≥2, 6) with contiguous REs | Alt 1, X*6 |
| Case 4 | (X≥2, 6) with contiguous REs | Alt 2, 6 |
| Case 5 | (X≥2, 6) with sub-PRB interlace | Alt 1, X*6 |
| Case 6 | (X≥2, 6) with sub-PRB interlace | Alt 2, 6 |

For example, when X =2, the RE mapping and sequence type for evaluation cases mentioned above are as illustrated in Figure 1, the different colours show the sequence with cyclic shifts across RBs, the same colour means a single sequence.

Figure 1 the RE mapping and sequence type for evaluation cases when X = 2

Evaluation results and analysis

There are some evaluation results provided according to the evaluation assumptions in section 2.1.1. We also provide some analysis for down-selecting or further discussion with the RE mapping for SCS 120kHz, sequence type for PUCCH format 0/1 and DMRS of PUCCH format 4 and block-wise spreading for PUCCH format 4, the number of RBs of each SCS of each PUCCH format, the UE multiplexing with misalignment or not.

Enhancement on PUCCH format 0/1

The sub-PRB of contiguous REs within each PRBs (case 3&4)

The following Figure 2 shows the MIL for case0 ~ case 6 when SCS is 120kHz, delay spread is 5ns, PRB number is 2 of PUCCH format 0/1. In most region including US, South Korea 1(when an equipment is <300m from an astronomical antenna) and South Korea 2(when an equipment is >=300m from an astronomical antenna), the MIL of case 1 ~ case 6 is higher than case 0 which means the multi-PRB PUCCH format 0/1 have better coverage performance than that with one PRB. Moreover, in US and South Korea, the MIL of case 3 and case 4 is lower than other multi-PRB based PUCCH cases. In Europe, even the MIL of case 3 and case 4 is higher than that of case 2, they are all lower than that of case 0. However, the MIL of case 1 and case 5 is higher than that of case 0.

Figure 2 The MIL for PUCCH format 0/1 when SCS is 120kHz, delay spread is 5ns, PRB number is 2

Observation 1: The case 3 and case 4 have worse MIL performance than other multi-PRB cases in US and South Korea, and have worse MIL performance than one PRB case in Europe.

The sequence type for enhanced PUCCH format 0/1

Considering the sequence type for enhanced PUCCH format 0/1, there are two alternatives are agreed in RAN1 104 e-meeting as following agreement. The Figure 3 and Figure 4 illustrate the MIL of enhanced PUCCH format 0/1 when SCS is {120kHz, 480kHz, 960kHz}, delay spread is {5ns, 10ns, 20ns}, PRB number is 2, and the cases 0/1/2/5/6 for SCS 120kHz, cases 0/1/2 for SCS 480kHz and 960kHz.

Agreement #RAN1 104-e: - For enhanced PF0/1, support Type-1 low PAPR sequences. Further study and strive to select one of the following alternatives: - Alt-1: A single sequence of length equal to the total number of mapped REs of of the PUCCH resource is used. Cyclic shifts for PF0/1 are defined in the same way as Rel-16 for the case that useInterlacePUCCH-PUSCH is not configured. - Alt-2: A single sequence of length equal to the number of mapped REs per RB of the PUCCH resource is used, and the sequence is repeated in each RB. At least the following scheme is considered for PAPR/CM reduction: - Cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured - At least the following aspects should be considered in the study - Coverage (maximum isotropic loss (MIL)), including - Required SNR to fulfil PUCCH detection criterion - PAPR/CM as a function of N_RB - Specification impact

Figure 3 The MIL of enhanced PUCCH format 0 when SCS is {120kHz, 480kHz, 960kHz}, delay spread is {5ns, 10ns, 20ns}, PRB number is 2

As we see from Figure 3, for enhanced PUCCH format 0, firstly, when SCS is 480kHz or 960kHz and PRB number is 2, the MIL of case 1 increases more than that of case 2 in US and South Korea. While in Europe the case 2 does not have MIL gain compared with case 0 while case 1 has slight MIL gain compared with case 0. Secondly, when SCS is 120kHz, the MIL of case 1 is higher than that of case 2 in all regions. For case 5 and case 6, similarly, the MIL of case 5 is obviously higher than that of case 6 in South Korea 1 when delay spread is 5ns or 10ns and in Europe when delay spread is 20ns. While the MIL of case 5 is slightly lower than that of case 6 in other situations.

Observation 2: For enhanced PUCCH format 0, when SCS is 480kHz or 960kHz and PRB number is 2, the MIL of case 1 increases more than that of case 2 in US and South Korea, and the case 2 does not have MIL gain compared with case 0 while case 1 has slight MIL gain compared with case 0 in Europe.

Observation 3: For enhanced PUCCH format 0, when SCS is 120kHz and PRB number is 2, the MIL of case 1 is higher than that of case 2 in all regions. Similarly, the MIL of case 5 is obviously higher than that of case 6 in South Korea 1 when delay spread is 5ns or 10ns and in Europe when delay spread is 20ns. While the MIL of case 5 is slightly lower than that of case 6 in other situations.

Figure 4 The MIL of enhanced PUCCH format 1 when SCS is {120kHz, 480kHz, 960kHz}, delay spread is {5ns, 10ns, 20ns}, PRB number is 2

As we see from Figure 4, for enhanced PUCCH format 1, firstly, when SCS is 480kHz and PRB number is 2, the MIL of case 1 increases more than that of case 2 in US and South Korea. While in Europe, the MIL of case 2 do not increase and the MIL of case 1 do not increase compared with case 0 in Europe either when delay spread is 20ns, and the MIL of case1 slightly increases compared with case 0 when delay spread is 5ns or 10ns. Secondly, when SCS is 960kHz, the MIL of case 2 decreases in all regions, and the MIL of case 1 decreases in all regions when delay spread is 20ns. When delay spread is 5ns or 10ns, the MIL of case 1 increases in US and South Korea but decreases in Europe. Thirdly, when SCS is 120kHz, in US and South Korea, the MIL of case 1 and case 2 are close to each other when delay spread is 10ns or 20ns, while the MIL of case 2 have slight increase than that of case 1 when delay spread is 5ns. Similarly, the MIL of case 6 is lower than that of case 5 when delay spread is 5ns or 10ns, while higher when delay spread is 20ns. In Europe, the MIL of case 1 and case 5 is obviously higher than that of case 2 and case 6 respectively.

Observation 4: For enhanced PUCCH format 1, when SCS is 480kHz and PRB number is 2, the MIL of case 1 increases more than that of case 2 in US and South Korea. While in Europe, the MIL of case 2 do not increase and the MIL of case 1 decrease in Europe either when delay spread is 20ns, and the MIL of case1 slightly increases when delay spread is 5ns or 10ns.

Observation 5: For enhanced PUCCH format 1, when SCS is 960kHz and PRB number is 2, the MIL of case 2 decreases in all regions, and the MIL of case 1 decreases in all regions when delay spread is 20ns. When delay spread is 5ns or 10ns, the MIL of case 1 increases in US and South Korea but decreases in Europe.

Observation 6: For enhanced PUCCH format 1, when SCS is 120kHz and PRB number is 2, in US and South Korea, the MIL of case 1 and case 2 are close to each other when delay spread is 10ns or 20ns, while the MIL of case 2 have slight increase than that of case 1 when delay spread is 5ns. Similarly, the MIL of case 6 is lower than that of case 5 when delay spread is 5ns or 10ns, while higher when delay spread is 20ns. In Europe, the MIL of case 1 and case 5 is obviously higher than that of case 2 and case 6 respectively.

Proposal 1: For enhanced PUCCH format 0/1, the alt 1 of a single sequence of length equal to the total number of mapped REs of the PUCCH resource is preferred.

The RE mapping for enhanced PUCCH format 0/1 when SCS 120kHz

Considering the RE mapping for enhanced PUCCH format 0/1 of SCS 120kHz, there are two alternatives are agreed in RAN1 104 e-meeting as following agreement. The Figure 5 and Figure 6 illustrate the MIL of enhanced PUCCH format 0/1 when SCS is 120kHz, delay spread is {5ns, 10ns, 20ns}, PRB number is {2,4,8}, and the cases 0/1/5.

Agreement #RAN1 104-e: For enhanced (multi-RB) PUCCH Formats 0/1/4 for 120/480/960 kHz SCS, support allocation of N_RB contiguous RBs - FFS: Values of N_RB for each SCS - For 480/960 kHz SCS, all REs within each RB are mapped - Note: PRB and sub-PRB interlaced mapping is not considered further - For 120 kHz SCS, further discuss the following two alternatives: - Alt-1: All REs within each RB are mapped - Note: PRB and sub-PRB interlaced mapping is not considered further - Alt-2: Subset of REs within each RB are mapped (sub-PRB interlaced mapping)

Figure 5 The MIL of enhanced PUCCH format 0 when SCS is 120kHz, delay spread is {5ns, 10ns, 20ns}, PRB number is {2,4,8}

As we see from Figure 5, for enhanced PUCCH format 0 with 120kHz SCS, the MIL of case 1 is slightly higher than that of case 5 when PRB number is 2. The MIL of case 5 increase significantly when PRB number is 4 or 8. Especially, compared with case 0, the MIL of case 1 decreases and case 5 increases when PRB number is 4 in Europe.

Observation 7: For enhanced PUCCH format 0 with 120kHz SCS, the MIL of case 1 is slightly higher than that of case 5 when PRB number is 2. While the MIL of case 5 increase significantly when PRB number is 4 or 8. Especially, compared with case 0, the MIL of case 1 decreases and case 5 increases when PRB number is 4 in Europe.

Figure 6 The MIL of enhanced PUCCH format 1 when SCS is 120kHz, delay spread is {5ns, 10ns, 20ns}, PRB number is {2,4,8}

As we see from Figure 6, for enhanced PUCCH format 1 with 120kHz SCS, firstly, when PRB number is 2, the MIL of case 1 is lower than that of case 5 when delay spread is 5ns in US and South Korea, slightly higher in other cases. Secondly, when PRB number is 4, the MIL of case 1 is higher than that of case 5 in US and Korea, while decreases in Europe. Thirdly, when PRB number is 8, the MIL of case 5 increase compared with case 1. Especially, the MIL of case 1 decreases in Europe.

Observation 8: For enhanced PUCCH format 1 with 120kHz SCS, the MIL of case 1 is lower than that of case 5 when delay spread is 5ns in US and South Korea, slightly higher in other cases when PRB number is 2. The MIL of case 1 is higher than that of case 5 in US and Korea when PRB number is 4, while decreases in Europe. While the MIL of case 5 increase obviously when PRB number is 8. Especially, the MIL of case 1 decreases when PRB number is 8 in Europe.

In general, for enhanced PUCCH format 0/1, case 0 has worse MIL performance than case 1 and case 5 in US and South Korea. However, case 0 has a comparable (when the number of PRB is 2 or 4 for case 1 and 5) or better MIL performance (when the number of PRB is 8 for case 1 and 5) than those of case 1 and case 5 in Europe. Comparing the MIL performance of case 1 and case 5 for enhanced PUCCH format 0/1 with multi-PRB, case 5 has better performance when the number of PRB is 4 in Europe and otherwise comparable performance for other number of PRB in Europe or in other regions. Based on the MIL performance evaluation results of single UE, we think both alternatives should be supported due to various regulations in different regions.

It is worth noting that the evaluation is done only with 1 user. The UE multiplexing impact on MIL need further discussion. The maximum number of multiplexing UE is 6 for case 0 when the payload is 1 bit. However, it increases to 12 for case 1 and case 5. Additionally, to compare case 1 with case 5 with the same UE multiplexing capacity, the UE number of case 5 should be half of case 1 owing to the FDM.

Proposal 2: For enhanced PUCCH format 0/1, for 120 kHz SCS, we support both of the two alternatives for RE mapping. Meanwhile, the UE multiplexing impact for MIL need further study.

Enhancement on PUCCH format 4

The RE mapping for UCI of enhanced PUCCH format 4 when SCS 120kHz

For UCI of PUCCH format 4, the Fourier basis can be used for the OCC spreading code {[1,1,1,1], [1, j, -1, -j], [1, -1, 1, -1], [1, -j, -1, j]} to multiplex 4 UEs, and {[1,1], [1, -1]} to multiplex 2 UEs, according to [3], we can conclude that the pre-DFT-OCC user multiplexing is equivalent to the comb-based user multiplexing. So, we think it is not necessary to use additional sub-PRB interlace RE mapping for UCI of enhanced PUCCH format 4.

Figure 7 Example of Rel.15 PUCCH format 4 for one PRBs using a single DFT-precoder.

Proposal 3: For UCI of enhanced PUCCH format 4, all REs within each RB are mapped is preferred for 120kHz SCS.

The sequence type for DMRS of enhanced PUCCH format 4

Considering the sequence type for DMRS of enhanced PUCCH format 4, there are two alternatives are agreed in RAN1 104 e-meeting as following agreement. The Figure 8,Figure 9 and Figure 10 illustrate the MIL of enhanced PUCCH format 4 when SCS is {120kHz, 480kHz, 960kHz}, delay spread is {5ns, 10ns, 20ns}, payload is {4 bits, 11 bits, 22 bits}, PRB number is 2, and the cases 0/1/2.

Agreement: - For DMRS of enhanced PF4, support Type-1 low PAPR sequences. Further study and strive to select one of the following alternatives for sequence construction: - Alt-1: A single sequence of length equal to the total number of mapped Res of of the PUCCH resource is used. Cyclic shifts are defined in the same was as Rel-15/16 for PF4. - Alt-2: A single sequence of length equal to the number of mapped Res per PRB of the PUCCH resource is used, and the sequence is repeated in each PRB. At least the following scheme is considered for PAPR/CM reduction: - Cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured - At least the following aspects should be considered in the study - Coverage (maximum isotropic loss (MIL)), including - Required SNR to fulfil PUCCH detection criterion - PAPR/CM as a function of N_RB - Specification impact

Figure 8 The MIL of enhanced PUCCH format 4 when SCS is {120kHz, 480kHz, 960kHz}, delay spread is {5ns, 10ns, 20ns}, PRB number is 2, payload is 4 bits

Figure 9 The MIL of enhanced PUCCH format 4 when SCS is {120kHz, 480kHz, 960kHz}, delay spread is {5ns, 10ns, 20ns}, PRB number is 2, payload is 11 bits

Figure 10 The MIL of enhanced PUCCH format 4 when SCS is {120kHz, 480kHz, 960kHz}, delay spread is {5ns, 10ns, 20ns}, PRB number is 2, payload is 22 bits

As illustrated in Figure 8, Figure 9 and Figure 10, to all provided payloads, in Europe, the MIL of case 1 and case 2 do not have gain compared with that of case 0 in Europe when SCS is 480kHz or 960kHz, the MIL of case 1 have more gain than that of case 2 when SCS is 120kHz. Additionally, In US and South Korea, the MIL of case 1 is obviously higher than that of case 2 when SCS is 960kHz, and minor difference between the two cases when SCS is 120kHz or 480kHz.

Observation 9: In Europe, the MIL of case 1 and case 2 do not have gain compared with that of case 0 in Europe when SCS is 480kHz or 960kHz, the MIL of case 1 have more gain than that of case 2 when SCS is 120kHz.

Observation 10: In US and South Korea, the MIL of case 1 is obviously higher than that of case 2 when SCS is 960kHz, and minor difference between the two cases when SCS is 120kHz or 480kHz.

Proposal 4: For DMRS of enhanced PUCCH format 4, a single sequence of length equal to the total number of mapped REs of the PUCCH resource is preferred is preferred.

The block-wise spreading for enhanced PUCCH format 4

Considering the block-wise spreading for enhanced PUCCH format 4, there are two alternatives are agreed in RAN1 104 e-meeting as following agreement. The Figure 11, Figure 12 and Figure 13 illustrate the MIL of enhanced PUCCH format 4 when SCS is {120kHz, 480kHz, 960kHz}, delay spread is {5ns, 10ns, 20ns}, payload is {4 bits, 11 bits, 22 bits}, PRB number is 2, and the cases 0/1-alt1/1-alt2. For alt 2, the PRB-specific phase rotation is used for PAPRA/CM reduction.

Agreement: - For UCI of enhanced PF4, support pre-DFT blockwise spreading using OCCs of length 2 and 4 as defined for Rel-16 PF4 - Further study the following and decide in RAN1#104-b: - Whether or not additional OCC lengths are supported - Down-select to one of the following alternatives for blockwise spreading - Alt-1: Blockwise spreading is performed across all allocated RBs - Alt-2: Blockwise spreading and DFT is performed per-RB followed by per-RB PAPR/CM reduction mechanism. - At least the following aspects should be considered in the study - Coverage (maximum isotropic loss (MIL)), including - Required SNR to fulfil PUCCH detection criterion - PAPR/CM as a function of N_RB - Specification impact

Figure 11 The MIL of enhanced PUCCH format 4 when SCS is {120kHz, 480kHz, 960kHz}, delay spread is {5ns, 10ns, 20ns}, PRB number is 2, payload is 4 bits for two alternatives of block-wise spreading

Figure 12 The MIL of enhanced PUCCH format 4 when SCS is {120kHz, 480kHz, 960kHz}, delay spread is {5ns, 10ns, 20ns}, PRB number is 2, payload is 11 bits for two alternatives of block-wise spreading

Figure 13 The MIL of enhanced PUCCH format 4 when SCS is {120kHz, 480kHz, 960kHz}, delay spread is {5ns, 10ns, 20ns}, PRB number is 2, payload is 22 bits for two alternatives of block-wise spreading

As illustrated in Figure 11, Figure 12 and Figure 13, the MIL of alt 1 is obviously higher than that of alt 2 when payload is 11 bits or 22 bits with all SCS and when payload is 4 bits with SCS 960kHz. When payload is 4 bits with SCS 120kHz, the MIL of alt 1 is higher than that of alt 2 when delay spread is 5ns or 10ns, slightly lower when delay spread is 20ns. When payload is 11 bits with SCS 480kHz, the MIL between the two alternatives have minor differences.

Moreover, the alt 1 has less specification impact than alt 2. The block-wise spreading formula for PUCCH 4 is also applied for interlace-based PUCCH format 3, which means the formula also supports PUCCH formats with multiple PRBs, the related parameter just to be changed for alt 1. Specifically, for PUCCH format 4, MscPUCCH,s=MRBPUCCH,sNscRB  with MRBPUCCH,s representing the bandwidth of the PUCCH in terms of resource blocks. In NR Release 16, MRBPUCCH,s=1 for PUCCH format 4 and NscRB=12. For alt 1, we consider MRBPUCCH,s>1, and MscPUCCH,s=MRBPUCCH,sNscsub_RB, where 1≤Nscsub_RB ≤12. Then the block-wise spreading applies with the NR release 16 spec:

ylMscPUCCH,s+k=wnkNSFPUCCH,sMscPUCCH,sdlMscPUCCH,sNSFPUCCH,s+k mod MscPUCCH,sNSFPUCCH,s
k=0,1,…, MscPUCCH,s-1
l=0,1,…,NSFPUCCH,sMsymbMscPUCCH,s-1

Therefore, the parameter related to block-wise spreading of PUCCH format 4 is dependent on the number of RBs and the number of REs in each RB. However, for alt 2, the per-RB PAPR/CM reduction mechanism brings more changes for signal processing flows.

Observation 11: The MIL of alt 1 is obviously higher than that of alt 2 when payload is 11 bits or 22 bits with all SCS and when payload is 4 bits with SCS 960kHz.

Observation 12: When payload is 4 bits with SCS 120kHz, the MIL of alt 1 is higher than that of alt 2 when delay spread is 5ns or 10ns, slightly lower when delay spread is 20ns. When payload is 11 bits with SCS 480kHz, the MIL between the two alternatives have minor differences.

Observation 13: For block-wise spreading for PUCCH format 4, alt 1 has less specification impact than alt 2.

Proposal 5: For UCI of enhanced PUCCH format 4, we support alt 1 that the block-wise spreading is performed across all allocated RBs.

In Rel.15, different PUCCH format are designed for different principles. For example, the PUCCH format 3/4 are used for larger UCI payloads and moderate UE multiplexing. And the PUCCH format 0/1 are used for smaller UCI payloads and large UC multiplexing. Even though there will be allocated multiple PRBs for enhanced PUCCH format 4, while PUCCH format 3 with interlace are allocated for multiple RBs with the OCC length 2 or 4 configured by RRC in Rel.16. Therefore, we think there is no strong motivation to support additional OCC length.

Proposal 6: The current OCC length for enhanced PUCCH format 4 is enough.

The RB number of enhanced PUCCH format 0/1/4

The minimum value of NRB is 1 for PF 0/1/4 for all subcarrier spacings, while the maximum value for each SCS and each of PF0/1/4 and the allowed values of NRB within the [min/max] range is not decided yet. The Figure 14 to Figure 18 illustrate the MIL trend with different values of NRB for PUCCH format 0/1 with payload of 1 bit for case 1/5 and PUCCH format 4 with different payload of {4 bits, 11 bits, 22 bits} for case 1.

Agreement #RAN1 104-e: - The configured number of RBs for enhanced PF 0/1/4 is denoted NRB - The minimum value of NRB is 1 for PF 0/1/4 for all subcarrier spacings - The maximum value of NRB depends on subcarrier spacing - FFS: maximum value for each SCS and each of PF0/1/4 - FFS: Allowed values of NRB within the [min/max] range - FFS: Details of indication of NRB by cell-specific (for PF0/1) and dedicated signaling (PF0/1/4) - FFS: Whether or not multiplexing of users with misaligned RB allocations is supported, where "misaligned" also includes users with different # of RBs. - For PF4: - The actual number of RBs used for a PUCCH transmission is equal to NRB, i.e., the actual number of RBs does not vary dynamically based on PUCCH payload - NRB fulfils the following: N=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers - Note: if frequency hopping is enabled, NRB is the number of RBs per hop Note: decisions on the maximum value of NRB for each SCS and PUCCH format shall take into account link budgets based at least on the agreed evaluation assumptions

Figure 14 The MIL of enhanced PUCCH format 0 for different numbers of RB

Figure 15 The MIL of enhanced PUCCH format 1 for different numbers of RB

Figure 16 The MIL of enhanced PUCCH format 4 with payload is 4 bits for different numbers of RB

Figure 17 The MIL of enhanced PUCCH format 4 with payload is 11 bits for different numbers of RB

Figure 18 The MIL of enhanced PUCCH format 4 with payload is 22 bits for different numbers of RB

The principle of the maximum value of NRB is dependent on the relatively large value of MIL. For example, as illustrated in Figure 14, the growth rate when NRB is 16 is lower than when NRB is 8, even the MIL is higher, then the maximum value of NRB is decided by 8. Moreover, the MIL has different trends in different regions, even if the MIL increases in one region, it should be considered for the maximum value of NRB. Therefore, the maximum value is decided according to the trends in Figure 14 to Figure 18 and listed in Table 4. The allowed value of NRB is within the [min/max] range which the minimum value is 1 and the maximum value is listed. within the [min/max] range. Moreover, the allowed NRB for PUCCH format 4 fulfils the following: N=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers. So, the value of 7 is excluded and the allowed values of NRB for each SCS of each PUCCH format are listed in Table 5.

Table 4 Maximum value of NRB for each SCS and each of PF0/1/4

|  | PF 0 | PF 1 | PF 4 (4bits) | PF 4 (11bits) | PF 4 (22bits) |
| --- | --- | --- | --- | --- | --- |
| 120 kHz (case 1& 5) | 8 | 8 | 8 | 8 | 8 |
| 480 kHz | 2 | 2 | 4 | 4 | 4 |
| 960 kHz | 2 | 2 | 2 | 2 | 2 |

Table 5 Allowed values of NRB within the [min/max] range

|  | PF 0 | PF 1 | PF 4 (4bits) | PF 4 (11bits) | PF 4 (22bits) |
| --- | --- | --- | --- | --- | --- |
| 120kHz (case 1&5) | {1,2,3,4,5,6,7,8} | {1,2,3,4,5,6,7,8} | {1,2,3,4,5,6,8} | {1,2,3,4,5,6,8} | {1,2,3,4,5,6,8} |
| 480kHz | {1,2} | {1,2} | {1,2,3,4} | {1,2,3,4} | {1,2,3,4} |
| 960kHz | {1,2} | {1,2} | {1,2} | {1,2} | {1,2} |

Proposal 7: The maximum value of NRB for each SCS of each PUCCH format 0/1/4 are listed in Table 4.

Proposal 8: The allowed values of NRB within the [min/max] range for each SCS of each PUCCH format 0/1/4 are listed in Table 5.

The UE multiplexing

Agreement #RAN1 104-e: - FFS: Whether or not multiplexing of users with misaligned RB allocations is supported, where "misaligned" also includes users with different # of RBs.

We provided some evaluations results for whether the UE multiplexing with misaligned RB allocations should be supported. The opt.1 is for multiplexing of users with aligned RB allocations, oppositely, the opt.2 is for multiplexing of users with misaligned RB allocations. The RB allocations for the two options are show in Figure 19. Figure 20 shows the MIL performance for the two options of PUCCH format 0, when SCS is 120kHz, payload is 1 bit, delay spread is {5ns, 10ns, 20ns}.

Figure 19 The RB allocations for the two options

Figure 20 The multiplexing of 2 UEs with alignment or misalignment

As illustrated in Figure 20, the MIL of opt.1 is always higher than that of opt.2. Therefore, the multiplexing of users with misaligned RB allocations should not be supported.

Proposal 9: The multiplexing of users with misaligned RB allocations should not be supported.

The other issues of enhanced PUCCH format 0/1/4

The cyclic shift for a single long sequence

The cyclic shift α varies as a function of the symbol and slot number according to [4]

αl=2πNscRBm0+mcs+mint+ncsns,fμ,l+l' mod NscRB

For PUCCH format 0/1, where m0 is RRC configured (the value ranges from 0 to 11) for dedicated PUCCH resource and predefined for common PUCCH resource as in Table 6-1. NscRB is 12 which is the total RE number within one PRB.  except for PUCCH format 0 when it depends on the information to be transmitted as Table 6-2 to Table 6-5 below.

As we discussed in section 2.1.2.1, the alt 1 of a single sequence of length equal to the total number of mapped REs of the PUCCH resource is preferred for PUCCH format 0/1. For example, the sequence length is 24 when PRB number is 2 for SCS 480kHz. To support more and better UE multiplexing, NscRB should be change to 24. Addtionally, the value range of m0 should change from 0～11 to 0～23 for dedicated PUCCH resource, the value candicates of m0 in Table 6-1 should also change for common PUCCH resource. Similarily, and the value of mcs in Table 6-2 to Table 6-5 should be changed dependent on the length of sequence.

Table 6 Copy of Table 9.2.1-1 and Table 9.2.3-3&4 and Table 9.2.5-1&2 from TS 38.213[5]

Table 6-1 Copy of Table 9.2.1-1: PUCCH resource sets before dedicated PUCCH resource configuration Index; PUCCH format; First symbol; Number of symbols; PRB offset; Set of initial CS indexes. 0; 0; 12; 2; 0; {0, 3}. 1; 0; 12; 2; 0; {0, 4, 8}. 2; 0; 12; 2; 3; {0, 4, 8}. 3; 1; 10; 4; 0; {0, 6}. 4; 1; 10; 4; 0; {0, 3, 6, 9}. 5; 1; 10; 4; 2; {0, 3, 6, 9}. 6; 1; 10; 4; 4; {0, 3, 6, 9}. 7; 1; 4; 10; 0; {0, 6}. 8; 1; 4; 10; 0; {0, 3, 6, 9}. 9; 1; 4; 10; 2; {0, 3, 6, 9}. 10; 1; 4; 10; 4; {0, 3, 6, 9}. 11; 1; 0; 14; 0; {0, 6}. 12; 1; 0; 14; 0; {0, 3, 6, 9}. 13; 1; 0; 14; 2; {0, 3, 6, 9}. 14; 1; 0; 14; 4; {0, 3, 6, 9}. 15; 1; 0; 14; {0, 3, 6, 9}. Table 6-2 Copy of Table 9.2.3-3: Mapping of values for one HARQ-ACK information bit to sequences for PUCCH format 0 HARQ-ACK Value; 0; 1. Sequence cyclic shift. Table 6-3 Copy of Table 9.2.3-4: Mapping of values for two HARQ-ACK information bits to sequences for PUCCH format 0 HARQ-ACK Value; {0, 0}; {0, 1}; {1, 1}; {1, 0}. Sequence cyclic shift. Table 6-4 Copy of Table 9.2.5-1: Mapping of values for one HARQ-ACK information bit and positive SR to sequences for PUCCH format 0 HARQ-ACK Value; 0; 1. Sequence cyclic shift. Table 6-5 Copy of Table 9.2.5-2: Mapping of values for two HARQ-ACK information bits and positive SR to sequences for PUCCH format 0 HARQ-ACK Value; {0, 0}; {0, 1}; {1, 1}; {1, 0}. Sequence cyclic shift.

Proposal 10：For a single sequence of length equal to the total number of mapped REs of the PUCCH resource, the cyclic shift should be adapted with the length of the sequence at least for PUCCH format 0/1.

The indication of PRB number

Agreement #RAN1 104-e: - FFS: Details of indication of NRB by cell-specific (for PF0/1) and dedicated signaling (PF0/1/4)

Indication of NRB by cell-specific (for PF0/1)

The indication of NRB for common PUCCH format 0/1 can be predefined in Table 6-1 or configured by SIB1. In Rel.15, the PUCCH frequency resource allocation is decided once the first PRB index is decided, because there is constant one PRB allocated for PUCCH 0/1. However, the PUCCH frequency resource for enhanced PUCCH 0/1 with multi-PRB should be dependent on not only the first PRB index but also the NRB. Moreover, the first PRB index is also dependent on the NRB.

For example, when the NRB indicated by SIB1 is 4, if rPUCCH8=0 ,	the UE determines the PRB index of the PUCCH transmission in the first hop as (RBBWPoffset+rPUCCHNCS)*4 and the PRB index of the PUCCH transmission in the second hop as NBWPsize-1-(RBBWPoffset-rPUCCHNCS)*4, where NCS is the total number of initial cyclic shift indexes in the set of initial cyclic shift indexes. If rPUCCH8=1 ,	the UE determines the PRB index of the PUCCH transmission in the first hop as NBWPsize-1-(RBBWPoffset-rPUCCHNCS)*4 and the PRB index of the PUCCH transmission in the second hop as (RBBWPoffset+rPUCCHNCS)*4.

Moreover, for sub-PRB interlace RE mapping when SCS is 120kHz, the RE pattern candidates should be indicated by SIB1. Then the specific sub-PRB mapping pattern is dynamically indicated to UE by DCI format.

Proposal 11: The indication of NRB for common PUCCH format 0/1 can be configured in the predefined table of PUCCH resource sets before dedicated PUCCH resource configuration or configured by SIB1.

Proposal 12: The PUCCH frequency resource and the first PRB index are dependent on the NRB.

Proposal 13: The sub-PRB interlace RE mapping pattern candidates should be configured by SIB1 and then dynamically indicated to UE by DCI format.

Indication of NRB by dedicated signaling (PF0/1/4)

For dedicated PUCCH resource, the frequency resource is RRC configured in Rel.15&Rel.16, so the NRB should be indicated by UE-specific RRC configuration. Similarity, the sub-PRB interlace RE mapping pattern can be also indicated by UE-specific RRC configuration. For example, the following IE is listed:

PUCCH-Resource ::=                      SEQUENCE {

pucch-ResourceId                        PUCCH-ResourceId,

startingPRB                             PRB-Id,

comboffset                                    {2}

comb                                          {4}

PRBnumofmultiPRB                              {2}

intraSlotFrequencyHopping               ENUMERATED { enabled }                                                OPTIONAL, -- Need R

secondHopPRB                            PRB-Id                                                                OPTIONAL, -- Need R

format                                  CHOICE {

format0                                 PUCCH-format0,

format1                                 PUCCH-format1,

format2                                 PUCCH-format2,

format3                                 PUCCH-format3,

format4                                 PUCCH-format4

}

}

Proposal 14: The indication of NRB (PF0/1/4) and sub-PRB interlace RE mapping pattern are by dedicated RRC signalling.

The shortage of common PUCCH resource

According to section 2.1.2.3, the maximum value of NRB is 8 when SCS is 120kHz for 400 MHz carrier. As shown in Table 6, for row index 0, 3,7,11, the PUCCH resource set is allocated 8 PRBs when one PRB is allocated for PUCCH format 0/1. Therefore, there are at least 64 PRBs requirement for the BWP so that there should be further design for the PUCCH resource sets before dedicated PUCCH resource configuration when the bandwidth of BWP is not enough. Table 7 is an example of additional SLIV for row index 0, and the PUCCH resource are listed below. The frequency resource is saved owing to the additional SLIV.

Table 7 The example of additional SLIV for index 0

| Index | PUCCH format | First symbol, Number of symbols | PRB offset | Set of initial CS indexes |
| --- | --- | --- | --- | --- |
| 0 | 0 | (12,2) (10,2) | 0 | {0, 3} |

| rPUCCH | PRB index for the first hop | CS | SLIV |  | rPUCCH | PRB index for the first hop | CS | SLIV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | （12,2） |  | 0 | 0 | 0 | （12,2） |
| 1 | 0 | 3 | （12,2） |  | 1 | 0 | 3 | （12,2） |
| 2 | 1 | 0 | （12,2） |  | 2 | 1 | 0 | （12,2） |
| 3 | 1 | 3 | （12,2） |  | 3 | 1 | 3 | （12,2） |
| 4 | 2 | 0 | （12,2） |  | 4 | 2 | 0 | （12,2） |
| 5 | 2 | 3 | （12,2） |  | 5 | 2 | 3 | （12,2） |
| 6 | 3 | 0 | （12,2） |  | 6 | 3 | 0 | （12,2） |
| 7 | 3 | 3 | （12,2） |  | 7 | 3 | 3 | （12,2） |
| 8 | NBWPsize-1 | 0 | （12,2） |  | 8 | NBWPsize-1 | 0 | （10,2） |
| 9 | NBWPsize-1 | 3 | （12,2） |  | 9 | NBWPsize-1 | 3 | （10,2） |
| 10 | NBWPsize-2 | 0 | （12,2） |  | 10 | NBWPsize-2 | 0 | （10,2） |
| 11 | NBWPsize- 2 | 3 | （12,2） |  | 11 | NBWPsize- 2 | 3 | （10,2） |
| 12 | NBWPsize- 3 | 0 | （12,2） |  | 12 | NBWPsize- 3 | 0 | （10,2） |
| 13 | NBWPsize- 3 | 3 | （12,2） |  | 13 | NBWPsize- 3 | 3 | （10,2） |
| 14 | NBWPsize- 4 | 0 | （12,2） |  | 14 | NBWPsize- 4 | 0 | （10,2） |
| 15 | NBWPsize- 4 | 3 | （12,2） |  | 15 | NBWPsize- 4 | 3 | （10,2） |

Proposal 15: The additional SLIV or OCC should be included in the PUCCH resource sets before dedicated PUCCH resource configuration.

- Conclusion

In this contribution, we focus on the evaluation methodology, result analysis and the enhancement methods on the increasing of the number of RBs of PUCCH format 0/1/4, and have the following observations and proposals:

Observation 1: The case 3 and case 4 have worse MIL performance than other multi-PRB cases in US and South Korea, and have worse MIL performance than one PRB case in Europe.

Observation 2: For enhanced PUCCH format 0, when SCS is 480kHz or 960kHz and PRB number is 2, the MIL of case 1 increases more than that of case 2 in US and South Korea, and the case 2 does not have MIL gain compared with case 0 while case 1 has slight MIL gain compared with case 0 in Europe.

Observation 3: For enhanced PUCCH format 0, when SCS is 120kHz and PRB number is 2, the MIL of case 1 is higher than that of case 2 in all regions. Similarly, the MIL of case 5 is obviously higher than that of case 6 in South Korea 1 when delay spread is 5ns or 10ns and in Europe when delay spread is 20ns. While the MIL of case 5 is slightly lower than that of case 6 in other situations.

Observation 4: For enhanced PUCCH format 1, when SCS is 480kHz and PRB number is 2, the MIL of case 1 increases more than that of case 2 in US and South Korea. While in Europe, the MIL of case 2 do not increase and the MIL of case 1 decrease in Europe either when delay spread is 20ns, and the MIL of case1 slightly increases when delay spread is 5ns or 10ns.

Observation 5: For enhanced PUCCH format 1, when SCS is 960kHz and PRB number is 2, the MIL of case 2 decreases in all regions, and the MIL of case 1 decreases in all regions when delay spread is 20ns. When delay spread is 5ns or 10ns, the MIL of case 1 increases in US and South Korea but decreases in Europe.

Observation 6: For enhanced PUCCH format 1, when SCS is 120kHz and PRB number is 2, in US and South Korea, the MIL of case 1 and case 2 are close to each other when delay spread is 10ns or 20ns, while the MIL of case 2 have slight increase than that of case 1 when delay spread is 5ns. Similarly, the MIL of case 6 is lower than that of case 5 when delay spread is 5ns or 10ns, while higher when delay spread is 20ns. In Europe, the MIL of case 1 and case 5 is obviously higher than that of case 2 and case 6 respectively.

Observation 7: For enhanced PUCCH format 0 with 120kHz SCS, the MIL of case 1 is slightly higher than that of case 5 when PRB number is 2. While the MIL of case 5 increase significantly when PRB number is 4 or 8. Especially, compared with case 0, the MIL of case 1 decreases and case 5 increases when PRB number is 4 in Europe.

Observation 8: For enhanced PUCCH format 1 with 120kHz SCS, the MIL of case 1 is lower than that of case 5 when delay spread is 5ns in US and South Korea, slightly higher in other cases when PRB number is 2. The MIL of case 1 is higher than that of case 5 in US and Korea when PRB number is 4, while decreases in Europe. While the MIL of case 5 increase obviously when PRB number is 8. Especially, the MIL of case 1 decreases when PRB number is 8 in Europe.

Observation 9: In Europe, the MIL of case 1 and case 2 do not have gain compared with that of case 0 in Europe when SCS is 480kHz or 960kHz, the MIL of case 1 have more gain than that of case 2 when SCS is 120kHz.

Observation 10: In US and South Korea, the MIL of case 1 is obviously higher than that of case 2 when SCS is 960kHz, and minor difference between the two cases when SCS is 120kHz or 480kHz.

Observation 11: The MIL of alt 1 is obviously higher than that of alt 2 when payload is 11 bits or 22 bits with all SCS and when payload is 4 bits with SCS 960kHz.

Observation 12: When payload is 4 bits with SCS 120kHz, the MIL of alt 1 is higher than that of alt 2 when delay spread is 5ns or 10ns, slightly lower when delay spread is 20ns. When payload is 11 bits with SCS 480kHz, the MIL between the two alternatives have minor differences.

Proposal 1: For enhanced PUCCH format 0/1, the alt 1 of a single sequence of length equal to the total number of mapped REs of the PUCCH resource is preferred.

Proposal 2: For enhanced PUCCH format 0/1, for 120 kHz SCS, we support both of the two alternatives for RE mapping. Meanwhile, the UE multiplexing impact for MIL need further study.

Proposal 3: For UCI of enhanced PUCCH format 4, all REs within each RB are mapped is preferred for 120kHz SCS.

Proposal 4: For DMRS of enhanced PUCCH format 4, a single sequence of length equal to the total number of mapped REs of the PUCCH resource is preferred is preferred.

Proposal 5: For UCI of enhanced PUCCH format 4, we support alt 1 that the block-wise spreading is performed across all allocated RBs.

Proposal 6: The current OCC length for enhanced PUCCH format 4 is enough.

Proposal 7: The maximum value of NRB for each SCS of each PUCCH format 0/1/4 are listed in Table 4.

Proposal 8: The allowed values of NRB within the [min/max] range for each SCS of each PUCCH format 0/1/4 are listed in Table 5.

Proposal 9: The multiplexing of users with misaligned RB allocations should not be supported.

Proposal 10：For a single sequence of length equal to the total number of mapped REs of the PUCCH resource, the cyclic shift should be adapted with the length of the sequence at least for PUCCH format 0/1.

Proposal 11: The indication of NRB for common PUCCH format 0/1 can be configured in the predefined table of PUCCH resource sets before dedicated PUCCH resource configuration or configured by SIB1.

Proposal 12: The PUCCH frequency resource and the first PRB index are dependent on the NRB.

Proposal 13: The sub-PRB interlace RE mapping pattern candidates should be configured by SIB1 and then dynamically indicated to UE by DCI format.

Proposal 14: The indication of NRB (PF0/1/4) and sub-PRB interlace RE mapping pattern are by dedicated RRC signalling.

Proposal 15: The additional SLIV or OCC should be included in the PUCCH resource sets before dedicated PUCCH resource configuration.

- Appendix

Agreement #RAN1 104-e:

Tables 1, 2, and 3 in Section 2.3 of R1-2102127 are agreed as a common set of assumptions for link level simulations and link budget calculations for evaluating enhancements to PUCCH formats 0/1/4

Note: Other parameters can be additionally considered in the evaluations

Table 1: Simplified Evaluation Assumptions

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 60 GHz |
| Subcarrier Spacing [kHz] | 120, 480, 960 kHz |
| Number of usable RBs per carrier | 256 for 120 kHz SCS (corresponds to ~400 MHz carrier) 256 for 480 kHz SCS (corresponds to ~1600 MHz carrier) 160 for 960 kHz SCS (corresponds to ~2000 MHz carrier) Note: If other values used, companies to report values |
| PUCCH Frequency Hopping | On |
| PUCCH Frequency Domain Resource Mapping | N_RB contiguous RBs per hop (with all REs allocated per PRB) Note: If alternative RE allocation per PRB is used, companies to report details |
| Waveform | CP-OFDM for PF0/1 DFT-s-OFDM for PF4 |
| CP Type | Normal CP |
| Channel Model | TDL-A model as defined in of TR38.901 Section 7.7.2: - Delay spread (DS) = {5ns, 10ns, 20ns} - Optional: DS = 40ns |
| BS Antenna Configuration (Mg,Ng,M,N,P) | {1,1,1,1,2} |
| UE Antenna Configuration (Mg,Ng,M,N,P) | {1,1,1,1,1} |
| Mobility | 3 km/hr |
| PA Model | None |
| gNB TRP PN Model | Zero phase noise |
| UE PN Model | Zero phase noise |
| Pre-loaded Tx EVM | 0% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |

Table 2: Reporting metrics

| Parameter | Value | Notes |
| --- | --- | --- |
| PUCCH Format |  | PF0, PF1, PF4 |
| Subcarrier spacing, SCS [kHz] |  | SCS = {120, 480, 960} kHz |
| Frequency hopping details |  | Frequency offset between hops, |
| Number of RBs used per hop (N_RB) |  | N_RB contiguous RBs per hop |
| PUCCH bandwidth per hop, BW [MHz] |  | BW = N_RB * 12 * SCS / 1e6 |
| Number of OFDM symbols used for PUCCH resource |  | 1 or 2 for PF0 {4 .. 14} for PF1/4 |
| Sequence construction details |  | Sequence type for PF0/1 Sequence type for DMRS of PF4 |
| OCC configuration details |  | Applicable for PF1, PF4 |
| Cyclic shift configuration details |  | For PF0/1 For DMRS of PF4 |
| Number of multiplexed users, e.g., by code division, if applicable |  | 1 user Note: Companies to report if other cases if evaluated |
| PUCCH payload encoder type |  | Reed Muller or Polar for PF4 |
| PUCCH payload size(s) (bits) |  | For PF4, at least the following values should be considered: - Low: 4 bits - Moderate: 11 bits - High: 22 bits Maximum isotropic loss (see calculation below) to be reported for each PUCCH payload size |
| PUCCH encoding rate(s) |  | Applicable for PF4 If multiple payload sizes evaluated, encoding rates to be reported for each payload size |
| Required SNR (dB) |  | Required SNR needed to fulfil detection criterion, from link level simulations based on Table 1 (see Notes (1) and (2) at bottom of table for definition of detection criteria for PF 0/1/4). |
| Cubic Metric, CM (dB) |  | Reported value is the 95th percentile, i.e., the CM for which 95% of all sequences of the design fall below |
| UE Tx Beamforming gain (dBi) |  | TxBF = 6 dBi Notes: - TxBF includes antenna element gain - If other TxBF value(s) used, companies to report value(s) |
| BS Rx Beamforming gain (dBi) |  | RxBF = 20 dBi Notes: - RxBF includes antenna element gain - If other RxBF value(s) used, companies to report value(s) |
| UE Power Limitations |  | Maximum EIRP: UE_EIRP = 25 dBm Maximum conduced power (prior to consideration of backoff): UE_P = 21 dBm Optional: - UE_EIRP = 40dBm - UE_P = 21 dBm Note: Companies to report if other cases evaluated |
| Pmax (dBm) |  | Maximum allowed conducted power considering combined limit per region (from Table 3). Note:Companies should report if Pmax is considered per region or a combined limit is considered across multiple regions |
| Backoff (dB) |  | Power backoff is equal to the cubic metric, CM Note: If cubic metric is not used, information on the backoff metric used should be provided. |
| Transmit power, P_TX (dBm) |  | Maximum allowed transmit power including UE power limitation and backoff P_TX = min(Pmax, UE_EIRP – TxBF, UE_P – Backoff) |
| Noise power, P_N (dBm) |  | BS Noise Figure, NF = 7 dB Noise PSD = -174 dBm/Hz P_N = Noise PSD + 10*log10(BW * 1e6) + NF Note: BW is the PUCCH bandwidth per hop in MHz |
| Maximum Isotropic Loss, MIL (dB) |  | MIL = P_TX – P_N – Required SNR + TxBF + RxBF |
| Definition of detection criteria for PF0/1/4: (1) For PF0/1 (payload of 1 or 2 bits) the detection criterion assumes that the PUCCH payload consists of randomly drawn HARQ ACK/NACK bits and the criterion is defined as the SNR for which P(ACK to Error) ≤ 1% AND P(NACK to ACK) ≤ 0.1%. Error is defined as NACK or DTX where the decision region for DTX is determined to ensure that the maximum P(DTX to ACK) ≤ 1% for the case when the input to the receiver is noise only. (2) For PF4 (payload greater than 2 bits): the detection criterion is the UCI block error probability BLER ≤ 1% (as in TS38.104 Section 8.3.6) |  |  |

Table 3: Regulatory Power Limits by Region

| Region | Maximum Conducted Power, Pmax (dBm) |
| --- | --- |
| US | Conducted power limit due to EIRP limit: Pmax_EIRP = 40 dBm - TxBF Conducted power limit as a function of PUCCH BW per hop: Pmax_P = 27 dBm – max(0, 10*log10(100 / BW)) Combined limit: Pmax = min(Pmax_P, Pmax_EIRP) |
| Europe | Conducted power limit due to EIRP limit: Pmax_EIRP = 40 dBm – TxBF Conducted power limit due to PSD limit (assumes N_RB contiguous RBs with all REs allocated per PRB): Pmax_PSD = 23 dBm/MHz + max(0, 10*log10(BW)) - TxBF Combined limit: Pmax = min(Pmax_PSD, Pmax_EIRP) |
| South Korea | Conducted power limit due to EIRP limit: Pmax_EIRP = 43 dBm – TxBF when an equipment is >=300m from an astronomical antenna Pmax_EIRP = 27 dBm – TxBF when an equipment is <300m from an astronomical antenna Conducted power limit due to PSD limit (assumes N_RB contiguous RBs with all REs allocated per PRB): Pmax_PSD = 13 dBm/MHz + max(0, 10*log10(BW)) - TxBF Combined limit: Pmax = min(Pmax_PSD, Pmax_EIRP) |
| Other regions | … |
| Note: BW is the PUCCH bandwidth per hop in MHz |  |

Agreement #RAN1 104-e:

For enhanced (multi-RB) PUCCH Formats 0/1/4 for 120/480/960 kHz SCS, support allocation of N_RB contiguous RBs

- FFS: Values of N_RB for each SCS

- For 480/960 kHz SCS, all REs within each RB are mapped

  - Note: PRB and sub-PRB interlaced mapping is not considered further

- For 120 kHz SCS, further discuss the following two alternatives:

  - Alt-1: All REs within each RB are mapped

    - Note: PRB and sub-PRB interlaced mapping is not considered further

  - Alt-2: Subset of REs within each RB are mapped (sub-PRB interlaced mapping)

Agreement #RAN1 104-e:

- The configured number of RBs for enhanced PF 0/1/4 is denoted NRB

  - The minimum value of NRB is 1 for PF 0/1/4 for all subcarrier spacings

  - The maximum value of NRB depends on subcarrier spacing

    - FFS: maximum value for each SCS and each of PF0/1/4

  - FFS: Allowed values of NRB within the [min/max] range

  - FFS: Details of indication of NRB by cell-specific (for PF0/1) and dedicated signaling (PF0/1/4)

  - FFS: Whether or not multiplexing of users with misaligned RB allocations is supported, where "misaligned" also includes users with different # of RBs.

  - For PF4:

    - The actual number of RBs used for a PUCCH transmission is equal to NRB, i.e., the actual number of RBs does not vary dynamically based on PUCCH payload

    - NRB fulfils the following: N=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers

- Note: if frequency hopping is enabled, NRB is the number of RBs per hop

- Note: decisions on the maximum value of NRB for each SCS and PUCCH format shall take into account link budgets based at least on the agreed evaluation assumptions

Agreement #RAN1 104-e:

- For enhanced PF0/1, support Type-1 low PAPR sequences. Further study and strive to select one of the following alternatives:

  - Alt-1: A single sequence of length equal to the total number of mapped REs of of the PUCCH resource is used. Cyclic shifts for PF0/1 are defined in the same way as Rel-16 for the case that useInterlacePUCCH-PUSCH is not configured.

  - Alt-2: A single sequence of length equal to the number of mapped REs per RB of the PUCCH resource is used, and the sequence is repeated in each RB. At least the following scheme is considered for PAPR/CM reduction:

    - Cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured

- At least the following aspects should be considered in the study

  - Coverage (maximum isotropic loss (MIL)), including

    - Required SNR to fulfil PUCCH detection criterion

    - PAPR/CM as a function of N_RB

  - Specification impact

Agreement #RAN1 104-e:

- For DMRS of enhanced PF4, support Type-1 low PAPR sequences. Further study and strive to select one of the following alternatives for sequence construction:

  - Alt-1: A single sequence of length equal to the total number of mapped Res of of the PUCCH resource is used. Cyclic shifts are defined in the same was as Rel-15/16 for PF4.

  - Alt-2: A single sequence of length equal to the number of mapped Res per PRB of the PUCCH resource is used, and the sequence is repeated in each PRB. At least the following scheme is considered for PAPR/CM reduction:

    - Cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured

- At least the following aspects should be considered in the study

  - Coverage (maximum isotropic loss (MIL)), including

    - Required SNR to fulfil PUCCH detection criterion

    - PAPR/CM as a function of N_RB

  - Specification impact

Agreement #RAN1 104-e:

- For UCI of enhanced PF4, support pre-DFT blockwise spreading using OCCs of length 2 and 4 as defined for Rel-16 PF4

- Further study the following and decide in RAN1#104-b:

  - Whether or not additional OCC lengths are supported

  - Down-select to one of the following alternatives for blockwise spreading

    - Alt-1: Blockwise spreading is performed across all allocated RBs

    - Alt-2: Blockwise spreading and DFT is performed per-RB followed by per-RB PAPR/CM reduction mechanism.

- At least the following aspects should be considered in the study

  - Coverage (maximum isotropic loss (MIL)), including

    - Required SNR to fulfil PUCCH detection criterion

    - PAPR/CM as a function of N_RB

  - Specification impact

- References

- RP-202925, 3GPP TSG RAN Meeting #90-e, Electronic Meeting, 7-11 December 2020.

- Chairman Notes, 3GPP TSG RAN WG1 #104-e meeting.

- R1-1912937, 3GPP TSG RAN WG1 Meeting #99, November 18th – November 22nd, 2019

- 3GPP TS 38.211 V16.5.0 (2021-03), Physical channels and modulation.

- 3GPP TS 38.213 V16.5.0 (2021-03), Physical layer procedures for control.