3GPP TSG RAN WG1 Meeting #104bis-e	R1-2103919

e-Meeting, April 12– April  20, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1389 | rev | - | Current version: | V16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on MPDCCH assignment procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMTC-Core |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The value for determining the CCEs for any PDCCH candidate of Type1A-MPDCCH common search space and Type2A-MPDCCH common search space is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the definition of for Type1A-MPDCCH common search space and Type2A-MPDCCH common search space in the spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CCEs for Type1A-MPDCCH common search space and Type2A-MPDCCH common search space can not be obtained. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.1.5	MPDCCH assignment procedure

<Unchanged parts are omitted>

For aggregation level  or  ECCEs, the number of ECCEs refers to the MPDCCH mapping to the REs of the 2+4 PRB set as defined in [3]. An MPDCCH search space at aggregation level  and repetition level is defined by a set of MPDCCH candidates where each candidate is repeated in a set of  consecutive BL/CE downlink subframes starting with subframe . For an MPDCCH-PRB-set , the ECCEs corresponding to MPDCCH candidate m of the search space  are given by

where

,

is the number of MPDCCH candidates to monitor at aggregation level in MPDCCH-PRB-set  in each subframe in the set of  consecutive subframes.

for MPDCCH UE-specific search space is determined as described in Clause 9.1.4, and for Type0-MPDCCH common search space, Type1-MPDCCH common search space, Type1A-MPDCCH common search space, Type2-MPDCCH common search space and Type2A-MPDCCH common search space.

For , if subframe is a special subframe that does not support MPDCCH according to table 6.8B.1-1 in [3], the UE shall calculate  by assuming for normal cyclic prefix and for extended cyclic prefix.

<Unchanged parts are omitted>