## Study on NB-IoT/eMTC support for Non-Terrestrial Network

Please refer to RP-202689 for detailed scope of the WI

R1-2102753	FS_LTE_NBIOT_eMTC_NTN work plan	MediaTek Inc.

[post-104b-e-NR-NB_IoT_eMTC] Email discussion/approval on update to TR to capture latest agreements until Apr–26 - Gilles (MediaTek)

### Scenarios applicable to NB-IoT/eMTC

R1-2102343	Application scenarios of IoT in NTN	Huawei, HiSilicon

R1-2102422	Discussion on scenarios applicable to NB-IoT/eMTC	OPPO

R1-2102550	Discussion on scenarios applicable to NB-IoT_eMTC for NTN	vivo

R1-2102617	Applicable scenario and initial evaluation result to NB-IoT/eMTC	CATT

R1-2102750	Discussion on NTN-IoT scenario with MEO	Hughes/EchoStar

R1-2102754	Scenarios applicable to IoT NTN	MediaTek Inc.

R1-2102831	Link budget evaluations for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

R1-2102905	Discussion on scenarios applicable to NB-IoT/eMTC	CMCC

R1-2102916	Discussion on the scenarios and assumption for IoT-NTN	ZTE

R1-2102972	Discussion on the link budget of NBIoT and eMTC over NTN	Xiaomi

R1-2103060	On scenarios and evaluations for NB-IoT and eMTC based NTN	Ericsson

R1-2103070	Scenarios applicable to NB-IoT/eMTC 	Qualcomm Incorporated

R1-2103132	Link Budget Analysis of IoT NTN	Apple

R1-2103266	Initial link budget evaluation for NB-IoT/eMTC	Samsung

R1-2103318	Scenarios for IoT-NTN	Sony

R1-2103716	Link budget analysis for Set-4	Sateliot, Gatehouse, Thales

R1-2103826	Summary #1 of AI 8.15.1 Scenarios applicable to NB-IoT/eMTC Document for: Discussion and Decision		Moderator (MediaTek)

R1-2103962	Summary #3 of AI 8.15.1 Scenarios applicable to NB-IoT/eMTC Document for: Discussion and Decision		Moderator (MediaTek)

[104b-e-NR-NB_IoT_eMTC-01] Email discussion/approval on scenarios applicable to NB-IoT/eMTC with checkpoints for agreements on Apr-15, Apr-20 – Gilles (MediaTek)

Agreement:

Capture in TR 36.763 the summary of link budget results from contributing companies in Appendix 1, Section 6.1.1

NOTE 1: The summary in Appendix 1, Section 6.1.1 can be further checked and revised during the drafting of Text Proposal as necessary.

NOTE 2: The summary of link budget results will be captured with alignment between contributing companies

Agreement:

Capture the detailed link budget results from contributing companies in a separate spreadsheet

Agreement:

Capture parameters associated with Set 4 for maximum beam diameter of 1700 km in a separate table in TR 36.763:

NOTE: There is no impact on Table 6.1-1 : IoT NTN reference scenario parameters in TR 36.763

| 3GPP TR 36.763 V0.1.0 Table 6.1-1 parameters that could be impacted by the beam size revision: | Current values in TR 36.763 V0.1.0 for LEO 600 km | Assumed and computed values under the consideration of a beam of 1700 km in diameter pointed at Nadir: | Comment |
| --- | --- | --- | --- |
| Min Elevation angle for both sat-gateway and C-IoT device | 10° for service link and 10° for feeder link | 30° for service link and 10° for feeder link | Assumed value for service link is higher than value in Table 6.1-1 in TR 36.763. No revision needed. |
| Max distance between satellite and C-IoT device at min elevation angle | 1,932 km | 1075.8 km (Computed for a terminal located at the beam edge, corresponding to an elevation angle of 30 degrees) | Computed value is lower than current value in Table 6.1-1 in TR 36.763. No revision needed. |
| Max Round Trip Delay (propagation delay only) | 25.77 ms (service and feeder links) | 20.05 ms (Computed for a terminal located at the beam edge, corresponding to an elevation angle of 30 degrees. Feeder link elevation angle kept at 10º) | Computed value is lower than current value in Table 6.1-1 in TR 36.763. No revision needed. |
| Max differential delay within a cell | 3.12 ms | 1.58 ms (Computed as the maximum differential delay between a device at beam edge and one at beam center) | Computed value is lower than current value in Table 6.1-1 in TR 36.763. No revision needed. |
| Max Doppler shift variation (earth fixed user equipment) (NOTE 6)” | 24 ppm | 19,95 ppm (Computed for a terminal at beam edge, corresponding to an elevation angle of 30 degrees) | Computed value is lower than current value in Table 6.1-1 in TR 36.763. No revision needed. |

Agreement:

Include the following in TR36.763

Add MEO scenario D in Table 4.2-1 in TR 36.763.

Table 4.2-1: IoT NTN reference scenarios (TR 36.763)

| NTN Configurations | Transparent satellite |
| --- | --- |
| GEO based non-terrestrial access network | Scenario A |
| LEO based non-terrestrial access network generating steerable beams (altitude 1200 km and 600km) | Scenario B |
| LEO based non-terrestrial access network generating fixed beams whose footprints move with the satellite (altitude 1200 km and 600km) | Scenario C |
| MEO based non-terrestrial access network generating fixed beams whose footprints move with the satellite (altitude 10000 km) | Scenario D |

Agreement:

Add MEO IoT NTN reference scenario parameters in Table 6.1-1 in TR 36.763:

Table 6.1-1: IoT NTN reference scenario parameters (TR 36.763)

| Scenarios | GEO based non-terrestrial access network - scenario A | LEO based non-terrestrial access network -Scenario B & C | MEO based non-terrestrial access network -Scenario D |
| --- | --- | --- | --- |
| Orbit type | station keeping a nominally fixed position in terms of elevation/azimuth with respect to a given earth point | circular orbiting at low altitude around the earth | circular orbiting at medium altitude around the earth |
| Altitude | 35,786 km | 600 km 1,200 km | 10,000 km |
| Frequency Range | < 6 GHz (e.g. 2 GHz in S band) |  |  |
| Device channel Bandwidth (service link) (NOTE 7) | - NB-IoT 180 kHz (DL), Up to 180 kHz with all permissible smaller resource allocations 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz (UL) - eMTC: 1080 kHz (DL), Up to 1080 kHz with all permissible smaller resource allocations, including 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz (UL) |  |  |
| Payload | Transparent type | Transparent Type | Transparent type |
| Earth-fixed beams | Yes | Scenario B: Yes (steerable beams), see NOTE 1 Scenario C: No (the beams move with the satellite) | Scenario D: The beams move with the satellite |
| Max beam footprint size (edge to edge) regardless of the elevation angle | 3500 km (NOTE 3) | 1000 km (NOTE 2) | 4018 km |
| Min Elevation angle for both sat-gateway and C-IoT device | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and C-IoT device at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) | 14018 km |
| Max Round Trip Delay (propagation delay only) | 541.46ms (service and feeder links) | 25.77 ms (600km) (service and feeder links) 41.77 ms (1200km) (service and feeder links) | 95.19 ms (service and feeder links) |
| Max differential delay within a cell | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km | 13.4 ms |
| Max Doppler shift (earth fixed user equipment) (NOTE 6) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) | 7.5 ppm |
| Max Doppler shift variation (earth fixed user equipment) (NOTE 6) | 0.000 045 ppm/s | 0.27 ppm/s (600km) 0.13 ppm/s (1200km) | 0.003 ppm/s |
| C-IoT device motion on the earth | Min 0 km/s (stationary device), max 120 km/h | Min 0 km/s (stationary device), max 120 km/h | Min 0 km/s (stationary device), max 120 km/h |
| C-IoT device antenna types | Omnidirectional antenna with 0 dBi TX antenna gain and 0 dBi RX antenna gain (NOTE 4) |  |  |
| C-IoT device max Tx power | UE power class 3 with up to 200 mW (23dBm), UE power class 5 with up to 100 mW (20 dBm) |  |  |
| C-IoT device Noise Figure | Omnidirectional antenna: 7 dB or 9 dB (NOTE 5) |  |  |
| Service link | 3GPP defined Narrow Band IoT and eMTC |  |  |
| NOTE 1: Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite. NOTE 2: This beam size refers to the Nadir pointing of the satellite. NOTE 3: The Maximum beam footprint size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation) or a single wide-beam. NOTE 4: The use of a Circular polarized antenna is optional. NOTE 5: Same Noise Figure of 7 dB as in Release 16 TR 38.821 or 9 dB as in Release 12 TR 36.888 for device can be assumed for link budget. The noise figure is device vendor implementation specific. NOTE 6: Max Doppler shift and Max Doppler shift variation in the absence of any device pre-compensation of satellite Doppler shift on the service link. NOTE 7: System bandwidth is FFS |  |  |  |

Agreement:

Include MEO Set-5 parameters for link budget analysis in a new Table 6.2-8 in TR 36.763, as a representative characterization of NTN-IoT scenarios with MEO altitude and characteristics:

Table 6.2-8: Sets of satellite parameters for link budget and system level evaluations

|  | Proposed MEO Scenarios (Set 5) |
| --- | --- |
| Satellite orbit | MEO |
| Satellite altitude | 10,000 km |
| Payload characteristics for DL transmission |  |
| Frequency band | S-band (i.e. 2 GHz) |
| Equivalent satellite antenna aperture (NOTE1) | 1.5 m |
| Satellite EIRP density | 45.4 dBW/MHz |
| Satellite Tx max Gain | 28.1 dBi |
| 3dB beamwidth | 6.5 degrees |
| Satellite beam diameter (at nadir pointing) | 1140 km |
| Payload characteristics for UL reception |  |
| Frequency band | S-band (i.e. 2 GHz) |
| Equivalent satellite antenna aperture (NOTE1) | 1.5 m |
| G/T | 3.8 dB/K |
| Satellite Rx max Gain | 28.1 dBi |
| NOTE 1: This value is equivalent to the antenna diameter for the parabolic reflector modelled in Sec. 6.4.1 of TR 38.811. Other antenna models can be considered. |  |

Agreement:

Add MEO Set-5 satellite parameters for system level simulator calibration   in a new Table 6.2-9 in TR 36.763:

Table 6.2-9: Set-5 parameters for link budget analysis

| Set 5 | MEO |
| --- | --- |
| 3 dB Beam width (HPBW) | 6.5 degrees |
| Central beam center elevation | 90 degrees |
| Central beam edge elevation | 81.6 degrees |
| Central beam edge satellite-UE distance | 10042 km |

Agreement:

Add observation in TR 36.763:

The doppler shift/variation and the delay variation for MEO are smaller than for LEO. The maximum delay for MEO is smaller than for GEO. The IoT-NTN enhancements for LEO and GEO should be sufficient to support MEO.

NOTE: The parameter set for MEO is only for information/reference and evaluation/enhancements are mainly considered for GEO and LEO. These enhancements can be applicable for MEO.

### Enhancements to time and frequency synchronization

Including random access procedure/signals

R1-2102758	"Summary #1 of AI 8.15.2  Enhancements to time and frequency

synchronization"	MediaTek Inc.

R1-2102344	Discussion on time and frequency synchronization enhancement for IoT in NTN	Huawei, HiSilicon

R1-2102423	Discussion on enhancements to time and frequency synchronization	OPPO

R1-2102473	Consideration on enhancements to time and frequency synchronization	Spreadtrum Communications

R1-2102618	Time and frequency synchronization for NB-IoT/eMTC	CATT

R1-2102736	Enhancements to time and frequency synchronization	Asia Pacific Telecom, FGI, ITRI, III

R1-2102755	Enhancements to time and frequency synchronization for IoT NTN	MediaTek Inc.

R1-2102832	Enhancement to time and frequency synchronization for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

R1-2102906	Enhancements to time and frequency synchronization for IoT  NTN	CMCC

R1-2102917	Discussion on the synchronization for IoT-NTN	ZTE

R1-2102973	Discussion on time and frequency synchronization for IoT NTN	Xiaomi

R1-2103056	On synchronization for NB-IoT and eMTC NTN	Intel Corporation

R1-2103061	On time and frequency synchronization enhancements for IoT NTN	Ericsson

R1-2103071	Enhancements to time and frequency synchronization	Qualcomm Incorporated

R1-2103133	On Time and Frequency Synchronization in IoT NTN	Apple

R1-2103267	On enhancements to time and frequency synchronization	Samsung

R1-2103273	Time/Frequency Synchronization for IoT NTN	InterDigital, Inc.

R1-2103319	Time and frequency synchronisation enhancements for IoT-NTN	Sony

R1-2103528	Time and frequency synchronization for IoT NTN	Lenovo, Motorola Mobility

R1-2102758	Summary #1 of AI 8.15.2 	Enhancements to time and frequency synchronization	Moderator (MediaTek)

R1-2103908	Summary #3 of AI 8.15.2 	Enhancements to time and frequency synchronization	Moderator (MediaTek)

R1-2103950	Summary #3 of AI 8.15.2 	Enhancements to time and frequency synchronization	Moderator (MediaTek)

R1-2103964	Summary #4 of AI 8.15.2 	Enhancements to time and frequency synchronization	Moderator (MediaTek)

[104b-e-NR-NB_IoT_eMTC-02] Email discussion/approval on enhancements to time and frequency synchronization with checkpoints for agreements on Apr-15, Apr-20 – Gilles (MediaTek)

Agreement:

- Capture in TR 36.763, moderator’s summary of GNSS Position fix impact on UE power consumption based on Appendix A Section 5.1

- Capture in TR 36.763, individual companies battery life analysis in Appendix A

Agreement:

UE pre-compensation done per N time units for long PUSCH is the baseline solution.

- The pre-compensation does not vary within a block of N time units

- FFS: the definition and value of N

Agreement:

UE pre-compensation done per N time units for long PRACH is the baseline solution.

- The pre-compensation does not vary within a block of N time units

- FFS: the definition and value of N

Agreement:

For DL synchronization in the Rel-17 timeframe, the following should be considered

- New Channel raster with a step size increased to be greater than 100 kHz

- (part of) ARFCN-indication-in-MIB

Agreement:

Capture the following in the TR:

The required power consumption to read SIB containing satellite ephemeris information for the short sporadic connections use case is not significant.

- Note: For this conclusion, it is assumed that the UE need not read broadcast SIB for the purpose of obtaining satellite ephemeris information in CONNECTED mode.

### Timing relationship enhancements

R1-2102345	Discussion on timing relationship enhancement for IoT in NTN	Huawei, HiSilicon

R1-2102424	Discussion on timing relationship enhancements	OPPO

R1-2102474	Consideration on timing relationship enhancements	Spreadtrum Communications

R1-2102619	Timing relationship enhancement for NB-IoT/eMTC	CATT

R1-2102737	Timing relationship enhancements	Asia Pacific Telecom, FGI, ITRI, III

R1-2102756	Timing relationship enhancements for IoT NTN	MediaTek Inc.

R1-2102800	Timing relationship enhancements to support NB-IoT eMTC in Non-Terrestrial Network	Zhejiang Lab

R1-2102833	Timing relationship enhancements for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

R1-2102907	Timing relationship enhancements  for IoT  NTN	CMCC

R1-2102918	Discussion on timing relationship for IoT-NTN	ZTE

R1-2102974	Discussion on the timing relationship enhancement for IoT NTN	Xiaomi

R1-2103057	On timing relationship for NB-IoT and eMTC NTN	Intel Corporation

R1-2103062	On timing relationship enhancements for IoT NTN	Ericsson

R1-2103072	Timing relationship enhancements	Qualcomm Incorporated

R1-2103134	On Timing Relationship Enhancement in IoT NTN	Apple

R1-2103268	Timing relationship enhancements	Samsung

R1-2103274	Timing relationship enhancement for IoT NTN	InterDigital, Inc.

R1-2103320	Timing relationships for IoT-NTN	Sony

R1-2103529	Timing relationship enhancements for IoT NTN	Lenovo, Motorola Mobility

R1-2103847	FL summary #1 of AI 8.15.3 Timing relationship for IoT-NTN		Moderator (Sony)

R1-2103887	FL summary #2 of AI 8.15.3 Timing relationship for IoT-NTN		Moderator (Sony)

R1-2103957	FL summary #3 of AI 8.15.3 Timing relationship for IoT-NTN		Moderator (Sony)

R1-2104083	FL summary #4 of AI 8.15.3: Timing relationship for IoT-NTN	Moderator (Sony)

[104b-e-NR-NB_IoT_eMTC-03] Email discussion/approval on timing relationship enhancements with checkpoints for agreements on Apr-15, Apr-20 – Sam (Sony)

Agreement:

The following NB-IoT timing relationships need enhancing for essential minimum functionality of IoT NTN:

- NPDCCH to NPUSCH format 1

- RAR grant to NPUSCH format 1

- NPDSCH to HARQ-ACK on NPUSCH format 2

- Timing advance command activation

- FFS: NPDCCH order to NPRACH

- FFS: Other NB-IoT timing relationships

Agreement:

The enhancement based on extending the timing relationship, by e.g. Koffset, adopted in NR NTN should be the starting point for enhancement of NB-IoT timing relationships in IoT NTN. Details can be further discussed considering IoT NTN.

Agreement:

The following eMTC timing relationships need enhancing for essential minimum functionality of IoT NTN:

- MPDCCH to PUSCH

- RAR grant to PUSCH

- MPDCCH to scheduled uplink SPS

- PUSCH to HARQ-ACK on PUCCH

- CSI reference resource timing

- MPDCCH to aperiodic SRS

- Timing advance command activation

- FFS: MPDCCH order to PRACH

- FFS: Other eMTC timing relationships

Agreement:

The enhancement based on extending the timing relationship, by e.g. Koffset, adopted in NR NTN should be the starting point for enhancement of eMTC timing relationships in IoT NTN. Details can be further discussed considering IoT NTN.

Agreement:

For NB-IoT over NTN, the following timing relationship needs to be studied to check whether enhancement is necessary and beneficial:

- PRACH preamble retransmission

Agreement:

For eMTC over NTN, the following timing relationship needs to be studied to check whether enhancement is necessary and beneficial:

- PRACH preamble retransmission

Agreement:

Capture the following in the TR:

The UE-specific TA and/or K_offset can be used by the eNB in its scheduling to avoid UL-DL collisions in FDD-HD.

Agreement:

The following aspects of Koffset are not to be studied further and can at least rely on decisions made in the NR NTN WI:

- Explicit or implicit indication in system information

- Support UE-specific Koffset after initial access

### Enhancements on HARQ

R1-2102346	Discussion on HARQ enhancement for IoT in NTN	Huawei, HiSilicon

R1-2102425	Discussion on HARQ enhancements	OPPO

R1-2102475	Consideration on enhancements on HARQ	Spreadtrum Communications

R1-2102551	Discussion on HARQ enhancements on NB-IoT/eMTC for NTN	vivo

R1-2102620	HARQ operation enhancement for NB-IoT/eMTC	CATT

R1-2102738	Enhancements on HARQ	Asia Pacific Telecom, FGI, ITRI, III

R1-2102757	Enhancements on HARQ for IoT NTN	MediaTek Inc.

R1-2102834	HARQ for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

R1-2102908	Enhancements on HARQ for IoT  NTN	CMCC

R1-2102919	Discussion on HARQ for IoT-NTN	ZTE

R1-2102975	Discussion on the HARQ enhancement for IoT NTN	Xiaomi

R1-2103063	On HARQ enhancements for IoT NTN	Ericsson

R1-2103073	Enhancements on HARQ	Qualcomm Incorporated

R1-2103135	On HARQ Enhancement in IoT NTN	Apple

R1-2103269	On enhancements on HARQ	Samsung

R1-2103275	HARQ enhancement for IoT NTN	InterDigital, Inc.

R1-2103321	HARQ issues for IoT-NTN	Sony

R1-2103530	HARQ enhancement for IoT NTN	Lenovo, Motorola Mobility

R1-2103803	Summary#1 of enhancements on HARQ	Moderator (Samsung)

R1-2103804	Summary#2 of enhancements on HARQ	Moderator (Samsung)

R1-2103805	Summary#3 of enhancements on HARQ	Moderator (Samsung)

[104b-e-NR-NB_IoT_eMTC-04] Email discussion/approval on enhancements on HARQ with checkpoints for agreements on Apr-15, Apr-20 – Carmela (Samsung)

Agreement:

Increasing the number of HARQ processes for NB-IoT and for eMTC in NTN is recommended not to be supported in Rel-17.

### Others

R1-2102426	Discussion on other aspects	OPPO

R1-2102476	Consideration on other design aspects for IOT NTN	Spreadtrum Communications

R1-2102835	Applicability of NB-IoT/eMTC features for NTN	Nokia, Nokia Shanghai Bell

R1-2102920	Discussion on additional enhancement for IoT-NTN	ZTE

R1-2102976	Discussion on the other design aspects for IoT NTN	Xiaomi

R1-2103064	On evaluation assumptions for NB-IoT and eMTC based NTN	Ericsson

R1-2103074	Other aspects for NTN IOT	Qualcomm Incorporated

R1-2103396	Other aspects to support IoT in NTN	Huawei, HiSilicon