## Supporting NR from 52.6GHz to 71 GHz

Please refer to RP-210862  for detailed scope of the WI

### Initial access aspects

Including SSB, initial BWP, PRACH, etc.

R1-2102327	Initial access signals and channels for 52-71GHz spectrum	Huawei, HiSilicon

R1-2102385	Discussion on initial access aspects	OPPO

R1-2102448	Discussion on initial access aspects for NR for 60GHz	Spreadtrum Communications

R1-2102514	Discussions on initial access aspects for NR operation from 52.6GHz to 71GHz	vivo

R1-2102558	Initial access aspects	Nokia, Nokia Shanghai Bell

R1-2102621	Initial access aspects for up to 71GHz operation	CATT

R1-2102688	Discussion on initial access of 52.6-71 GHz NR operation	MediaTek Inc.

R1-2102715	Considerations on initial access for NR from 52.6GHz to 71 GHz	Fujitsu

R1-2102772	Further considerations on initial access for additional SCS in Beyond 52.6GHz	FUTUREWEI

R1-2102788	Initial Access Aspects	Ericsson

R1-2102977	On initial access aspects for NR from 52.6GHz to 71GHz	Xiaomi

R1-2102996	Initial access aspects for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2103021	Discussion on initial access aspects for extending NR up to 71 GHz	Intel Corporation

R1-2103096	Discussion on Initial access signals and channels	Apple

R1-2103157	Initial access aspects for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2103229	Initial access aspects for NR from 52.6 GHz to 71 GHz	Samsung

R1-2103294	Considerations on initial access aspects for NR from 52.6 GHz to 71 GHz	Sony

R1-2103339	Initial access aspects to support NR above 52.6 GHz	LG Electronics

R1-2103411	NR Initial Access from 52.6 GHz to 71 GHz	Convida Wireless

R1-2103442	Further Discussion of Initial Access Aspects	AT&T

R1-2103448	Discussions on initial access aspects	InterDigital, Inc.

R1-2103472	Initial access aspects	Sharp

R1-2103487	Discussion on the initial access aspects for 52.6 to 71GHz	ZTE, Sanechips

R1-2103519	Discussion on initial access aspects supporting NR from 52.6 to 71 GHz	NEC

R1-2103567	Initial access aspects for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2103691	Discussion on initial access aspects for NR beyond 52.6GHz	WILUS Inc.

R1-2103801	Issue Summary for initial access aspects of NR extension up to 71 GHz	Moderator (Intel Corporation)

[104b-e-NR-52-71GHz-01] Email discussion/approval on initial access aspects with checkpoints for agreements on Apr-15, Apr-20 – Daewon (Intel)

Agreement:

For the case where SSB location and SCS are explicitly provided to the UE (non-initial access) and SSB does not configure Type-0 PDCCH, support 480 kHz and 960 kHz numerologies for the SSB

- Note: Strive to minimize specification impact due to the new SCS for SSB

Proposal:

For initial access,

- no new SCSs

- support 480 kHz or 960 kHz only

- support both 480 kHz and 960 kHz

- support 240 kHz, 480 kHz and 960 kHz

### PDCCH monitoring enhancements

R1-2102328	Enhancement on PDCCH monitoring	Huawei, HiSilicon

R1-2102386	Discussion on PDCCH monitoring	OPPO

R1-2102449	Discussion on PDCCH monitoring enhancement for NR beyond 52.6 GHz	Spreadtrum Communications

R1-2102515	Discussions on PDCCH monitoring enhancements for NR operation from 52.6GHz to 71GHz			vivo

R1-2102559	PDCCH monitoring enhancements	Nokia, Nokia Shanghai Bell

R1-2102622	PDCCH monitoring enhancements for up to 71GHz operation	CATT

R1-2102704	PDCCH monitoring enhancement for 52.6-71 GHz NR operation	MediaTek Inc.

R1-2102773	Further considerations on PDCCH monitoring enhancements	FUTUREWEI

R1-2102789	PDCCH Monitoring Enhancements	Ericsson

R1-2102809	PDCCH monitoring enhancement for NR from 52.6GHz to 71GHz	Panasonic

R1-2102978	PDCCH monitoring enhancement for NR 52.6-71GHz	Xiaomi

R1-2102997	PDCCH monitoring enhancements for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2103022	Discussion on PDCCH monitoring enhancements for extending NR up to 71 GHz	Intel Corporation

R1-2103097	Discussion on PDCCH Enhancements for above 52.6 GHz	Apple

R1-2103158	PDCCH monitoring enhancements for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2103230	PDCCH monitoring enhancements for NR from 52.6 GHz to 71 GHz	Samsung

R1-2103295	PDCCH enhancement for NR from 52.6GHz to 71GHz	Sony

R1-2103340	PDCCH monitoring enhancements to support NR above 52.6 GHz	LG Electronics

R1-2103412	PDCCH Monitoring Considerations for Supporting NR from 52.6 GHz to 71 GHz	Convida Wireless

R1-2103449	Discussions on PDCCH monitoring enhancements	InterDigital, Inc.

R1-2103488	Discussion on the PDCCH monitoring enhancements for 52.6 to 71GHz	ZTE, Sanechips

R1-2103512	Discussion on PDCCH monitoring enhancements supporting NR from 52.6GHz to 71 GHz			NEC

R1-2103568	PDCCH monitoring enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2103689	Feature lead summary for B52.6 GHz PDCCH monitoring enhancements	Moderator (Lenovo)

[104b-e-NR-52-71GHz-02] Email discussion/approval on PDCCH monitoring enhancements with checkpoints for agreements on Apr-15, Apr-20 – Alex (Lenovo)

Agreement:

Previous agreement is modifed as follows:

Choose one of the following alternatives for defining the multi-slot PDCCH monitoring capability

- Alt 1: Use a fixed pattern of slot groups as the baseline to define the new capability.

  - Each slot group consists of X slots

  - Slot groups are consecutive and non-overlapping

  - The capability indicates the BD/CCE budget within Y consecutive [symbols or slots] in each slot group separately

  - FFS: Supported values/constraints of X and Y, e.g. Y<=X, Y=X

  - FFS: Restrictions on location of the Y [symbols or slots] within a slot group, e.g. the Y [symbols or slots] always start at the first slot within a slot group

  - FFS: Further definition of capabilities

- Alt 2: Use an (X, Y) span as the baseline to define the new capability

  - X is the minimum time separation between the start of two consecutive spans

  - The capability indicates the BD/CCE budget within a span of at most Y consecutive [symbols or slots]

  - Y <= X

  - FFS: Exact values of X and Y and units in which they are defined (e.g., symbols, slots), including cases where a span is longer than one slot or crosses a slot boundary.

  - FFS: What is a span pattern, how it is defined and whether it is supported. If it is supported, whether number of slots within which the span pattern is repeated is needed, and if needed, the value of the number of slots.

  - FFS: Further definition of capabilities

- Alt 3: Use a sliding window of X slots as the baseline to define the new capability.

  - The capability indicates the BD/CCE budget within the sliding window

  - The sliding unit of the sliding window is [1] slot.

  - FFS: Further definition of capabilities

- Specific numbers for X, Y may depend on UE capability and gNB configuration

  - Examples:

    - X = [4] slots for 480 kHz SCS and X = [8] slots for 960 kHz SCS

### Enhancements for PUCCH formats 0/1/4

R1-2102329	Enhancement on PUCCH formats	Huawei, HiSilicon

R1-2102387	Discussion on enhancements for PUCCH format 0/1/4	OPPO

R1-2102450	Discussion on enhancements for PUCCH format 0/1/4 for above 52.6GHz	Spreadtrum Communications

R1-2102516	Discussions on PUCCH enhancements for NR operation from 52.6GHz to 71GHz	vivo

R1-2102560	Enhancements for PUCCH formats	Nokia, Nokia Shanghai Bell

R1-2102623	Enhancements for PUCCH formats for up to 71GHz operation	CATT

R1-2102706	On Enhancements for PUCCH formats 0/1/4	MediaTek Inc.

R1-2102774	Considerations on PUCCH Channel enhancements	FUTUREWEI

R1-2102790	PUCCH enhancements	Ericsson

R1-2102998	Enhancements to PUCCH formats 0/1/4 for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2103023	Discussion on PUCCH enhancements for extending NR up to 71 GHz	Intel Corporation

R1-2103098	Discussion on Enhancements for PUCCH formats 0/1/4	Apple

R1-2103159	Enhancements for PUCCH for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2103231	Enhancements for PUCCH format 0/1/4 for NR from 52.6 GHz to 71 GHz	Samsung

R1-2103296	Considerations on enhancements for PUCCH formats 0/1/4	Sony

R1-2103341	Enhancements for PUCCH formats 0/1/4 to support NR above 52.6 GHz	LG Electronics

R1-2103450	Discussions on enhancements for PUCCH formats 0/1/4	InterDigital, Inc.

R1-2103489	Discussion on the PUCCH enhancements for 52.6 to 71GHz	ZTE, Sanechips

R1-2103569	PUCCH format 0/1/4 enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2103692	Discussion on PUCCH enhancement for PUCCH format 0/1/4	WILUS Inc.

R1-2103784	FL Summary for Enhancements for PUCCH formats 0/1/4	Moderator (Ericsson)

[104b-e-NR-52-71GHz-03] Email discussion/approval on PUCCH format 0/1/4 enhancements with checkpoints for agreements on Apr-15, Apr-20 – Steve (Ericsson)

Agreement:

- The maximum values for the configured number of RBs, NRB, for enhanced PF0/1/4 are at least:

  - 12 RBs for 120 kHz SCS

  - 3 RBs for 480 kHz SCS

  - 2 RBs for 960 kHz SCS

- FFS: Whether or not the above values need to be revised to support larger values (and any associated signaling impact), e.g., to support lower UE Tx beamforming gain and/or larger UE EIRP and conducted power limits for different UE power classes, different from those in the agreed evaluation assumptions

### Beam management for new SCSs

Including timing associated with beam-based operation

R1-2102330	Discussion on the beam management procedures for 52-71GHz spectrum	Huawei, HiSilicon

R1-2102388	Discussion on beam management	OPPO

R1-2102451	Discussion on beam manangement for above 52.6GHz	Spreadtrum Communications

R1-2102517	Discussions on beam management for new SCSs for NR operation from 52.6GHz to 71GHz			vivo

R1-2102561	Beam Management Aspects	Nokia, Nokia Shanghai Bell

R1-2102624	Beam management for new SCSs for up to 71GHz operation	CATT

R1-2102705	Beam management discussion for 52.6-71 GHz NR operation	MediaTek Inc.

R1-2102775	Beam management for shared spectrum access in Beyond 52.6GHz	FUTUREWEI

R1-2102791	Beam Management for New SCSs	Ericsson

R1-2102979	Discussion on beam management in NR from 52.6 GHz to 71GHz	Xiaomi

R1-2102999	Beam-management enhancements for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2103024	Discussion on Beam management aspects for extending NR up to 71 GHz	Intel Corporation

R1-2103099	Beam Management for New SCSs	Apple

R1-2103160	Beam management for new SCS for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2103232	Beam management for new SCSs for NR from 52.6 GHz to 71 GHz	Samsung

R1-2103297	Beam management enhancement for NR from 52.6GHz to 71GHz	Sony

R1-2103342	Enhancements for beam management to support NR above 52.6 GHz	LG Electronics

R1-2103413	On Beam Management for Supporting NR from 52.6 GHz to 71 GHz	Convida Wireless

R1-2103451	Discussions on beam management for new SCSs	InterDigital, Inc.

R1-2103490	Discussion on the beam management for 52.6 to 71GHz	ZTE, Sanechips

R1-2103570	Beam based operation for new SCSs for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2103821	FL Summary for Beam Management for new SCSs	Moderator (InterDigital, Inc.)

[104b-e-NR-52-71GHz-04] Email discussion/approval on beam management for new SCSs with checkpoints for agreements on Apr-15, Apr-20 – Youngwoo (InterDigital)

Proposal:

For timeDurationForQCL, beamSwitchTiming and beamReportTiming,

- Following candidate values of FR2 are reused for 120 kHz:

  - timeDurationForQCL: 14 and 28 symbols

  - beamSwitchTiming: 14, 28, 48, 224 and [336] symbols

  - beamReportTiming: 14, 28 and 56 symbols

- Reuse the absolute time duration defined for 120kHz as the maximum reportable value for 480 kHz and 960 kHz

  - Down select one of the following alternatives for UE capability indication method used to report the values

    - Alt-1: UE reports preferred values in number of symbols

    - Alt-2: Introduce a factor to scale the reference values of 120kHz for 480 kHz and 960 kHz respectively

  - FFS: Whether absolute time duration defined for 480 kHz and 960 kHz can be further reduced

### PDSCH/PUSCH enhancements

Including defining maximum bandwidth for new SCSs, time line related aspects adapted to each of the new numerologies 480kHz and 960kHz, reference signals, scheduling particularly w.r.t. multi-PDSCH/PUSCH with a single DCI, HARQ, etc.

R1-2102331	PDSCH/PUSCH enhancements for 52-71GHz spectrum	Huawei, HiSilicon

R1-2102389	Discussion on PDSCH/PUSCH enhancements	OPPO

R1-2102452	Discussion on PDSCH and PUSCH enhancements for above 52.6GHz	Spreadtrum Communications

R1-2102518	Discussions on PDSCH/PUSCH enhancements for NR operation from 52.6GHz to 71GHz			vivo

R1-2102562	PDSCH/PUSCH enhancements	Nokia, Nokia Shanghai Bell

R1-2102569	Discussions on scheduling enhancements for PDSCH and PUSCH	CAICT

R1-2102625	PDSCH/PUSCH enhancements for up to 71GHz operation	CATT

R1-2102716	Considerations on multi-PDSCH/PUSCH with a single DCI and HARQ for NR from 52.6GHz to 71 GHz	Fujitsu

R1-2102776	Considerations on PDSCH/PUSCH enhancements	FUTUREWEI

R1-2102792	PDSCH-PUSCH Enhancements	Ericsson

R1-2102980	PDSCH and PUSCH enhancements for NR 52.6-71GHz	Xiaomi

R1-2103000	PDSCH/PUSCH scheduling enhancements for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2103012	PT-RS enhancements for NR from 52.6GHz to 71GHz	Mitsubishi Electric RCE

R1-2103025	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

R1-2103100	Discussion on PDSCH/PUSCH enhancements for above 52.6 GHz	Apple

R1-2103161	PDSCH/PUSCH enhancements for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2103233	PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz	Samsung

R1-2103298	PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz	Sony

R1-2103343	PDSCH/PUSCH enhancements to support NR above 52.6 GHz	LG Electronics

R1-2103407	Discussion on PDSCH and PUSCH enhancements for 52.6GHz – 71GHZ band	CEWiT

R1-2103414	PDSCH Considerations for Supporting NR from 52.6 GHz to 71 GHz	Convida Wireless

R1-2103452	Discussions on PDSCH/PUSCH enhancements for 52.6 GHz to 71 GHz Band	InterDigital, Inc.

R1-2103463	Discussion on multi-PDSCH/PUSCH scheduling for NR 52.6-71 GHz	Panasonic Corporation

R1-2103491	Discussion on the data channel enhancements for 52.6 to 71GHz	ZTE, Sanechips

R1-2103513	Discussion on PDSCH enhancements supporting NR from 52.6GHz to 71 GHz	NEC

R1-2103571	PDSCH/PUSCH enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2103693	Discussion on multi-PDSCH/PUSCH scheduling for NR from 52.6GHz to 71GHz	WILUS Inc.

R1-2103726	PDSCH-PUSCH Enhancement Aspects for NR beyond 52.6 GHz	Charter Communications

R1-2103818	Discussion summary #1 of [104b-e-NR-52-71GHz-05]	Moderator (vivo)

[104b-e-NR-52-71GHz-05] Email discussion/approval on defining maximum bandwidth for new SCSs, timeline related aspects adapted to each of the new numerologies 480kHz and 960kHz and reference signals with checkpoints for agreements on Apr-16, Apr-20 – Huaming (Vivo)

Agreement:

A model-based approach is not used to derive the timelines for single PDSCH/PUSCH and multi-PDSCH/PUSCH scheduling for NR operation in 52.6 GHz to 71 GHz.

Agreement:

Continue study at least the following aspects for potential PTRS enhancement for DFT-s-OFDM for NR operation in 52.6 to 71 GHz

- The need of potential PTRS enhancement

- PTRS pattern with more PTRS groups within one DFT-s-OFDM symbol when a large number of PRBs is scheduled

  - (Ng = 8, Ns = 4, L = 1), (Ng = 16, Ns = 2, L = 1), (Ng = 16, Ns = 4, L = 1),

  - Note: Ng number of PT-RS groups, Ns number of samples per PT-RS group, and PTRS every L number of DFT-s-OFDM symbols

  - Other patterns are not precluded

- Other aspects of PTRS enhancements are not precluded from further study

Proposal:

At least for PDSCH processing time (N1), PUSCH preparation time (N2) and HARQ-ACK multiplexing timeline (N3), RAN1 strives to define a single value per SCS. This single value per SCS applies to both single PDSCH/PUSCH and multi-PDSCH/PUSCH scheduling for NR operation in 52.6 GHz to 71 GHz.

[104b-e-NR-52-71GHz-06] Email discussion/approval on scheduling particularly w.r.t. multi-PDSCH/PUSCH with a single DCI, HARQ, with checkpoints for agreements on Apr-16, Apr-20 – Seonwook (LGE)

R1-2103344	Summary #1 of PDSCH/PUSCH enhancements (Scheduling/HARQ)	Moderator (LG Electronics)

Agreement:

- The maximum number of PDSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 480 and 960 kHz.

  - FFS: Further restrictions for 480 kHz to 4

  - FFS: A UE capability to select between 4 and 8 for 480 kHz SCS

  - Note: Multi-PDSCH scheduling for the case of 120 kHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided.

- The maximum number of PUSCHs that can be scheduled with a single DCI in Rel-17 is 8.

  - FFS: Further restrictions for 120 kHz and 480 kHz SCS

  - FFS: A UE capability to select between different values for 120 kHz and 480 kHz SCS

Agreement:

For a DCI that can schedule multiple PDSCHs,

- MCS for the 1st TB: This appears only once in the DCI and applies commonly to the first TB of each PDSCH

- NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH

- RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH

- HARQ process number: This applies to the first scheduled PDSCH and is incremented by 1 for subsequent PDSCHs (with modulo operation, if needed)

- FFS:

  - MCS/NDI/RV for the 2nd TB for each PDSCH, including whether scheduling of the 2nd TB for each PDSCH can be supported or not

  - Details of resource allocation related fields such as VRB-to-PRB mapping, PRB bundling size indicator, rate matching indicator, and ZP CSI-RS trigger

  - Whether/how to signal CBGFI/CBGTI if CBGFI/CBGTI is supported for multi-PDSCH scheduling

  - Details of fields that are common with multi-PUSCH scheduling, e.g., TDRA, FDRA, priority indicator, including potential enhancements

### Channel access mechanism

R1-2102332	Channel access mechanism for 60 GHz unlicensed operation	Huawei, HiSilicon

R1-2102390	Discussion on channel access mechanism	OPPO

R1-2102453	Discussion on channel access mechanism for above 52.6GHz	Spreadtrum Communications

R1-2102519	Discussions on channel access mechanism for NR operation from 52.6GHz to 71 GHz	vivo

R1-2102563	Channel access mechanism	Nokia, Nokia Shanghai Bell

R1-2102570	Discussions on channel access mechanism enhancements for 52.6G-71 GHz	CAICT

R1-2102626	Channel access mechanism for up to 71GHz operation	CATT

R1-2102689	On the channel access mechanisms for 52.6-71 GHz NR operation	MediaTek Inc.

R1-2102717	Considerations on channel access mechanism for NR  from 52.6GHz to 71 GHz	Fujitsu

R1-2102777	Further considerations on channel access for shared spectrum Beyond 52.6 GHz	FUTUREWEI

R1-2102793	Channel Access Mechanisms	Ericsson

R1-2102981	Channel access mechanism for NR on 52.6-71 GHz	Xiaomi

R1-2103001	Channel access mechanisms for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2103026	Discussion on channel access mechanism for extending NR up to 71 GHz	Intel Corporation

R1-2103101	Channel access mechanisms for unlicensed access above 52.6GHz	Apple

R1-2103162	Channel access mechanism for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2103234	Channel access mechanism for NR from 52.6 GHz to 71 GHz	Samsung

R1-2103299	channel access mechanism for 60 GHz unlicensed spectrum	Sony

R1-2103345	Channel access mechanism to support NR above 52.6 GHz	LG Electronics

R1-2103415	Channel Access for Supporting NR from 52.6 GHz to 71 GHz	Convida Wireless

R1-2103427	Channel access for multi-beam operation	Panasonic

R1-2103443	Further Discussion of Channel Access Mechanisms	AT&T

R1-2103453	Discussion on channel access mechanisms	InterDigital, Inc.

R1-2103492	Discussion on the channel access for 52.6 to 71GHz	ZTE, Sanechips

R1-2103520	Discussion on channel access mechanism supporting NR from 52.6 to 71GHz	NEC

R1-2103572	Channel access mechanism for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2103631	Discussion on multi-beam operation	ITRI

R1-2103694	Discussion on channel access mechanism for NR from 52.6GHz to 71GHz	WILUS Inc.

R1-2103727	Channel access mechanisms for above 52.6 GHz	Charter Communications

R1-2103791	Email discussion summary for channel access mechanism for 52.6GHz-71GHz band, ver01			Moderator (Qualcomm Incorporated)

[104b-e-NR-52-71GHz-07] Email discussion/approval on channel access mechanism with checkpoints for agreements on Apr-16, Apr-20 – Jing (Qualcomm)

Working assumption:

For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT.

Agreement:

- Contention Exempt Short Control Signaling rules can be applicable to the transmission of SS/PBCH.

  - FFS: What are the other DL signals and channels that can be multiplexed with SS/PBCH transmission under Contention Exempt Short Control Signaling rule

  - FFS: Whether this can be applied to all supported SCS or specific SCS.

  - FFS: Extension to discovery burst if it is defined including signals other than SS/PBCH

  - Note: Restriction for short control signalling transmissions apply (10% over any 100ms interval)

- FFS: Other DL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as PDCCH, broadcast PDSCH, PDSCH without user plain data, CSI-RS, PRS, etc

### Others

Including analysis or recommendation to RAN#92e (June) on how to introduce the 52.6-71GHz frequency range

R1-2103102	On How to Introduce the 52.6-71GHz Frequency Range	Apple

R1-2103493	Discussion on the frequency range definition for above 52.6GHz	ZTE, Sanechips

R1-2102391	Discussion on other aspects	OPPO

R1-2102520	Discussions on frequency range definition and sync raster for 52.6-71GHz	vivo

R1-2102564	Extending current NR operation to 71 GHz	Nokia, Nokia Shanghai Bell

R1-2102627	Views on Frequency Range for 52.6  to 71 GHz	CATT

R1-2102794	NR channelization for 52.6 – 71 GHz band	Ericsson

R1-2103235	Discussion on defining frequency range for NR from 52.6 to 71 GHz	Samsung

R1-2103346	Link level evaluation results for NR above 52.6 GHz	LG Electronics

R1-2103393	Link level and system evaluation results for 52-71 GHz	Huawei, HiSilicon

R1-2103418	Discussion on frequency range determination for NR extension up to 71GHz	Intel Corporation

R1-2103454	Efficient beam management for NR in 52.6 – 71 GHz	InterDigital, Inc.

R1-2103573	Views on how to introduce 52.6 - 71 GHz frequency range in NR	NTT DOCOMO, INC.