3GPP TSG-RAN WG1 Meeting #104bis-e	R1-21xxxxx

e-Meeting, 12th – 20th April 2021

Agenda Item:	6.2.1

Source:	Moderator (Ericsson)

Title:	FL summary for Multi-TB and PUR issues for Rel-16 LTE-MTC

Document for:	Discussion, Decision

1	Introduction

This document provides a summary of the following RAN1 email discussion.

[104b-e-LTE-eMTC5-02] Multi-TB and PUR issues – Johan (Ericsson) - Issue #1: Correction of PUCCH transmit power control for multi-TB scheduling (R1-2102848) - Issue #2: Correction of bundling parameter for multi-TB scheduling (R1-2102849) - Issue #3: Clarification of PUSCH PRB resources for PUR (R1-2103721) - Discussion and decision by April 15, TPs by April 20

Issue #1: Correction of PUCCH transmit power control for multi-TB scheduling

Contribution [1] proposes to clarify that the PUCCH transmit power is the same for all PUCCH transmissions corresponding to TBs scheduled by the same DCI.

Question 1-1: Companies are invited to comment below on the 36.213 TP in [1] for correction of PUCCH transmit power control for multi-TB scheduling.

| Company | Comments |
| --- | --- |
| Lenovo, MotoM | PUCCH can be configured to transmit periodically w/o DCI indication and PUCCH may not be associated with PDSCH(e.g., not ACK/NACK), so we don’t think the CR is OK. How about the following updated CR For a BL/CE UE configured with CEModeA, if the PUCCH, or multiple PUCCHs corresponding to PDSCH scheduled by one DCI, are transmitted in more than one subframe i0, i1, …, iN-1 where i0< i1< …< iN-1, the PUCCH(s) transmit power in subframe ik , k=0, 1, …, N-1is determined by |

Issue #2: Correction of bundling parameter for multi-TB scheduling

Contribution [2] proposes to change the parameter name multi-TB-DL-HARQ-bundling to harq-AckBundling in one place in 36.212 in order to align with 36.331.

Question 2-1: Companies are invited to comment below on the 36.212 TP in [2] for correction of bundling parameter for multi-TB scheduling.

| Company | Comments |
| --- | --- |
| Lenovo, MotoM | We are fine with the CR |

Issue #3: Clarification of PUSCH PRB resources for PUR

Contribution [3] proposes to clarify how to obtain the PUSCH PRB resource configuration for PUR.

Question 3-1: Companies are invited to comment below on the 36.211 TP in [3] for clarification of PUSCH PRB resources for PUR.

| Company | Comments |
| --- | --- |
| Lenovo, MotoM | We are fine with the CR |

References

- R1-2102848, “Correction on PUCCH transmit power control for LTE-M”, ZTE, Sanechips

- R1-2102849, “Correction on bundling parameter for multi-TB scheduling for LTE-M”, ZTE, Sanechips

- R1-2103721, “PUR maintenance issues for Rel-16 LTE-MTC”, Ericsson