3GPP TSG-RAN WG1 Meeting #104bis-e	R1-21xxxxx

e-Meeting, 12th – 20th April 2021

Agenda Item:	6.2.1

Source:	Moderator (Ericsson)

Title:	FL summary for Multi-TB and PUR issues for Rel-16 LTE-MTC

Document for:	Discussion, Decision

# 1	Introduction

This document provides a summary of the following RAN1 email discussion.

[104b-e-LTE-eMTC5-02] Multi-TB and PUR issues – Johan (Ericsson) - Issue #1: Correction of PUCCH transmit power control for multi-TB scheduling (R1-2102848) - Issue #2: Correction of bundling parameter for multi-TB scheduling (R1-2102849) - Issue #3: Clarification of PUSCH PRB resources for PUR (R1-2103721) - Discussion and decision by April 15, TPs by April 20

# Issue #1: Correction of PUCCH transmit power control for multi-TB scheduling

Contribution [1] proposes to clarify that the PUCCH transmit power is the same for all PUCCH transmissions corresponding to TBs scheduled by the same DCI.

Question 1-1: Companies are invited to comment below on the 36.213 TP in [1] for correction of PUCCH transmit power control for multi-TB scheduling.

| Company | Comments |
| --- | --- |
| Lenovo, MotoM | PUCCH can be configured to transmit periodically w/o DCI indication and PUCCH may not be associated with PDSCH(e.g., not ACK/NACK), so we don’t think the CR is OK. How about the following updated CR For a BL/CE UE configured with CEModeA, if the PUCCH, or multiple PUCCHs corresponding to PDSCH scheduled by one DCI, are transmitted in more than one subframe i0, i1, …, iN-1 where i0< i1< …< iN-1, the PUCCH(s) transmit power in subframe ik , k=0, 1, …, N-1is determined by |
| Qualcomm | We think Lenovo’s version is slightly more clear, but we don’t have a strong view. |
| Ericsson | In Lenovo’s TP above, “PDSCH” can be replaced with “PDSCHs”. |
| ZTE,Sanechips | We are OK with the modification from Lenovo and Ericsson. |
| Moderator (Ericsson) | Proposal 1-2: Agree the following 36.213 TP in principle, and draft a corresponding CR: For a BL/CE UE configured with CEModeA, if the PUCCH, or multiple PUCCHs corresponding to PDSCHs scheduling by one DCI, is/are transmitted in more than one subframe i0, i1, …, iN-1 where i0< i1< …< iN-1, the PUCCH(s) transmit power in subframe ik , k=0, 1, …, N-1is determined by |
| Huawei, HiSilicon | We are fine the proposal from Moderator, maybe it’s better to have a bracket there as “or multiple PUCCHs corresponding to PDSCH(s) scheduling by one DCI” as it’s possible the PUCCH corresponds a single PDSCH. |
| Ericsson 2 | Regarding the comment from Huawei/HiSilicon, if there are multiple PUCCHs, there are also multiple PDSCHs, so it should probably say “PDSCHs” without parentheses. |

# Issue #2: Correction of bundling parameter for multi-TB scheduling

Contribution [2] proposes to change the parameter name multi-TB-DL-HARQ-bundling to harq-AckBundling in one place in 36.212 in order to align with 36.331.

Question 2-1: Companies are invited to comment below on the 36.212 TP in [2] for correction of bundling parameter for multi-TB scheduling.

| Company | Comments |
| --- | --- |
| Lenovo, MotoM | We are fine with the CR |
| FUTUREWEI | OK with the CR. It should be clear that this parameter is for the -r16 feature. |
| Nokia, NSB | Fine with CR |
| Qualcomm | OK with the CR |
| Ericsson | We support the proposed change. |
| ZTE,Sanechips | OK with the CR |
| Moderator (Ericsson) | Proposal 2-2: Agree the draft 36.212 CR in R1-2102849 in principle, with a modification to make it clear that the text concerns harq-AckBundling-r16, and draft a corresponding CR. |
| ZTE2 | ce-HARQ-AckBundling is for R14 feature. harq-AckBundling is for R16 feature and it is used in TS36.213 subclause 7.3 For a BL/CE UE, if the UE is configured with CEModeA, and if the UE is configured with higher layer parameter harq-AckBundling in ce-PDSCH-MultiTB-Config and multiple TB are scheduled in the corresponding DCI format 6-1A with CRC scrambled by C-RNTI,. Therefore, actually there is no confusion. In order to keep aligned with 36.213, we hope the harq-AckBundling can be kept. |
| Huawei, HiSilicon | We are fine with the CR. |
| Ericsson 2 | The ZTE2 comment seems to make sense, so perhaps there is no need for any modification to make it clearer that the text concerns the -r16 variant. |

# Issue #3: Clarification of PUSCH PRB resources for PUR

Contribution [3] proposes to clarify how to obtain the PUSCH PRB resource configuration for PUR.

Question 3-1: Companies are invited to comment below on the 36.211 TP in [3] for clarification of PUSCH PRB resources for PUR.

| Company | Comments |
| --- | --- |
| Lenovo, MotoM | We are fine with the CR |
| Nokia, NSB | Fine with CR |
| Ericsson | We support the proposed change. |
| ZTE,Sanechips | OK with the CR |
| Moderator (Ericsson) | Proposal 3-2: Agree the 36.211 TP in R1-2103721 in principle, and draft a corresponding CR. |
| Huawei, HiSilicon | We are fine with the CR. |

# References

- R1-2102848, “Correction on PUCCH transmit power control for LTE-M”, ZTE, Sanechips

- R1-2102849, “Correction on bundling parameter for multi-TB scheduling for LTE-M”, ZTE, Sanechips

- R1-2103721, “PUR maintenance issues for Rel-16 LTE-MTC”, Ericsson