3GPP TSG RAN WG1 Meeting #104bis-e	R1-210xxxx

E-Meeting, April 12th – April 20th, 2021

Agenda Item:	6.2.2

Source:	Moderator (Huawei)

Title:	Feature summary on 104b-e-LTE-NB_IoTenh3-01

Document for:	Discussion and Decision

# Introduction

This documents provides the proposals and summary of discussions of the following email discussion:

[104b-e-LTE-NB_IoTenh3-01] PUR issues – Xiang (Huawei)

- Issue 1: Clarification on subcarrier indication for PUR. (R1-2103722)

- Issue 2: Clarification on scheduling/processing gaps for NB-IoT. (R1-2102850, R1-2103762)

- Discussion and decision by April 15, TPs by April 20

# Discussion

### Issue 1: Clarification on subcarrier indication for PUR

- As explained in [2], the the set of contiguously allocated subcarriers () is missed for PUR cases.

- Proposal: Endorse one of the following text proposal

------------------------------------------------- Text omitted (TS 36.213 Clause 16.5.1.1)---------------------------------------

For NPUSCH transmission with subcarrier spacing, where  is the subcarrier indication field or nsc = npusch-SubCarrierSetIndex in PUR-Config-NB in case of NPUSCH transmissions using preconfigured uplink resources. is reserved.

For NPUSCH transmission with subcarrier spacing, the subcarrier indication field () in the DCI or npusch-SubCarrierSetIndex in PUR-Config-NB in case of NPUSCH transmissions using preconfigured uplink resources determines the set of contiguously allocated subcarriers () according to Table 16.5.1.1-1.

---------------------------------------------------- Text end (TS 36.213 Clause 16.5.1.1)-----------------------------------------

Please input your comments in the following table:

| Companies | Comments |
| --- | --- |
| Lenovo, MotoM | We are fine with the CR in general. Hope to consider the following update. since we can’t use n_sc = a RRC parameter. For NPUSCH transmission with subcarrier spacing, where is the subcarrier indication field, is reserved, or nsc equals to the value of parameter npusch-SubCarrierSetIndex in PUR-Config-NB in case of NPUSCH transmissions using preconfigured uplink resources. |
| Qualcomm | Although the TP is technically correct, we are a bit concerned about the “double specification” that we are introducing by these changes: In TS 36.331, the following is mentioned: npusch-SubCarrierSetIndex For NPUSCH transmission with subcarrier spacing 3.75 kHz, indicates the subcarrier used for PUR specified in TS 36.213 [23]. For NPUSCH transmission with subcarrier spacing 15 kHz, index to a table specified in TS 36.213 [23], Table 16.5.1.1-1, that defines the set of subcarriers for NPUSCH for PUR. We are just repeating the same specification now in TS 36.213. If in the future we need to make a change to this behavior, we would need to change both 36.213 and 36.331. Either way, we can go with the majority view. One intermediate approach would be to modify as follows: For NPUSCH transmission with subcarrier spacing, where is the subcarrier indication field or is configured by higher layers for a NPUSCH transmissions using preconfigured uplink resources. is reserved. For NPUSCH transmission with subcarrier spacing, the subcarrier indication field () in the DCI, or configured by higher layers for a NPUSCH transmissions using preconfigured uplink resources, determines the set of contiguously allocated subcarriers () according to Table 16.5.1.1-1. |

### Issue 2: Clarification on scheduling/processing gaps for NB-IoT.

- As explained in [1][3], in current spec, the consecutive subframes for multicast does not consider the scheduling gaps or processing gaps.

- In [1], the following is proposed:

---------------------------------------------- Start of Text Proposal to 36.213 ------------------------------

### 16.4.1	UE procedure for receiving the narrowband physical downlink shared channel

<Unchanged parts are omitted>

A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N1, N2 ending in subframe n intended for the UE, decode, starting in

-	n+5 DL subframe for FDD,

-	n+5 subframe for TDD,

the corresponding NPDSCH transmission in N NB-IoT DL subframe(s) ni with i = 0, 1, …, N-1 according to the NPDCCH information, where

-	subframe n is the last subframe in which the NPDCCH is transmitted and is determined from the starting subframe of NPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI;

-	, where the value of  is determined by the repetition number field in the corresponding DCI (see Clause 16.4.1.3), the value of is determined by the resource assignment field in the corresponding DCI (see Clause 16.4.1.3), and the value of is determined by the Number of scheduled TB for Unicast field or Number of scheduled TB for SC-MTCH field, if present, in the corresponding DCI,  otherwise,

-	k0 is the number of NB-IoT DL subframe(s) starting in DL subframe n+5 for FDD or subframe n+5 for TDD, until DL subframe n0, where k0 is determined by the scheduling delay field () for DCI format N1, and k0 = 0 for DCI format N2. For DCI CRC scrambled by G-RNTI, k0 is determined by the scheduling delay field () according to Table 16.4.1-1a, otherwise k0 is determined by the scheduling delay field () according to Table 16.4.1-1. The value of is according to Clause 16.6 for the corresponding DCI format N1,

-  if  and NPDSCH corresponding to an NPDCCH with DCI CRC scrambled by G-RNTI,

-  if multiTB-Gap is not configured and ,  subframe(s) ni is the first NB-IoT DL subframe after subframe , where  is processing gap with 20 subframes, δd=1, d=00,d≠0, and   is the number of overlapped subframes if the jth processing gap overlaps with the NPDSCH transmission gap defined in [3] and .

- otherwise, subframe(s) ni is the first NB-IoT DL subframe after subframe , where is given by higher layer parameter multiTB-Gap,  δd=1, d=00,d≠0, and  is the number of overlapped subframes if the jth scheduling gap overlaps with the NPDSCH transmission gap defined in [3] and .

-	otherwise, subframe(s) ni with i=0,1,…,N-1 are N consecutive NB-IoT DL subframe(s) excluding subframes used for SI messages where, n0<n1<…,nN-1 ,

-	for ,

-	if the UE is configured with higher layer parameter multiTB-Config in npdsch-MultiTB-Config set to 'interleaved', and NPDSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI, and

-	NB-IoT DL subframes  with  are associated with TBr+1 ,

-	otherwise,

-	NB-IoT DL subframes  with  are associated with TBr+1 ,

<Unchanged parts are omitted>

----------------------------------------------- End of Text Proposal to 36.213 ------------------------------

- In [3], the following is proposed:

---------------------------------------------- Start of Text Proposal to 36.213 ------------------------------

<Unchanged part omitted>

### 16.4.1	UE procedure for receiving the narrowband physical downlink shared channel

A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N1, N2 ending in subframe n intended for the UE, decode, starting in

-	n+5 DL subframe for FDD,

-	n+5 subframe for TDD,

the corresponding NPDSCH transmission in N consecutive NB-IoT DL subframe(s) ni with i = 0, 1, …, N-1 according to the NPDCCH information, where

-	subframe n is the last subframe in which the NPDCCH is transmitted and is determined from the starting subframe of NPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI;

-	subframe(s) ni with i=0,1,…,N-1 are N consecutive NB-IoT DL subframe(s) excluding subframes used for SI messages or scheduling gap (if any) or processing gap (if any) where, n0<n1<…,nN-1 ,

-	, where the value of  is determined by the repetition number field in the corresponding DCI (see Clause 16.4.1.3), the value of is determined by the resource assignment field in the corresponding DCI (see Clause 16.4.1.3), and the value of is determined by the Number of scheduled TB for Unicast field, if present, in the corresponding DCI,  otherwise,

-	k0 is the number of NB-IoT DL subframe(s) starting in DL subframe n+5 for FDD or subframe n+5 for TDD, until DL subframe n0, where k0 is determined by the scheduling delay field () for DCI format N1, and k0 = 0 for DCI format N2. For DCI CRC scrambled by G-RNTI, k0 is determined by the scheduling delay field () according to Table 16.4.1-1a, otherwise k0 is determined by the scheduling delay field () according to Table 16.4.1-1. The value of is according to Clause 16.6 for the corresponding DCI format N1,

-	for ,

-	if the UE is configured with higher layer parameter multiTB-Config in npdsch-MultiTB-Config set to 'interleaved', and NPDSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI, and

-	NB-IoT DL subframes  with  are associated with TBr+1 ,

-	otherwise,

-	NB-IoT DL subframes  with  are associated with TBr+1 ,

-	for  and NPDSCH corresponding to an NPDCCH with DCI CRC scrambled by G-RNTI,

-	if multiTB-Gap is not configured and , a processing gap of 20ms is inserted after every 2 TBs

-	otherwise, a scheduling gap with a length equal to the indicated value of multiTB-Gap is inserted between TBr and TBr+1, .

-	If the scheduling gap or the processing gap overlaps with the NPDSCH transmission gap defined in [3], the overlapped part of the scheduling gap or processing gap is also counted as the part of NPDSCH transmission gap.

<Unchanged part omitted>

----------------------------------------------- End of Text Proposal to 36.213 ------------------------------

Please input your comments regarding the above text proposals:

| Companies | Comments |
| --- | --- |
| Lenovo, MotoM | We agree the issue proposed by ZTE and Huawei. Consider the difference between NBIoT and eMTC, there is NPDSCH transmission gap in NBIoT, so we think it is not clear to specify the subframes used for NPDSCH transmission as ZTE proposal, especially why do we have shorter gaps if there is overlapped NPDSCH transmission gap. subframe(s) ni is the first NB-IoT DL subframe after subframe To make things easier, we agree to simply use the general description as Huawei proposal. |
| Qualcomm | We think the Huawei TP is enough in this case, as mentioned by Lenovo. |

# Summary

To be added.

# References

- R1-2102850	Correction on multi-TB gap for multicast in NB-IoT	ZTE

- R1-2103722	PUR maintenance issues for Rel-16 NB-IoT	Ericsson

- R1-2103762	Correction on multi-TB scheduling for NB-IoT	Huawei, HiSilicon