3GPP TSG- RAN WG1 Meeting #104b-e                                                 R1-2xxxxxx

# e-Meeting, April 12th – 20th, 2021

Agenda Item:	7.1

Source:	Moderator (Apple Inc.)

Title:	Summary on 104b-e-NR-7.1CRs-08

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide a summary on 104bis-e-NR-7.1CRs-08.

# Discussion

In R1-2103076, Apple proposes to clarify the scope for the UE FG 2-24, which is as follows:

| beamManagementSSB-CSI-RS Defines support of SS/PBCH and CSI-RS based RSRP measurements. The capability comprises signalling of - maxNumberSSB-CSI-RS-ResourceOneTx indicates maximum total number of configured one port NZP CSI-RS resources and SS/PBCH blocks that are supported by the UE to measure L1-RSRP as specified in TS 38.215 [13] within a slot and across all serving cells (see NOTE). On FR2, it is mandatory to report >=8; On FR1, it is mandatory with capability signalling to report >=8. - maxNumberCSI-RS-Resource indicates maximum total number of configured NZP-CSI-RS resources that are supported by the UE to measure L1-RSRP as specified in TS 38.215 [13] across all serving cells (see NOTE). It is mandated to report at least n8 for FR1. - maxNumberCSI-RS-ResourceTwoTx indicates maximum total number of two ports NZP CSI-RS resources that are supported by the UE to measure L1-RSRP as specified in TS 38.215 [13] within a slot and across all serving cells (see NOTE). - supportedCSI-RS-Density indicates density of one RE per PRB for one port NZP CSI-RS resource for RSRP reporting, if supported. On FR2, it is mandatory to report either "three" or "oneAndThree"; On FR1, it is mandatory with capability signalling to report either "three" or "oneAndThree". - maxNumberAperiodicCSI-RS-Resource indicates maximum number of configured aperiodic CSI-RS resources across all serving cells (see NOTE). For FR1 and FR2, the UE is mandated to report at least n4. NOTE: If the UE sets a value other than n0 in an FR1 band, it shall set that same value in all FR1 bands. If the UE sets a value other than n0 in an FR2 band, it shall set that same value in all FR2 bands. The UE supports a total number of resources equal to the maximum of the FR1 and FR2 value, but no more than the FR1 value across all FR1 serving cells and no more than the FR2 value across all FR2 serving cells. | Band | Yes | No | Yes |
| --- | --- | --- | --- | --- |

This FG defines several elements for RSRP related measurement, but currently RSRP measurement could be with the following usages:

- U1: The RS is used for beam report, i.e. L1-RSRP is associated with a CSI-reportConfig

- U2: The RS is used for candidate beam detection for CFRA based BFR

- U3: The RS is used for candidate beam detection for CBRA based BFR

Proposal for discussion: Clarify the usage for each element in UE FG 2-24 with U1, U2 and/or U3

Companies’ views and comments

| Company | View |
| --- | --- |
| Apple | Our initial understanding is as follows, but we are open to see other views. According to understanding, the mandatory value for 1 port CSI-RS was discussed based on assumption that the element is used for normal beam report only, but the 2 port CSI-RS is general for all cases, as this element includes a candidate value that UE does not support 2-port CSI-RS based RSRP measurement. - maxNumberSSB-CSI-RS-ResourceOneTx: U1 - maxNumberCSI-RS-Resource: U1 - maxNumberCSI-RS-ResourceTwoTx: U1+U2+U3 - supportedCSI-RS-Density: U1 - maxNumberAperiodicCSI-RS-Resource: U1 |