3GPP TSG-RAN WG1 Meeting #104b-e											R1-210xxxx

e-Meeting, April 12th – 20th, 2021

Agenda Item:	7.2.1

Source:			Moderator (ZTE)

Title:			FL summary on the maintenance of 2-step RACH

Document for:	Discussion

# Introduction

This document contains the summary of issues related to the maintenance of Rel-16 2-step RACH WI in RAN1#104b-e meeting.

# Maintenance issues

The following 3 issues are identified based on the submitted contributions in RAN1#104b-e.

| Issue # | Description | Related TDoc # |
| --- | --- | --- |
| 1 | Correction on the configuration of RACH-related power control parameters | R1-2103403 |
| 2 | Editorial corrections on the DMRS description for MsgA | R1-2103495 |
| 3 | Determination of power control parameter in case of 2-step RACH only operation | R1-2103680, TP#1 |
| 4 | Determination of PUSCH waveform in case of 2-step RACH only operation | R1-2103680, TP#2 |

# Summary

It seems be rather straightforward to include all the above issues in a single email thread. Therefore the following email discussion is proposed (will be updated based on companies’ feedback, if any).

Proposed Email thread #1:

Corrections of 2-step RACH related issues

- CR in R1-2103403, CR in R1-2103495 and two TPs in R1-2103680

Any comments?

| Company | Comment |
| --- | --- |

# References

- R1-2103403	Correction on the configuration of RACH-related power control parameters	Huawei, HiSilicon

- R1-2103495	Editorial corrections on the DMRS description for MsgA		ZTE, Sanechips

- R1-2103680	Discussion on corrections for 2-step RACH	Ericsson

# Appendix

List of proposals in the submitted contributions.

| TDoc | Proposals |
| --- | --- |
| R1-2103403 Huawei | Reason for change: - The msgA-Alpha and msg3-Alpha should be used for the PUSCH transmission of 2-step RACH and 4-step RACH, respectively. However, according to current TS 38.213, when both 2-step RACH and 4-step RACH are configured, and ΔMsgA_PUSCH is equal to ΔPREAMBLE_Msg3, msgA-Alpha will be used for both 2-step RACH and 4-step RACH. ---------------------------- # Uplink Power control ## 7.1 Physical uplink shared channel ### 7.1.1 UE behaviour *** Unchanged text is omitted *** - For - For , - if PO_NOMINAL_PUSCH,f,c0 is determined by =PO_PRE +ΔMsgA_PUSCHand ΔMsgA_PUSCH, and msgA-Alpha is provided, αb,f,c(0) is the value of msgA-Alpha - elseif - PO_NOMINAL_PUSCH,f,c0 is determined by =PO_PRE+ΔPREAMBLE_Msg3 and ΔPREAMBLE_Msg3, or msgA-Alpha is not provided, and - msg3-Alpha is provided, - is the value of msg3-Alpha - else, *** Unchanged text is omitted *** |
| R1-2103495 ZTE | Reason for change: Some typos and copy-paste errors were found in the latest specification for the description of DMRS configurations for MsgA. ----------------------------------------- < Unchanged parts are omitted > For PUSCH mapping type A, - the case dmrs-AdditionalPosition equals to 'pos3' is only supported when dmrs-TypeA-Position is equal to 'pos2'; - ld=4 symbols in Table 6.4.1.1.3-4 is only applicable when dmrs-TypeA-Position is equal to 'pos2'. For msgA transmitted using PUSCH mapping type A, - the case msgA-DMRS-AdditionalPosition equals to 'pos3' is only supported when dmrs-TypeA-Position is equal to 'pos2'; - 'dmrs-AdditionalPosition' in Tables Tables 6.4.1.1.3-3 to 6.4.1.1.3-6 shall be replaced by msgA-DMRS-AdditionalPosition; - only PUSCH DM-RS configuration type 1 is supported. For msgA transmitted using PUSCH mapping type B, - 'dmrs-AdditionalPosition' in Tables 6.4.1.1.3-3 to 6.4.1.1.3-6 shall be replaced by msgA-DMRS-AdditionalPosition; - only PUSCH DM-RS configuration type 1 is supported. < Unchanged parts are omitted > |
| R1-2103680 Ericsson | In case of 2-step RACH only operation, when p0-AlphaSets is not provided, for power control of normal PUSCH, P0-nominal and alpha for msgA PUSCH are used, according to TP1. ------------------------- TP1 of 38.213 V16.5.0 ----------------------------------------- 7.1.1 UE behaviour *** unchanged text omitted*** - For - For , - if PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH and msgA-Alpha is provided, αb,f,c(0) is the value of msgA-Alpha - elseif PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔPREAMBLE_Msg3 or msgA-Alpha is not provided, and msg3-Alpha is provided, is the value of msg3-Alpha - else, - For , is provided by alpha obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig providing an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP of carrier of serving cell - For , a set of values are provided by a set of alpha in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP of carrier of serving cell - If the UE is provided SRI-PUSCH-PowerControl and more than one values of p0-PUSCH-AlphaSetId, and if a DCI format scheduling the PUSCH transmission includes an SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in the DCI format [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values and determines the values of from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value - If the PUSCH transmission except for the PUSCH retransmission corresponding to a RAR UL grant is scheduled by a DCI format that does not include an SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, , and the UE determines from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets - For j=1 or j∈SJ, if P0-PUSCH-AlphaSet is not configured, the UE uses the P0-nominal and msg3-Alpha configured for msg3 PUSCH if a Type-1 random access is configured for the BWP or uses the P0-nominal and msgA-Alpha for msgA PUSCH if a Type-1 random access procedure is not configured for the BWP. *** unchanged text omitted*** ---------------------------------- TP1 of 38.213 V16.5.0 ---------------------------------------- In case of 2-step RACH only operation, when transformPrecoder is not provided, waveform of normal PUSCH is determined based on msgA-transformPrecoder according to TP2. ------------------------------- TP2 of 38.214 V16.5.0 ------------------------------------------ *** unchanged text omitted*** For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI: - If the DCI with the scheduling grant was received with DCI format 0_0, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder if a Type-1 random access is configured for the BWP or msgA-transformPrecoder if a Type-1 random access is not configured for the BWP. - If the DCI with the scheduling grant was not received with DCI format 0_0 - If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter. - If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder if a Type-1 random access is configured for the BWP or msgA-transformPrecoder if a Type-1 random access is not configured for the BWP. For PUSCH transmission with a configured grant - If the UE is configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter. - If the UE is not configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder if a Type-1 random access is configured for the BWP or msgA-transformPrecoder if a Type-1 random access is not configured for the BWP. *** unchanged text omitted*** ------------------------------ TP2 of 38.214 V16.5.0 ------------------------------------- |