3GPP TSG RAN WG1 Meeting #104b-e	R1- 210xxxx

e-Meeting, 12th – 20th April 2021

Title:	Summary of NR UE Power Saving

Agenda item:	7.2.7

Source:	CATT

Document for:	Discussion

# Final Summary of Email Discussions and Agreements

# Email Discussion [104b-e-NR_UE_Pow_Sav_01]

# Email Discussion during Preparation[104b-e-Prep_NR_UE_Pow_Sav]

| Company | Supporting Issues and draft CR | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Issue 1, Issue 2, Issue 3 | - Issue1 is a valid issue that needs to be resolved. The detailed change needs to be further discussed; - We are open to discuss issue2. However, it seems for connected mode UE, C-RNTI shall be used to schedule PDSCH rather than TC-RNTI during RACH procedure. - Issue3 should be discussed to align RAN1 and RAN2 specification. |

# Summary of Open Issues

- Issue 1: A-CSI-RS triggering offset for cross-slot scheduling [1]

- Proposal Hardcode A-CSI-RS triggering offset to K0min when A-CSI-RS is triggered by SRS request field in non-codebook-based UL transmission.

##### 5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology

======skipped text======

The UE does not expect that aperiodic CSI-RS is transmitted before the OFDM symbol(s) carrying its triggering DCI. When the minimum scheduling offset restriction is applied, UE is not expected to be triggered by CSI triggering state indicated by the CSI request field in DCI in which CSI-RS triggering offset is smaller than the currently applicable minimum scheduling offset restriction K0min. When the minimum scheduling offset restriction is applied, aperiodic CSI-RS triggering offset is set to K0min, when triggered by SRS request indicated by the value of the SRS request field is not '00' as in Table 7.3.1.1.2-24 of [5, TS 38.212] and if the scheduling DCI is not used for cross carrier or cross bandwidth part scheduling.

#### 6.1.1.2	Non-Codebook based UL transmission

======skipped text======

If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, the presence of the associated CSI-RS is indicated by the SRS request field if the value of the SRS request field is not '00' as in Table 7.3.1.1.2-24 of [5, TS 38.212] and if the scheduling DCI is not used for cross carrier or cross bandwidth part scheduling. The CSI-RS is located in the same slot as the SRS request field if currently applicable minimum scheduling offset restriction K0min=0. When the minimum scheduling offset restriction is applied, CSI-RS triggering offset is set to  K0min. If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, any of the TCI states configured in the scheduled CC shall not be configured with qcl-Type set to 'typeD'.

#### 6.1.1.2	Non-Codebook based UL transmission

======skipped text======

If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, the presence of the associated CSI-RS is indicated by the SRS request field if the value of the SRS request field is not '00' as in Table 7.3.1.1.2-24 of [5, TS 38.212] and if the scheduling DCI is not used for cross carrier or cross bandwidth part scheduling. The CSI-RS is located in the same slot as the SRS request field if currently applicable minimum scheduling offset restriction K0min=0. When the minimum scheduling offset restriction is applied, CSI-RS triggering offset is set to  K0min. If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, any of the TCI states configured in the scheduled CC shall not be configured with qcl-Type set to 'typeD'.

Issue 2:  Including TC-RNTI for the RNTI not supporting cross-slot scheduling [2]

----------------------------------------------- Start of TP of TS 38.214 -------------------------------------------------------- 5.1.2.1 Resource allocation in time domain *** Unchanged text is omitted *** When the UE is configured with minimumSchedulingOffsetK0 in an active DL BWP it applies a minimum scheduling offset restriction indicated by the 'Minimum applicable scheduling offset indicator' field in DCI format 1_1 or DCI format 0_1 if the same field is available. When the UE is configured with minimumSchedulingOffsetK0 in an active DL BWP and it has not received 'Minimum applicable scheduling offset indicator' field in DCI format 0_1 or 1_1, the UE shall apply a minimum scheduling offset restriction indicated based on 'Minimum applicable scheduling offset indicator' value '0'. When the minimum scheduling offset restriction is applied the UE is not expected to be scheduled with a DCI in slot n to receive a PDSCH scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI with K0 smaller than K0min⋅2μ'2μ, where K0min and μ are the applied minimum scheduling offset restriction and the numerology of the active DL BWP of the scheduled cell when receiving the DCI in slot n, respectively, and μ' is the numerology of the new active DL BWP in case of active DL BWP change in the scheduled cell and is equal to μ, otherwise. The minimum scheduling offset restriction is not applied when PDSCH transmission is scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI in common search space associated with CORESET0 and default PDSCH time domain resource allocation is used, in the search space set provided by recoverySearchSpaceId when monitoring PDCCH as described in [6, TS 38.213] or when PDSCH transmission is scheduled with SI-RNTI, MSGB-RNTI, or RA-RNTI or TC-RNTI. The application delay of the change of the minimum scheduling offset restriction is determined in Clause 5.3.1. *** Unchanged text is omitted *** ----------------------------------------------- End of TP of TS 38.214 --------------------------------------------------------

Issue 3: Clarification of Dormancy group indication by higher layer parameter dormancyGroupOutsideActiveTime [3]

10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ======skipped part======= - a bitmap, when the UE is provided a number of groups of configured SCells by high layer parameter(s) dormancyGroupOutsideActiveTime, where - the bitmap location is immediately after the Wake-up indication bit location - the bitmap size is equal to the number of groups of configured SCells where each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured SCells - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE [11, TS38.321] for each activated SCell in the corresponding group of configured SCells - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstOutsideActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP ======skipped part======= If a UE is provided search space sets to monitor PDCCH for detection of DCI format 0_1 and DCI format 1_1 and if one or both of DCI format 0_1 and DCI format 1_1 include a SCell dormancy indication field, - the SCell dormancy indication field is a bitmap with size equal to a number of groups of configured SCells, provided by according to high layer parameter(s) dormancyGroupWithinActiveTime, - each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured Scells - if the UE detects a DCI format 0_1 or a DCI format 1_1 that does not include a carrier indicator field, or detects a DCI format 0_1 or DCI format 1_1 that includes a carrier indicator field with value equal to 0 - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP - - the UE sets the active DL BWP to the indicated active DL BWP

# Contributions summary and proposals

| Apple [1] | Proposal 1: Hardcode A-CSI-RS triggering offset to K0min when A-CSI-RS is triggered by SRS request field in non-codebook-based UL transmission. |
| --- | --- |
| ZTE, Sanechips [2] | Including TC-RNTI for the RNTI not supporting cross-slot scheduling |
| Huawei, HiSilicon [3] | Clarification of Dormancy group indication by higher layer parameter dormancyGroupOutsideActiveTime |

# Reference

- R1-2103086		Maintenance of UE power saving for NR	Apple

- R1-2103602		Remaining issues of Rel-16 power saving	ZTE,Sanechips

- R1-2103748		Remaining issues for Rel-16 UE power saving		Huawei, HiSilicon