3GPP TSG RAN WG1 #104bis-e						R1-210xxxx

e-Meeting, April 12th – 20th, 2021

______________________________________________________________________Agenda item: 8.11.1.2

Source: Moderator (LG Electronics)

Title: Feature lead summary for AI 8.11.1.2 Inter-UE coordination for Mode 2 enhancements

Document for: Discussion and information

- Contents to be discussed in Tuesday’s GTW (Apr. 13th)

After reviewing contributions submitted in this meeting, FL observed that the following two approaches can be considered for the inter-UE coordination.

- Approach 1: Inter-UE coordination to help UE-B’s resource selection procedure (i.e., in a semi-static manner)

  - UE-A explicitly sends the set of resources preferred and/or non-preferred for UE-B’s transmission

    - The information includes time-and-frequency resources preferred and/or non-preferred for UE-B’s transmission

  - SCI or higher layer signaling is used to transmit the coordination information

  - Request signaling or pre-defined event/condition triggers the transmission of coordination information

- Approach 2:	Inter-UE coordination to confirm a validity of UE-B’s selected/reserved resources (i.e., in a dynamic manner)

  - UE-A implicitly sends the set of resources non-preferred for UE-B’s transmission and/or the set of resources where the resource conflict is detected

    - The information includes the presence of expected/potential and/or detected resource conflict on UE-B’s transmission resource

  - PSFCH format is used to transmit the coordination information

  - Pre-defined event/condition triggers the transmission of coordination information

Based on the observations mentioned above, the following draft proposals were made from FL’s perspective:

FL’s proposal:

- Support the following schemes of inter-UE coordination in Mode 2:

  - Inter-UE Coordination Scheme 1:

    - UE-A sends to UE-B the set of resources preferred and/or non-preferred for UE-B’s transmission

      - The coordination information includes time-and-frequency resources preferred and/or non-preferred for UE-B’s transmission

        - FFS on details including a possibility of down-selection between the preferred resource set and the non-preferred resource set

        - FFS whether or not to introduce additional information

      - Down select to one among the following tree options for the container of coordination information

        - 1st SCI

        - 2nd SCI

        - Higher layer signaling (e.g., MAC CE, PC5 RRC)

  - Inter-UE Coordination Scheme 2:

    - UE-A sends to UE-B the set of resources non-preferred for UE-B’s transmission and/or the set of resources where the resource conflict is detected

      - The coordination information includes the presence of expected/potential and/or detected resource conflict on UE-B’s transmission resource

        - FFS on details including a possibility of down-selection between the expected/potential conflict and the detected resource conflict

        - FFS whether or not to introduce additional information

      - PSFCH format is used to convey the coordination information

        - FFS on details including whether to (pre)configure separately PSFCH resource set from that of SL HARQ feedback

FL’s proposal:

- For Inter-UE Coordination Scheme 1, at least the following information is used to determine the time-and-frequency resources preferred and/or non-preferred for UE-B’s transmission

  - UE-A’s sensing result

    - FFS on details including how to obtain it

  - UE-A’s SL resources selected for multiple transmissions of different TBs

  - UE-A’s configured resources for UL

- For Inter-UE Coordination Scheme 2, at least the following information is used to determine the set of resources non-preferred for UE-B’s transmission and/or the set of resources where the resource conflict is detected

  - Time resource conflict between UE-B and other UE(s) in the same group

    - FFS on details including which information (e.g., destination ID) is used to determine it

  - UE-A’s sensing result

    - FFS on details including how to obtain it

FL’s proposal:

- Down select one or more of following options for determining UE-A (transmitting the inter-UE coordination information) and UE-B (receiving and using the inter-UE coordination information):

  - Option 1: UE-A is the intended receiver of UE-B

  - Option 2: UE-A (e.g., RSU, platooning header) and UE-B are determined by higher layer

  - FFS on applicable scenarios/inter-UE coordination schemes for each option

- Summary of contributions

- How UE-A and UE-B are determined

  - Option 1: UE-B is a PSCCH/PSSCH TX UE for data transmission, and UE-A is the intended receiver of UE-B [OPPO,3] [Spreadtrum,4] [vivo,5] [Fraunhofer,12] [CMCC,14] [Xiaomi,16] [Intel,17] [Samsung,20] [Sony,22] [LG,24] [Lenovo,29] [DCM,30]

  - Option 2: UE-A and UE-B are determined via higher layer (e.g. application layer) [Huawei,1] [vivo,5] [Apple,18] [Sony,22] [LG,24]

  - Option 3: UE-A is pre-defined, and UE-B is UEs that can receive inter-UE coordination information from other UE [LG,24]

- How/when UE-A determines the contents of “A set of resources”, including consideration of UL scheduling?

  - Type of “A set of resources”

    - For Type A and/or Type B

      - based on its sensing result of UE-A [Huawei,1] [Nokia,2] [OPPO,3] [vivo,5] [MediaTek,8] [Fujitsu,9] [Fraunhofer,12] [CMCC,14] [ZTE,15] [Xiaomi,16] [Intel,17] [Apple,18] [InterDigital,28] [Lenovo,29]

      - based on UE-A’s transmission [vivo,5] [Fraunhofer,12] [ZTE,15] [Apple,18] [Qualcomm,19] [LG,24] [NEC,27]

        - SL transmission [vivo,5] [Qualcomm,19] [LG,24]

        - UL transmission [vivo,5] [LG,24]

      - based on UE-A’s decision in higher layer [Huawei,1] [LG,24]

      - based on semi-static information [Intel,17] [LG,24]

    - For Type B and/or Type C

      - based on expected/potential resource conflict [vivo,5] [MediaTek,8] [Fujitsu,9] [Fraunhofer,12] [Xiaomi,16] [Intel,17] [Apple,18] [Qualcomm,19] [LG,24] [NEC,27] [DCM,30] [Ericsson,34]

        - PSSCH TX and PSSCH RX [vivo,5] [Intel,17] [Apple,18] [LG,24] [DCM,30]

        - PSSCH TX and PSSCH TX [vivo,5] [Apple,18] [LG,24]

        - PSFCH TX and PSFCH RX [vivo,5] [Apple,18] [DCM,30]

        - PSFCH TX and PSFCH TX [vivo,5] [Apple,18] [DCM,30]

        - SL TX and UL TX [vivo,5] [Intel,17] [LG,24] [DCM,30]

        - SL RX and UL TX [vivo,5] [Intel,17] [LG,24] [DCM,30]

      - based on detected resource conflict [Intel,17] [Apple,18] [Qualcomm,19] [LG,24] [NEC,27] [Ericsson,34]

        - PSSCH TX and PSSCH RX [Intel,17] [Apple,18] [Qualcomm,19] [LG,24]

        - PSSCH TX and PSSCH TX [Apple,18] [Qualcomm,19] [LG,24]

        - PSFCH TX and PSFCH RX [Apple,18]

        - PSFCH TX and PSFCH TX [Apple,18]

        - SL TX and UL TX [Intel,17] [Qualcomm,19] [LG,24]

        - SL RX and UL TX [Intel,17] [Qualcomm,19] [LG,24]

  - Other information in the inter-UE coordination information

    - For Type A and/or B

      - Recommended TX parameters [MediaTek,8]

      - Sensing information [Fujitsu,9] [Apple,18] [Hyundai,32] [ASUSTeK,33]

      - Source ID of UE-B [Fujitsu,9] [Hyundai,32]

      - Destination ID associated with UE-B [Fujitsu,9] [Hyundai,32]

      - Resource conflict type [Fujitsu,9] [Apple,18]

      - Indication of information type (e.g. Type A or Type B) [Fraunhofer,12] [Convida,25]

      - Indication about the intended recipient UE [Fraunhofer,12] [LG,24]

      - Resource pool index [Fraunhofer,12]

    - For Type B and/or Type C

      - Resource conflict type (e.g., resource collision or half-duplex restriction) [Intel,17] [LG,24]

      - Indication of information type (e.g. Type B or Type C) [Intel,17] [LG,24]

- When UE-A sends ”A set of resources” to UE-B, including which UE(s) sends it

  - For Type A and/or Type B

    - Explicit Trigger-based based coordination procedures [Huawei,1] [Nokia,2] [OPPO,3] [Spreadtrum,4] [vivo,5] [CATT,7] [Fujitsu,9] [FUTUREWEI,10] [Zhejiang Lab,11] [Fraunhofer,12] [Mitsubishi,13] [CMCC,14] [ZTE,15] [Xiaomi,16] [Apple,18] [Samsung,20] [ITL,21] [Sony,22] [LG,24] [Sharp,26] [NEC,27] [InterDigital,28] [Lenovo,29]

      - Information carried by the explicit triggering

        - the parameters related to the sensing procedure of UE-B [Huawei,1] [Nokia,2] [OPPO,3] [CATT,7] [Fujitsu,9] [Fraunhofer,12] [ZTE,15] [Xiaomi,16] [InterDigital,28] [Lenovo,29]

        - A set of preferred or non-preferred resources determined at UE-B [Nokia,2] [OPPO,3]

      - Container of the explicit triggering

        - SCI format [Huawei,1] [OPPO,3] [Fujitsu,9] [FUTUREWEI,10]

        - PSFCH format [Apple,18]

        - Higher layer signaling [OPPO,3] [Fujitsu,9] [LG,24]

    - Event-trigger based coordination procedures [Huawei,1] [Fujitsu,9] [FUTUREWEI,10] [Zhejiang Lab,11] [Fraunhofer,12] [Mitsubishi,13] [CMCC,14] [Xiaomi,16] [ITL,21] [Sony,22] [LG,24] [NEC,27] [InterDigital,28] [Lenovo,29]

      - Based on (pre)configured periodicity [Huawei,1] [ITL,21] [LG,24]

      - Based on RSRP measurement [MediaTek,8] [CMCC,14] [ITL,21]

      - Based on distance between UE-A and UE-B [Mitsubishi,13] [CMCC,14] [Xiaomi,16] [ITL,21]

      - When the coordination information is updated for UE-B [LG,24]

      - Based on decision in higher layer [LG,24]

      - Based on congestion status [LG,24]

      - Based on SL HARQ-ACK state at UE-A side [Lenovo,29]

  - For Type B and/or Type C

    - Explicit Trigger-based based coordination procedures  [Intel,17]

      - Container of the explicit triggering

        - 1st SCI format [Intel,17]

    - Event-trigger based coordination procedures

      - Based on detection of resource conflict [Spreadtrum,4] [vivo,5] [MediaTek,8] [Fujitsu,9] [Xiaomi,16] [Intel,17] [Apple,18] [LG,24] [InterDigital,28] [Lenovo,29]

    - Validity check of transmitting inter-UE coordination information

      - Based on distance between UE-A and UE-B(s) [Intel,17] [LG,24]

      - Based on RSRP from UE-A to UE-B(s) [Intel,17] [LG,24] [Lenovo,29]

      - Based on L2 ID(s) from UE-B [Huawei,1] [LG,24]

      - Based on distance between UE-Bs [LG,24]

      - Based on communication range requirement [LG,24]

- How UE-A sends ”A set of resources” to UE-B, including container used for carrying it, implicitly or explicitly or both

  - For Type A and/or Type B

    - 1st SCI format [Nokia,2] [Spreadtrum,4] [MediaTek,8] [FUTUREWEI,10] [Xiaomi,16] [Lenovo,29]

    - 2nd SCI format [Huawei,1] [OPPO,3] [Spreadtrum,4] [vivo,5] [CAICT,6] [MediaTek,8] [Fraunhofer,12] [CMCC,14] [Xiaomi,16] [Samsung,20] [Sony,22] [Lenovo,29]

    - Higher layer signaling (e.g. MAC CE and/or PC5-RRC) [OPPO,3] [Spreadtrum,4] [vivo,5] [MediaTek,8] [Fraunhofer,12] [ZTE,15] [Intel,17] [LG,24] [NEC,27] [Lenovo,29]

    - PSFCH format [OPPO,3] [Sony,22]

  - For Type B and/or Type C

    - PSFCH format [vivo,5] [CAICT,6] [MediaTek,8] [Xiaomi,16] [Intel,17] [Apple,18] [LG,24] [NEC,27] [Lenovo,29] [DCM,30]

      - Priority is inherited by the priority indicated by TX UE [Intel,17] [LG,24] [Lenovo,29]

  - Further consideration of using a single signaling to transmit one or multiple “set of resources” to multiple of UEs  [OPPO,3] [LG,24]

  - Retransmission of the inter-UE coordination information

  - Further consideration on whether shared or dedicated resource is used for inter-UE coordination signaling [Nokia,2]

- How/when/whether UE-B receives “A set of resources” and takes it into account in the resource selection for its own transmission

  - For Type A and/or Type B

    - How UE-B performs resource (re)selection procedure upon receiving the inter-UE coordination information

      - Combine UE-B’s sensing results and resource set provided from UE-A [Huawei,1] [OPPO,3] [vivo,5] [CATT,7] [FUTUREWEI,10] [Fraunhofer,12] [CMCC,14] [Xiaomi,16] [Apple,18] [ETRI,23] [LG,24] [Convida,25] [NEC,27] [InterDigital,28] [Lenovo,29] [Hyundai,32]

      - Use resource set provided from UE-A without a consideration of UE-B’s sensing results [Huawei,1] [vivo,5] [FUTUREWEI,10] [Fraunhofer,12] [Apple,18] [ETRI,23] [Convida,25] [InterDigital,28] [Hyundai,32]

      - Reselect UE-B’s reserved resources [OPPO,3] [Apple,18] [LG,24] [Lenovo,29]

      - It is up to UE-B how to use it [Zhejiang Lab,11] [ZTE,15] [Samsung,20]

    - Cast type of UE-B that can use inter-UE coordination information

      - Unicast [Huawei,1] [OPPO,3] [Spreadtrum,4] [vivo,5] [Fujitsu,9] [Fraunhofer,12] [Mitsubishi,13] [Xiaomi,16] [Samsung,20] [ETRI,23] [LG,24] [Convida,25] [Lenovo,29]

      - Groupcast with HARQ-ACK feedback Option 1 [Spreadtrum,4] [vivo,5] [Fujitsu,9] [Fraunhofer,12] [Mitsubishi,13] [Samsung,20] [LG,24] [Convida,25] [Lenovo,29]

      - Groupcast with HARQ-ACK feedback Option 2 [Huawei,1] [OPPO,3] [Spreadtrum,4] [vivo,5] [Fujitsu,9] [Fraunhofer,12] [Mitsubishi,13] [Samsung,20] [ETRI,23] [LG,24] [Lenovo,29]

      - Broadcast [Spreadtrum,4] [vivo,5] [Fujitsu,9] [Mitsubishi,13] [LG,24] [Convida,25] [Lenovo,29]

    - Validity check of the received inter-UE coordination information at UE-B side

      - Based on RSRP in coordination information [Fujitsu,9]

      - Based on distance between UE-A and UE-B [Fujitsu,9] [Fraunhofer,12] [Mitsubishi,13] [LG,24]

      - Based on RSRP from UE-A to UE-B [Fraunhofer,12] [LG,24]

      - Based on information about target UE of the inter-UE coordination information [Fraunhofer,12] [LG,24]

      - Based on whether the indicated resource set is inside UE-B’s selection window

  - For Type B and/or Type C

    - How UE-B performs resource (re)selection procedure upon receiving the inter-UE coordination information

      - UE-B performs retransmission on the already selected resource(s) [Intel,17] [Qualcomm,19]

      - UE-B reselect all or a subset of its own selected resource(s) [vivo,5] [MediaTek,8] [Fujitsu,9] [Intel,17] [LG,24] [NEC,27] [DCM,30]

      - Continue to use the selected resource(s) [Intel,17]

      - Skip all or a subset of its own selected resource(s) [Intel,17]

      - Further consideration on what is the non-preferred resource set for the resource conflict indication [LG,24]

    - Cast type of UE-B that can use inter-UE coordination information

      - Unicast

      - Groupcast with HARQ-ACK feedback Option 1 [Fujitsu,9] [Qualcomm,19]

      - Groupcast with HARQ-ACK feedback Option 2

      - Broadcast

    - Validity check of the received inter-UE coordination information at UE-B side

      - Resources for initial transmission of UE-B [LG,24]

      - Resources for retransmission of UE-B of which HARQ-ACK state is not ACK [LG,24]

      - Based on HARQ-ACK state at UE-B side [LG,24]

      - Based on the number of (re)transmission of the same TB at UE-B side [LG,24]

- Others

  - Further consideration of indication to UE-A of ID(s) used by UE-B and the intended receiver(s) of UE-B’s transmission [Nokia,2]

  - Further consideration of congestion control for inter-UE coordination signaling [Fujitsu,9] [Zhejiang Lab,11] [Intel,17] [LG,24]

  - Further consideration on the unmonitored slot at UE-B side [Fujitsu,9] [LG,24]

  - Further consideration on the impact on Rel-16 UE sharing the same resource pool with UEs using inter-UE coordination operation [Samsung,20] [Panasonic,31]

  - Further consideration on SL DRX to determine “A set of resources” at UE-A side [ASUSTeK,33]

- Reference

- R1-2102324	Inter-UE coordination in sidelink resource allocation	Huawei, HiSilicon

- R1-2102362	Inter-UE coordination in mode 2 sidelink resource allocation	Nokia, Nokia Shanghai Bell

- R1-2102412	Inter-UE coordination in mode 2 of NR sidelink	OPPO

- R1-2102468	Discussion on inter-UE coordination in sidelink resource allocation	Spreadtrum Communications

- R1-2102540	Discussion on mode-2 enhancements	vivo

- R1-2102576	Considerations on mode 2 enhancements	CAICT

- R1-2102607	Discussion on inter-UE coordination in mode 2 enhancement	CATT, GOHIGH

- R1-2102690	Discussion on Mode 2 enhancements	MediaTek Inc.

- R1-2102720	Considerations on inter-UE coordination for mode 2 enhancements	Fujitsu

- R1-2102781	Discussion on techniques for inter-UE coordination	FUTUREWEI

- R1-2102798	Inter-UE coordination for mode 2 enhancements	Zhejiang Lab

- R1-2102812	Resource Allocation Enhancements for Mode 2	Fraunhofer HHI, Fraunhofer IIS

- R1-2102826	Inter-UE coordination for enhanced resource allocation	Mitsubishi Electric RCE

- R1-2102898	Discussion on enhancements for mode-2 resource allocation	CMCC

- R1-2102921	Discussion on the inter-UE coordination	ZTE

- R1-2102966	Discussion on inter-UE coordination	Xiaomi

- R1-2103049	Inter-UE coordination solutions for sidelink resource allocation mode-2	Intel Corporation

- R1-2103122	Discussion on Inter-UE Coordination	Apple

- R1-2103185	Reliability and Latency Enhancements for Mode 2	Qualcomm Incorporated

- R1-2103258	On Inter-UE Coordination for Mode2 Enhancements	Samsung

- R1-2103271	Inter-UE coordination for mode 2 enhancement	ITL

- R1-2103315	Discussion on reliability and latency enhancements for mode 2	Sony

- R1-2103332	Discussion on mode 2 enhancements	ETRI

- R1-2103379	Discussion on inter-UE coordination for Mode 2 enhancements	LG Electronics

- R1-2103417	On Inter-UE Coordination for Mode 2 Enhancements 	Convida Wireless

- R1-2103484	Discussion on inter-UE coordination for Mode 2 enhancements	Sharp

- R1-2103518	Discussion on mode 2 enhancements	NEC

- R1-2103538	On Inter-UE coordination for Mode 2 enhancement	InterDigital, Inc.

- R1-2103549	Discussion on inter-UE coordination for Mode 2 enhancements	Lenovo, Motorola Mobility

- R1-2103593	Resource allocation for reliability and latency enhancements	NTT DOCOMO, INC.

- R1-2103605	Inter-UE coordination for Mode 2 enhancements	Panasonic Corporation

- R1-2103636	Discussion on mode 2 enhancements	Hyundai Motors

- R1-2103648	Discussion on V2X mode 2 enhancements	ASUSTeK

- R1-2103705	Mode 2 enhancements using Inter-UE coordination	Ericsson

- Appendix

# 4.1	Conclusions made in RAN1#103-e meeting

- Conclusion:

  - The schemes of inter-UE coordination in Mode 2 are categorized as being based on the following types of “A set of resources” sent by UE-A to UE-B:

    - UE-A sends to UE-B the set of resources preferred for UE-B’s transmission

        - e.g., based on its sensing result

    - UE-A sends to UE-B the set of resources not preferred for UE-B’s transmission

        - e.g., based on its sensing result and/or expected/potential resource conflict

    - UE-A sends to UE-B the set of resource where the resource conflict is detected

    - FFS: details of resource conflict, e.g., including type of resource conflict

    - FFS: details of sensing operation at UE-A side

    - FFS: which type(s) of resource set information is(are) beneficial/feasible to which cast type(s)

    - Note: these different types may be used in combination with each other

  - From RAN1 perspective, further study on the feasibility/benefit of inter-UE coordination is required

  - Send an LS to RAN plenary

    - Final LS in R1-2009841

- Conclusion:

  - For the schemes of inter-UE coordination identified as feasible/beneficial, at least the following aspects are further discussed.

    - How/when UE-A determines the contents of ”A set of resources”, including consideration of UL scheduling

    - When UE-A sends ”A set of resources” to UE-B, including which UE(s) sends it

    - How UE-A and UE-B are determined

    - How UE-A sends ”A set of resources” to UE-B, including container used for carrying it, implicitly or explicitly or both

    - How/when/whether UE-B receives “A set of resources” and takes it into account in the resource selection for its own transmission

    - How/whether to define the relationship between support/signaling of inter-UE coordination and cast type

# 4.2	Conclusions made in RAN1#104-e meeting

- Conclusion:

  - RAN1 concludes that the inter-UE coordination in Mode 2 is feasible, and is beneficial (e.g., reliability, etc.) compared to Rel-16 Mode 2 RA, and thus recommends specification of the feature.

    - The detailed observations can be found in the attachment of the LS

- Draft LS in R1-2102165, along with the attachment R1-2102166, is approved (with a typo fix)

  - Final LS in R1-2102168