3GPP TSG RAN WG1 #105-e	R1-2104264

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.3.3

Source:	Huawei, HiSilicon

Title:	Intra-UE multiplexing enhancements

Document for:	Discussion and Decision

Introduction

In the RAN plenary meeting #88e, the scope of Industrial IoT and URLLC was revised in [1]. The description for intra-UE multiplexing and prioritization is captured as follows:

- Intra-UE multiplexing and prioritization of traffic with different priority based on work done in Rel.16 [RAN1]: - Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH. - Specify PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH of different PHY priorities on a BWP of a serving cell including the related cancelation behavior for the PUSCH of lower PHY priority, taking the solution developed during Rel-16 as the baseline

In previous meetings, some agreements have been achieved, mainly on potential multiplexing methods for HP SR and LP HARQ-ACK, and some mechanisms to multiplex HP HARQ-ACK and LP HARQ-ACK on PUCCH/PUSCH [2][3][4]. In this paper, we will discuss the remaining issues for these two aspects and provide our views on the details of the overall procedure for UCI/data multiplexing. Additionally, the prioritization rule for CG PUSCH and DG PUSCH of different priorities as well as the simultaneous PUCCH/PUSCH transmission are also discussed in this paper.

Mechanism to enable/disable intra-UE MUX

In the RAN1#103-e meeting, the following agreements have been achieved for multiplexing HARQ-ACK of different priorities on PUCCH and on PUSCH [3].

Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, support a mechanism for gNB to enable/disable the multiplexing. - FFS the type of the mechanism, e.g. DCI indication and/or RRC configuration - FFS: Interaction between the enable/disable mechanism and other multiplexing conditions - FFS for other types of UCI. Agreements: For HARQ-ACK multiplexing on PUSCH of different priority in R17, support a mechanism for gNB to enable/disable the multiplexing. - FFS the type of the mechanism, e.g. DCI indication and/or RRC configuration, beta_offset=0 - FFS: Interaction between the enable/disable mechanism and other multiplexing conditions - FFS for other types of UCI.

In principle, a unified solution is preferred to enable/disable the multiplexing of HP UCI and LP UCI, regardless if they are carried on PUCCH or on PUSCH. This would minimize the standard effort and simplify the implementation and network management complexity. In the following we discuss further details and present a proposal for common handling:

For the case of multiplexing HP HARQ-ACK and LP HARQ-ACK on PUCCH, it is not preferred to introduce DCI indication. The reason is that such a mechanism could not be applicable for some situations, e.g. for HARQ-ACK feedback for PDSCH(s) that are scheduled by fallback DCI or for SPS HARQ-ACKs. In addition, an extra DCI field would be needed for indicating the multiplexing, which brings unnecessary overhead. Hence, it is better to use RRC signaling to semi-statically enable/disable the MUX.

For the case of multiplexing HARQ-ACK of different priorities on PUSCH, similar to the case of multiplexing HARQ-ACK of different priorities on PUCCH, newly introduced DCI indication is not preferred since it is not applicable in some cases similar to the PUCCH scenario.

Thus, RRC configuration is preferred as a unified method that the gNB can use to enable/disable the multiplexing. In addition, extra conditions on latency and reliability can be defined to judge whether the multiplexing should be allowed or not allowed for certain cases as discussed in section 3.1.4.

Proposal 1: Adopt RRC configuration to enable/disable the multiplexing of HP HARQ-ACK and LP HARQ-ACK on PUCCH, and the multiplexing of HARQ-ACK on PUSCH with different priorities.

- Additional conditions shall be specified to further guarantee the latency/reliability of the HP HARQ-ACK for the overlapping case.

UCI multiplexing on PUCCH

In the RAN1 #102-e meeting [2], three use cases are identified as high priority, i.e. HP HARQ-ACK vs LP HARQ-ACK, HP SR vs LP HARQ-ACK, and HP HARQ-ACK/SR vs LP HARQ-ACK. This section provides our views on the details of multiplexing for each case.

3.1 Case 1: HP HARQ-ACK vs LP HARQ-ACK

In the RAN1 #103-e [3] and #104bis-e meeting [5], the following agreements have been achieved for multiplexing HP HARQ-ACK and LP HARQ-ACK on PUCCH:

Agreements: - For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits are more than 2 bits, down-select from the following options in RAN1#104-e: - Option 1: Support joint coding. - Option 2: Support separate coding. - Option 3: Combination of Option1 and 2. - FFS the details - For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2 bits, provide design details for decision for the following cases in RAN1#104-e: - Multiplexing on a PUCCH format 0 - Multiplexing on a PUCCH format 1

Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, support separate coding for the two HARQ-ACKs. - FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s). - (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N. - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1. - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2.

Based on the above agreements, it can be seen that the following two sub-cases are defined for discussing the multiplexing methods for HP HARQ-ACK and LP HARQ-ACK:

- Case 1-1: the total number of bits is 2.

- Case 1-2: the total number of bits is more than 2.

3.1.1 HP HARQ-ACK vs LP HARQ-ACK with 2 bits total payload

For Case 1-1, as shown in the agreement we need to discuss both the multiplexing on PUCCH format 0 and PUCCH format 1 [3]. To keep it simple, the existing mechanism should be reused as much as possible. For the case of multiplexing on PUCCH format 0, the 1 bit HP HARQ-ACK and 1 bit LP HARQ-ACK can be regarded as 2 bits, and can be transmitted on PUCCH format 0 following the existing sequence based mechanisms. Note that the current PUCCH format 0 can sufficiently guarantee the reliability requirement for 2 bits HP HARQ-ACK. Therefore, we do not see a motivation to further enhance the reliability of HP HARQ-ACK through an unequal-distance CS allocation for 1 bit HP HARQ-ACK plus 1 bit LP HARQ-ACK with respect to its implementation complexity. Similarly, for the case of multiplexing on PUCCH format 1, the 1 bit HP HARQ-ACK and 1 bit LP HARQ-ACK can be regarded as 2 bits in Rel-15 and can be transmitted on PUCCH format 1 using the existing CS and OCC mechanisms.

One potential impact on the performance of HP HARQ-ACK is due to missing LP DCI, which may result in performance loss for the HP HARQ-ACK because there is ambiguity on the HARQ-ACK codebook size. One way to resolve the issue is to use a dedicated PUCCH resource set for the multiplexed HP/LP transmission, as explained in Section 3.1.3.

Proposal 2: For multiplexing HP HARQ-ACK and LP HARQ-ACK on PUCCH in case that the total number of bits is 2 bits, the 1-bit HP HARQ-ACK and the 1-bit LP HARQ-ACK are concatenated and transmitted on PUCCH format 0 or PUCCH format 1 following the existing mechanism.

3.1.2 HP HARQ-ACK vs LP HARQ-ACK with more than 2 bits total payload

For Case 1-2, in the RAN1 #104bis-e meeting, it has been agreed to support separate coding in case the total number of LP and HP HARQ-ACK bits is more than 2, with the target of trying to reuse the legacy encoders. In the following we will discuss the FFS issues for the separate coding method. It should be noted that the same rule can be also applied for HP HARQ-ACK and HP SR vs LP HARQ-ACK with more than 2 bits total payload.

The first remaining issue is the general principle on how to arrange and to reuse current encoders for multiplexing HP HARQ-ACK and LP HARQ-ACK. We discuss this below from the perspectives of encoding, rate matching and RE mapping.

Encoder

In general, 2 separate encoder chains exist for the multiplexing of the UCIs with the same priority on PUCCH. The concatenated bit sequence of HARQ-ACK/SR/CSI part 1 is encoded through encoder chain #1, and the information bit sequence of CSI part 2, if any, is encoded independently through encoder chain #2. So for the multiplexed HP HARQ-ACK and LP HARQ-ACK, it is straightforward to reuse the encoder chain #1 for HP HARQ-ACK, and to reuse the encoder chain #2 for LP HARQ-ACK. As the HARQ-ACK and CSI part 1 are jointly encoded in Rel-15/Rel-16, it is not reasonable to jointly encode HP HARQ-ACK and LP CSI part 1 in encoder chain#1. Therefore, both CSI part 1 and part 2 can be dropped for simplicity.

Rate matching

For rate matching, since HP HARQ-ACK uses encoder chain #1 which is originally intended for HARQ-ACK/SR/CSI part 1, it is straightforward that the rule of rate matching for HP HARQ-ACK follows the equation applied to CSI part 1 in 38.212, restricted by a configured maximum PUCCH coding rate as shown below, where L=0 in case of the HP payload no larger than 11. In addition, to achieve a differentiated protection for the HP HARQ-ACK, separate PUCCH coding rates should be introduced for HP HARQ-ACK and LP HARQ-ACK.

EHP AN=min⁡(Etot,(OHP AN+L)/RHP ANmax/Qm∙Qm)

For LP HARQ-ACK, the number of coded bits after rate matching is equal to the total number of coded bits on all REs minus the number of coded bits of HP HARQ-ACK:

ELP AN=Etot-min⁡(Etot,(OHP AN+L)/RHP ANmax/Qm∙Qm)

Proposal 3: For the encoders of LP and HP HARQ-ACK bits with more than 2 bits total payload,

- Confirm the working assumption, i.e., drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N.

- HP HARQ-ACK can reuse the encoder and rate matching equation of Rel-15 HARQ-ACK+CSI part 1.

- LP HARQ-ACK can reuse the encoder and rate matching equation of Rel-15 CSI part 2.

- Separate PUCCH coding rates should be adopted for HP HARQ-ACK and LP HARQ-ACK.

RE mapping

Different from the encoding/rate matching procedure which is generally implemented by hardware for which the changes are challenging and complex, the RE mapping procedure is generally achieved by software and can be flexibly changed with the evolving of the 3GPP releases. Therefore, from our understanding it is feasible to consider flexible and enhanced RE mapping rule in Rel-17 that could be different from Rel-15/Rel-16.

Observation 1: It is feasible to consider enhanced RE mapping rule in Rel-17 as the changing on RE mapping would be less challenging in implementation.

Another issue that had been discussed in the last meeting is that the existing PUCCH format 2 does not support the transmission of CSI part 2 for Rel-15/Rel-16., i.e., there is only one coding chain for PUCCH format 2. However, regarding the PUCCH format 2 is typically adopted for serving latency sensitive traffics, it is useful to relax the restriction by supporting PUCCH format 2 to carry separately coded HP HARQ-ACK and LP HARQ-ACK in Rel-17. From the implementation perspective, the two coding chains for PUCCH format 3/4 can be applied for format 2 without much challenge, and the RE mapping for HP HARQ-ACK and LP HARQ-ACK on format 2 can also be introduced as per Observation 1.

One more remaining issue is for HP HARQ-ACK or LP HARQ-ACK with 1-2 bit(s) payload. For the Rel-15/Rel-16 the HARQ-ACK with payload 1-2 bit(s) should be carried on format 0/1, so when it comes to Rel-17, we need to consider whether and how to perform the separate coding and multiplexing for the case of LP 1-2bit(s) or HP 1-2bit(s). There could be two options in the following.

- Option 1: Drop the LP HARQ-ACK.

- Option 2: LP HARQ-ACK multiplexed with HP HARQ-ACK by reusing some existing coding method for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s).

1 bit repetition code and 2 bits simplex code have already been supported as UCI encoding behaviors in Rel-15 for UCI on PUSCH. Therefore, the same UCI encoder can be applied to PUCCH without increasing much complexity. As an alternative, the < 3 bits CSI part 2 coding method (e.g., for PMI of codebookType=typeI-SinglePanel with 2 CSI-RS ports) for UCI on PUCCH in Rel-15 can also be considered, where zeros are appended to the UCI bit sequence to expand it to 3 bits before performing the RM coding. From the implementation perspective, both of the two above encoding approaches have been achieved in hardware for UCI encoders irrespective of the channel (PUSCH/PUCCH) or the content (CSI part 2/HARQ-ACK), so it can be understood that Option 2 also reuses the legacy encoder with the minor change of relaxing the restriction of the applicable channel or the content.

The 1-2 bit(s) LP or HP HARQ-ACK cases may generally happen in case of FDD, so dropping the HARQ-ACK as option 1 would have serious impact on system performance and thus it is not preferred.

Proposal 4: For multiplexing of HP HARQ-ACK and LP HARQ-ACK with more than 2 bits total payload and 1-2 bit(s) HP or LP HARQ-ACK, one of the following two alternatives can be considered for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s):

- Alt.1: Reuse the Rel-15 1-2 bits UCI coding methods for UCI on PUSCH, i.e., repetition code/simplex code.

- Alt.2: Reuse the Rel-15 less than 3 bits CSI part 2 coding method for UCI on PUCCH, where zero(s) are appended to the UCI bit sequence to expand it to 3 bits before performing the RM coding.

3.1.3 PUCCH resource determination

In the RAN1 #104-e meeting [4], the following agreements are achieved.

Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK) at least in case the total number of LP and HP HARQ-ACK bits is more than 2. - FFS: The PUCCH resource is configured dedicated for multiplexing of HP HARQ-ACK and LP HARQ-ACK. - FFS in case the total number of LP and HP HARQ-ACK bits is 2. - FFS details

As shown in the agreement above, whether to also use a PUCCH resource in the second PUCCH-Config for multiplexing a HP HARQ-ACK and a LP HARQ-ACK into a PUCCH is still open for the case that the total number of LP and HP HARQ-ACK bits is 2. In this situation, HP HARQ-ACK and LP HARQ-ACK would be multiplexed on PUCCH format 0 or format 1, and thus it is straightforward to select the PUCCH resource from the second PUCCH-Config. The major benefit is that it can be guaranteed that the selected PUCCH resource uses the same power control as well as spatial processing as the PUCCH for HP HARQ-ACK only, and hence ensures the reliability of the HP transmission.

Another open issue is how to determine the PUCCH resource for the multiplexed of HP HARQ-ACK and LP HARQ-ACK. One candidate is to transmit the multiplexed HP HARQ-ACK and LP HARQ-ACK on to the HP PUCCH resource which is the same resource with HP HARQ-ACK only. However, the ambiguity of the existence of LP HARQ-ACK due to missing LP DCI will harm the reliability of the HP decoding. For example, for Case 1-1 (i.e. 1 bit HP HARQ-ACK and 1 bit LP HARQ-ACK) on PUCCH format 0, if the UE misses the LP HARQ-ACK and transmits the sequence with a CS #0 (for ‘0’) or CS #6 (for ‘1’), the gNB still makes a decision among CS #0 (for ‘00’), #3 (for ‘01’), #6 (for ‘11’), #9 (for ‘10’) has been transmitted and would be easier to mistake CS #0 (HP state ‘0’) as CS #9 (HP state ‘1’), or CS #6 (HP state ‘1’) as CS #3 (HP state ‘0’), compared to 1 bit case, as Fig.1 shows below, leading to a larger error probability for HP. Similarly, if the UE transmits only 1 bit BPSK (‘0’ or ‘1’) but the gNB still makes decision based on 2 bits QPSK (‘00’, ‘01’, ‘10’, ‘11’), then it would be easier to mistake ‘00’ as ‘10’ or ‘11’ as ‘01’ compared to the BPSK case, leading to a larger error probability for HP.

Figure 1. UE transmits sequence with CS #0 or CS #6, while gNB receives the sequence with CS #0/#3/#6/#9 pattern, leading to judgement error of HP when the received sequence falls in the red line part

For Case 1-2 (i.e. more than 2 bits HP HARQ-ACK and LP HARQ-ACK on PUCCH format 2/3/4), different rate matching outcome, i.e., a different number of coded bits E, would result into different polar coding parameters. If the LP DCI is missed, the rate matching result may be different, i.e., the HP HARQ-ACK would occupy all REs from the UE view. But the gNB may wrongly assume that the HP HARQ-ACK only occupies parts of the REs according to the configured maximum coding rate and applies a wrong parameter (due to a wrong rate matching result) for decoding, leading to decoding errors for HP HARQ-ACK.

As an alternative, a dedicated PUCCH resource should be configured for the multiplexed HP+LP HARQ-ACKs to avoid a decoding error of HP HARQ-ACK due to the ambiguity of the LP HARQ-ACK number. The gNB can configure different PUCCH resources (RB/CS/OCC) for HP only and for hybrid HP+LP, respectively, and simply perform the blind detection of PUCCH DMRS on the two hypotheses for an easy verification if the LP DCI has been missed. Following this approach, in addition to the up to 4 PUCCH resource sets currently configured for HP HARQ-ACK only, up to 4 extra PUCCH resource sets can be additionally configured for multiplexing  hybrid HP+LP in the second PUCCH-Config.

Fig. 2 below is an illustration of the PUCCH resource configuration in the second PUCCH-Config. The dedicated PUCCH set for hybrid HP+LP can be on different RBs or CSs than the HP only PUCCH resource, partially overlap or totally orthogonal, which can be up to the gNB configuration. This method can achieve flexible PUCCH configuration for hybrid HP+LP and avoid the ambiguity issue that would occur in the case of a missed LP DCI.

Proposal 5: Support a group of dedicated PUCCH sets in the second PUCCH-Configuration to carry the multiplexed HP HARQ-ACK and LP HARQ-ACK.

Figure 2 Different PUCCH resource sets for HP HARQ-ACK and multiplexed HARQ-ACKs

Meanwhile, to achieve differentiated reliability, in the second PUCCH-Config, one extra coding rate for LP HARQ-ACK should be configured for PUCCH format 2, 3, 4 respectively, and then the UE can apply the legacy coding rate and the newly configured coding rate for HP HARQ-ACK and LP HARQ-ACK, respectively. Then the UE can select the PUCCH resource set as well as the number of total RBs according to the separate payload sizes and the separate coding rates for HP HARQ-ACK and LP HARQ-ACK.

3.1.4 MUX conditions

As analyzed above, by configuring dedicated PUCCH resources for hybrid HP+LP at the second PUCCH-Config, the transmission reliability of HP HARQ-ACK can be guaranteed. Then the remaining key problem is how to guarantee the latency. The latency of the HP HARQ-ACK depends mainly on the ending symbol of the PUCCH resource carrying the multiplexed UCI, or more specifically, the ending symbols of REs carrying HP HARQ-ACK in the PUCCH resource. Therefore, if the multiplexing is only allowed when the ending symbol of the PUCCH resource carrying multiplexed UCI or the ending symbol of REs carrying HP HARQ-ACK is no later than X symbols after the ending symbol of the PUCCH resource carrying HP HARQ-ACK, then the impact on the latency for HP HARQ-ACK can be avoided. For simplicity, it is preferred to define X = 0 and to select the ending symbol of the PUCCH resource for the HP HARQ-ACK as the reference symbol to make it immune to mapping rules.

Proposal 6: For HP HARQ-ACK overlapping with LP HARQ-ACK, the multiplexing is allowed only when the PUCCH carrying the multiplexed UCI ends no later than the PUCCH carrying HP HARQ-ACK.

3.2 Case 2: HP SR vs LP HARQ-ACK

In Rel-15/Rel-16, the multiplexing of SR and HARQ-ACK has been discussed in two sub-cases. Similarly, the study of Case 2 can still follow these two sub-cases.

- Case 2-1: HARQ-ACK is of 1~2 bits and carried on PUCCH format 0 or 1

- Case 2-2: HARQ-ACK is of more than 2 bits and carried on PUCCH format 2, 3 or 4

For Case 2-1, the following agreements were achieved in the RAN1 #104-e meeting [4].

Agreements: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis): - Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource. - Opt.1a: The UE does not transmit negative SR. - Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource. - Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource - FFS: whether with power boost to transmit multiplexed payload or not. - Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource. - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource. - Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit. - Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource. - Opt.3: No enhancement over Rel-16. - Other options not excluded. - FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK? Agreements: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, further study the following options (proponents are encouraged to provide more details and analysis): - Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource. - Opt.1a: The UE does not transmit negative SR. - Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource. - Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource - FFS: whether with power boost to transmit multiplexed payload or not. - Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource. - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource. - Opt.2b: Applying QPSK for SR+1-bit HARQ-ACK. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit. - FFS on conditions of multiplexing. - Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource. - Opt.4: For positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource. - Opt.5: No enhancement over Rel-16. - Other options not excluded. - FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK? Agreements: When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis): - Opt.1: The SR and HARQ-ACK are multiplexed and transmitted on the SR resource. - Opt.1a: For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 1 for SR. The value of cyclic shift of sequence, i.e., , of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR. For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR. - Opt.1b: SR and HARQ-ACK are multiplexed and modulated to be transmitted on the SR resource - Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource. - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource. - Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit. - Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource. - Opt.2d: HP SR and LP HARQ-ACK are multiplexed by the Rel-15 cyclic shift only if latency requirement for HP SR is met. Otherwise, drop the LP HARQ-ACK and only transmit the HP SR on its resource. - Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource. - Opt.4: No enhancement over Rel-16. - Other options not excluded. - FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

In the above agreements, the option to transmit SR and HARQ-ACK on SR resources is listed for all cases. It is true that transmitting HP SR on SR resources can guarantee the reliability by reusing the power control of HP SR and also guarantee the latency since the same resource is used. However, for SR with PF0, this would lead to a great resource waste since the PUCCH resource for SR is pre-configured by the gNB and the gNB needs to reserve more RBs/CSs which are not useful in most of the time. Such waste is not negligible for the IIoT scenario where massive robots/AGVs would be connected. Similarly, for SR with PF1, this would lead to multiplexing as many as three bits on PF1 resource.

In principle, we prefer to reuse the Rel-15 mechanism as much as possible unless the R15 way will result in the dropping of HP SR.

- For HP SR of PF0/PF1 and LP HARQ-ACK of PF0, SR can be multiplexed on the PUCCH resource of the LP HARQ-ACK through CS changing. The latency increase is at most 1 OFDM symbol and hence can be negligible since the PUCCH resource of PF0 spans at most two symbols. The reliability can be guaranteed by employing power control for HP UCI. So we think Opt.2c should be supported for cases that HP SR of PF0/PF1 and LP HARQ-ACK of PF0.

- For HP SR of PF1 and LP HARQ-ACK of PF1, the resource selection method in Rel-15 can guarantee the latency and reliability of the multiplexed UCI since the multiplexed UCI is transmitted on the PUCCH resource of SR.

- For HP SR of PF0 and LP HARQ-ACK of PF1, the Rel-15 rule would lead to SR dropping and hence is unacceptable in Rel-17. The simplest solution would be to drop the LP HARQ-ACK when the HP SR is positive, i.e., the prioritization rule of HP UCI vs LP UCI in Rel-16 should be applied.

Proposal 7: For multiplexing HP SR and LP HARQ-ACK with PF0/PF1,

- For the case of HP SR with PF0 vs LP HARQ-ACK with PF0, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15 if SR is positive and transmit only HARQ-ACK on HARQ-ACK resource if SR is negative (i.e. option 2c);

- For the case of HP SR with PF0 vs LP HARQ-ACK with PF1, drop LP HARQ-ACK if HP SR is positive (i.e. option 4/5);

- For the case of HP SR with PF1 vs LP HARQ-ACK with PF0, Opt.2c should be supported.  That is SR is multiplexed on HARQ-ACK resource in the same way as Rel-15 if SR is positive and transmit only HARQ-ACK on HARQ-ACK resource if SR is negative (i.e. option 2c);

- For the case of HP SR with PF1 vs LP HARQ-ACK with PF1, reuse the resource selection method in Rel-15.

For Case 2-2, the multiplexing rule in Rel-15 is to add log2(K+1) bits to represent which SR configuration is positive into the HARQ-ACK bit sequence where K denotes the number of SR configurations whose PUCCH resources overlap with the PUCCH resource carrying HARQ-ACK. In Rel-17, a straightforward way is to employ separate coding for the HP SR and the LP HARQ-ACK on one PUCCH resource to provide different reliability protection and improved resource utilization. For resource determination, we can reuse the dedicated PUCCH resource for hybrid HP HARQ-ACK and LP HARQ-ACK mentioned in 3.1.3 as a unified solution. With respect to the MUX condition, we can reuse the condition for multiplexing the HP HARQ-ACK and the LP HARQ-ACK explained above. That is, the multiplexing is only allowed when the Rel-15 timeline requirement is satisfied and the latency of HP SR is not enlarged.

Proposal 8: For multiplexing HP SR and LP HARQ-ACK with format2/3/4,

- Adopt separate coding to HP SR and LP HARQ-ACK on one PUCCH resource

- The PUCCH resource is selected from dedicated PUCCH resource sets in the second PUCCH-Config for multiplexing HP HARQ-ACK and LP HARQ-ACK

- The multiplexing is only allowed if the ending symbol of the PUCCH resource carrying multiplexed SR and HARQ-ACK is no later than the ending symbol of the PUCCH resource carrying SR.

3.3 Case 3: collision of more than two PUCCHs

For the collision of more than 2 PUCCHs, e.g., one LP HARQ-ACK overlapping with two HP HARQ-ACKs in two sub-slots or one HP SR and one HP HARQ-ACK in one or two sub-slots, the multiplexing rules are more complicated since the multiplexing timeline may only be satisfied by parts of them. Also, the multiplexing order may impact the final result. At the current stage, we think the R16 multiplexing order should be adopted as the starting point but the detailed discussion should be delayed until the multiplexing methods for Case 1 and Case 2 above are clear and have become agreed.

Proposal 9: The discussion about multiplexing more than 2 PUCCHs should be postponed until the multiplexing rules for two colliding PUCCHs are agreed.

UCI multiplexing on PUSCH

In the RAN1 #102-e meeting [2], three use cases are identified as high priority, i.e. LP HARQ-ACK vs HP PUSCH (conveying data only), HP HARQ-ACK vs LP PUSCH (conveying data only), and LP HARQ-ACK and HP HARQ-ACK vs HP/LP PUSCH (conveying data and A-CSI).

4.1 Case 4: LP HARQ-ACK vs HP PUSCH

In the RAN1 #104-e meeting [4], it was agreed to support 0< beta-offset <1 to enable better protection of data reliability.

From RAN1#104-e:

Agreements: For multiplexing LP HARQ-ACK in a HP PUSCH, support 0< beta-offset <1. - FFS value(s) - FFS to additionally support beta-offset =0 or a value disabling the multiplexing - Aim to NOT increase the corresponding bitwidth in the DCI (compared to Rel-16)

Since the bit-field for the beta-offset already exists in the DCI, it is useful to also support beta-offset = 0 to implicitly disable multiplexing of UCI on HP PUSCH. However, enabling/disabling multiplexing based on RRC signaling should be supported also considering uniform solution for all scenarios, since using beta-offset=0 for disabling is not applied in some cases, e.g. configured grant PUSCH.

Proposal 10: For multiplexing LP HARQ-ACK on HP PUSCH scheduled dynamically by UL grant, support beta-offset = 0 to disable the multiplexing.

4.2 Case 5: HP HARQ-ACK vs LP PUSCH

For multiplexing HP HARQ-ACK on LP PUSCH, the reliability can be guaranteed by configuring/indicating a large beta-offset, but the latency may be greatly enlarged, especially when frequency hopping is configured for the LP PUSCH. A simple solution would be to introduce extra conditions and to allow multiplexing only for the case when the latency would not be extended. For that, the method in proposal 6 could be re-used, i.e., allowing the multiplexing only if the ending symbol of the LP PUSCH (or ending of the symbols used for carrying HP HARQ-ACK in LP PUSCH) is no later than the ending symbol of the PUCCH carrying HP HARQ-ACK.

Proposal 11: For HP HARQ-ACK overlapping with LP PUSCH, the multiplexing is only allowed when the ending symbol of the LP PUSCH is no later than the ending symbol of the PUCCH carrying HP HARQ-ACK.

4.3 Case 6: LP HARQ-ACK/CSI & HP HARQ-ACK/CSI vs one PUSCH

The following agreements [5] have been achieved in the RAN1 #104bis-e meeting.

Agreement: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, support separate coding for the two HARQ-ACKs. - It is understood that it is intended that the number of encoding chains for all UCI multiplexing combinations in Rel-17 should not exceed that in Rel-15/16.

Since separate beta-offsets can be configured for different multiplexing combinations, e.g. HP UCI vs HP PUSCH, HP UCI vs LP PUSCH, LP UCI vs HP PUSCH, LP UCI vs LP PUSCH, it is straightforward to still use two different beta-offsets for HP HARQ-ACK/CSI and LP HARQ-ACK/CSI when being multiplexed on one PUSCH.

Proposal 12: For multiplexing HP HARQ-ACK/CSI and LP HARQ-ACK/CSI on one PUSCH, support different beta-offsets for these two UCIs.

For the configuration/indication of different beta-offsets, for fallback DCI or when the DCI format 0_1/0_2 does not include the beta-offset bit-field, the beta-offset values for HP HARQ-ACK and LP HARQ-ACK can be configured by RRC. For DCI format 0_1/0_2 with existing beta-offset bit-field, the bit-file is used to dynamically indicate one of beta-offset values configured by RRC. Specifically, one codepoint value n of the beta-offset field is linked to a triple {βoffsetHARQ-ACK(n), βoffsetCSI-1(n), βoffsetCSI-2(n)}. For Rel-17, if both HP HARQ-ACK and LP HARQ-ACK would be multiplexed on one PUSCH, we can split the column for HARQ-ACK into two columns for HP HARQ-ACK and LP HARQ-ACK separately. That is, one codepoint value is linked to a quadruple {βoffsetHARQ-ACK,βoffsetCSI-1,βoffsetCSI-2, βoffsetHARQ-ACK,1} as shown in the following table.

Table 1 R17 beta-offset indication methods

| Beta-offset bit-field | HARQ-ACK of the same priority | CSI part 1 (of same priority) | CSI part 2 (of same priority) | HARQ-ACK of the different priority |
| --- | --- | --- | --- | --- |
| 0 (1-bit beta-offset indicator) 00 (2-bit beta-offset indicator) | βoffsetHARQ-ACK(0) | βoffsetCSI-1(0) | βoffsetCSI-2(0) | βoffsetHARQ-ACK,1(0) |
| 1 (1-bit beta-offset indicator) 01 (2-bit beta-offset indicator) | βoffsetHARQ-ACK(1) | βoffsetCSI-1(1) | βoffsetCSI-2(1) | βoffsetHARQ-ACK,1(1) |
| 10 (2-bit beta-offset indicator) | βoffsetHARQ-ACK(2) | βoffsetCSI-1(2) | βoffsetCSI-2(2) | βoffsetHARQ-ACK,1(2) |
| 11 (2-bit beta-offset indicator) | βoffsetHARQ-ACK(3) | βoffsetCSI-1(3) | βoffsetCSI-2(3) | βoffsetHARQ-ACK,1(3) |

Proposal 13: For DCI format 0_1/0_2 with existing beta-offset bit-field, one codepoint of the field is linked to a quadruple {βoffsetHARQ-ACK,βoffsetCSI-1,βoffsetCSI-2, βoffsetHARQ-ACK,1} to jointly indicate the beta-offset values for HP HARQ-ACK, LP HARQ-ACK, CSI part 1 and CSI part 2.

Another issue that needs to be discussed is how to reuse the PUSCH encoder chains for HP UCI and LP UCI multiplexed on PUSCH in Rel-17. In the legacy system, there are in total 3 encoder chains available for UCI on PUSCH: encoder chain #1 used to encode HARQ-ACK and SR, encoder chain #2 for CSI part 1, and encoder chain #3 for CSI part 2. For Rel-17, the rules for reusing the legacy encoder chains and the dropping rules in case of collision with the LP HARQ-ACK need to be discussed, to avoid exceeding the number of the UCI encoder chains.

HP HARQ-ACK, and LP HARQ-ACK would be transmitted on HP/LP PUSCH

In Rel-17, if the HP HARQ-ACK, LP HARQ-ACK collide on one PUSCH without CSI, there is no need to drop UCI as the maximum encoder number is not exceeded. The LP HARQ-ACK can reuse the encoder chain #2 or #3 in Rel-15/Rel-16, and it can be FFS whether it should be encoder chain #2 or #3. We recommend to adopt the same rule as for other cases to achieve a unified solution for simplicity.

HP HARQ-ACK, LP HARQ-ACK, and CSI would be transmitted on LP PUSCH

CSI should be dropped to avoid an increased number of encoders. But as opposed to CSI on PUCCH, the HARQ-ACK, CSI part 1 and CSI part 2 are all separately encoded on PUSCH in Rel-15/Rel-16. Thus, we recommend to only drop CSI part 2, while CSI part 1 can be kept. Two candidate principles can be considered for the reuse of the legacy encoders, where the encoder chain in the following includes the behaviors of encoding, rate matching and RE mapping:

- Candidate 1: HP HARQ-ACK reuses the encoder chain #1, and LP HARQ-ACK reuses the encoder chain #3, i.e., LP HARQ-ACK is swapped in the CSI part 2 encoder. It has minor impact to the specification, where only the CSI part 2 related step in multiplexing is replaced with the content of LP HARQ-ACK, while the CSI part 1 is left unchanged.

- Candidate 2: HP HARQ-ACK reuses the encoder chain #1, LP HARQ-ACK reuses encoder chain #2, and CSI part 1 reuses encoder chain #3. This principle can achieve a better performance for LP HARQ-ACK compared to Candidate 1. For example, there is no risk for the LP HARQ-ACK to be punctured by HP HARQ-ACK. But it results in more spec efforts where the CSI part 1 related step also needs to be revisited.

In addition, for LP HARQ-ACK with 1-2 bit(s) payload, it should be rate matched with UL-SCH and/or CSI part 1 instead of puncturing for minimizing the specification impact.

HP HARQ-ACK, LP HARQ-ACK, and CSI would be transmitted on HP PUSCH

It includes two sub-cases:

1) The CSI is semi-static CSI, i.e., the CSI is categorized as LP CSI. As the priority of CSI is lower than that of the HARQ-ACK with the same priority, it is intuitive to drop the CSI to give the priority to LP HARQ-ACK. Similar to the multiplexing of HP HARQ-ACK and LP HARQ-ACK on LP PUSCH, the CSI part 2 should be dropped, and the reorganizing of the encoder chains can refer to Candidate 1 or Candidate 2.

2) The CSI is A-CSI triggered on the HP PUSCH, i.e. the CSI is categorized as HP CSI. The HP CSI triggered by HP DCI should be prioritized over the LP HARQ-ACK, thus the LP HARQ-ACK should be dropped in this sub-case.

Proposal 14: If HP HARQ-ACK and LP HARQ-ACK would be transmitted on HP/LP PUSCH without CSI, the LP HARQ-ACK can be multiplexed by reusing the encoder chain for legacy CSI part 1 or CSI part 2.

Proposal 15: If HP HARQ-ACK, LP HARQ-ACK, and CSI would be transmitted on LP PUSCH, or, if HP HARQ-ACK, LP HARQ-ACK, and semi-static CSI would be transmitted on HP PUSCH, the CSI part 2 should be dropped, and following two candidates can be further studied:

- Candidate 1: HP HARQ-ACK reuses the encoder chain for legacy HARQ-ACK, and LP HARQ-ACK reuses the encoder chain for legacy CSI part 2.

- Candidate 2: HP HARQ-ACK reuses the encoder chain for legacy HARQ-ACK, LP HARQ-ACK reuses the encoder chain for legacy CSI part 1, and CSI part 1 reuses the encoder chain for legacy CSI part 2.

Proposal 16: If HP HARQ-ACK, LP HARQ-ACK, and A-CSI would be transmitted on HP PUSCH, the LP HARQ-ACK should be dropped.

Proposal 17: For collision of HP HARQ-ACK and LP HARQ-ACK with PUSCH, if the LP HARQ-ACK is to be multiplexed on PUSCH, it should be rate matched with the UL-SCH and/or CSI regardless of the LP HARQ-ACK payload.

Prioritization between CG PUSCHs and DG PUSCHs

In the RAN1 #102-e meeting, the following agreement was achieved [2].

Agreements: Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH in R17. - FFS details - Clarify R16 baseline if needed.

In the RAN1 # 103-e meeting, the following agreements are achieved [3].

Agreements: Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell in R17. - FFS the related cancelation behavior for the PUSCH of lower PHY priority and other details. - First clarify what is the scope of this feature, e.g. if overlapping between more than 2 channels is considered. - FFS the timeline requirements. - First clarify what is the behavior of Rel-16 UE in case of DG/CG/UCI overlapping, with and without uplink skipping enabled. - FFS UE capability for this feature. Note: The main bullet has been agreed in the WID by RAN Plenary

For collision between HP CG vs LP DG, the LP DG should be cancelled at least from the first overlapping symbol in case of CG PUSCH begins later than the DG PUSCH (if CG would begin earlier, then the UE can directly stop the DG PUSCH). With respect to the cancellation timeline, we think it is up to UE implementation to guarantee that there is enough time in the PHY layer to cancel a LP DG and to prepare the transmission HP CG.

Proposal 18: For collision between HP CG PUSCH and LP DG PUSCH, PHY layer can make the prioritization so that the UE is expected to transmit the CG PUSCH and cancel the overlapping DG PUSCH at latest from the first symbol that is overlapping with the CG PUSCH.

For collision between LP CG PUSCH and HP DG PUSCH, the R16 timeline for HP DG PUSCH and LP PUCCH for SPS HARQ-ACK can be reused, i.e. the UE is expected to cancel the LP CG PUSCH at least from the first symbol of the HP DG PUSCH, and the cancellation should start no earlier than Tproc,2+d1 after the last symbol of the PDCCH scheduling the DG PUSCH. Meanwhile, an extra processing time of d2 would be introduced for the preparation of the DG PUSCH due to the cancellation process.

Proposal 19: For collision between HP DG PUSCH and LP CG PUSCH, PHY layer can make the prioritization so that the UE is expected to transmit the DG PUSCH and cancel the CG PUSCH by the first overlapping symbol at the latest.

- The UE expects to transmit the DG PUSCH no earlier than Tproc,2+d1 after the last symbol of the PDCCH scheduling the DG PUSCH.

Simultaneous PUCCH /PUSCH transmission on different cells

For simultaneous PUCCH/PUSCH transmission on different cells, the following agreements were achieved in the previous meeting.

Agreements (RAN1#102-e meeting[2]): Support simultaneous PUCCH/PUSCH transmissions on different cells at least for inter-band CA. - FFS how to trigger this function. - FFS for intra-band CA. Agreements (RAN1#104-e meeting[4]): Per UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells can be RRC configured within the same PUCCH group - FFS: dynamic indication

The focus of the feature for simultaneous PUCCH/PUSCH transmission on different cells is limited to the inter-band CA case and also the PUCCH and PUSCH must have different priorities. Note that we don't see the motivation to support simultaneous PUCCH/PUSCH transmission on different cells for same priority. Based on this, we discuss how this feature will work with the Rel-16 intra-UE prioritization rule.

In Rel-16, the processing of intra-UE prioritization is as follows:

- Step 1: Perform the multiplexing of LP PUCCH(s)/PUSCH assuming no overlapping HP UL channels and determine the final LP PUCCH/PUSCH;

- Step 2: Judge whether the final LP PUCCH/PUSCH is overlapping with any HP UL channels before and/or after multiplexing of HP UL channels, and if an overlapping happens, the LP PUCCH/PUSCH is dropped

With the support of simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells, the processing can be slightly changed: the UE performs Step 1 as in Rel-16 and gets the final LP PUCCH/PUSCH; and in Step 2, the UE would still check whether the final LP PUCCH/PUSCH overlaps with any HP UL channel, but only when the overlapping happens and the HP UL channel is in a CC within the same band as the final LP PUCCH/PUSCH, the LP PUCCH/PUSCH will be dropped. That is, if a HP UL channel overlaps with the final LP PUCCH/PUSCH but lies in a CC in a different band, the LP PUCCH/PUSCH can remain and is transmitted simultaneously with the HP UL channel.

Observation 2: If simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells is configured, the prioritization of LP PUCCH/PUSCH can be performed as follows.

- Step 1: Perform the multiplexing of LP PUCCH(s)/PUSCH assuming no overlapping HP UL channels and determine the final LP PUCCH/PUSCH;

- Step 2: Judge whether the final LP PUCCH/PUSCH is overlapping with any HP UL channels before and/or after multiplexing of HP UL channels, and if an overlapping happens on the same serving cell or cells within the same band, the LP PUCCH/PUSCH is dropped.

Conclusions

In this paper, we discussed the possible multiplexing rules for PUCCHs/PUSCHs with different priorities. The following observations and proposals are given:

Proposal 1: Adopt RRC configuration to enable/disable the multiplexing of HP HARQ-ACK and LP HARQ-ACK on PUCCH, and the multiplexing of HARQ-ACK on PUSCH with different priorities.

- Additional conditions shall be specified to further guarantee the latency/reliability of the HP HARQ-ACK for the overlapping case.

Proposal 2: For multiplexing HP HARQ-ACK and LP HARQ-ACK on PUCCH in case that the total number of bits is 2 bits, the 1-bit HP HARQ-ACK and the 1-bit LP HARQ-ACK are concatenated and transmitted on PUCCH format 0 or PUCCH format 1 following the existing mechanism.

Proposal 3: For the encoders of LP and HP HARQ-ACK bits with more than 2 bits total payload,

- Confirm the working assumption, i.e., drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N.

- HP HARQ-ACK can reuse the encoder and rate matching equation of Rel-15 HARQ-ACK+CSI part 1.

- LP HARQ-ACK can reuse the encoder and rate matching equation of Rel-15 CSI part 2.

- Separate PUCCH coding rates should be adopted for HP HARQ-ACK and LP HARQ-ACK.

Observation 1: It is feasible to consider enhanced RE mapping rule in Rel-17 as the changing on RE mapping would be less challenging in implementation.

Proposal 4: For multiplexing of HP HARQ-ACK and LP HARQ-ACK with more than 2 bits total payload and 1-2 bit(s) HP or LP HARQ-ACK, one of the following two alternatives can be considered for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s):

- Alt.1: Reuse the Rel-15 1-2 bits UCI coding methods for UCI on PUSCH, i.e., repetition code/simplex code.

- Alt.2: Reuse the Rel-15 less than 3 bits CSI part 2 coding method for UCI on PUCCH, where zero(s) are appended to the UCI bit sequence to expand it to 3 bits before performing the RM coding.

Proposal 5: Support a group of dedicated PUCCH sets in the second PUCCH-Configuration to carry the multiplexed HP HARQ-ACK and LP HARQ-ACK.

Proposal 6: For HP HARQ-ACK overlapping with LP HARQ-ACK, the multiplexing is allowed only when the PUCCH carrying the multiplexed UCI ends no later than the PUCCH carrying HP HARQ-ACK.

Proposal 7: For multiplexing HP SR and LP HARQ-ACK with PF0/PF1,

- For the case of HP SR with PF0 vs LP HARQ-ACK with PF0, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15 if SR is positive and transmit only HARQ-ACK on HARQ-ACK resource if SR is negative (i.e. option 2c);

- For the case of HP SR with PF0 vs LP HARQ-ACK with PF1, drop LP HARQ-ACK if HP SR is positive (i.e. option 4/5);

- For the case of HP SR with PF1 vs LP HARQ-ACK with PF0, Opt.2c should be supported.  That is SR is multiplexed on HARQ-ACK resource in the same way as Rel-15 if SR is positive and transmit only HARQ-ACK on HARQ-ACK resource if SR is negative (i.e. option 2c);

- For the case of HP SR with PF1 vs LP HARQ-ACK with PF1, reuse the resource selection method in Rel-15.

Proposal 8: For multiplexing HP SR and LP HARQ-ACK with format2/3/4,

- Adopt separate coding to HP SR and LP HARQ-ACK on one PUCCH resource

- The PUCCH resource is selected from dedicated PUCCH resource sets in the second PUCCH-Config for multiplexing HP HARQ-ACK and LP HARQ-ACK

- The multiplexing is only allowed if the ending symbol of the PUCCH resource carrying multiplexed SR and HARQ-ACK is no later than the ending symbol of the PUCCH resource carrying SR.

Proposal 9: The discussion about multiplexing more than 2 PUCCHs should be postponed until the multiplexing rules for two colliding PUCCHs are agreed.

Proposal 10: For multiplexing LP HARQ-ACK on HP PUSCH scheduled dynamically by UL grant, support beta-offset = 0 to disable the multiplexing.

Proposal 11: For HP HARQ-ACK overlapping with LP PUSCH, the multiplexing is only allowed when the ending symbol of the LP PUSCH is no later than the ending symbol of the PUCCH carrying HP HARQ-ACK.

Proposal 12: For multiplexing HP HARQ-ACK/CSI and LP HARQ-ACK/CSI on one PUSCH, support different beta-offsets for these two UCIs.

Proposal 13: For DCI format 0_1/0_2 with existing beta-offset bit-field, one codepoint of the field is linked to a quadruple {βoffsetHARQ-ACK,βoffsetCSI-1,βoffsetCSI-2, βoffsetHARQ-ACK,1} to jointly indicate the beta-offset values for HP HARQ-ACK, LP HARQ-ACK, CSI part 1 and CSI part 2.

Proposal 14: If HP HARQ-ACK and LP HARQ-ACK would be transmitted on HP/LP PUSCH without CSI, the LP HARQ-ACK can be multiplexed by reusing the encoder chain for legacy CSI part 1 or CSI part 2.

Proposal 15: If HP HARQ-ACK, LP HARQ-ACK, and CSI would be transmitted on LP PUSCH, or, if HP HARQ-ACK, LP HARQ-ACK, and semi-static CSI would be transmitted on HP PUSCH, the CSI part 2 should be dropped, and following two candidates can be further studied:

- Candidate 1: HP HARQ-ACK reuses the encoder chain for legacy HARQ-ACK, and LP HARQ-ACK reuses the encoder chain for legacy CSI part 2.

- Candidate 2: HP HARQ-ACK reuses the encoder chain for legacy HARQ-ACK, LP HARQ-ACK reuses the encoder chain for legacy CSI part 1, and CSI part 1 reuses the encoder chain for legacy CSI part 2.

Proposal 16: If HP HARQ-ACK, LP HARQ-ACK, and A-CSI would be transmitted on HP PUSCH, the LP HARQ-ACK should be dropped.

Proposal 17: For collision of HP HARQ-ACK and LP HARQ-ACK with PUSCH, if the LP HARQ-ACK is to be multiplexed on PUSCH, it should be rate matched with the UL-SCH and/or CSI regardless of the LP HARQ-ACK payload.

Proposal 18: For collision between HP CG PUSCH and LP DG PUSCH, PHY layer can make the prioritization so that the UE is expected to transmit the CG PUSCH and cancel the overlapping DG PUSCH at latest from the first symbol that is overlapping with the CG PUSCH.

Proposal 19: For collision between HP DG PUSCH and LP CG PUSCH, PHY layer can make the prioritization so that the UE is expected to transmit the DG PUSCH and cancel the CG PUSCH by the first overlapping symbol at the latest.

- The UE expects to transmit the DG PUSCH no earlier than Tproc,2+d1 after the last symbol of the PDCCH scheduling the DG PUSCH.

Observation 2: If simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells is configured, the prioritization of LP PUCCH/PUSCH can be performed as follows.

- Step 1: Perform the multiplexing of LP PUCCH(s)/PUSCH assuming no overlapping HP UL channels and determine the final LP PUCCH/PUSCH;

- Step 2: Judge whether the final LP PUCCH/PUSCH is overlapping with any HP UL channels before and/or after multiplexing of HP UL channels, and if an overlapping happens on the same serving cell or cells within the same band, the LP PUCCH/PUSCH is dropped.

- References

- RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”, Nokia, Nokia Shanghai Bell, RAN #83, June 2020.

- RAN1 Chairman’s notes, 3GPP TSG RAN WG1 Meeting 102-e, August 2020.

- RAN1 Chairman’s notes, 3GPP TSG RAN WG1 Meeting 103-e, October 2020.

- RAN1 Chairman’s notes, 3GPP TSG RAN WG1 Meeting 104-e, January 2021.

- RAN1 Chairman’s notes, 3GPP TSG RAN WG1 Meeting 104bis-e, April 2021.