3GPP TSG RAN WG1#105-e                              	                      R1- 2104418

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.2.5

Source:	Spreadtrum Communications

Title:	Discussion on PDSCH and PUSCH enhancements for above 52.6GHz

Document for:	Discussion and decision

Introduction

In RAN1#104b-e meeting, the following conclusions were made to further clarify the three alternatives of enhanced type 2 codebook mechanism: [1]

Conclusion: The following is observed for alternative 1 from prior agreement. - For Alt 1 (C-DAI/T-DAI is counted per DCI) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, - C-DAI/T-DAI in DL DCI: Same DAI overhead with Rel-16 single-PDSCH DCI - T-DAI in UL DCI: - In case of single codebook handling feedback for both single and multi-PDSCH scheduling, same DAI overhead with Rel-16 UL DCI - In case of separate sub-codebooks, need additional DAI field (with same bit-width of DAI with Rel-16 UL DCI), in UL DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI - Note that DAI field increment for this case is similar for the case in Rel-15 where CBG is configured - HARQ-ACK codebook generation: - A separate sub-codebook can be generated when multi-PDSCH DCI is configured for a serving cell, similar to the way as 2nd sub-codebook is defined to handle CBG-based scheduling - FFS: whether single codebook or separate sub-codebooks is(are) generated when multi-PDSCH DCI is configured for a serving cell - FFS: how many sub-codebooks are generated when multi-PDSCH DCI is configured for a serving cell and CBG is configured for the serving cell and/or the other serving cell(s) - HARQ-ACK payload size is increased compared to single PDSCH scheduling only, since the number of HARQ-ACK bits corresponding to each DAI of the (sub-)codebook for multi-PDSCH DCI in case of separate sub-codebooks (or for all DL DCIs in case of single codebook) depends on the maximum configured number of PDSCHs for multi-PDSCH DCI across serving cells belonging to the same PUCCH cell group. - The number of HARQ-ACK bits for multi-PDSCH DCI in case of separate sub-codebooks, or for all DL DCIs in case of single codebook, does not depend on the number of actually scheduled PDSCHs, rather, it is fixed as the maximum configured number of PDSCHs. - FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH Conclusion: The following is observed for alternative 2 from prior agreement. - For Alt 2a (C-DAI/T-DAI is counted per PDSCH with a single codebook) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, - C-DAI/T-DAI in DL DCI: Bit-width can be increased (FFS: by how much), in DL DCI not only for multi-PDSCH DCI but also for single-PDSCH DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI - T-DAI in UL DCI: Bit-width can be increased (FFS: by how much), in UL DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI - C-DAI/T-DAI in DL DCI and T-DAI in UL DCI shall be designed such that at most 3 consecutive DCI missing can be resolved, same as in Rel-15/16 NR. - FFS: details on increment of DAI field size - FFS: whether/how to handle the case where different DCI formats (e.g., DCI format 1_0 and DCI format 1_1) have different field sizes for C-DAI/T-DAI - HARQ-ACK codebook generation: - The number of HARQ-ACK bits depends on the number of scheduled PDSCHs. - FFS: ordering of the PDSCHs for DAI counting - FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH Conclusion: The following is observed for alternative 3 from prior agreement. - For Alt 3 (C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, - If M equals to the maximum configured number of PDSCHs, Alt 3 is the same with Alt 1, if the same number of codebooks is assumed. - Else if M equals to 1, Alt 3 is the same with Alt 2. - Otherwise (i.e., 1<M<the maximum configured number of PDSCHs), Alt 3 is similar to Alt 2, except that - The number of HARQ-ACK bits corresponding to each DAI increases by M times. - NACK bits may be padded if the number of scheduled PDSCHs is not an integer multiple of M. - FFS: details on DAI field size - FFS: whether single codebook or separate sub-codebooks is(are) generated when multi-PDSCH DCI is configured for a serving cell - In addition, new RRC parameter to configure M needs to be introduced. - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH

In this contribution, we focus on the method of single DCI scheduling multi-PDSCH/PUSCH in the 60GHz unlicensed band.

Discussion

Single DCI scheduling Multi-PDSCH/PUSCH

In RAN1#104b-e meeting the single DCI scheduling multiple PDSCHs/PUSCHs related issues had been discussed and the following had been agreed [1]:

Agreement: For a DCI that can schedule multiple PDSCHs, - MCS for the 1st TB: This appears only once in the DCI and applies commonly to the first TB of each PDSCH - NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH - RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH - HARQ process number: This applies to the first scheduled PDSCH and is incremented by 1 for subsequent PDSCHs (with modulo operation, if needed) - FFS: - MCS/NDI/RV for the 2nd TB for each PDSCH, including whether scheduling of the 2nd TB for each PDSCH can be supported or not - Details of resource allocation related fields such as VRB-to-PRB mapping, PRB bundling size indicator, rate matching indicator, and ZP CSI-RS trigger - Whether/how to signal CBGFI/CBGTI if CBGFI/CBGTI is supported for multi-PDSCH scheduling - Details of fields that are common with multi-PUSCH scheduling, e.g., TDRA, FDRA, priority indicator, including potential enhancements Agreement: - For a DCI that can schedule multiple PUSCHs, - TDRA: Alt 2 (TDRA table is extended such that each row indicates up to 8 multiple PUSCHs (that can be non-continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.), as per agreement made in RAN1#104-e - FFS: signaling details - Note: Alt 2 does not preclude continuous resource allocation in time-domain. - For a DCI that can schedule multiple PDSCHs, - TDRA: TDRA table is extended such that each row indicates up to 8 multiple PDSCHs (that can be non-continuous in time-domain). Each PDSCH has a separate SLIV and mapping type. The number of scheduled PDSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI. - FFS: signaling details - Note: This does not preclude continuous resource allocation in time-domain. - Note: Multi-PDSCH scheduling for the case of 120 kHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided. Agreement: For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following options can be considered, - Option 1: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table and based on extension of K1 set - Option 1a: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table - Option 2: The set of candidate PDSCH reception occasions is determined according to the last SLIV of each row in the TDRA table - FFS: Codebook generation details, including how to handle the collision with TDD DL/UL configuration and whether/how to extend K1 set based on K1 and slot offset between last PDSCH and other PDSCHs in a row in the TDRA table

In Rel-16 NR-U, an interlaced resource assignment mechanism is introduced to fulfill OCB requirement and compete PSD limitation. Interlace is evenly distributed over the entire bandwidth, so there is no need to perform frequency hopping to obtain frequency selective gain. If interlaced resource assignment mechanism is not supported for 60 GHz band operation, frequency hopping is needed to improve the performance of PUSCH transmission. To simplify the design of frequency hopping, the Rel-15 design principle should be reused, e.g. intra-PUSCH hopping should be supported.

Proposal 1: Frequency hopping should be supported for scheduled PUSCH.

From last RAN1 meeting, the issue of whether to support CBG based retransmission for multi-PDSCH/PUSCHs scheduling or not has been discussed. Obviously, CBG based retransmission for multiple PDSCH/PUSCHs scheduling will lead to large overhead for DCI, e.g. assuming 8 bit CBGTI for single PDSCHs/PUSCHs, then for 8 PDSCHs/PUSCHs scheduling, 64 bits are required. On the other hand, considering the general use case for 60 GHz unlicensed band, the channel state will not change dramatically and if the retransmission is needed, it is more likely that the whole TB should be retransmitted, and then the CBGTI indication will be redundant. Even if the CBG based retransmission is needed for some TBs, gNB could still use single-PDSCH/PUSCH scheduling to indicate the retransmission for each TB. Therefore, there is no need to support CBG based retransmission for multi-PDSCHs/PUSCHs scheduling. Proposal 2: CBG (re)transmission should not be supported when more than one PDSCHs/PUSCHs are scheduled.

Regarding CSI-request and FDRA, in order to simplify the design and reduce the standardization work, the Rel-16 multi-PUSCH scheduling mechanism should be reused.

Proposal 3:

- Apply same rule compared to Rel-16 NR-U for CSI request.

- Apply same method rule compared to Rel-16 NR-U for FDRA.

Regarding whether to support the indication of the MCS/NDI/RV for the 2nd TB for each PDSCH, considering that there is already a distinction between 1st and 2nd TB in legacy DCI 1_1, and the DCI 1_1 is reused for multi-PDSCHs scheduling, the indication of the MCS/NDI/RV for the 2nd TB for each PDSCH should be retained.

Proposal 4: Support to indicate the MCS/NDI/RV for the 2nd TB for multi-PDSCH scheduling.

In RAN1#104-e meeting the Type 2 codebook related issues had been discussed and the following had been agreed [2]:

Agreement: For generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following alternatives can be considered to DAI counting and will be down-selected in RAN1#104bis-e. - Alt 1: C-DAI/T-DAI is counted per DCI. - Alt 2: C-DAI/T-DAI is counted per PDSCH. - Alt 3: C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable (e.g., 1, 2, 4, …). - FFS: Codebook generation details - FFS: How to signal DAI values (e.g., increase of DAI bits for Alt 2 and Alt 3) - FFS: Whether to apply time domain bundling of HARQ-ACK feedback

However, different from the method of the multi-PUSCH scheduling, some issues related HARQ-ACK feedback should be further studied and specified. For example, how to interpret the value of C-DAI/T-DAI and consequently how to determine the HARQ codebook size. Three alternatives were discussed and further clarified in last two meetings with the consideration of PDCCH overhead, PUCCH overhead PDCCH recovery robustness and specification impact. From our observation, For Alt 1, the number of HARQ-ACK bits per DAI is always fixed as the maximum configured number of scheduling PDSCHs. While the DAI size in DL DCI remains the same as single PDSCH DCI. For Alt 2, the number of HARQ-ACK bits depends on the number of scheduled PDSCHs, while the DAI size in DL DCI should be increased. From the perspective of coverage, the coverage of UL channel like PUCCH is usually worse than that of DL channels (especially PDCCH). Therefore, compared with increasing the redundant PUCCH payload, increasing PDCCH overhead may have less impact on the coverage.

Proposal 5: Regarding the generation of type 2 codebook, C-DAI/T-DAI should be counted per PDSCH.

Conclusion

In this contribution, we have discussed PDSCH/PUSCH enhancements in the 60GHz unlicensed band. Based on the discussion in section 2, we provide the following proposals.

Proposal 1: Frequency hopping should be supported for scheduled PUSCH.

Proposal 2: CBG (re)transmission should not be supported when more than one PDSCHs/PUSCHs are scheduled.

Proposal 3:

- Apply same rule compared to Rel-16 NR-U for CSI request.

- Apply same method rule compared to Rel-16 NR-U for FDRA.

Proposal 4: Support to indicate the MCS/NDI/RV for the 2nd TB for multi-PDSCH scheduling.

Proposal 5: Regarding the generation of type 2 codebook, C-DAI/T-DAI should be counted per PDSCH.

Reference

- RAN1#104b-e Chairman notes, 2021-04

- RAN1#104-e Chairman notes, 2021-02