3GPP TSG RAN WG1 #105-e                                                              	       R1-2104889

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.1.2.1

Source: 	Intel Corporation

Title:	Multi-TRP enhancements for PUCCH and PUSCH

Document for: 	Discussion/Decision

# Introduction

According to NR MIMO WID RP-193133, the following is the scope of work in this AI.

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline

In this contribution we provide our views on the features that should be considered to improve reliability for PUSCH and PUCCH channels using multi-TRP/multi-panel framework.

# PUSCH

## PHR

Agreement For PHR reporting related to M-TRP PUSCH repetition, select one from the following options in RAN1 #105-e meeting. - Option 1: Calculate one PHR associated with the first PUSCH occasion (earliest repetition that overlaps with the first slot in which the PUSCH that carries the PHR MAC-CE is transmitted) - Option 2: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, but report one of them - FFS: How to select the PHR for reporting. - Option 4: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, and report two PHRs - Option 5: No changes to legacy PHR reporting

According to 38.321, PHR triggering timers are specific to a MAC entity and a PHR can be triggered if at least in one serving cell the pathloss has changed more than phr-Tx-PowerFactorChange dB. It is also noted that the path loss variation for one cell is assessed by the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time.

Option 2 and Option 4 as agreed above is suited for TRP-specific PHR reporting, however, the existing philosophy of PHR triggering, determination and reporting in RAN2 does not seem to be TRP-specific. In view of the above, we think it is beneficial to consult RAN2 on this matter before taking a decision in RAN1.

Proposal-1: Check with RAN2 whether current options list in RAN1 (esp. options 2 and 4) conflict with RAN2 PHR design philosophy.

## Configured grant operation

Agreement For type 1 or type 2 CG based multi-TRP PUSCH repetition, - Introduce the second fields of 'p0-PUSCH-Alpha' and 'powerControlLoopToUse' in 'ConfiguredGrantConfig’ - For type 1 CG based m-TRP PUSCH repetition, introduce the second fields of ‘pathlossReferenceIndex’, 'srs-ResourceIndicator' and 'precodingAndNumberOfLayers' in 'rrc-ConfiguredUplinkGrant'. - For type 2 CG based M-TRP PUSCH, two SRIs/TPMIs are indicated via the activating DCI. - FFS1: UL PT-RS port(s) and DM-RS port(s) for CG type 1 (vivo, apple. ) - FFS3: Details on RV mapping. (Nokia) - FFS4: Possible transmission occasion for initial transmission (LG) - FFS5: Other TRP specific parameters in 'rrc-ConfiguredUplinkGrant', e.g., 'dmrs-SeqInitialization'

In Rel-16, for a PUSCH corresponding to a CG Type 1 transmission, the UE may assume the association between UL PT-RS port(s) and DM-RS port(s) defined by value “0” in Table 7.3.1.1.2-25 or value "00" in Table 7.3.1.1.1.2-26 described in Clause 7.3.1 of [TS38.212]. The association between PTRS ports and DMRS ports are predefined due to semi static nature of RRC configuration. We think the same philosophy (default relation) can be used for CG PUSCH transmission towards both TRPs.

In terms of RV mapping, the same philosophy as agreed for DG can be reused. The configured RV sequence can be applied separately for the two sets of PUSCH repetitions (towards different TRPs); with a possibility of configuring an offset for the starting redundancy version corresponding to the second TRP.

We also note that in FR2 operation, each PUSCH repetition is TDM-ed and is contributing interference to primarily a single target TRP (unlike FR1). In this case we propose to consider DMRS sequence to be cycled in consecutive repetitions in a TRP specific manner. This allows interference orthogonalization (MU-MIMO) with traffic scheduled in the same cell and interference randomization with traffic scheduled in the neighbor cells.

Proposal-2: For CG operation, the configured RV sequence can be applied separately for the two sets of PUSCH repetitions (towards different TRPs) with a possibility of configuring an offset.

Proposal-3: For CG operation, support RRC configuration of 'dmrs-SeqInitialization' to be TRP specific

## Open loop power control

Agreement For the indication of open-loop power control parameter (OLPC) in DCI format 0_1/0_2, support enhanced open-loop power control parameter (OLPC) set indication by indicating per-TRP OLPC set. - FFS: Details of indication.

In current specification for single-TRP based transmission, the open-loop power control (OLPC) parameter set indication field is 1 bit when SRI field is present in the DCI, and it can be either 1 or 2 bits when the SRI field is not present in the DCI. In particular, if the SRI field does not exist in the DCI, a 1-bit OLPC field can indicate two OLPC values, i.e., 1) a first ‘P0-PUSCH-AlphaSet’ in ‘p0-AlphaSets’, and 2) a first value in ‘P0-PUSCH-Set-r16’ with the lowest ‘p0-PUSCH-SetID’value, a 2-bit OLPC field can indicate three OLPC values, i.e., 1) a first ‘P0-PUSCH-AlphaSet’ in ‘p0-AlphaSets’, 2) a first value in ‘P0-PUSCH-Set-r16’ with the lowest ‘p0-PUSCH-SetID’value, and 3) a second value in ‘P0-PUSCH-Set-r16’ with the lowest ‘p0-PUSCH-SetID’ value.

It is possible that no SRI field exists in the DCI that schedules multi-TRP PUSCH repetition. Since per TRP OLPC set is agreed in last 104be meeting, the total size the OLPC field for multi-TRP transmission can be either 2 or 4 bits without reducing resolution. However, as we have described, the 2-bit OLPC field per TRP is only needed to provide the same maximum flexibility of OLPC parameters (for URLLC) which may not be necessary since it only corresponds to the scenario that no SRI field exists in the DCI. Thus, we prefer to limit the OLPC field per TRP to be 1-bit length, so that the total OLPC field occupies 2 bits no matter the SRI fields exist in DCI or not.

Proposal-4: If SRI field is present, a total of 2 bits can be supported for OLPC parameter set indication where the first and second bit field is associated with the first and second SRS resource sets respectively. The case of SRI field not present case is also limited to a total of 2 bits (1 bit per TRP).

## Dynamic switching field for CB/NCB

Working Assumption For indicating STRP/MTRP dynamic switching for non-CB/CB based MTRP PUSCH repetition, - Introduce a new field in DCI to indicate at least the S-TRP or M-TRP operation - FFS: Whether the new field is 1 bit or 2 bits

In general, we can associate the first/second SRI fields including SRS-SpatialRelationInfo and SRI-PUSCH-PowerControl configurations with the first/second SRS resource sets respectively. Also, we think the new field can be configurable to be 1-bit or 2-bits long.

Proposal-5: Associate the first/second SRI fields including SRS-SpatialRelationInfo and SRI-PUSCH-PowerControl configurations with the first/second SRS resource sets respectively. Support 1-bit or 2-bit for the new dynamic switching bit field according to the following table:

| index | SRI field application to PUSCH | PINL fields (CB only) |
| --- | --- | --- |
| 0 | 1st SRI field applied to all PUSCH Tx occasions | 1st PINL field used |
| 1 | 1st SRI field and 2nd SRI field applied to first and second set of PUSCH Tx occasions respectively | 1st and 2nd PINL fields used |
| 2 | 2nd SRI field applied to all PUSCH Tx occasions | 1st and 2nd PINL fields used |
| 3 | 1st SRI field and 2nd SRI field applied to second and first set of PUSCH Tx occasions respectively | 1st and 2nd PINL fields used |

## PT-RS DMRS association

Agreement For single DCI based M-TRP PUSCH Type B repetition, the indication of PTRS-DMRS association for maxRank > 2 is supported, down select one of the following options in RAN1 #105-e meeting, - Option 1 (4 bits): with a second PTRS-DMRS association field (similar to the existing field), and each field separately indicating the association between PTRS port and DMRS port for two TRPs. - Option 2 (2 bits): using the existing PTRS-DMRS association field in DCI for the first TRP, and using reserved entries/bits in DM-RS port indication field for the second TRP. - Option 3 (2 bits): 1 bit MSB is used to indicate PTRS-DMRS association for the first TRP, and 1 bit LSB is used to indicate PTRS-DMRS association for the second TRP - if maxNrofPorts = 1, the 1 bit indicates one of the first two DMRS ports. - if maxNrofPorts = 2, the 1 bit indicates one of two DMRS ports sharing the same PTRS port.

In current specification, the PTRS-DMRS association is according to Table 7.3.1.1.2-25 and Table 7.3.1.1.2-26 for one and two PTRS ports, respectively with a maximum of 2 bits DCI field size. We slightly prefer that for MaxRank > 2, the PTRS-DMRS association field size can be kept at 2 bits while reducing the resolution of PTRS-DMRS indication with MSB and LSB associated to the 2 TRPs.

Proposal-6:  For UL PTRS port 0 indication (1 PTRS port) and for port 0 and port 1 indication (2 PTRS ports) only values of 0, 1 can be indicated per TRP (reduced resolution) with MSB and LSB associated with the 2 TRPs respectively.

## A-CSI and SP-CSI

Agreement For multiplexing A-CSI on two PUSCH repetitions in the case of multi-TRP PUSCH repetition, - For S-DCI based multi-TRP PUSCH repetition Type B, support multiplexing A-CSI on the first PUSCH repetition corresponding to the first beam and the first (X = 1) PUSCH repetition corresponding to the second beam. - The UE is expected to follow the above operation for multiplexing A-CSI on two PUSCH repetitions only if - the first actual repetition corresponding to the first beam and the first actual repetition corresponding to the second beam does not have the same number of symbols, and - UCIs other than the A-CSI are not multiplexed on any of the two PUSCH repetitions. - When the UE does not follow the above operation, UE multiplexes A-CSI only on the first PUSCH repetition similar to Rel. 15/16. - The content for the two A-CSI should be the same - Note: RAN1 has the assumption on CSI timelines are followed as rel-15/16, including UE shall expect the timeline for the first A-CSI meets Z and Z’ requirement - FFS: For s-DCI based multi-TRP PUSCH repetition Type A and B, support multiplexing of A-CSI on the first PUSCH repetition corresponding to the first beam and the first PUSCH repetition corresponding to the second beam when there is no TB carried in the PUSCH. - The UE assumes that the number of repetitions is 2 regardless of the indicated number of repetitions. - For PUSCH repetition Type B, the first and second nominal repetitions are expected to be the same as the first and second actual repetitions, respectively (no segmentation).

In the current specification when activation DCI is present, A-CSI/SP-CSI is transmitted on the first actual repetition which is expected to be the same length as a nominal repetition. We propose to use the same principle as PUSCH multiplexing case such that A-CSI/SP-CSI is transmitted in the first actual repetition for first beam and the first actual repetition for the second beam and such PUSCH length is expected to be equal to a nominal repetition.

Proposal-7: For the case of multi- TRP PUSCH type B repetition with no scheduled data, support A-CSI/SP-CSI transmission in the first actual repetition for first beam and the first actual repetition for the second beam and such PUSCH length is expected to be equal to a nominal repetition.

# PUCCH

## Frequency hopping

Agreement When inter-slot frequency hopping is configured with Scheme 1, decide one from the below options in RAN1#105-e meeting, - Option 1 - If sequential mapping pattern is configured, frequency hopping is performed on slot level (as in Rel-15). - If cyclical mapping pattern is configured, frequency hopping is performed among the repetitions with the same beam. - Option 2: - gNB always configures sequential mapping pattern and frequency hopping is performed on slot level. (no spec impact) - Option 3: - Frequency hopping is performed on slot level as in Rel-15 (no spec impact).

We think option1- first sub-bullet can be achieved with the current spec, i.e., inter slot frequency hopping. The second sub-bullet can be achieved with configuring intra-slot frequency hopping with cyclical mapping pattern. No specification impact is necessary for supporting frequency hopping with inter/intra slot repetition.

# Conclusions

Proposal-1: Check with RAN2 whether current options list in RAN1 (esp. options 2 and 4) conflict with RAN2 PHR design philosophy.

Proposal-2: For CG operation, the configured RV sequence can be applied separately for the two sets of PUSCH repetitions (towards different TRPs) with a possibility of configuring an offset.

Proposal-3: For CG operation, support RRC configuration of 'dmrs-SeqInitialization' to be TRP specific

Proposal-4: If SRI field is present, a total of 2 bits can be supported for OLPC parameter set indication where the first and second bit field is associated with the first and second SRS resource sets respectively. The case of SRI field not present case is also limited to a total of 2 bits (1 bit per TRP)

Proposal-5: Associate the first/second SRI fields including SRS-SpatialRelationInfo and SRI-PUSCH-PowerControl configurations with the first/second SRS resource sets respectively. Support 1-bit or 2-bit for the new dynamic switching bit field according to the following table:

| index | SRI field application to PUSCH | PINL fields (CB only) |
| --- | --- | --- |
| 0 | 1st SRI field applied to all PUSCH Tx occasions | 1st PINL field used |
| 1 | 1st SRI field and 2nd SRI field applied to first and second set of PUSCH Tx occasions respectively | 1st and 2nd PINL fields used |
| 2 | 2nd SRI field applied to all PUSCH Tx occasions | 1st and 2nd PINL fields used |
| 3 | 1st SRI field and 2nd SRI field applied to second and first set of PUSCH Tx occasions respectively | 1st and 2nd PINL fields used |

Proposal-6:  For UL PTRS port 0 indication (1 PTRS port) and for port 0 and port 1 indication (2 PTRS ports) only values of 0, 1 can be indicated per TRP (reduced resolution) with MSB and LSB associated with the 2 TRPs respectively.

Proposal-7: For the case of multi- TRP PUSCH type B repetition with no scheduled data, support A-CSI/SP-CSI transmission in the first actual repetition for first beam and the first actual repetition for the second beam and such PUSCH length is expected to be equal to a nominal repetition

# References

[1] RP-193133, New WID: Further enhancements on MIMO for NR, Samsung, RAN#86