3GPP TSG RAN WG1 #105-e		R1-2105160

e-Meeting, May 10th – 27th, 2021

Agenda Item: 	8.3.1.1

Source: 	Sony

Title: 	Retransmission of dropped HARQ-ACK for URLLC

Document for: 	Discussion / decision

# Introduction

In RAN1#102e we made the following agreements:

Agreements:

Study further at least the following schemes:

- SPS HARQ skipping for ‘skipped’ SPS PDSCH

- PUCCH repetition enhancements (at least for HARQ-ACK), e.g., sub-slot based, etc.

- Retransmission of cancelled HARQ

- SPS HARQ payload size reduction and / or skipping for ‘non-skipped’SPS PDSCH

- Type 1 HARQ codebook based on sub-slot PUCCH config

- PUCCH carrier switching for HARQ feedback

Companies are encouraged to provide detailed analysis and comparison accordingly

One of the schemes agreed for further study is the retransmission of dropped HARQ-Ack.  This contribution discusses some considerations on this topic.

# Retransmission of Cancelled HARQ-ACK

The following methods were considered for the retransmission of cancelled HARQ-ACK [1]:

- Method 1: Enhanced Type 3 HARQ-ACK Codebook

- Method 2: New Type 4 HARQ-ACK Codebook with a one-shot trigger

Type 3 Codebook was introduced in Rel-16 for NR-U to enable the UE to retransmit HARQ-ACKs due to LBT failure.  Type 3 Codebook is triggered by a DL Grant via the “One-shot HARQ-ACK request” DCI field, where the UE transmits HARQ-ACK for all HARQ-ACK process IDs across configured CCs.  It was argued that this has high overhead and therefore some companies proposed to enhance Type 3 Codebook by reducing the overhead.  We do not see the need for such enhancements and if overhead is an issue, it is better to introduce a new HARQ-ACK codebook for this purpose, e.g. Method 2 rather than try to enhance Type 3 Codebook which would produce a sub-optimal solution.  However, we do think that it is beneficial to support Type 3 Codebook also for DCI Format 1_2 to improve reliability of HARQ-ACK in NR-U.

Proposal 1: Do not support enhancements for Type 3 HARQ-ACK Codebook.

Proposal 2: Consider introducing one shot trigger for Type 3 Codebook in DCI Format 1_2.

In Method 2, a new “one-shot trigger” is introduced to retransmit only dropped HARQ-ACK, i.e. in a new Type 4 HARQ-ACK Codebook, which has significantly less overhead compared to the method used in Type 3 Codebook or any of the enhancements proposed to reduce Type 3 Codebook overheads.  If overhead is an issue, we prefer to address it directly rather than trying to enhance Type 3 Codebook since Type 3 Codebook has a semi-static size and therefore any enhancements would not provide as effective an overhead reduction as a codebook that retransmits only dropped HARQ-ACKs.

Observation 1: Since the size of Type 3 HARQ-ACK Codebook is semi-static, any enhancement made to Type 3 HARQ-ACK Codebook for overhead reduction can never be as effective as a new HARQ-ACK codebook that retransmits only dropped HARQ-ACK.

For the triggering of the new Type 4 HARQ-ACK Codebook, some companies proposed that it is triggered using a new “one-shot trigger” in a DL Grant scheduling a PUCCH only [2] and/or a new “one-shot trigger” in an UL Grant scheduling a PUSCH to retransmit the dropped HARQ-ACKs [3] [4].  The PUSCH can be triggered to carry only dropped HARQ-ACK, i.e. without any uplink data.  This trigger would allow the gNB to trigger for HARQ-ACK retransmission without the need to also schedule data transmission to or from the UE, which can reduce latency.

Proposal 3: Consider a new “one-shot trigger” to trigger a new Type 4 HARQ-ACK Codebook to retransmit only dropped HARQ-ACK(s), where this “one-shot trigger” can be transmitted using a DL Grant that can schedule a PUCCH without a PDSCH and/or an UL Grant that can schedule a PUSCH without any uplink data.

The following options were considered in RAN1#104bis-e on the definition of “dropped HARQ-ACK” that are to be retransmitted using the new “one-shot trigger”:

- Option 1: the last dropped PUCCH occasion is to be re-transmitted

- Option 2: dynamic indication of the PUCCH occasion that is to be re-transmitted

- Option 3: based on a timing window of the PUCCH occasion(s) that is/are to be re-transmitted

Option 1 retransmits the HARQ-ACK of the last dropped PUCCH which may be reasonable for URLLC but is limited to only being applicable to 1 dropped PUCCH.  Option 2 allows dynamic indication of which PUCCH occasion to retransmit, which provides high flexibility but requires additional overhead in the DCI.  Option 3 retransmits HARQ-ACK of PUCCHs within a time window, which is not as limited as Option 1 but lack sthe flexibility of Option 2.  Option 1 can be implemented using Option 3 by configuring a short time window such that it captures only 1 dropped PUCCH.  These 3 options should be further discussed.

Proposal 4: Further consider the following 3 options on the definition of “dropped HARQ-ACK” under the context of HARQ-ACK retransmission:

- Option 1: the last dropped PUCCH occasion is to be re-transmitted

- Option 2: dynamic indication of the PUCCH occasion that is to be re-transmitted

- Option 3: based on a timing window of the PUCCH occasion(s) that is/are to be re-transmitted

# Conclusion

In this contribution, we discuss some aspects on retransmissions of dropped HARQ-ACK.  We observe the following:

Observation 1: Since the size of Type 3 HARQ-ACK Codebook is semi-static, any enhancement made to Type 3 HARQ-ACK Codebook for overhead reduction can never be as effective as a new HARQ-ACK codebook that retransmits only dropped HARQ-ACK.

We therefore propose the following:

Proposal 1: Do not support enhancements for Type 3 HARQ-ACK Codebook.

Proposal 2: Consider introducing one shot trigger for Type 3 Codebook in DCI Format 1_2.

Proposal 3: Consider a new “one-shot trigger” to trigger a new Type 4 HARQ-ACK Codebook to retransmit only dropped HARQ-ACK(s), where this “one-shot trigger” can be transmitted using a DL Grant that can schedule a PUCCH without a PDSCH and/or an UL Grant that can schedule a PUSCH without any uplink data.

Proposal 4: Further consider the following 3 options on the definition of “dropped HARQ-ACK” under the context of HARQ-ACK retransmission:

- Option 1: the last dropped PUCCH occasion is to be re-transmitted

- Option 2: dynamic indication of the PUCCH occasion that is to be re-transmitted

- Option 3: based on a timing window of the PUCCH occasion(s) that is/are to be re-transmitted

# References

[1] R1-2104039, “Final moderator summary on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT,” Moderator (Nokia), RAN1#104bis-e

[2] R1-2102392, “HARQ-ACK enhancements for Rel-17 URLLC/IIoT,” OPPO, RAN1#104bis-e

[3] R1-2102819, “HARQ-ACK Feedback Enhancements for URLLC/IIoT,” Nokia, Nokia Shanghai Bell, RAN1#104bis-e

[4] R1-2102493, “Discussion on HARQ-ACK enhancements for eURLLC,” ZTE, RAN1#104bis-e