3GPP TSG RAN WG1 #105-e	R1-2105493

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.10.1

Source: 	LG Electronics

Title: 	Discussions on IAB resource multiplexing enhancements

Document for:	Discussion and decision

Introduction

In RAN1#104b-e meeting, several agreements were made regarding on the applicability of a given multiplexing capability, frequency domain resource for simultaneous operation, spatial domain resource for simultaneous operation, and dual-connectivity to support multiple parent DUs [1]. Following the previous discussion, we continue to discuss on specification of resource multiplexing enhancement between child and parent links of an IAB node based on above agreements.

Simultaneous operation schemes between MT and DU

Adaptation of an IAB-node’s multiplexing operation

The multiplexing operation of an IAB node at a specific time resource can be determined by various factors, where the followings can be considered as candidate factors.

- D/U/F resource type at the DU and MT

A main element that determines the multiplexing operation of the IAB node is the combination of the Tx/Rx direction of the MT and DU. Depending on D/U/F resource type of MT and DU, IAB node and its parent node can determine the multiplexing operation of IAB node in a time resource based on semi-static configurations (i.e., TDD configuration for MT and DU resource configuration for DU) and dynamic indications (i.e., slot format indication by DCI format 2_0, dynamic grant for DL/UL scheduling).

However, the actual resource direction in the DU flexible resource is determined by the IAB-DU dynamically and indicated to the child-MT/UE. Thus, the parent node does not know the resource direction of the DU in the flexible resource, so it cannot determine multiplexing operation of the IAB-node. In order for the parent node to know the multiplexing operation of the IAB node in DU flexible resource, it is necessary for the IAB node to report related information to the parent node.

- Timing mode

Information on the timing mode of the IAB node should be shared with the parent node in advance, and the IAB node needs to operate by applying its own timing based on the information. For this, for example, the timing mode according to the time resource of the IAB node can be configured from the network.

But, if it is possible for the IAB node to operate at a different timing from the planned timing mode, the multiplexing operation of the IAB node may be different. When the multiplexing operation of the IAB node is changed due to the dynamic change of the timing mode, it is necessary to discuss the necessity and effect of the reporting related information to the parent node.

- Power control

In the case of dynamic power sharing between the MT and the DU, the transmission power of the DU or the MT may be reduced or the transmission may not be performed depending on the power allocation between MT and DU of the IAB-node. Due to this, the IAB-node could be forced to dynamically perform half-duplex operation on the resource for which simultaneous operation was planned. As such, the multiplexing operation of the IAB node can be dynamically changed due to the power control issue, and it is necessary to discuss the need and effect of the dynamic reporting related information to the parent node.

Basically, we think that the multiplexing operation of the IAB node, which is determined semi-statically, can be implicitly determined by network configurations. For example, it is possible to determine the multiplexing operation based on the resource direction of the IAB DU and MT and the timing mode of the IAB node in the corresponding resource.

On the other hand, as discussed above, the multiplexing operation of the IAB node may be dynamically determined, or the applicability of the semi-statically determined multiplexing operation may be changed. In this case, it is questionable whether dynamic reporting of information about the multiplexing operation of the IAB node to the parent node will have a practical effect compared to the specification effort. In our view, at least dynamic reporting of the multiplexing operation in DU flexible resource is considerable. When the DU periodically indicates the resource direction in its flexible resource to a child MT through DCI format 2_0, the operation of reporting related information to the parent node also seems feasible.

Proposal 1: The multiplexing operation of an IAB node can be determined implicitly based on semi-static D/U/F resource type of MT and DU at least.

Proposal 2: Discuss how to align the understanding on the multiplexing operation of an IAB node in DU flexible resource with its parent node.

Frequency domain resource for simultaneous operation

In this section, we discuss on frequency domain resource utilized for MT and DU for simultaneous operation.

- Frequency domain H/S/NA configuration

For frequency domain H/S/NA, frequency granularity for resource configuration is to be discussed. In our view, frequency resource for an attribute does not need to finely divided and distributed. Instead, it seems better to divide frequency resources of DU and MT into large chunks. For an approach, multiple sets of frequency resource can be defined which is similar to the BWP configuration scheme (i.e., frequency resource indication by starting PRB position and the number of PRBs). In DU perspective, one or multiple group(s) of consecutive RBs can be defined and a RB group can correspond to a Hard, Soft, or NA resource type. Further, multiple sets of frequency resource can be designated as the same resource type.

Since the frequency domain H/S/NA attributes of a DU are related to the operating BWP of the MT and the child MT of the DU, it is not desirable to change the attributes frequently. Therefore, rather than changing the frequency domain H/S/NA attributes for each time unit (slot/multi-slots), it would be desirable to keep the same the attributes before reconfiguration. If dynamic change is required according to the situation, it is more appropriate to use a soft resource rather than changing the H/NA resource attributes.

It is also necessary to consider frequency domain H/S/NA configuration according to DU and UL resources. Since the DL BWP and UL BWP of the MT can be configured independently, it seems natural that the DU frequency domain H/S/NA attributes can be configured differently for DL and UL.

Proposal 3: In a DU cell perspective, one or multiple group(s) of consecutive PRBs can be defined where a PRB group corresponds to a Hard, Soft, or NA resource type for frequency domain multiplexing.

Proposal 4: Frequency domain H/S/NA attributes can be configured independently for DL and UL.

- Frequency domain resource for TDM operation

Even if an IAB-node has FDM capability, a transition may occur between TDM and FDM operation.

When MT and DU of the IAB-node are operated in FDM, the IAB-node applies a frequency domain H/S/NA configuration to determine the operating frequency resources of the DU and MT.

On the other hand, when the IAB-node operates in TDM, it is necessary to discuss whether to apply frequency domain H/S/NA configuration and to limit the operating frequency resources of DU and MT also, or to enable MT and DU to operate in all frequency resources by not applying frequency domain H/S/NA configuration.

When the IAB-node is operated in TDM, it seems better not limiting the operating frequency resources by frequency domain H/S/NA attributes in terms of resource utilization.

Proposal 5: Frequency domain H/S/NA configuration does not applied when the IAB-node is operated in TDM.

- Dynamic availability indication for soft resource

Soft resource type in frequency domain is supported for flexible resource utilization between MT and DU. In this meeting, it is to be discussed the indication of the frequency domain soft resource availability.

In our view, the availability indication of time domain soft resource and frequency domain soft resource should be independently applicable depending on the situation. For example, as discussed prior sections, only frequency domain availability information can be applied to time resources with FDM operation. Also, only time domain availability information can be applied to resources for TDM operation. Therefore, the time domain and frequency domain soft resource availability indication may be transmitted through the same DCI, but it is preferred to be indicated independently within the DCI.

For this, we propose that the mapping table between values of resourceAvailability elements and types of soft symbol availability is specified independently for time domain and frequency domain resource. In this case, both a method in which time domain and frequency domain soft resource availability are indicated in the same field and a method in which they are transmitted through different fields within a DCI may be considered.

When the time domain and frequency domain soft resource availability for a DU cell are indicated through different fields, the frequency domain soft resource availability can be indicated by a different field from the AI index field that indicates the time domain soft resource availability for the DU cell. For detail, the followings can be considered.

- In order to indicate the location of a field indicating frequency domain soft resource availability, an additional positionInDCI-AI is introduced.

- Frequency domain soft resource availability is indicated through the next field of the AI index field indicated by the existing positionInDCI-AI.

Meanwhile, when time domain and frequency domain soft resource availability for a DU cell are indicated through the same field in a DCI, the following schemes may be specifically considered.

- availabilityCombinations linked to a positionInDCI-AI is configured for time domain and frequency domain, respectively.

- resourceAvailability linked to a availabilityCombinationId is configured for time domain and frequency domain, respectively.

Proposal 6: The mapping table between values of resourceAvailability elements and types of frequency domain soft resource availability is specified independently from the table for time domain soft resource defined in Rel-16.

Spatial domain resource for simultaneous operation

In terms of spatial domain resource for IAB node, it is agreed to support dynamic indication to facilitate simultaneous operation and interference management [1].

In the case of simultaneous transmission (DU DL-Tx and MT UL-Tx) or simultaneous reception (DU UL-Rx and MT DL-Rx) operation between DU and MT of the IAB node, interference may occur between multiple beams. It would be desirable to use a method of reducing inter-layer interference by using beams in which DU and MT are orthogonal to each other.

At this time, it is considered that the beam resource of the DU to be dynamically controlled by the parent DU is not desirable. This is because the scheduling subject to the child link of the IAB node is the DU, and the parent node cannot accurately know the required beam direction for the DU. Instead, it is desirable that the parent DU determines and indicated the MT beam based on the information on the desired beam direction from the IAB node. To this end, the parent DU needs to be shared information on beam pair(s) capable of simultaneous operation between DU and MT of the IAB node.

- For simultaneous Tx, the IAB node can report information about the pair(s) between the MT UL-Tx beam and the DU DL-Tx beam to the parent DU. As an example, as information on the UL-Tx beam used for the time resource of simultaneous Tx, the MT may report the SRI or Tx panel ID (if the ID is defined and multi-panels are used). Parent DU can instruct the UL transmission of the MT based on the reported SRI (or Tx panel ID) to help the simultaneous Tx operation between the DU and the MT of the IAB node. This mechanism for reporting SRI (or Tx panel ID) which reflects MT preferred Tx beam paired with beam used in a resource for DU Tx (or Rx) can be applied for the case of simultaneous MT-Tx and DU-Rx operation.

- For simultaneous Rx, the IAB node can report information to the parent DU about the pair(s) between the MT DL-Rx beam and the DU UL-Rx beam. For example, as an information on the DL-Rx beam used for the time resource of the simultaneous Rx, the MT may report the TCI-RS information to the parent DU. Then, the parent DU may indicate the TCI state and QCL condition for DL ​​reception to the MT based on the reported TCI-RS information to help the simultaneous Rx operation between the DU and the MT of the IAB node. This mechanism for reporting TCI-RS information which reflects MT preferred Rx beam paired with beam used in a resource for DU Rx (or Tx) can be applied for the case of simultaneous MT-Rx and DU-Tx operation.

Additionally, when considering the difficulty and overhead of dynamic information sharing for the desired beam pair of the IAB node, it may be considered to limit the beam pairs that can be used by IAB node depending on frequency resources.

Proposal 7: It is supported that IAB node reports information on beam pair(s) capable of simultaneous operation between DU and MT (i.e., simultaneous Tx-Tx, simultaneous Rx-Rx, simultaneous Tx-Rx,  simultaneous Rx-Tx) to its parent DU

Dual-connectivity to support multiple parent DUs

For dual-connectivity of MT to support two parent DUs, we discuss solutions to address resource coordination/scheduling collision issues between parent nodes.

Slot format collision between two parent DUs

At first, we address scheduling collision issue between two parent DUs due to indication of the slot format by DCI format 2_0.

In case of semi-static TDD configuration of MT by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, the coordination between two parent DUs can be assumed for both of intra-donor and inter-donor DC scenarios.

However, slot format indication by DCI format 2_0 is determined and indicated from each parent DU with dynamic manner. Since dynamic information sharing between two parent DUs cannot be assumed, it is difficult to avoid slot format collision or dynamic DL/UL scheduling conflict between two parent DUs.

Therefore, for the case that a MT is scheduled to receive downlink and transmit uplink simultaneously, the MT behaviour should be discussed and defined. Since the MT can perform scheduled transmission to/reception from only one of two parent DUs, and the priority rule between transmission and reception can be defined.

For example, when a MT is scheduled to transmit and receive simultaneously, transmission to/reception from a specific parent node (e.g., MCG) can be prioritized. For the other example, when a symbol is configured as Hard for parent DU_1 and NA for parent DU_2, it can be considered to give priority to transmission/reception with parent DU_1, since parent DU_2 may not perform transmission/reception in the symbol.

Proposal 8: The coordination of TDD configuration of MT between two parent DUs can be assumed for both of intra-donor and inter-donor DC scenarios.

Proposal 9: In case of DL/UL collision between two parent links, the priority rule should be defined in MT perspective.

Non-simultaneous operation between two parent DUs

In Rel-15 and Rel-16, NR designed Dual Connectivity for FR1/FR1 and FR1/FR2. In this scenarios, it was assumed that the carriers for dual connectivity are located far way, and inter-carrier interference is negligible. So, it was assumed that UE can receive simultaneously channel/signal from two cells with different carriers in FR1/FR1 or FR1/FR2.

But, for operating inter-carrier, intra-band DC scenario for FR2, it needs to clarify MT operation for transmission and reception in terms of antenna configurations (e.g., same panel/separate panel for multiple carriers) and combination of carriers. In our view, simultaneous transmission or reception between two parent links may not be available in some cases. For example, Tx/Rx timing and/or beam direction for two parent links may not be aligned. Then, depending on RF/antenna implementation, the MT cannot perform transmission/reception for two parent links simultaneously.

Considering these aspects, MT behaviour for non-simultaneous Tx/Tx and Rx/Rx between two parent links also should be discussed and supported.

Proposal 10: It is necessary to support a MT which cannot perform simultaneous Tx/Tx and Rx/Rx for two parent links.

Soft symbol availability determination

The collision issue between the availability indications of soft resource from each parent DU also should be discussed. When a MT is dual connected to two parent DUs, we discuss from whom the availability indication of DU is received and related behaviour of MT and DU.

For discussion, an example of multi-parent scenario is depicted in Figure 1. In this example, it is assumed that MT3 with multi-carrier is connected two multiple parents (DU1 and DU2).

Figure 1. An example of multi-parent scenario

Inter-band inter-carrier DC scenario

Regarding on soft resource availability indication of a DU cell, it is necessary to clarify that which of the two parent DUs or both of the information can be indicated. It is desirable that the soft resource availability information is given by corresponding parent DU(s) that is affected by the operation of the DU cell. In case of inter-band inter-carrier DC scenario, only one parent DU of the two parent DUs will be affected by the operation of the cell, and it make sense to receive the soft resource availability indication for the DU cell from this parent DU.

For example, when the carrier frequencies of DU1 and DU2 are f1 and f2 respectively (f1 and f2 are in inter-band) and a cell of DU3 has carrier frequency f1 in Figure 1, the soft resource availability for the cell of DU3 can be indicated from DU1 only. Then, the DU cell can determine its soft resource availability based on the indication. MT3 also takes into account the indication to determine its operation in carrier frequency f1. MT operation in f2 may not associated with the DU cell, then the soft resource availability indication does not need to be considered.

Intra-band inter-carrier DC scenario

For intra-band inter-carrier DC scenario, the operation of a DU3 cell affects to MT operation in both carrier frequencies f1 and f2.  In other words, it corresponds to both parent DUs. In this case, it seems necessary to discuss whether one parent DU or both of two parent DUs can indicate the soft resource availability for a DU cell.

When the soft resource availability for a DU cell is indicated from one parent DU only, the other parent DU does not control the availability of the corresponding DU cell at all, and it cannot determine the availability of the DU cell correctly.

On the other hand, when the soft resource availability for a DU cell can be indicated from two parent DUs, collision may occur in the availability indication received from the two parent DU cells. For example, one parent DU may indicate that a soft resource of the DU cell is available by DCI format 2_5, but the other parent DU may not indicate that it is available. In this case, the DU cell needs to determine its final availability based on availability indication from two parent DUs. This approach make that both parent DUs may be involved in the availability of the DU cell. However, as in the previous case, if the availability indicated by the parent DU and the final availability determined by the DU cell are different, the parent DU cannot accurately determine the operation of the DU cell.

In overall, the MT may be configured to monitor DCI format 2_5 for a DU cell only in one carrier frequency or multiple carrier frequencies.

When the MT monitors DCI format 2_5 for a DU cell only in one carrier frequency, the availability of the soft resource of the corresponding DU cell can be determined based on the indication.

On the other hand, when the MT is configured to monitor DCI 2_5 for a DU cell from two carrier frequencies, it is possible to determine the availability of the soft resource of the corresponding DU cell as follows.

- Alt 1. The soft resource is determined as available, if at least one parent node indicates that the DU soft resource is available. Otherwise, it is determined that the soft resource is not indicated as available.

- Alt 2. The soft resource is determined as available, if both parent nodes indicate that the DU soft resource is available. Otherwise, it is determined that the soft resource is not indicated as available.

Proposal 11: Availability determination rule when a MT monitors DCI format 2_5 for a DU cell from multiple carrier frequencies needs to be defined. Discuss further following alternatives to determine for availability determination rule.

- Alt 1. The soft resource is determined as available, if at least one parent node indicates that the DU soft resource is available. Otherwise, it is determined that the soft resource is not indicated as available.

- Alt 2. The soft resource is determined as available, if both parent nodes indicate that the DU soft resource is available. Otherwise, it is determined that the soft resource is not indicated as available.

Conclusion

In this contribution, we discussed on specification of resource multiplexing enhancement between child and parent links of an IAB node. From the discussion, we obtained following proposals.

- Simultaneous operation schemes between MT and DU

Proposal 1: The multiplexing operation of an IAB node can be determined implicitly based on semi-static D/U/F resource type of MT and DU at least.

Proposal 2: Discuss how to align the understanding on the multiplexing operation of an IAB node in DU flexible resource with its parent node.

Proposal 3: In a DU cell perspective, one or multiple group(s) of consecutive PRBs can be defined where a PRB group corresponds to a Hard, Soft, or NA resource type for frequency domain multiplexing.

Proposal 4: Frequency domain H/S/NA attributes can be configured independently for DL and UL.

Proposal 5: Frequency domain H/S/NA configuration does not applied when the IAB-node is operated in TDM.

Proposal 6: The mapping table between values of resourceAvailability elements and types of frequency domain soft resource availability is specified independently from the table for time domain soft resource defined in Rel-16.

Proposal 7: It is supported that IAB node reports information on beam pair(s) capable of simultaneous operation between DU and MT (i.e., simultaneous Tx-Tx, simultaneous Rx-Rx, simultaneous Tx-Rx,  simultaneous Rx-Tx) to its parent DU

- Dual-connectivity to support multiple parent DUs

Proposal 8: The coordination of TDD configuration of MT between two parent DUs can be assumed for both of intra-donor and inter-donor DC scenarios.

Proposal 9: In case of DL/UL collision between two parent links, the priority rule should be defined in MT perspective.

Proposal 10: It is necessary to support a MT which cannot perform simultaneous Tx/Tx and Rx/Rx for two parent links.

Proposal 11: Availability determination rule when a MT monitors DCI format 2_5 for a DU cell from multiple carrier frequencies needs to be defined. Discuss further following alternatives to determine for availability determination rule.

- Alt 1. The soft resource is determined as available, if at least one parent node indicates that the DU soft resource is available. Otherwise, it is determined that the soft resource is not indicated as available.

- Alt 2. The soft resource is determined as available, if both parent nodes indicate that the DU soft resource is available. Otherwise, it is determined that the soft resource is not indicated as available.

Reference

- RAN1 chairman’s notes, RAN1 #104b-e, e-Meeting, April 12th – 20th, 2021

Agreement in RAN1#104b-e meeting [1]

Agreement The extension of the semi-static DU resource type indication to frequency-domain resources within a carrier (in addition to existing Rel-16 per-carrier granularity) for H/S/NA resource types is supported Agreement Adaptation of an IAB-node’s multiplexing operation is supported. The adaptation may be based on multiple factors, for example (not necessary to support all of the following): - Resource type (D/U/F) at the IAB-DU and IAB-MT - Specific sets of time/frequency resources - Certain conditions being met (e.g. supported timing modes, power control enhancements (if supported), etc.) FFS: Mechanisms for informing/coordination the change in multiplexing operation(s) between child and parent nodes (including whether the adaptation is dynamic or semi-static) FFS: Need for explicit linkage between indicated multiplexing operations and other features/enhancements – e.g. number of required guard symbols, supported timing modes, and power control enhancements (if supported) Agreement For the semi-static DU resource configuration in the frequency domain within a carrier, the frequency-domain granularity is configurable - FFS: minimum resource size e.g. N PRBs/N RBGs - FFS: Separate or joint TDM and FDM semi-static DU resource configurations Agreement Soft resource availability indications for frequency-domain resources are supported - FFS enhancements to DCI Format 2_5 - FFS: Separate or joint TDM and FDM indications Agreement To facilitate simultaneous operations and interference management, dynamic indication for restriction/usage/availability of beams (in upstream and/or downstream directions) is supported - FFS: Applicability to specific multiplexing cases or specific time-frequency resources - FFS: Whether IAB-specific enhancements beyond the existing beam management framework are needed to the support the functionality - FFS: Impact on the semi-static resource configurations (e.g., extending the H/S/NA resource attributes to the spatial domain) - FFS: Whether panel-based granularity is additionally supported Agreement The following enhancements to support intra-band inter-carrier dual connectivity for both inter-donor and intra-donor scenarios are considered (in addition to reusing solutions for inter-band dual connectivity) to support simultaneous Tx and/or Rx at the child IAB-MT to/from both parent links: - Extending the Rel-16 CA TDD conflict resolution framework for synchronous intra-band NR-DC operation Coordinating TDD configurations for the parent nodes (for both intra-donor and inter-donor operation) and coordinating H/S/NA configurations for the child node between donors (at least for inter-donor operation)