3GPP TSG RAN WG1 Meeting #105-e			     R1-2105689

Electronic Meeting, May 10th – May 27th, 2021

Source:	NTT DOCOMO, INC.

Title:	PUCCH format 0/1/4 enhancements for NR from 52.6 to 71 GHz

Agenda Item:	8.2.3

Document for: 	Discussion and Decision

- Introduction

At RAN#90-e meeting, an updated work item description [1] on supporting NR from 52.6 GHz to 71 GHz was approved, which include the following objective:

- Physical layer aspects including [RAN1]: - In addition to 120 kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz ... - Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation. …

In this contribution, we discuss on PUCCH resource allocation under PSD limitation to support NR from 52.6 GHz to 71 GHz.

- Discussion

  - Evaluation for PUCCH format 0/1/4 with multiple RBs

Agreement:

- For enhanced PF0/1, support Type-1 low PAPR sequences. Further study and strive to select one of the following alternatives:

  - Alt-1: A single sequence of length equal to the total number of mapped REs of the PUCCH resource is used. Cyclic shifts for PF0/1 are defined in the same way as Rel-16 for the case that useInterlacePUCCH-PUSCH is not configured.

  - Alt-2: A single sequence of length equal to the number of mapped REs per RB of the PUCCH resource is used, and the sequence is repeated in each RB. At least the following scheme is considered for PAPR/CM reduction:

    - Cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured

- At least the following aspects should be considered in the study

  - Coverage (maximum isotropic loss (MIL)), including

    - Required SNR to fulfil PUCCH detection criterion

    - PAPR/CM as a function of N_RB

    - Specification impact

Agreement:

- The maximum values for the configured number of RBs, NRB, for enhanced PF0/1/4 are at least:

  - 12 RBs for 120 kHz SCS

  - 3 RBs for 480 kHz SCS

  - 2 RBs for 960 kHz SCS

- FFS: Whether or not the above values need to be revised to support larger values (and any associated signaling impact), e.g., to support lower UE Tx beamforming gain and/or larger UE EIRP and conducted power limits for different UE power classes, different from those in the agreed evaluation assumptions

Agreement:

Down select to one of the following two alternatives for the configuration of the number of RBs, NRB, for enhanced PUCCH formats 0/1/4:

- Alt-1:

  - For enhanced PF0/1

    - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS

  - For enhanced PF4

    - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS that fulfill the requirement NRB=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers.

- Alt-2:

  - Same as Alt-1, but with coarser granularity, i.e., not all integer values of NRB can be configured

FFS: Which values of NRB are supported values in the range [1 .. max(NRB)]

Agreement:

For addressing the FFS from the prior agreement in RAN1#104bis-e on the maximum values for the configured number RBs, send an LS to RAN4 asking for feasible maximum values for UE_EIRP and UE_P for operation in 52.6-71 GHz.

Agreement:

User-multiplexing can be considered but as lower priority compared to maximum isotropic loss for PUCCH as a design criterion.

At RAN1#104-e and #104bis-e meeting, the followings were agreed.

According to the agreement, to specify the number of RB(s) for PUCCH format 0/1/4 and the sequence design for PUCCH format 0/1 with multiple RBs, we evaluate the performance of PUCCH format 0/1/4 based on the evaluation assumptions in Table 1.

For PUCCH format 0/1/4, PRB and sub-PRB interlaced mapping are not considered for all SCSs. The sequences are constructed based on the Type-1 low PAPR sequence specified in 38.211 section 5.2.2 [2], i.e., the computer generated sequence is used when the number of allocated RBs is at most two RB(s) and the ZC-sequence is used otherwise. We evaluate both sequence designs of Alt-1 and Alt-2 provided in the agreement above for PUCCH format 0/1.

For PUCCH format 1 and 4, OCC is configured according to 38.211 section 6.3.2 [2].

During the calculation of the required SNR for PUCCH sequence detection, the detection criteria we use is the same as what was suggested in the agreement as follows.

For PF0/1 (payload of 1 or 2 bits): the detection criterion assumes that the PUCCH payload consists of randomly drawn HARQ ACK/NACK bits and the criterion is defined as the SNR for which P(ACK to Error) ≤ 1% AND P(NACK to ACK) ≤ 0.1%. Error is defined as NACK or DTX where the decision region for DTX is determined to ensure that the maximum P(DTX to ACK) ≤ 1% for the case when the input to the receiver is noise only.

For PF4 (payload greater than 2 bits): the detection criterion is the UCI block error probability BLER ≤ 1% (as in TS38.104 Section 8.3.6)

Note that, the maximum allowed conducted power Pmax is calculated considering combined limit per region (US, Europe and South Korea regulatory are evaluated separately) provided in table 5 of Appendix A. Other simulation assumptions are provided in table 6 of appendix B.

Table 1: Evaluation assumptions

| Parameter | Value |
| --- | --- |
| PUCCH Format | PF0, PF1, PF4 |
| Subcarrier spacing, SCS [kHz] | {120, 480, 960} kHz |
| Frequency hopping details | Intra-frequency hopping |
| Number of RBs used per hop (N_RB) | 1 to 40 |
| PUCCH bandwidth per hop, BW [MHz] | BW = N_RB * 12 * SCS / 1e6 |
| Number of OFDM symbols used for PUCCH resource | 2 for PF0 4 for PF1/4 |
| Number of multiplexed users | 1 user |
| UE Tx Beamforming gain (dBi) | 6 dBi |
| BS Rx Beamforming gain (dBi) | 20 dBi |
| UE Power Limitations | Maximum EIRP: UE_EIRP = 25, 40 dBm Maximum conduced power (prior to consideration of backoff): UE_P = 21, 25 dBm |
| Backoff (dB) | Power backoff is equal to the cubic metric |
| Transmit power, P_TX (dBm) | P_TX = min(Pmax, UE_EIRP – TxBF, UE_P – Backoff) |
| Noise power, P_N (dBm) | BS Noise Figure, NF = 7 dB Noise PSD = -174 dBm/Hz P_N = Noise PSD + 10*log10(BW * 1e6) + NF |
| Maximum Isotropic Loss, MIL (dB) | MIL = P_TX – P_N – Required SNR + TxBF + RxBF |

    - Required SNR for PUCCH sequence detection

Table 2 and 3 show the required SNR for PUCCH format 0 and 1 sequence detection, respectively, with multiple RBs with the Alt-1 and Alt-2 sequence design provided in the agreement. According to the Table 2 and 3, the required SNR decreases as the number of RBs increases for each SCS/alternative since the PUCCH detection probability is improved by the gains of frequency diversity and longer length of sequences. In addition, it can be observed that the differences between Alt-1 and Alt-2 are less than 1.0 dB in many cases in terms of the required SNR.

Observation 1: For PUCCH format 0, there is no big difference between Alt-1 and Alt-2 of sequence design in terms of the required SNR for PUCCH sequence detection.

Table 2: Required SNR [dB] for PUCCH format 0 with multiple RBs (TDL-A DS 5 ns)

| # of RBs |  | 1 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120 kHz | Alt.1 | 3.0 | 0 | -2.7 | -4.4 | -6.0 | -6.6 | -7.0 | -8.0 | -8.5 |
|  | Alt.2 | 2.3 | 0 | -3 | -4.7 | -4.0 | -6.5 | -7.5 | -8.0 | -8.5 |
| 480 kHz | Alt.1 | 3.5 | 0.6 | -2.6 | -4.2 | -5.4 | -5.7 | -6.4 | -6.7 | -7.1 |
|  | Alt.2 | 3.6 | 0.9 | -2.2 | -3.8 | -5.2 | -5.7 | -6.3 | -6.7 | -7.1 |
| 960 kHz | Alt.1 | 4.0 | 0.7 | -1.7 | -3.3 | -4.4 | -4.5 | -5.0 | -5.0 | -4.8 |
|  | Alt.2 | 4.3 | 1.5 | -1.4 | -3.0 | -3.7 | -4.2 | -4.4 | -4.5 | -5.0 |

Table 3: Required SNR [dB] for PUCCH format 1 with multiple RBs (TDL-A DS 5 ns)

| # of RBs |  | 1 | 2 | 4 | 6 | 8 | 10 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120 kHz | Alt.1 | 2.2 | 0.3 | -2.3 | -0.5 | -2.0 | -4.0 | -4.1 |
|  | Alt.2 | 3.6 | 1.4 | 0 | -1.7 | -3.1 | -3.6 | -4.0 |
| 480 kHz | Alt.1 | 3.8 | 1.3 | -3 | -3.5 | -3.7 | -4.6 | -5.9 |
|  | Alt.2 | 3.0 | -0.2 | -2.5 | -3.5 | -4.5 | -6.0 | -6.7 |
| 960 kHz | Alt.1 | 3.2 | 0 | -3.5 | -4.0 | -5.8 | -6.4 | -7.2 |
|  | Alt.2 | 3.5 | -0.7 | -2.9 | -4.9 | -5.2 | -6.0- | -6.9 |

    - Cubic Metric Evaluations on PUCCH format 0/1 base sequence

We evaluate the Cubic Metric of type-1 low PAPR sequence of Alt-1 and Alt-2 provided in the agreement for PUCCH format 0/1 base sequence.

Figure 1 shows the Cubic Metric for PUCCH format 0/1 with multiple PRBs and the sequence constructed based on the Alt-1 and Alt-2. According to figure 1, it can be observed that Alt-1 shows better CM characteristics than Alt-2 over a wide range of number of RBs while the CM characteristics of Alt-2 deteriorate greatly as the number of RBs increases.

Observation 2: For PUCCH format 0/1, Alt-1 sequence design can provide better results on cubic metric evaluations for most cases with different number of RBs compared with Alt-2 sequence design.

Figure 1: Cubic Metric [dB] for PUCCH format 0/1 base sequences with multiple RBs

    - MIL Evaluations on PUCCH format 0/1/4

Here we show MIL performances of PUCCH format 0/1/4 with multi-PRBs based on the evaluation metric shown above.

Figure 2 to 4 show the MIL performances for PUCCH format 0/1/4 with multiple RBs considering the US, Europe and South Korea transmit power regulation separately. For calculation of MIL, we evaluate two UE power limit cases {UE_EIRP, UE_P} = {25, 21 dBm}, {40, 25 dBm} assuming two different UE power classes.

For all PUCCH formats, under the US and South Korea regulation of maximum allowed conducted power, two or more RBs are required to achieve the maximum MIL performance for each SCS/UE power class while MIL is almost constant regardless of number of RBs from 1 to 30 considering the regulation in Europe.

When UE_EIRP and UE_P are 25 dBm and 21 dBm respectively, 12/3/2 RBs for 120/480/960 kHz SCS each are large enough to achieve the max transmit power as agreed at RAN1#104bis-e meeting, however, larger number of RBs such as 30/8/4 RBs for 120/480/960 kHz SCS each are required when UE_EIRP and UE_P are 40 dBm and 25 dBm respectively.

It has been agreed that UE power limit values would be updated according to the response from RAN4 at RAN1#104bis-e meeting, and the required number of RBs to transmit at the maximum allowed conducted power may exceed 16 depending on the UE power limit values based on the evaluation results shown in figure 2 to 4. Given that the required number of RBs can be more than 16 for PUCCH format 0/1/4, PUCCH format 2/3 may also need to be enhanced in terms of available number of RBs e.g., specify the minimum value for number of RBs or allocate more than 16 RBs.

Proposal 1: PUCCH format 2/3 may need to be enhanced according to the updated evaluation assumption for UE power limits depending on LS response from RAN4.

For the sequence design of PUCCH format 0/1, it can be observed that Alt-1 is slightly better than Alt-2 in terms of the MIL performances, while the performance gaps seem not large. From specification impact perspective, there is no big difference between Alt-1 and Alt-2 since both of them can be realized by reusing Rel-15 and 16 sequence design respectively. Furthermore, the user multiplexing aspect has been discussed that Alt-2 achieves larger user multiplexing capacity than Alt-1, however, it has been agreed that it can be considered but as lower priority compared to maximum isotropic loss for PUCCH as a design criterion. Therefore, considering the evaluation results and specification impact perspective, Alt-1 which shows better CM performances should be supported as the PUCCH format 0/1 base sequence design with multiple RBs.

Proposal 2: According to the evaluation results of Cubic Metric, required SNR on sequence detection and the MIL performance with sequence designs of Alt-1 and Alt-2, Alt-1 should be supported as the PUCCH format 0/1 base sequence design with multiple RBs.

- 120 kHz SCS

- 480 kHz SCS

- 960 kHz SCS

Figure 2: MIL [dB] performance evaluation results for PUCCH format 0 with multiple RBs

- 120 kHz SCS

- 480 kHz SCS

- 960 kHz SCS

Figure 3: MIL [dB] performance evaluation results for PUCCH format 1 with multiple RBs

- 120 kHz SCS

- 480 kHz SCS

- 960 kHz SCS

Figure 4: MIL [dB] performance evaluation results for PUCCH format 4 with multiple RBs

  - Indication of number of PRB for PUCCH format 0/1/4

In Rel-15 and 16, the frequency domain resources for dedicated PUCCH resources are indicated via RRC signaling, i.e., via startingPRB field of PUCCH-Resource and nrofPRBs field of PUCCH-format2/3 in PUCCH-Config IE. In the same way as the current specification, the number of RBs for PUCCH format 0/1/4 with multi-PRB allocation should be indicated via UE dedicated RRC signaling.

The granularity of available RBs up to the maximum value has been discussed at the previous meeting. Based on the evaluation results shown in section 2.1.3, the required number of RBs to transmit at maximum conducted power can be different depending on the region and the UE power class, and hence the number of RBs should be configurable flexibly. To address this, all integer values for PUCCH format 0/1 and all integer values that fulfill the requirement NRB=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers for PUCCH format 4 should be supported same as PUCCH format 2/3.

Proposal 3: For the dedicated PUCCH resources, the number of RBs for PUCCH format 0/1/4 should be indicated via UE dedicated RRC signaling.

Proposal 4: All integer values for PUCCH format 0/1 and all integer values that fulfill the requirement NRB=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers for PUCCH format 4 (Alt-1 in RAN1#104bis-e agreement) should be supported.

  - PUCCH resource sets before dedicated PUCCH resource configuration

For the PUCCH resource sets before dedicated PUCCH resource configuration, the PUCCH resource is provided by the table 4 and one row index of the table is indicated by 4 bits from remaining minimum system information (RMSI). This table specifies a PUCCH format, common PRB offset, a first symbol, a duration and cyclic shift index. In a same way, a PUCCH resource set table for the 60 GHz unlicensed band operation which includes the number of RBs for each PUCCH resource can be specified and one resource can be indicated by RMSI if multi-PRB allocation is applied for the PUCCH before dedicated PUCCH resource configuration.

For another option to indicate the number of RBs before dedicated PUCCH resource configuration, it can be indicated via RRC signaling, e.g., the number of RB is signaled by SIB1 aside from PUCCH resource related information specified in table 4, considering that the required number of RB can be different depending on regions/UE power classes.

Proposal 5: For the PUCCH resource sets before dedicated PUCCH resource configuration, the number of RBs for PUCCH format 0/1 can be indicated by using a PUCCH resource sets table for the 60 GHz unlicensed band operation or SIB1.

PUCCH format 0/1 are used for PUCCH resource sets before dedicated PUCCH resource configuration with single-PRB allocation. As shown above, it has been agreed that more than one RBs can be allocated for PUCCH format 0/1/4 to achieve the maximum transmit power, and the multi-PRB allocation should be supported even for PUCCH resource sets before dedicated PUCCH resource configuration considering the coverage during the initial access.

As mentioned above, the PUCCH resource sets before dedicated PUCCH resource configuration are provide by table 4 and it assumes single-PRB allocation of PUCCH format 0/1. Accordingly, at least frequency domain resource allocation related field, i.e. PRB offset values should be extended depending on the number of RBs for the PUCCH resource sets if the multi-PRB allocation is supported for the PUCCH resource sets.

Proposal 6: More than one PRB should be supported even for PUCCH resource sets before dedicated PUCCH resource configuration considering the coverage during the initial access.

Proposal 7: At least PRB offset values in PUCCH resource sets before dedicated PUCCH resource table should be enhanced based on the number of RBs for the PUCCH resource sets.

In RAN1#104bis-e meeting, some companies pointed out that the bandwidth of initial UL BWP may not be large enough to FDM with multi-PRB allocation for PUCCH format 0/1 before dedicated PUCCH resource configuration if the large value(s) of the number of RBs for the PUCCH resource is supported such as 40 RBs for 120 kHz SCS. This shortage of FDM resources issue with multi-PRB allocation can be occurred depending on the number of RBs for PUCCH resource sets before dedicated PUCCH resource configuration and the bandwidth of initial UL BWP for 52.6 to 71 GHz band. Hence, this potential issue should be discussed when the number of RBs for the PUCCH resource sets for 52.6 to 71 GHz band is agreed.

Proposal 8: The potential shortage of PUCCH resource sets before dedicated should be discussed depending on the specified number of RBs for the PUCCH resource sets.

Table 4: PUCCH resource sets before dedicated PUCCH resource configuration (table 9.2.1-1 from TS 38.213)

| Index | PUCCH format | First symbol | Number of symbols | PRB offset | Set of initial CS indexes |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 12 | 2 | 0 | {0, 3} |
| 1 | 0 | 12 | 2 | 0 | {0, 4, 8} |
| 2 | 0 | 12 | 2 | 3 | {0, 4, 8} |
| 3 | 1 | 10 | 4 | 0 | {0, 6} |
| 4 | 1 | 10 | 4 | 0 | {0, 3, 6, 9} |
| 5 | 1 | 10 | 4 | 2 | {0, 3, 6, 9} |
| 6 | 1 | 10 | 4 | 4 | {0, 3, 6, 9} |
| 7 | 1 | 4 | 10 | 0 | {0, 6} |
| 8 | 1 | 4 | 10 | 0 | {0, 3, 6, 9} |
| 9 | 1 | 4 | 10 | 2 | {0, 3, 6, 9} |
| 10 | 1 | 4 | 10 | 4 | {0, 3, 6, 9} |
| 11 | 1 | 0 | 14 | 0 | {0, 6} |
| 12 | 1 | 0 | 14 | 0 | {0, 3, 6, 9} |
| 13 | 1 | 0 | 14 | 2 | {0, 3, 6, 9} |
| 14 | 1 | 0 | 14 | 4 | {0, 3, 6, 9} |
| 15 | 1 | 0 | 14 |  | {0, 3, 6, 9} |

- Conclusion

In this contribution, we evaluated and discussed the base sequence design and the number of RB(s) for enhanced PUCCH format 0/1/4 to support NR from 52.6 GHz to 71 GHz. Based on the evaluation results and discussion above, we derived following observations and proposals.

Observation 1: For PUCCH format 0, there is no big difference between Alt-1 and Alt-2 of sequence design in terms of the required SNR for PUCCH sequence detection.

Observation 2: For PUCCH format 0/1, Alt-1 sequence design can provide better results on cubic metric evaluations for most cases with different number of RBs compared with Alt-2 sequence design.

Proposal 1: PUCCH format 2/3 may need to be enhanced according to the updated evaluation assumption for UE power limits depending on LS response from RAN4.

Proposal 2: According to the evaluation results of Cubic Metric, required SNR on sequence detection and the MIL performance with sequence designs of Alt-1 and Alt-2, Alt-1 should be supported as the PUCCH format 0/1 base sequence design with multiple RBs.

Proposal 3: For the dedicated PUCCH resources, the number of RBs for PUCCH format 0/1/4 should be indicated via UE dedicated RRC signaling.

Proposal 4: All integer values for PUCCH format 0/1 and all integer values that fulfill the requirement NRB=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers for PUCCH format 4 (Alt-1 in RAN1#104bis-e agreement) should be supported.

Proposal 5: For the PUCCH resource sets before dedicated PUCCH resource configuration, the number of RBs for PUCCH format 0/1 can be indicated by using a PUCCH resource sets table for the 60 GHz unlicensed band operation or SIB1.

Proposal 6: More than one PRB should be supported even for PUCCH resource sets before dedicated PUCCH resource configuration considering the coverage during the initial access.

Proposal 7: At least PRB offset values in PUCCH resource sets before dedicated PUCCH resource table should be enhanced based on the number of RBs for the PUCCH resource sets.

References

- RP-2002925, “Revised WID:  Extending current NR operation to 71 GHz”.

- 3GPP TS 38.211, “NR; Physical channels and modulation”, V16.0.0.

- 3GPP TS 38.213, “NR; Physical layer procedures for control”, V16.4.0.

Appendix  A: Regulatory power limits by region

Table 5: Regulatory power limits of US and Europe

| Region | Maximum Conducted Power, Pmax (dBm) |
| --- | --- |
| US | Conducted power limit due to EIRP limit: Pmax_EIRP = 40 dBm - TxBF Conducted power limit as a function of PUCCH BW per hop: Pmax_P = 27 dBm – max(0, 10*log10(100 / BW)) Combined limit: Pmax = min(Pmax_P, Pmax_EIRP) |
| Europe | Conducted power limit due to EIRP limit: Pmax_EIRP = 40 dBm – TxBF Conducted power limit due to PSD limit (assumes N_RB contiguous RBs with all REs allocated per PRB): Pmax_PSD = 23 dBm/MHz + max(0, 10*log10(BW)) - TxBF Combined limit: Pmax = min(Pmax_PSD, Pmax_EIRP) |
| South Korea | Conducted power limit due to EIRP limit: Pmax_EIRP = 43 dBm – TxBF Conducted power limit due to PSD limit (assumes N_RB contiguous RBs with all REs allocated per PRB): Pmax_PSD = 13 dBm/MHz + max(0, 10*log10(BW)) - TxBF Combined limit: Pmax = min(Pmax_PSD, Pmax_EIRP) |
| Note: BW is the PUCCH bandwidth per hop in MHz |  |

Appendix  B: Evaluation assumptions

Table 6: Evaluation assumptions

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 60 GHz |
| Subcarrier Spacing [kHz] | 120, 480, 960 kHz |
| Number of usable RBs per carrier | 256 for 120 kHz SCS (corresponds to ~400 MHz carrier) 256 for 480 kHz SCS (corresponds to ~1600 MHz carrier) 160 for 960 kHz SCS (corresponds to ~2000 MHz carrier) |
| PUCCH Frequency Hopping | On |
| PUCCH Frequency Domain Resource Mapping | N_RB contiguous RBs per hop (with all REs allocated per PRB) |
| Waveform | CP-OFDM for PF0/1 DFT-s-OFDM for PF4 |
| CP Type | Normal CP |
| Channel Model | TDL-A model as defined in of TR38.901 Section 7.7.2: - Delay spread (DS) = {5ns, 20ns} |
| BS Antenna Configuration (Mg,Ng,M,N,P) | {1,1,1,1,2} |
| UE Antenna Configuration (Mg,Ng,M,N,P) | {1,1,1,1,1} |
| Mobility | 3 km/hr |
| PA Model | None |
| gNB TRP PN Model | Zero phase noise |
| UE PN Model | Zero phase noise |
| Pre-loaded Tx EVM | 0% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |