3GPP TSG RAN WG1 Meeting #105-e	R1-2105923

e-Meeting, May 10th – 27th, 2021

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on RAN2 LS on DRX impact

Document for:	Discussion and Decision

- Introduction

RAN2 sent an LS to RAN1 [1], asking for feedback if there is any concern on the working assumption as below:

SL DRX should take PSCCH monitoring also for sensing (in addition to data reception) into account if SL DRX is used.

In the previous meeting discussions, most companies suggest that more discussion about SL DRX is necessary before replying LS to RAN2. In this contribution, we provide some further investigations based on the agreements from RAN2, and indicate how to reply.

- Discussions

Based on RAN2 agreement in RAN2#113-e:

For SL groupcast/broadcast, SL DRX configuration can be configured in common. FFS on granularity of SL DRX configuration.

For data reception, RAN2 defines the behaviour for monitoring the SCI reception (i.e., PSCCH and 2nd SCI on PSSCH) during the SL active time for SL DRX. For data reception, the UE may skip monitoring of PSCCH and 2nd SCI on PSSCH during inactive time for SL DRX. Sensing aspect is not considered in this agreement.

RAN2 kindly agree that for groupcast and broadcast communication further granularity to multiple sets of DRX configurations (beyond just cast type) is required i.e. more than two DRX Cycle configurations should be supported in specification.

RAN2 will study/discuss how PQI and/or L2 destination ID is used to derive groupcast and broadcast DRX configuration.

SL DRX Command MAC CE is introduced for SL DRX operation in unicast.

Based on RAN2 agreement in RAN2#113b:

For unicast, the TX UE behaviors should be specified to keep aligned with the RX UE regarding the DRX Active time. FFS the specific Spec impacts needed at the TX side.

For unicast, the RX UE maintains a separate SL inactivity timer for each pair of src/dest L2 ID.

In order to reduce power consumption, DRX is configured to constrain the behavior of data reception. When DRX is configured to a RX UE, the relationship between DRX and resource allocation should be discussed.

When a TX UE would like to communicate with a RX UE with SL DRX configuration, the RX UE is expected to receive data only during SL DRX active time. As per the agreement of RAN2#113b, at least for unicast, the TX UE behaviors should be specified to keep aligned with the RX UE regarding the DRX Active time. Then there would be a constraint for the selection window of a TX UE to make sure a RX UE can receive the data during SL DRX active time.

Observation 1: When DRX is configured, the corresponding TX UE(s) and RX UE(s) have a common understanding of the SL DRX active time and the SL DRX inactive time.

From the view of a UE with SL DRX configuration, for data reception, RAN2 defines that the UE only monitors SCIs for data during the SL DRX active time, and may skip monitoring them during SL DRX inactive time. However a working assumption is agreed in RAN2 that SL DRX should take PSCCH monitoring also for sensing (in addition to data reception) into account if SL DRX is used. There seem to be divergent understandings for meaning of the working assumption cannot be aligned in RAN1, as follows:

Alt 1: When SL DRX is configured/determined, it should take into account of UE’s sensing operation.

Alt 2: When UE is performing the sensing operation, it should take into account of the configured SL DRX.

Alt 3: It should be jointly considered between the configuration of SL DRX and UE’s sensing operation.

Alt 4:  It’s up to UE implementation.

Alt 5: The SL DRX refer to the SL DRX of TX UE (i.e., the sensing UE) or RX UE

Alt 1: This alternative is only usable in the limited set of case. For SL groupcast/broadcast, SL DRX configuration can be configured in common as the agreement of RAN2#113-e. Since the traffic profiles, e.g. period and latency, of all UE’s packets in a resource pool cannot be aligned, it is infeasible to take into account a UE’s sensing operation during SL DRX configuration for SL groupcast/broadcast. For unicast, since the RX UE maintains a separate SL inactivity timer for each pair of src/dest L2 IDs based on the agreement of RAN2 #113b, SL DRX configuration can take sensing operation into account only when there is only one pair of src/dest L2 ID between a TX UE and a RX UE, and the traffic profiles, e.g. period and trigger timing of transmission data from the two UEs is fixed. Otherwise, SL DRX would be frequently reconfigured because of the non-fixed trigger timing of resource selection for a UE. Furthermore, the additional latency is introduced and the signaling overhead for frequently exchanging information would be rapidly increased.

Alt 2: SL DRX is configured for power saving, thus it is reasonable to take the SL DRX configuration into account when sensing is performed by a UE with SL DRX configuration. Sensing enhancement should be discussed considering the impact of SL DRX configuration. The description about how to take into account of the configured SL DRX when a UE with SL DRX configuration perform sensing is unclear. There are two understanding as follows:

- Option 1: Sensing operation for a UE is limited within SL active time when the UE is configured with SL DRX.

- Option 2: Sensing operation for a UE can be performed within SL inactive time conditionally when the UE is configured with SL DRX.

When there is an overlap between sensing windows/occasions and SL DRX inactive time, the available sensing results would be reduced based on the assumption of option 1. If there is no additional PSCCH monitoring during SL DRX inactive time, i.e., the reservation information is lost due to no sensing in SL DRX inactive time as shown in Figure 1 (a). Then, there would be more SL transmission collision in a resource pool, and the reliability of SL transmission is decreased if DRX is configured for a given UE. Thus option 2 is a reasonable solution to keep the reliability requirement when SL DRX is configured for a UE.

Alt3: The alternative is the same as the Alt1, and there is no clear description about how to jointly consider the SL DRX configuration and UE’s sensing requirement.

Alt4: This should not be left fully up to UE implementation where it could allow the UE to abandon any resource allocation configurations that may exist (e.g. partial sensing, full sensing, random selection, or any combination of), and do whatever it likes during SL DRX inactive time. This will lead to an unpredictable baseline system-level performance.

Alt5: The constraint for the TX UE and RX UE in a SL physical link with SL DRX configuration is different. When a TX UE would like to communicate with a RX UE with SL DRX configuration, the selection window of  the TX UE should be limited in SL DRX active time to make sure that the associated RX UE has an opportunity to perform SL receiving. When a UE with SL DRX configuration would perform sensing for SL transmission, the sensing procedure can be performed within SL inactive time in order to guarantee enough sensing results for reliable SL transmission.

(a)

(b)

Figure 1: Overlap between sensing window and DRX OFF

Observation 2: Sensing and resource selection would be impacted for a UE with SL DRX configuration when there is an overlap between sensing windows/occasions and SL DRX inactive time.

Observation 3: Alt2 is a reasonable solution subject to power saving.

Proposal 1: When SL DRX is configured for a UE, sensing of the UE is not limited to be within SL DRX active time.

In NR SL, HARQ-ACK feedback is supported to decrease the number of re-transmissions of one TB. As a result, HARQ-ACK reception is beneficial for power saving as well as resource usage efficiency. The time gap between a PSSCH and its PSFCH is fixed, thus the associated PSFCH reception slot associated with a PSSCH is deterministically known to a transmitting UE, and the additional power consumption for PSFCH reception is low (50 power units), especially when compared to always performing blind retransmission of PSSCH (300 power units). Thus, the associated PSFCH slot of a PSSCH should be received even in SL DRX OFF durations.

Observation 3: PSFCH reception in SL DRX inactive time is beneficial for power saving, as it reduces the need for re-transmissions.

Proposal 2:  Reply to RAN2 as follows:

- When sensing UE would like to communicate to a UE with SL DRX configuration,  it should ensure that the data is transmitted during SL DRX active time of the UE according to the DRX configuration.

- When sensing UE itself has a SL DRX configuration, it should take SL DRX configuration into account for sensing operation.

    - Support PSCCH reception within the sensing during SL DRX inactive time.

- In addition, SL reception of PSCCH for sensing, and PSFCH, during the SL DRX inactive time should be considered.

- Conclusions

Based on the discussion above, it is shown that sensing would be impacted if there is no PSCCH monitoring during DRX OFF state. PSFCH reception is beneficial for power saving and higher resource usage efficiency. Thus, the associated PSFCH slot of a PSSCH should be received even in SL DRX OFF durations. Following proposals were made:

Observation 1: When DRX is configured, the corresponding TX UE(s) and RX UE(s) have a common understanding of the SL DRX active time and the SL DRX inactive time.

Observation 2: Sensing and resource selection would be impacted for a UE with SL DRX configuration when there is an overlap between sensing windows/occasions and SL DRX inactive time.

Observation 3: Alt2 is a reasonable solution subject to power saving.

Proposal 1: When SL DRX is configured for a UE, sensing of the UE is not limited to be within SL DRX active time.

Observation 3: PSFCH reception in SL DRX inactive time is beneficial for power saving, as it reduces the need for re-transmissions.

Proposal 2:  Reply to RAN2 as follows:

- When sensing UE would like to communicate to a UE with SL DRX configuration,  it should ensure that the data is transmitted during SL DRX active time of the UE according to the DRX configuration.

- When sensing UE itself has a SL DRX configuration, it should take SL DRX configuration into account for sensing operation.

  - Support PSCCH reception within the sensing during SL DRX inactive time.

- In addition, SL reception of PSCCH for sensing, and PSFCH, during the SL DRX inactive time should be considered.

# References

- R2-2010961, “LS to RAN1 on SL DRX design”, RAN2, RAN2 #112-e, November, 2020.