3GPP TSG RAN WG1 #105-e	R1-2105928

e-Meeting, May 10th – 27th, 2021

Agenda Item:	7.2.5

Source:	Huawei, HiSilicon

Title:	Remaining issues on PDCCH enhancements

Document for:	Discussion and Decision

Introduction

This contribution discusses the following issues:

- Correction on UE PDSCH processing time for DCI format 1_2

- Clarification on new SLIV reference for Type-1 HARQ-ACK codebook

Discussions

  - Correction on UE PDSCH processing time for DCI format 1_2

In the RAN1#104b-e meeting, an issue about the UE PDSCH processing time for DCI format 1_2 was discussed, and it is still open. During that discussion it was brought up whether a UE can have different processing times for different DCI formats. We firstly state our understanding about Rel-15 and then propose how to handle DCI format 1_2 for Rel-16.

Legacy behavior in Rel-15

In Rel-15, a PDSCH can be scheduled with DCI format 1_0 and shall always assume dmrs-AdditionalPosition=’pos2’. This means that the processing time for a PDSCH scheduled with DCI format 1_0 is always given by the 3rd column of table 5.3-1 in 38.214 [1] which corresponds to the relaxed UE processing time capability 1. At the same time, the UE can be configured with processing time capability 2 and dmrs-AdditionalPosition=’pos0’ for DCI format 1_1. This means that when the PDSCH is scheduled with DCI format 1_1, the processing time is given by table 5.3-2 in 38.214 [1].

Observation 1: In Rel-15, for a given cell, the PDSCH can be scheduled with two different processing times, this includes a switch between processing time capability 1 and capability 2.

Based on the above observation, it is our view that different PDSCH processing times depending on the DCI format (including a switch between capability 1 and capability 2) are supported in Rel-15 and that the specification is clear.

Processing times for DCI format 1_2 in Rel-16

In Rel-16 URLLC, the new DCI format 1_2 has been introduced. It is configured by a new set of parameters. The motivation for introducing new RRC parameters is to be able to handle DCI format 1_1 and DCI format 1_2 separately.

Because Rel-15 already supports that different processing time capabilities follow different DCI formats, we suggest that this behavior also shall apply for DCI format 1_1 and DCI format 1_2.

If the UE is configured with processing time capability 1 (i.e. processingType2Enabled is disabled) we suggest that, depending on the DMRS configuration for the respective DCI format, the PDSCH scheduled by DCI format 1_1 or 1_2 shall follow the following processing times of table 5.3-1 in 38.214 [1] as summarized in Table 1 below:

Table 1 – Suggested different PDSCH processing capabilities dependent on RRC parameters if processingType2Enabled is disabled

|  | RRC parameter configuration |  |  |  |
| --- | --- | --- | --- | --- |
|  | dmrs-AdditionalPosition = 'pos0' in dmrs-DownlinkForPDSCH-MappingTypeA, and dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1 and dmrs-AdditionalPosition = 'pos0' in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2 | dmrs-AdditionalPosition = 'pos0' in dmrs-DownlinkForPDSCH-MappingTypeA, and dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1 and dmrs-AdditionalPosition ≠ 'pos0' in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 or dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2 | dmrs-AdditionalPosition ≠ 'pos0' in dmrs-DownlinkForPDSCH-MappingTypeA, or dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1 and dmrs-AdditionalPosition = 'pos0' in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2 | dmrs-AdditionalPosition ≠ 'pos0' in dmrs-DownlinkForPDSCH-MappingTypeA, or dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1 and dmrs-AdditionalPosition ≠ 'pos0' in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 or dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2 |
|  | Suggested applicable PDSCH processing times from 38.214 |  |  |  |
| DCI format 1_1 | The 2nd column in table 5.3-1 | The 2nd column in table 5.3-1 | The 3rd column in table 5.3-1 | The 3rd column in table 5.3-1 |
| DCI format 1_2 | The 2nd column in table 5.3-1 | The 3rd column in table 5.3-1 | The 2nd column in table 5.3-1 | The 3rd column in table 5.3-1 |

If the UE is configured with processing time capability 2 (i.e. processingType2Enabled is enabled) we suggest that, depending on the DMRS configuration for the respective DCI format, the PDSCH scheduled by DCI format 1_1 or 1_2 shall follow the following processing times of table 5.3-1 and 5.3-2 in 38.214 [1] as summarized in Table 2 below:

Table 2 – Suggested different PDSCH processing capabilities dependent on RRC parameters if processingType2Enabled is enabled

|  | RRC parameter configuration |  |  |  |
| --- | --- | --- | --- | --- |
|  | dmrs-AdditionalPosition = 'pos0' in dmrs-DownlinkForPDSCH-MappingTypeA, and dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1 and dmrs-AdditionalPosition = 'pos0' in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2 | dmrs-AdditionalPosition = 'pos0' in dmrs-DownlinkForPDSCH-MappingTypeA, and dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1 and dmrs-AdditionalPosition ≠ 'pos0' in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 or dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2 | dmrs-AdditionalPosition ≠ 'pos0' in dmrs-DownlinkForPDSCH-MappingTypeA, or dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1 and dmrs-AdditionalPosition = 'pos0' in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2 | dmrs-AdditionalPosition ≠ 'pos0' in dmrs-DownlinkForPDSCH-MappingTypeA, or dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1 and dmrs-AdditionalPosition ≠ 'pos0' in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 or dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2 |
|  | Suggested applicable PDSCH processing times from 38.214 |  |  |  |
| DCI format 1_1 | Table 5.3-2 (Processing capability 2) | Table 5.3-2 (Processing capability 2) | Table 5.3-1 (Processing capability 1) | Table 5.3-1 (Processing capability 1) |
| DCI format 1_2 | Table 5.3-2 (Processing capability 2) | Table 5.3-1 (Processing capability 1) | Table 5.3-2 (Processing capability 2) | Table 5.3-1 (Processing capability 1) |

Based on the above discussion and as seen in Table 1 and Table 2, we support independent processing times of DCI format 1_1 and DCI format 1_2. A simple way to capture this in the specification is to introduce two new tables for DCI format 1_2. Therefore, we propose to adopt the following TP:

Proposal 1: Support independently RRC configured PDSCH processing times for DCI format 1_1 and DCI format 1_2.  Endorse the following text proposal for TS 38.214 Section 5.3.

------------------------------------ Start of TP 38.214 V16.5.0 section 5.3--------------------------------- <unchanged text omitted> - N1 is based on µ of table 5.3-1, and table 5.3-2, table 5.3-3 and table 5.3-4 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µPDCCH, µPDSCH, µUL) resulting with the largest Tproc,1, where the µPDCCH corresponds to the subcarrier spacing of the PDCCH scheduling the PDSCH, the µPDSCH corresponds to the subcarrier spacing of the scheduled PDSCH, and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is to be transmitted, and κ is defined in clause 4.1 of [4, TS 38.211]. - For operation with shared spectrum channel access, is calculated according to [4, TS 38.211], otherwise =0. - If the PDSCH DM-RS position l1 for the additional DM-RS in Table 7.4.1.1.2-3 in clause 7.4.1.1.2 of [4, TS 38.211] is l1=12 then N1,0=14 in Table 5.3-1 and Table 5.3-3, otherwise N1,0=13. <unchanged text omitted> Table 5.3-1: PDSCH processing time for PDSCH processing capability 1 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB or if the higher layer parameter is not configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. Table 5.3-2: PDSCH processing time for PDSCH processing capability 2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB. 0; 3. 1; 4.5. 2; 9 for frequency range 1. Table 5.3-3: PDSCH processing time for PDSCH processing capability 1 for DCI format 1_2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 or if the higher layer parameter is not configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. Table 5.3-4: PDSCH processing time for PDSCH processing capability 2 for DCI format 1_2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2. 0; 3. 1; 4.5. 2; 9 for frequency range 1. <unchanged text omitted> ------------------------------------ End of TP 38.214 V16.5.0 section 9.1.2.1---------------------------------

  - Clarification on new SLIV reference for Type-1 HARQ-ACK codebook

In RAN1#104-e, an issue was raised whether the new SLIV reference (i.e. the starting symbol of the PDCCH monitoring occasion) can be applied to the Type-1 HARQ-ACK codebook. This topic was then continued in RAN1#104b-e but without reaching consensus.

Based on the current specification text in 38.213, if the UE is configured with referenceOfSLIVDCI-1-2, then for each row index with K0=0 and each PDCCH monitoring occasion with starting symbol S0>0, a new row index is added into the TDRA table.

From 38.213v16,5,0[2], Section 9.1.2.1

b) on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c a) if the UE is provided referenceOfSLIVDCI-1-2, for each row index with slot offset K0=0 and PDSCH mapping Type B in a set of row indexes of a table for DCI format 1_2 [6, TS 38.214], for each PDCCH monitoring occasion in a set of PDCCH monitoring occasions with different starting symbols within a slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0, if S+S0+L≤14 for normal cyclic prefix and S+S0+L≤12 for extended cyclic prefix, add a new row index in the set of row indexes of the table by replacing the starting symbol S of the row index by S+S0

Companies have two different interpretations of the above description in the specification:

- Interpretation 1: The extended SLIV applies in every slot (i.e. even for the slot(s) with no PDCCH monitoring occasion with starting symbol S0>0). Interpretation 1 ensures the same number of the set of row indexes of the TDRA tables for type-1 codebook construction for every slot. With this interpretation, there is no problem related to the Type 1 codebook size.

- Interpretation 2: The extended SLIV is only applied to the slot(s) with PDCCH monitoring occasion with starting symbol S0>0. The candidate PDSCHs in a slot can be impacted by both PDCCH MOs and the number of PDSCH repetitions as illustrated in [3]. With this understanding, there is a problem with the Type-1 codebook size in case that PDSCH repetition is used and a TP is needed to resolve it.

Based on the discussions during the RAN1#104b-e meeting, most companies agreed with Interpretation 1. However, it seems companies feel that the current specification is not clear enough to reflect interpretation 1 above. For simplicity from the specification perspective, it was suggested to add restriction(s) on the PDCCH monitoring occasions to ensure that each slot has the same set of row indices, i.e. the new reference is only applied if the starting symbols of PDCCH monitoring occasions are configured the same in all the slots with PDCCH monitoring occasion with starting symbol S0>0.  Although most companies expressed that they can accept it, several companies also shared that with this restriction the feature is not that useful.

To avoid limiting the applicable scenario for the feature of new reference, one way is trying to change the specification to make it reflecting interpretation 1. For example, the following changes in Section 9.1.2.1 in 38.214 can be considered. With such changes, the new row index due to the new SLIV reference will be added into the set of row indexes of the TDRA table with any PDCCH monitoring occasion in any slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0. Then in the pseudo code for determining the set of occasions for candidate PDSCH receptions for certain PUCCH, the extended TDRA table with new row indexes is applied for all the DL slots corresponding to the PUCCH according to the current specification, including the slot(s) with no PDCCH monitoring occasion with starting symbol S0>0.

b) on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c a) if the UE is provided referenceOfSLIVDCI-1-2, for each row index with slot offset K0=0 and PDSCH mapping Type B in a set of row indexes of a table for DCI format 1_2 [6, TS 38.214], for eachany PDCCH monitoring occasion in any slotin a set of PDCCH monitoring occasions with different starting symbols within a slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0, if S+S0+L≤14 for normal cyclic prefix and S+S0+L≤12 for extended cyclic prefix, add a new row index in the set of row indexes of the table by replacing the starting symbol S of the row index by S+S0

Proposal 2: The following TP for 38.213 V16.5.0 section 9.1.2.1 is captured in specification to support new SLIV reference for Type-1 HARQ-ACK codebook.

------------------------------------ Start of TP 38.213 V16.5.0 section 9.1.2.1--------------------------------- <unchanged text omitted> b) on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c a) if the UE is provided referenceOfSLIVDCI-1-2, for each row index with slot offset K0=0 and PDSCH mapping Type B in a set of row indexes of a table for DCI format 1_2 [6, TS 38.214], for eachany PDCCH monitoring occasion in any slotin a set of PDCCH monitoring occasions with different starting symbols within a slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0, if S+S0+L≤14 for normal cyclic prefix and S+S0+L≤12 for extended cyclic prefix, add a new row index in the set of row indexes of the table by replacing the starting symbol S of the row index by S+S0 <unchanged text omitted> ------------------------------------ End of TP 38.213 V16.5.0 section 9.1.2.1---------------------------------

Conclusions

Based on the above discussions, the following proposal is given:

For Correction on UE PDSCH processing time for DCI format 1_2:

Observation 1: In Rel-15, for a given cell, the PDSCH can be scheduled with two different processing times, this includes a switch between processing time capability 1 and capability 2.

Proposal 1: Support independently RRC configured PDSCH processing times for DCI format 1_1 and DCI format 1_2.  Endorse the following text proposal for TS 38.214 Section 5.3.

------------------------------------ Start of TP 38.214 V16.5.0 section 5.3--------------------------------- <unchanged text omitted> - N1 is based on µ of table 5.3-1, and table 5.3-2, table 5.3-3 and table 5.3-4 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µPDCCH, µPDSCH, µUL) resulting with the largest Tproc,1, where the µPDCCH corresponds to the subcarrier spacing of the PDCCH scheduling the PDSCH, the µPDSCH corresponds to the subcarrier spacing of the scheduled PDSCH, and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is to be transmitted, and κ is defined in clause 4.1 of [4, TS 38.211]. - For operation with shared spectrum channel access, is calculated according to [4, TS 38.211], otherwise =0. - If the PDSCH DM-RS position l1 for the additional DM-RS in Table 7.4.1.1.2-3 in clause 7.4.1.1.2 of [4, TS 38.211] is l1=12 then N1,0=14 in Table 5.3-1 and Table 5.3-3, otherwise N1,0=13. <unchanged text omitted> Table 5.3-1: PDSCH processing time for PDSCH processing capability 1 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB or if the higher layer parameter is not configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. Table 5.3-2: PDSCH processing time for PDSCH processing capability 2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB. 0; 3. 1; 4.5. 2; 9 for frequency range 1. Table 5.3-3: PDSCH processing time for PDSCH processing capability 1 for DCI format 1_2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 or if the higher layer parameter is not configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. Table 5.3-4: PDSCH processing time for PDSCH processing capability 2 for DCI format 1_2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2. 0; 3. 1; 4.5. 2; 9 for frequency range 1. <unchanged text omitted> ------------------------------------ End of TP 38.214 V16.5.0 section 9.1.2.1---------------------------------

For Clarification on new SLIV reference for Type-1 HARQ-ACK codebook:

Proposal 2: The following TP for 38.213 V16.5.0 section 9.1.2.1 is captured in specification to support new SLIV reference for Type-1 HARQ-ACK codebook.

------------------------------------ Start of TP 38.213 V16.5.0 section 9.1.2.1--------------------------------- <unchanged text omitted> b) on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c a) if the UE is provided referenceOfSLIVDCI-1-2, for each row index with slot offset K0=0 and PDSCH mapping Type B in a set of row indexes of a table for DCI format 1_2 [6, TS 38.214], for eachany PDCCH monitoring occasion in any slotin a set of PDCCH monitoring occasions with different starting symbols within a slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0, if S+S0+L≤14 for normal cyclic prefix and S+S0+L≤12 for extended cyclic prefix, add a new row index in the set of row indexes of the table by replacing the starting symbol S of the row index by S+S0 <unchanged text omitted> ------------------------------------ End of TP 38.213 V16.5.0 section 9.1.2.1---------------------------------

- References

- 3GPP TS 38.214 v16.5.0, NR: Physical layer procedures for data

- 3GPP TS 38.213 v16.5.0, Physical layer procedures for control

- R1-2104132, Summary of email discussion [104b-e-NR-L1enh-URLLC-01] on remaining issues on PDCCH enhancements,	Moderator (Huawei), RAN1#104b-e