3GPP TSG-RAN WG1 Meeting #105-e 	R1-2106092

# e-Meeting, 19th – 27th May 2021

Title:	[Draft] LS on RF switching time for RedCap UE

Response to:

Release:	Rel-17

Work Item:	Support of reduced capability NR devices (NR_redcap)

Source:	Ericsson [to be RAN1]

To:	RAN4

Cc:

Contact person:	Johan Bergman, johan (dot) bergman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN1 has discussed the RedCap WI objective on “Reduced maximum UE bandwidth”. It is RAN1’s understanding that the existing Rel-15/16 BWP switching framework and related requirements can be reused for RedCap UEs, e.g. that the UE supports two BWPs and the center frequency changes among the two BWPs. RAN1 would like RAN4 to confirm whether it is feasible to maintain the same BWP switching delays for RedCap UEs as currently specified for non-RedCap UEs.

Furthermore, RAN1 would like to ask RAN4 whether the switching delay for FR1 and FR2 could be reduced under the following assumptions (either as a mandatory or an optional UE capability):

- The RF switching takes place between two frequency locations with different centre frequencies.

  - Including cases such that the UL/DL center frequencies are different in a TDD scenario

  - Including cases such that the UE may assume the locations are selected from fewer number of candidates but not any raster currently required

- The maximum UE RF bandwidth is 20 MHz for FR1 and 100 MHz for FR2.

  - The frequency change is up to 80 MHz for FR1 and up to 300 MHz for FR2.

  - Are there any switching ranges that could be faster compared to some other switching ranges? If any, please state the frequency ranges for both FR1 and FR2.

- The RF bandwidth, SCS, QCL, and RRC configuration for the corresponding BWP can be the same before and after the RF switching, i.e. it is only the centre frequency that changes.

- The RF switching may take place during initial access or after initial access.

- The RF switching is either triggered by DCI or preconfigured and not triggered by DCI.

Other assumptions/cases can be fed back based on RAN4 discussion.

# 2	Actions

To RAN4:

ACTION: 	RAN1 respectfully asks RAN4 to provide feedback on the question above on RF switching time.

# 3	Dates of next TSG-RAN WG1 meetings

TSG-RAN WG1 Meeting #106-e 	16th – 27th August 2021		Electronic Meeting

TSG-RAN WG1 Meeting #106bis-e 	10th – 19th October 2021	Electronic Meeting